ESWIN Equipment

PLC Communication Specification

| DOCUMENT ID: | NUMBER OF PAGES: 254 |
| --- | --- |
| REVISION NO: v1.1.0 | CREATED ON: 2025-01-01 |
| FILENAME: Equipment Communication Specification(CC-LINK) | LAST REVISED: |
| STATUS:Release |  |
| PREPARED BY: Signature Date | REVIEWED BY: Signature Date |
| REVIEWED BY: Signature Date | REVIEWED BY: Signature Date |
| APPROVED BY Signature Date | APPROVED AND ACCEPTED BY Signature Date |

Revision History

| Item | Version | Page | Change Place | Writer | Date |
| --- | --- | --- | --- | --- | --- |
| 1 | v0.1 | All | Initial |  | 2025.01.01 |

※Some details are subject to cha

This document contains confidential information provided by ESWIN. No part of this document may be copied, reproduced, disclosed, or transferred by any means without prior written consent of ESWIN.

ESWIN reserves the right to make updates to the information in this document without prior notice or approval from others. Please consult the author of this document to ensure that you have the latest revision.

[ESWIN]

- Caution –

- This specification is distributed in pursuant to the business procedure in the ESWIN.

- If the publishing company takes losses due to illegal distribution of this specification outside the company by a party, it shall take all the civil and criminal responsibilities in pursuant to acts on preventing unfair competition, protecting business secrets, as well as punishment in pursuant to regulations of the company and compensation for losses.

- ESWIN have publishing rights and copyrights of this specification and it shall not be copied, amended or distributed without prior consent.

Table of Contents

REVISION HISTORY	2

TABLE OF CONTENTS	7

- SCOPE &BASIC DESCRIPTION	15

  - SCOPE	15

  - Basic Rule	15

  - Operation Specification	15

  - Equipment Operation Screen Specification	16

  - Inline Process	16

  - Equipment Type	16

  - Line Structure	17

- SYSTEM ARCHITECTURE	18

  - CC-Link Communication	18

    - CC-Link Structure	18

    - Data Communication	18

    - MASTER PLC	18

    - LOCAL PLC (Provide by Equipment Supplier)	19

    - Requirement of Cable Feed-in Hole & Path	19

    - Equipment CC-Link IE Network Concept	20

    - PLC Network Number	20

- DATA COMMUNICATION	21

  - Link Relay, Link Register Relation	21

  - EQP Event Report Timing chart	22

    - Event Report Timing Chart (Local to Master)	22

    - Request Timing Chart (Local to Master)	23

    - Consecutive Event Report Timing Chart (Local to Master)	24

    - Event Report Timing Chart (Local to Local)	25

  - EAS Command Relay Time Chart	26

    - EAS Command Timing Chart (Master to Local)	26

  - Timeout Error	26

    - T1 Timeout (Master ↔ Local)	26

    - T2 Timeout_Case1 (Master ↔ Local)	27

    - T2 Timeout_Case2 (Master ↔ Local)	27

    - Normal Timer Reset (Master ↔ Local)	28

  - Equipment Operations relating to the Timeout	28

    - Warning Alarm Case	28

    - Error Alarm Case	29

  - Interface Signals and Data	29

    - Data Expression	29

- TERMS DEFINITION	32

  - COMMON	32

    - Job	32

    - Recipe ID	32

    - Master Recipe ID (PPID)	32

  - Equipment	32

    - Inline Mode	32

    - CIM Mode	32

    - Equipment Status	32

    - Unit/Sub-Unit Status	33

    - Processing Time	33

    - Upstream Path No	33

    - Downstream Path No	33

    - VCR No	33

  - PORT	33

    - Port Type	33

    - Port Mode	33

    - Port Transfer Mode (STK / AGV/ MGV)	33

    - Port Enable Mode (Port Down)	33

    - Port Status	34

    - Cassette ID	34

  - JOB	34

    - Substrate Type	34

    - Glass ID	34

    - Panel ID	34

    - Cassette Sequence No (Lot index)	35

    - Job Sequence No (slot number)	35

    - Glass Code	36

    - Panel Position	37

    - Job Type	37

    - Dummy Type	38

    - Job Judge	38

    - Job Grade	39

    - Process Flag	40

    - Inspection Judge data	40

    - Skip Flag	40

    - Inspection Flag	41

    - Q-Time	41

  - JOB DATA	42

    - Job Data Items	42

- COMMON FUNCTION	51

  - Equipment Main Mode Management	51

    - Equipment Alive	51

    - CIM Mode	51

    - CIM Mode Change By EAS	51

    - Inline Mode	51

    - Local Alarm State	52

    - PLC MAP	52

    - Communication Scenarios	53

  - Equipment Mode Change	53

    - Equipment Mode Change Command	53

    - Equipment Mode Change Report	54

  - Equipment Management	55

    - Equipment Status	55

    - Unit/Sub-Unit Status	57

    - Equipment Status Reason Code	57

    - No Processing Time	58

    - PLC MAP	59

  - Alarm Management	61

    - Alarm State	62

    - Alarm Unit	62

    - Alarm ID	62

    - Alarm Code	62

    - Alarm Level	63

    - Alarm Text	63

    - Error Recovery and Remove Job	63

    - PLC MAP	64

  - Job Management	65

    - Job Data Transfer	65

    - Job Count	65

    - Job Event	66

    - Job Event Report Timing Condition	67

    - Received Job Event	67

    - Sent Out Job Event	67

    - Stored Job Event	67

    - Fetched Out Job Event	68

    - Removed Job Event	68

    - Removed Job Recovery Function	69

    - Process Start Report	69

    - Process End Report	69

    - PLC MAP	69

  - Process Data Management	73

    - Event Report Timing	73

    - DV and SV Naming Rule(recommend)	73

    - PLC MAP: By Glass(DV)	73

    - PLC MAP: By Glass(DV) – CF Only	74

    - PLC MAP: By Time (SV)	76

    - Tact-Time DV Data	76

    - FAC Data Report	77

    - PLC MAP: By Time (SV)	77

  - Job Information Management	79

    - Job Data Request	80

    - Job Data Change	80

    - PLC Map	80

  - Recipe Management	81

    - Auto Recipe Change Function	81

    - Current Recipe ID	81

    - Equipment Recipe Validation	82

    - Master Recipe List (auto change only)	83

    - Master Recipe List Report	84

    - Current Recipe Change Report	85

    - Recipe Key Parameter Validation	85

    - Recipe Key Parameter or Master Recipe List at abnormal	88

    - Local Recipe Request (Array Only)	88

    - PLC MAP (Cell Only)	88

    - PLC MAP (Array Only)	91

  - Recipe Management (CF Shop Only)	95

    - Recipe Table	95

    - Glass Flow by Recipe	96

    - Local Recipe List	97

    - Local Recipe Request	97

    - Recipe Parameter Data Request (Recipe Parameter Data Report)	97

    - Recipe Parameter Data Change Report	97

    - Processing Master Recipe ID / Local Recipe Change Report	97

    - Recipe State Change Event Report	98

    - PLC Map	98

  - CIM Message Management	101

    - CIM Message	102

    - Touch Panel Number	102

    - CIM Message ID	102

    - CIM Message Set Command	102

    - CIM Message Clear Command	103

    - CIM Message Confirm Event Report	103

    - CIM Message History Management	104

    - PLC MAP	104

    - Signal Tower Management	105

  - Date Time Set Command	105

    - PLC MAP	106

- PORT MANAGEMENT	107

  - Port Operation Mode Management	107

    - CST Operation Mode	107

    - Indexer Operation Mode(Line Operation Mode)	107

    - PLC Map	108

  - Port Type & Mode Management	111

    - Port Type	111

    - Port Mode	112

    - Port Transfer Mode	114

    - Port Enable Mode	114

    - Port Type Auto Change Mode	114

    - Port Cassette Type	115

    - PLC MAP	115

  - Port Q-Time Check (except End Cell)	120

    - PLC MAP	121

  - Port and Cassette Control Management	122

    - Port State	122

    - Cassette Status (Feeder Type & Independent Type Indexer Only)	124

    - Cassette Control Command	125

    - Return Code	126

    - Cassette Map Data	126

    - Cassette Mapping State	126

    - PLC MAP	127

    - Communication Scenario	133

  - Port WIP Management	141

    - PLC MAP	141

  - Port Cassette Size (only End Cell)	141

    - PLC MAP	142

- ROBOT MANAGEMENT	143

  - Robot Control Command	143

    - Command format	143

    - Data Description	144

    - Robot CMD Ack	145

    - PLC MAP	145

    - Robot Command Buffer	147

  - Command Fetch Report	148

    - PLC MAP	148

  - Command Result Report	149

    - PLC MAP	149

  - Robot Arm Event	150

    - PLC MAP	150

  - Robot Monitoring Block	151

    - PLC MAP	151

  - Dispatching Mode	151

    - Auto Dispatching	151

    - Robot Control Mode (Indexer Only)	152

    - PLC MAP	152

  - Robot Position	153

- MONITORING BLOCK	154

  - C/V MONITORING	154

  - Buffer Monitoring	154

  - Indexer Monitoring	157

  - Position State Monitoring	158

    - PLC MAP	158

- SIGNAL TOWER LAMP & BUZZER(PROVISIONAL)	160

  - Signal Light Tower Specification for Process Equipment	160

  - Signal Light Tower Specification for Indexer or Feeder	160

  - Buzzer Specification	161

  - The Configuration of the Signal Tower	161

10SPECIAL FUNCTION	162

  - VCR Management	162

    - PLC MAP	162

    - Communication Scenario	164

  - Panel Data Request	164

    - PLC Map	164

  - Panel Data Report	165

    - PLC Map	165

  - Material Management	166

    - Material ID	166

    - Material Status	166

    - PLC Map: Material	167

  - LOADING STOP (EQ TO EQ)	169

    - PLC Map	169

  - TRANSFER STOP (EQ TO EQ)	169

    - PLC Map	169

  - Cancel Abort Request (EQ to EAS)	170

    - PLC MAP	170

  - Lot End Glass Report (EQ to EQ)	171

    - PLC Map	171

  - Cassette Move In/Out Report	172

    - PLC MAP	172

  - Oven Initial Heating	173

    - PLC MAP	174

  - Dummy Glass Request	174

    - PLC MAP	174

  - Pre-action Signal (EQ to EQ)	175

    - Pre-action Signal Type	175

    - Pre-action Signal On/Off Condition	176

    - PLC Map	176

  - Oven Over Bake	177

    - Oven Over Bake Report	177

    - PLC MAP	177

  - DFS	177

    - Cassette ID Request	177

    - PLC MAP	177

  - Panel Repair Result Report	178

    - PLC Map	179

11SHOP SPECIAL FUNCTION	180

  - Array Special Function	180

    - Remain Glass inform (EAS to EQ) – CVD Only	180

    - Tray Data Management (EAS to EQ) - Sputter Only	180

    - Glass Start Signal (EQ to EQ) – Sputter Only	181

    - Crack Signal (EQ to EQ) - EGIS Only	181

    - Oven Auto Recipe Change	182

    - AOI Enable Signal ( EQ to EQ) : Photo Inline Only	183

    - EE/TT Removed Job Signal (EQ to EQ ) : Photo Inline Only	183

    - Mix Run Recipe Data Request/Reply	184

    - Mix Run Unit Recipe Change Command/Reply	184

    - Parts Management – Dry Etch	185

    - Parts Management – Wet Etch/Strip, HDC, Pre Cleaner, Initial Cleaner	188

    - Defect Code Report	189

  - Cell Special Function	190

    - Common	190

    - ODF	194

    - PI	196

    - Rubbing	198

    - Cutting	203

    - Panel Sorter Load	208

    - Cell Test Inline	209

    - CST Model Change	211

    - PS Height Value Management	213

  - CF Special Function	214

    - Job Route Decision	214

    - Product Stop Signal	216

    - PPID PreDownload	217

    - Block Stop	218

    - COAT PR	218

    - Block Stop Signal Off Command	219

    - Equipment Reason Code Event	219

    - EXPO Mask	222

    - Mask Data Management	224

    - Mask Data PLC MAP	228

    - Mask Clean Priority Management.	232

    - Mask Clean Priority PLC Map	232

    - CF Oven Reverse Flow	234

    - CF AOI Mask Info Request	235

- Scope &Basic Description

This is a PLC Communication Specification for Equipment Automation, outlining detailed rules required for communicating and operation between Equipment Automation Software and Equipment.

For this specification, CUSTOMER reserves the right of final decision in any disputes. During the testing term, weather it is specification discrepancy or software bug, CUSTOMER can change the specification without any charge by vendor.

Equipment should provide all equipment function without big modification when start address changed. There would be separate specification regarding signals for hand-shaking between two different equipment.

There will be separate test specification based on the latest version of this specification. Each and every equipment supplier is obligated to comply with the latest version of test specification before obtaining Final Acceptance from Customer.

  - Scope

This specification specifies basic requirement for PLC and defines the communication specification between the equipment and EAS in Customer’s Company.

The contents of this have been determined through continuous discussion with Customer’s Company, and equipment venders are required to comply with this specification.

  - Basic Rule

- Equipment maker should provide the associated modules for CIM PLC and Network module.

- CIM cable clamp mechanism should be provided at both feed-in position and PLC connection position for preventing cable damage from unexpected physical force.

- Equipment maker should provide the layout about CIM PLC, lead-in opening, and cable path, including relative position and approximate length of the path after design.

- All interfaces between EAS and equipment must be fully tested by equipment maker and EAS engineer. These tests must be completed first prior to the equipment being shipped to the FAB.

- Equipment maker should supply final specification documents and the manual for operation regarding CIM.

- Equipment maker should provide the Ethernet module for interface with EAS.(Optional)

  - Operation Specification

- EAS can control each cassette and EAS can control each command (Process start, Process abort, Process pause, Process resume, etc.) by each port.

- If equipment has multiple chambers or similar units, partial down (cycle stop, etc.) should be controllable by engineer.

- Operation screen of equipment regarding CIM is required to follow the Customer’s Company standard.

- Basically, EAS can control and manage for the Recipe of each Glass.

- Glass movement trace is needed. Equipment should report the current situation or location of glass.

- EAS message (CIM Message) can be sent to equipment touch panel.

- Setting function for BCR/VCR/DCR is needed to enable or disable by operator.

- BCR usage field (function key) must be prepared according to the port number for each port if BCR exists at port. But VCR usage field also should prepared according to the VCR number.

- Help function (in GUI) for alarm or error is needed to recover the problem more easily.

- Raw material data should be reported to EAS automatically or manually by operator using touch panel on the Equipment.

  - Equipment Operation Screen Specification

- The equipment should support GUI (Graphic User Interface) in the basic operational display.

- Bar-Code ID (Cassette ID) & Veri-Code ID (Glass ID or Panel ID) input fields are necessary in the screen.

- The equipment should support monitoring screen for a job (glass/Panel). The screen should include Job Data.

- The equipment should support monitoring screen for the interface status, which must include interface with EAS, with upstream equipment, and with downstream equipment.

  - Inline Process

A cassette is loaded to a loader of a process line by AMHS System (STK/MGV/AGV/RGV), and glass/Panel is fetched out from the cassette and sent to downstream equipment to continue the process. Finally, un-loader receives each glass/Panel from upstream equipment, and stores it into cassette, and then the cassette is unloaded by AMHS System to continue the next process. This consecutive sequence of processes is defined as inline process, and the processing of each glass/Panel in the inline process is defined as job. Job transfer between equipment is controlled by interconnecting signals, and equipment sends or receives Job Data such as Recipe Number via MELSECNET/G.

  - Equipment Type

The equipment types are divided into 4 types according to function.

- Loader (LD): In charge of normal input of glasses into equipment. It is generally composed of ports and robot.

- Un-loader (ULD): In charge of normal output of glasses from equipment. It is generally composed of ports and robot.

- Process Equipment (PM): Denotes equipment to process glasses, etc., rather than handling the job transferring. It also includes inspection or measurement equipment. It is generally composed of process unit or inspection unit.

- Buffer Equipment (BM): In charge of interleaving and buffering Job between equipment’s. It is generally composed of buffer port and robots.

  - Line Structure

Line is equipment formed by the interconnection of one or more units of equipment. Line is composed of the following.

- Sub Unit: The lowest component.

- Unit : Types in this category are process unit, chamber, etc.

- Block : logical group of dispatching at operation of process flow in EAS

- Equipment: Equipment is composed of one or more units. Equipment can be controlled independently. General types of equipment are loader, un-loader, process Equipment and buffer Equipment.

- Line: Line is composed of more than one Indexer (Cassette loading/Unloading) and more than one process equipment. This Line is regarded as one equipment to Host CIM program, and its communication with Host is regarded as such.

Figure 1-1 Line structure

- System Architecture

  - CC-Link Communication

    - CC-Link Structure

The basis of communication between EAS and Equipment is MITSUBISHI PLC network.

Figure 2-1 CC-Link IE Communication Structure

    - Data Communication

Equipment connects with upstream and downstream Equipment through link signal via MELSECNET/G Link Area. Link signal is used only to control job transferring. And Job Transfer Data is transferred via MELSECNET/G. Equipment connects with EAS via MELSECNET/G network of Mitsubishi to send/receive CIM data.

    - MASTER PLC

- PLC Module Base :

  - CPU Unit: MITSUBISHI Q03UDH, R08CPU or above CPU

  - Base Unit: MELSEC Q35B , R38B

  - Power Unit: MELSEC Q62P , R61P

  - Ether Net Unit : QJ71E71-100 (100BASE-TX/10BASE-T)

  - MELSECNET/G Unit: MELSEC QJ71BR11 unit version D

- PC Board Base:

  - MELSECNET PCI Card: Q80BD-J71GP21(s)-SX (PCI) or Q81BD-J71GP21(s)-SX (PCI Express) or NZ81GP21-SX

- Common

  - Optical Cable: MELSECNET/G convertible type (1 set, more than 50 meters)

  - Optical Connector: MELSECNET/G convertible type (2 sets) (F connector 1 set) (unused)

    - LOCAL PLC (Provide by Equipment Supplier)

- CPU Unit : CC-Link IE Controller Network convertible

- MELSECNET/G Unit : QJ71GP21(s)-SX, RJ71GP21-SX or compatible PLC card for PC base

* PC Card reference model information : “MELSECNET PCI Card: Q80BD-J71GP21(s)-SX (PCI) or Q81BD- J71GP21(s)-SX (PCI Express)”

- Optical Connector : CC-Link IE Controller Network convertible type (Duplex LC connector 1set)

- Optical Cable prepared by each EQP. Vendor

- Upstream Equipment vendor prepare the optical cable for communication to downstream Equipment.

- If Equipment has port or buffer port that can be loaded or unloaded a cassette, the Equipment maker should provide File Register (R area). EAS can read or write the area for cassette map data.

    - Requirement of Cable Feed-in Hole & Path

- Equipment should reserve a path for optical cable from the lead-in opening to MELSECNET/G module and the path must be isolated from any electric noise source, like power supply, inverter, high voltage device and so on.

- Equipment vendor must supply layout information about MELSECNET module position in Equipment. End user must supply information about the length of optical cable between upstream Equipment and downstream Equipment.

- Around the connector of MELSECNET/G module, at least 90 mm must be reserved for connecting optical cable. It is to make sure that PLC connection and connector should be easily accessed for maintenance.

- Equipment must prepare a lead-in opening on the Equipment bottom plate.

* Data Link Configuration

| Link Type | MELSEC PLC |  | Remarks |
| --- | --- | --- | --- |
| Unit | QJ71GP21(s)-SX | RJ71GP21-SX | ‘(s)’ option determine by maker |
| BPS | 1Gbps | 1Gbps |  |
| Cable | Optical Cable | Optical Cable |  |
| LX,LY | LX:8192 Points,LY:8192 Points | LX:8192 Points,LY:8192 Points | Use Device /1 Network |
| Link Device (B) | Max, 32768 Points (Bit) | Max, 65536 Points (Bit) |  |
| Link Device (W) | Max, 131072 Points (Word) | Max, 262144 Points (Word) |  |
| Link Device (B)/Local | Max, 16384 Points (Bit) | Max, 65536 Points (Bit) | Use Device /1 Unit (PLC) |
| Link Device (W)/Local | Max, 16384 Points (Word) | Max, 262144 Points (Word) |  |

    - Equipment CC-Link IE Network Concept

Figure 2-2 Communication Concept of Each Local PLC, Unit

    - PLC Network Number

Default Network Number is ‘1’

- Data Communication

Data Area of PLC memory is divided into two areas, namely Local Area and Master Area. The Local Area is used to send data from Local PLC to EAS, while the Master Area is used for data transfer from EAS to Local PLC. Items of each area are categorized into several types according to report or command characteristic, and the each item type is as below.

- Local Area

  - Status: Item that represents the current status of equipment, port, PLC.

  - Event: Item that represents events generated by equipment.

  - Request: Item that equipment requests a permission to do any action to EAS.

  - Command Reply: Item that equipment replies against the command from EAS.

- Master Area

  - Status: Item that represents the status of EAS.

  - Command: Item that can be sent to local PLC.

  - Event Reply: Item that is replied against the event from equipment.

  - Request Reply: Item that is replied against the request from equipment.

  - Link Relay, Link Register Relation

- When event occurs or some data are changed, local PLC must write the data to the specific link register (W Area) and then turn on the related local link relay for firing the event.

- When the data of link relay is changed, EAS should read the related link register area, and then turn on its own link relay for replying, which is corresponding to the local relay.

- When the response from EAS is OK, local PLC turns off its link relay.

- When the link relay of the local PLC is turn off, EAS turns off its link relay corresponding to the link relay of the local PLC.

  - EQP Event Report Timing chart

    - Event Report Timing Chart (Local to Master)

Figure 3-1 Diagram of Timing Chart for Normal Report Data

①  After setting the data into link register (W Area), local PLC turns on the link relay (Event Bit).

② Local PLC loads the timer to check the Timeout. (T1 : 4 seconds) –Local PLC doesn’t report any same event using this link relay during this time.

③ Master PLC(EAS)reads the data, and then turn on the link relay (Event Reply Bit).

④	Local PLC turns off the link relay and continues the next action.

⑤	Master PLC (EAS) turns off link relay (Event Reply), after the timer reset. Master PLC (EAS) scans the link of the local PLC during T2 time. If timeout occurs, Master (EAS) processes error routine (T2 Timeout is 2 second)

- EQP vendor provide T1 timer value setting function. (Default value = 4 : 4 seconds)

- Local PLC preserves Report Data once set in link register until new Report Data is generated.

- When timeout occur , the EQ doesn’t have to stop signal wait then continue the next action

    - Request Timing Chart (Local to Master)

Figure 3-2 Diagram of Timing Chart for Request Report Data

① After setting the data into link register (W Area), local PLC turns on the link relay (Event Bit).

② Local PLC loads the timer to check the Timeout. (T1 : 4 seconds) – Local PLC doesn’t report any same event using this link relay during this time.

③ Master (EAS) reads the data, and then turn on the link relay (Event Reply Bit).

④	Local PLC reads the data of Master (EAS) and turns off the link relay (Event Bit), then continues the next action.

⑤	Master (EAS) turns off link relay (Event Reply), after the timer reset. Master (EAS) scans the link of the local PLC during T2 time. If timeout occurs, Master (EAS) processes error routine (T2 Timeout is 2 second)

- EQ vendor provide T1 timer value setting function. (Default value = 4 : 4 seconds)

- Local PLC preserves Report Data once set in link register until new Report Data is generated.

- When timeout occur , the EQ doesn’t have to stop signal wait then continue the next action

    - Consecutive Event Report Timing Chart (Local to Master)

Figure 3-3 Diagram of Timing Chart for Normal Report Data – Consecutive Data Report

The above Timing Chart specifies data reporting procedure when Local PLC has more than one consecutive Data to report to Master (EAS) using one Event bit, such as Alarm or Job Event.

① If Local PLC has data (say, Data A) to report to Master (EAS), it sets the Data A in Buffer.

② If the Local PLC has no other previous data currently being reported to Master (EAS), moves Data A set in buffer to link register area and reports to Master (EAS).

③ If local PLC has new data (say, Data B) to report to Master (EAS), local PLC sets Data B in buffer. If, at this time, there is previous data (i.e. Data A) currently being reported to Master (EAS), local PLC waits until the Data A reporting process finishes.

④ After Master (EAS) turns off the link relay (Event Reply), the local PLC waits for 0.5 (T3: 0.5 second) second, and then the local PLC moves Data B to Link Word area and reports to Master (EAS).

- EQ vendor provide T3 timer value setting function. (Default value = 0.5 : 0.5 seconds)

- When timeout occur , the EQ doesn’t have to stop signal wait then continue the next action

    - Event Report Timing Chart (Local to Local)

Figure 3-4 Diagram of Timing Chart for Normal Report Data

① After setting the data into link register (W Area), Local-1(send) PLC turns on the link relay (Event Bit).

② Local-1(send) PLC loads the timer to check the Timeout. (T1) – Local-1(send) PLC doesn’t report any same event using this link relay during this time.

③ Local-2(receive) reads the data, and then turn on the link relay (Event Reply Bit), and loads the timer to check the Timeout (T2).

④  Local-1(send) PLC turns off the link relay and continues the next action.

⑤ Local-2(receive) turns off link relay (Event Reply)

⑥ After Local-2(receive) turns off the link relay (Event Reply), the Local-1(send) PLC waits for 0.5 (T3) second before turns on the link relay (Event Bit) again for the next Event Report, if any.

- EQP vendor provide T1 timer value setting function. (Default value = 4 : 4 seconds)

- Local PLC preserves Report Data once set in link register until new Report Data is generated.

- When timeout occur , the EQP turns off the signal, and then continue the next action. EQP should generate timeout alarm to notify, but should not stop normal processing until required to do so

  - EAS Command Relay Time Chart

    - EAS Command Timing Chart (Master to Local)

EAS Notice Area is related to Equipment Command Reply Area

Figure 3-5 Diagram of Timing Chart for Normal Command Data

After setting data into the link register area of the Master (EAS), then mater PLC turns on the link relay (Command).

① Master (EAS) loads timer to check the Timeout. (T1 : 4 seconds)

② Local PLC reads the data of master area. After reading, local PLC turns on the link relay of local PLC (Command Reply).

③ Master (EAS) turns off the link relay (Command) upon Command Reply on. If local PLC does not turn on the reply bit within T1 timeout, Master (EAS) processes error routine.

④ Local PLC turns off the link relay (Command Reply), after the timer reset. Local PLC scans the link relay of Master (EAS) during T2 time. If timeout (T2) occurs, local PLC processes error routine.

- EQ vendor provide T2 timer value setting function. (Default value = 2 : 2 seconds)

- When timeout occur , the EQ doesn’t have to stop signal wait then continue the next action

  - Timeout Error

    - T1 Timeout (Master ↔ Local)

T1 : duration to keep Event/Request/Command signal on without reply signal ,4sec

Figure 3-6 T1 Timeout Chart (Master (EAS) & Local PLC)

① After setting the data into link register, then Local or Master (EAS) turns on the link relay (Event or Command Bit).

② Local or Master (EAS) Loads timer to check Timeout. (T1 : 4 seconds) If the response of the partner does not exist, then Local or Master (EAS) makes Timeout.

③ Turn off the link relay (Event or Command Bit), after the timer reset.

    - T2 Timeout_Case1 (Master ↔ Local)

(Case 1)T2 : duration to wait Event/Request/Command signal off after reply signal on, 2sec

Figure 3-7 T2 Timeout Chart

① After reading the data into Link Word, then turn on the Link relay (Command or Event Reply Bit).

② Load the timer to check the Timeout. (T2: 2 second). If the response of the partner does not exist, then Local or Master (EAS) makes Timeout.

③ Turn off the Link relay (Event or Command Reply Bit), after the timer reset.

    - T2 Timeout_Case2 (Master ↔ Local)

(Case 2)T2 : duration to wait Event Reply signal off after Event or Command signal Off, 2sec

Figure 3-8 T2 Timeout Chart

① After reading the data into Link Word, then turn on the Link relay (Event Reply Bit).

② Turn off the Link relay (Event or Command Bit), after received Event or Command Reply.

③ Load the timer to check the Timeout. (T2: 2 second). If Reply BIT doesn't turn off,, then Local or Master (EAS) makes Timeout.

    - Normal Timer Reset (Master ↔ Local)

① When reply bit on , signal bit off then T1 timer reset

② When reply bit off, signal bit off then T2 timer reset

  - Equipment Operations relating to the Timeout

    - Warning Alarm Case

      - T1 Timeout

- When Equipment report event and EAS don’t set Reply bit ON, After set time over, Equipment set

Warning Alarm and Equipment continue the processing

      - T2 Timeout

- When EAS send the Command or Request and Equipment set the Reply bit ON. Since then EAS don’t set Command or Request bit OFF, After set time over, Equipment set Warning Alarm and Equipment continue the processing

    - Error Alarm Case

      - T1 Timeout

- When Equipment request and EAS don’t set Reply bit ON. After set time over(T1 Timeout),

Equipment set Error Alarm and Equipment continue the processing (exception : special operation data, job data, the related processing data)

      - Consecutive Event

- When Equipment set Event, Equipment check the reply bit of EAS, If reply bit set ON, Equipment set the Error Alarm and stop processing

  - Interface Signals and Data

Interface between devices consist of:

- Transfer Interlock and Data

: This is mutual interlock signal required for transfer processing such as glass-in and glass-out between upstream and downstream devices. The Interlock Type depends on the configuration of the device and each has different Interlock signal system and its content.

- Signal and Data for Reporting Device Status

: These include device modes, anomaly and Glass hold state

- Glass Processing Signal and Data

: Scrap report and data report for Glass process.

- Signal and Data for Reporting Glass-in or Glass-out

: This is to report data on Glass carried upon glass-in from upstream or transmitted to downstream.

- Data Request Signal

: This requests data necessary for devices like Equipment Recipe ID request, Glass Data request to EAS.

- Special Signal and Data for other devices

: This allows exchange with other devices of data required for operation in such a case as special mode change.

    - Data Expression

      - ASC Type

- Use 8 bits ASC Code.

- Fill the empty value with Blank (H20) value at ASC type.

- Order of inter-character in word: Low byte → High byte order

- Order of inter-word: Low word → High word order

- Character and numeric data form: Standard Character form

| ‘A’ | ‘B’ | ‘C’ | ‘D’ | ‘1’ | ‘2’ | ‘3’ | ‘4’ |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H42 | H41 | H44 | H43 | H32 | H31 | H34 | H33 | H20 | H20 | H20 | H20 |
| Word 1 (Low) |  | Word 2 |  | Word 3 |  | Word 4 |  | Word 5 |  | Word 6 (High) |  |

Examples)

- Character data: (ex) ‘R’ → H2052

- Character data: (ex) ‘I1’ → H3149 (H4931 is an error)

- Character data: (ex) ‘123’ → H3231 2033

- Character data: (ex) ‘2AA00101	‘ → H4132 3041 3130 3130 2020 2020

      - Numeric Type

| Format | Abbreviati on | Size | Range | Sample Value | Expression in PLC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | order | BIN | DEC | HEX |
| Unsigned Integer | INT | 1W | 0 ~ 65,535 | 324 | 0000 0001 0100 0100 |  | 324 | 0144 |
| Signed Integer | SINT | 1W | -32,768~ 32,767 | - 90 | 1111 1111 1010 0110 |  | 65,446 | FFA6 |
|  |  |  |  | 90 | 0000 0000 01011010 |  | 90 | 005A |
| Unsigned Long | LONG | 2W | 0 ~ 4,294,967,295 | 1234567 | 1st | 1101 0110 1000 0111 | 54,919 | D687 |
|  |  |  |  |  | 2nd | 0000 0000 0001 0010 | 18 | 0012 |
| Signed Long | SLONG | 2W | -2,147,483,648 ~ 2,147,483,647 | -1234567 | 1st | 0010 1001 0111 1001 | 10,617 | 2979 |
|  |  |  |  |  | 2nd | 1111 1111 1110 1101 | 65,517 | FFED |
| Float | F | 2W | sign : 1st bit exponent : next 8bit, fraction : next 23bit | 0.75 | 1st | 0011 1111 0100 0000 |  | 3F40 |
|  |  |  |  |  | 2nd | 0000 0000 0000 0000 |  | 0000 |

- In case of signed format, negative value should be described with two’s complement.

- Float type complies with IEEE 754

      - BCD Type

- Use 4 bits BCD Code

- Character data is not used

- The below is BCD code Sample

| 1Word | F | E | D | C | B | A | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Binary | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCD | 2 |  |  |  | 8 |  |  |  | 3 |  |  |  | 0 |  |  |  |

- Terms Definition

  - Common

    - Job

The Job means specific processing works with a material such as glass, Panel. Glass is physical a unit and job logical a unit. And also, it is managed job data

There are glass job data, Panel job data, dispatching job data that distinguish for useful type

    - Recipe ID

Each job is to be processed in its desired process condition, called recipe. Equipment should give IDs to represent and manage these recipes, and the ID is called Recipe ID.

- Custom Specific:

Array : Recipe ID consists of ASC 20 characters. 0~9, A~Z, a~z (except : I, O, i, o). CF,CELL : Recipe ID consists of INT.

    - Master Recipe ID (PPID)

Master Recipe represent the group of each local Recipe ID. Master Recipe ID and Recipe ID relation rule refer to Equipment Operation Scenario.

(Ex. 1~999 or ABCD123)

- Custom Specific:

Array, Ecell : will use ASC type (ASC 20 characters. 0~9, A~Z, a~z (except : I, O, i, o)).

CF, FCELL : will use Integer type (1~999)

  - Equipment

    - Inline Mode

The inline mode is related only with job transferring and is necessary only between upstream equipment and downstream equipment.

    - CIM Mode

CIM Mode is to specify the communication relation between equipment and EAS. In general, a local PLC should ignore the command of the EAS at CIM OFF Mode. But, Equipment should be allow for the Command of the CIM Mode On Change.

    - Equipment Status

The Equipment Status shows the current operational state of the equipment. It consists of Setup, Stop, Pause, Idle and Run.

    - Unit/Sub-Unit Status

The Unit Status shows the current operation state of the unit. Types of Unit Status and the definitions are same as those of Equipment Status.

    - Processing Time

It is actual processing time of a job in equipment or unit. It does not include time spent for transferring between upstream equipment and downstream equipment. Equipment include this item in process data report.

    - Upstream Path No

There would be multiple upstream equipment’s for any specific equipment. Equipment should manage its upstream equipment’s’ identity by assigning unique number to all upstream equipment’s, and the number is called Upstream Path No. When receiving a Job from upstream equipment, the equipment can know from which upstream equipment the Job is received by checking the Upstream Path No. If there is only one upstream equipment, Upstream Path No is always set ‘1’.

    - Downstream Path No

There would be multiple downstream equipment’s for any specific equipment. Equipment manages its downstream equipment’s identity by assigning unique number to all downstream equipment’s, and the number is called Downstream Path No. When sending a Job to downstream equipment, the equipment should indicate to which downstream equipment the Job is sending to by checking the Downstream Path No.

If there is only one downstream equipment, Downstream Path No is always set ‘1’.

    - VCR No

VCR No is used to distinguish the unique VCR in the equipment. If there is only one VCR, VCR No is always ‘1’.

  - Port

    - Port Type

It is to classify a Port based on the expected operation of the Cassette currently loaded on the Port: Cassette loaded on the port is whether to input Job into Inline, or to output Job from Inline after Job that is finished its process in a line, or to buffer a Job in a line temporary. There are 4 types: Loading Port, Unloading Port, Buffer Port(There are 3 types for Buffer Port; Buffer Type, Loader in Buffer Type, Un-loader in Buffer Type), and Both Port.

    - Port Mode

It is to classify a port based on Judge of job that has been included or is going to be sorted in a cassette on the port. Port Mode is defined by a combination of Job Type, Production Type and Judge/Grade values.

    - Port Transfer Mode (STK / AGV/ MGV)

It indicates the type of AMHS Equipment that connects with the port in transferring cassette to/from the port.

    - Port Enable Mode (Port Down)

It indicates that the port can be used or not. If this mode of a port is changed to ‘Disable Mode’, equipment does not process the cassette on the port. Operator can change its value in equipment screen.

    - Port Status

It is to indicate whether a Port is available for cassette loading, or a cassette loading is finished to a Port, or a Port is available for cassette unloading.

    - Cassette ID

Specific ID is assigned to individual cassettes, which is called Cassette ID. It can be read by Bar-Code Reader (BCR) or RFID.

  - Job

    - Substrate Type

The Substrate Type of job. It consists of 2 types: Glass, Panel.

    - Glass ID

A specific ID is assigned to a full size glass, which is called Glass ID. Veri Code is used to read the Glass ID of a job, and it consists of physical symbols.

    - Panel ID

After cut operation, the glass becomes a smaller piece, called Panel (It is also called cell ). Its ID is called Panel ID.

Example)

Glass ID = TESTGLS00, ID of a Panel = AA  Panel ID = TESTGLS00AA Glass ID = TESTGLS00, ID of a Panel= BB  Panel ID = TESTGLS00BB

Figure 4-1 Panel ID

    - Cassette Sequence No (Lot index)

The Cassette Sequence Number is a fictitious sequence number to represent a cassette. It should be increased when the Port Status of the port is changed to ‘In Use’. It is used to uniquely distinguish the cassette for the jobs. Regardless of CIM ON or CIM OFF, the Cassette Sequence Number is produced automatically by loader or un-loader. Buffer also produces Cassette Sequence Number as loader or un-loader. Equipment vendor should follow cassette sequence numbering rule specified in the equipment operation scenario.

| Port No | CST SeqNo.Range | Port No | CST SeqNo.Range |
| --- | --- | --- | --- |
| 1 | 1001 ~ 1999 | 7 | 7001 ~ 7999 |
| 2 | 2001 ~ 2999 | 8 | 8001 ~ 8999 |
| 3 | 3001 ~ 3999 | 9 | 9001 ~ 9999 |
| 4 | 4001 ~ 4999 | 10 | 10001 ~ 10999 |
| 5 | 5001 ~ 4999 | 11 | 11001 ~ 11999 |
| 6 | 6001 ~ 6999 | 12 | 12001 ~ 12999 |

    - Job Sequence No (slot number)

The Job Sequence Number is a fictitious sequence number. No matter it is in CIM ON or CIM OFF Mode, when a job is fetched out from a cassette on loader, Job Sequence Number should be produced by the loader/Un-loader/Buffer/Virtual Port automatically. Slot sequence of cassette is ordered from bottom to top, from Slot #1 to Slot #210. As same as slot order, the Job Sequence Number is numbered from bottom to top.

27 or 56

Top

Slot #27 or 56

…	…	…  …

1	Bottom

Slot #1

Job Sequence Number

Slot Number

Cassette

| 30 | 60 | Slot #30 | Slot #60 |
| --- | --- | --- | --- |
| ... | ... | ... | ... |
| 2 | 32 | Slot #02 | 1 2 Slot #32 |
| 1 | 31 | Slot #01 | Slot #31 |
|  |  | 1 | 2 |

| 30 | 60 | 90 | Slot #30 | Slot #60 | Slot #90 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ... | ... | ... | ... | ... | ... |  |  |  |  |
| 2 | 32 | 62 | Slot #02 | Slot #32 | Slot #62 |  | 1 | 2 | 3 |
| 1 | 31 | 61 | Slot #01 | Slot #31 | Slot #61 |  |  |  |  |
|  |  |  | 1 | 2 | 3 |  |  |  |  |

Figure 4-1 Relation between Job Sequence Number and Slot Number

This is Rule of Job Sequence No for Cell Cut only that’s in below

    - Glass Code

Glass Code is a combination of Cassette Sequence No. and Job Sequence No., and is used to identify a Job (Glass, Panel) instead of Glass ID or Panel ID. So, either Glass Code or Glass ID/Panel ID can be used to indicate a Glass/Panel.

| Word | Word 01 (Glass Code #1) | Word 02(Glass Code #2) |
| --- | --- | --- |
| Glass Code | CST Sequence Number (1~65535) | Job Sequence Number (1~65535) |

For example) Data Expression

Figure 4-2 Data Expression of Glass Code

    - Panel Position

It is mainly to indicate the specific position of a panel in a glass before cutting. After 1st cutting or 2nd cutting, one can know by this value the original position of each Panel in a glass from which the Panel has been cut.

Figure 4-2 Panel position

Figure 4-3 Multi Panel position

    - Job Type

Job Type is used to distinguish the kind of cassette or glass. It consists of Normal, Dummy and so on. Normally, glass is sorted to the cassette according to the Job Type of it.

Job type is distinguish by dispatching job type or Glass job type or Panel job type

    - Dummy Type

There are various types of Dummy Glasses according to its purpose of exist. This is to indicates the type of Dummy for a specific Dummy Glass.

When ‘Job Type’ is ‘Dummy’, this value should be set by Host.

| Dummy Type | Value | Format | EAS Map Down |
| --- | --- | --- | --- |
| General Dummy | 1 | INT | General |
| QC Dummy | 2 | INT |  |
| Through Dummy | 3 | INT |  |
| Mask Dummy | 4 | INT |  |
| Screen Dummy | 5 | INT |  |
| ODF Seal Dummy | 6 | INT | SD Dummy(ODF) |
| VAS Dummy | 7 | INT |  |
| Laser Seal Dummy | 8 | INT |  |
| ITO Dummy | 9 | INT |  |
| Photo Dummy | 10 | INT |  |
| TPCD Dummy | 11 | INT |  |
| Monitor Dummy | 12 | INT |  |
| Thickness Dummy | 13 | INT | Thickness |
| Rubbing ITO Dummy | 14 | INT | ITO(RubbingMonitor) |
| Rubbing Aging Dummy | 15 | INT | Rubbing Rubbing(Bare) Aging(RubbingBare) |
| MM Dummy | 16 | INT |  |
| Bare Dummy | 17 | INT | Particle Count(Bare) |
| ODF UVMask Dummy | 18 | INT | UV Mask(ODF) |
| ODF Bare Dummy | 19 | INT |  |
| ODF LC Dummy | 20 | INT |  |
| Particle Bare Dummy | 21 | INT |  |
| Rubbing Bare Dummy | 22 | INT |  |
| General ITO Dummy | 23 | INT |  |
| Thickness ITO Dummy | 24 | INT |  |
| Rubbing NIP Dummy | 25 | INT |  |
| Etch Strip Dummy | 26 | INT | For Array EQP |
| Metal Dummy | 27 | INT | For Array EQP |

    - Job Judge

It indicates the judgment of job. The value is to set by equipment automatically or by operator.

| Job Judge |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 0 | Inspect Skip or No Judge | G | Good | R | Repair |
| N | NG | A | Need Mac | I | Need Mic |

| S | Scrap | W | R/W | K | Process Skip |
| --- | --- | --- | --- | --- | --- |
| T | Retest | M | Mac/Mic | P | Pending |
| F | Special Good (CF only) | C | Need CSI | V | Need CVD |
| E | Test(CF Only) |  |  |  |  |

Each equipment may use different set of Judge codes – it will be determined by equipment engineer.

    - Job Grade

It indicates the grade of job. The value is to set by equipment automatically or by operator.

| Job Grade |  |  |  |
| --- | --- | --- | --- |
| Judge | Transfer Grade | Grade |  |
| 0 | 0 | 0 | Inspect Skip or No Judge or No Grade |
| G | G | AP | AP |
|  |  | AM | AM |
|  |  | AO | AO |
|  |  | AS | AS |
|  |  | AT | AT |
|  |  | AL | AL |
|  |  | AX | AX |
|  |  | AY | AY |
|  |  | AZ | AZ |
|  |  | AI | AI |
|  | T | TX | TX |
|  |  | TY | TY |
|  |  | TZ | TZ |
|  |  | TU | TU |
|  | Q | QS | QS |
|  |  | QT | QT |
|  |  | QQ | QQ |
|  |  | QL | QL |
|  |  | QV | QV |
|  |  | QW | QW |
|  |  | QU | QU |
|  |  | QP | QP |
|  |  | QR | QR |
| N | N | NW | NW |
|  |  | NU | NU |
|  |  | NR | NR |
|  |  | NP | NP |
|  |  | NV | NV |
|  |  | FL | FL |
|  |  | FO | FO |
|  |  | NI | NI |
|  |  | NS | NS |

|  |  | NT | NT |
| --- | --- | --- | --- |
|  |  | BR | BR |

Each equipment may use different set of Judge codes – it will be determined by equipment engineer.

    - Process Flag

It is historical record of processing status, indicating that job has been processed or skipped in specific equipment. Each and every equipment should update this data after processing or skip.

Every Equipment should set its processed data value at its own designated write area, and should not overwrite data value recorded by other equipment. And also it has to be set “Skipped” when job has been aborted by OP or EQP. It can be recognized by Process Reason Code.

| Process Flag | Value |
| --- | --- |
| Skipped | 0 |
| Processed | 1 |

    - Inspection Judge data

It is historical judge data that job has been judged in specific inspections. The specific inspections should update this data after inspection.

<Example of use>

| Inspection Jude Data |  |  |  |
| --- | --- | --- | --- |
| 8bit | Equipment01 Inspection Jude Data “0”= Inspect Skip or No Judge ,“G”=Good ,“R”=Repair ,“N”=NG “V”=Need CVD ,“C”=Need CSI ,“S”=Scrap ,“W”=R/W “K”=Process Skip ,“T”=Retest ,“M”=Mac/Mic | 8bit | Equipment03 Inspection Jude Data “0”= Inspect Skip or No Judge ,“G”=Good ,“R”=Repair,“N”=NG “V”=Need CVD ,“C”=Need CSI ,“S”=Scrap ,“W”=R/W “K”=Process Skip ,“T”=Retest ,“M”=Mac/Mic |
| 8bis | Equipment02 Inspection Jude Data “0”= Inspect Skip or No Judge ,“G”=Good ,“R”=Repair,“N”=NG “V”=Need CVD ,“C”=Need CSI ,“S”=Scrap ,“W”=R/W “K”=Process Skip ,“T”=Retest ,“M”=Mac/Mic | 8bit | Equipment04 Inspection Jude Data “0”= Inspect Skip or No Judge ,“G”=Good ,“R”=Repair,“N”=NG “V”=Need CVD ,“C”=Need CSI ,“S”=Scrap ,“W”=R/W “K”=Process Skip ,“T”=Retest ,“M”=Mac/Mic |

    - Skip Flag

This is Host instruction to equipment whether to skip processing for a specific Job. If this flag is On, equipment should by pass the Job without processing.

Skip flag is composed of bits where each bit is assigned to each equipment. Therefore equipment should check its own Skip flag only.

| Skip Flag | Value |
| --- | --- |
| To normal Process | 0 (OFF) |
| To Skip | 1(ON) |

    - Inspection Flag

This flag indicate is to be set and be used within Equipment or Inline for inspection purpose.

While a Job is transferred or being processed in equipment, Operator or Equipment can set this flag to indicate that the specific Job is required inspection or measurement somewhere in downstream.

When receiving a Job with this flag is On, Transfer Unit is required to transfer the Job to designated Inspection Unit/Equipment, and the Inspection Unit/Equipment processes the inspection.

- Which equipment has to provide function to set this flag: To be determined by engineer, if need.

- Which equipment need to use this flag : To be determined by engineer, if need.

- After Inspection, whether to turn Off this flag after inspection : to be determined by engineer, if need.

| Inspection Flag | Value |
| --- | --- |
| Default | 0 |
| Inspection is required | 1 |

    - Q-Time

Q-Time is defined as duration of Job between two processing Equipment. It is calculated in time from the processing end of a Job in one equipment to the processing start of the same Job in target equipment.

- Equipment in Q-Time path : equipment records the in/out duration into Job Data before sending the Job to downstream. The in/out duration record should be cumulative so that it has to be added not overwritten.

- Q-Time Target Equipment : after receiving a Job, equipment checks the Q-Time of the Job before starting processing. If Q-Time is over, equipment raises alarm and set Q-Time over flag to the Job, and continues the next processing.

- Q-Time Target Equipment has to provide Q-Time validation setting function on touch panel.

- There are maximum three of Q-Time define points in equipment. Refer to Equipment Operation Scenario for each Q-Time Start and Target equipment.

- Time duration should be recorded in second.

Figure 4-3 Process Q-Time

  - Job Data

Job Data is characteristics of a Job and should be managed and maintained in equipment.

- When Job is transferred between equipment, the Job Data also has to transferred along with the Job.

- Equipment has to maintain the Job Data as long as the Job is in equipment, even when equipment is down or power off.

    - Job Data Items

      - Job Data A

It has 96 word in default without optional Item. This Data is used for Job Data transferring between Equipment (Local) and is used to report Equipment-level Job Event Report to Host (Received Job Event, Sent Out Job Event)

| Item Name | Format | Word Points | Bit Points | Sub Items | Writable |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | EAS | LD | EQP |  |
| Cassette Sequence No | INT | 1 |  | 1~65535 |  | W |  | It is mainly to distinguish which kind of job the job is and from which cassette the job is fetched out |
| Job Sequence No | INT | 1 |  | 1~65535 |  | W |  | It indicates the position of the job in the cassette that is Slot No |
| Lot ID | ASC | 10 |  |  | W | W |  |  |
| Product ID | ASC | 10 |  |  | W | W |  |  |
| Operation ID | ASC | 10 |  |  | W | W |  |  |
| GlassID or PanelID | ASC | 10 |  |  | W | W | VCR | The Glass ID or Panel ID (after Cut) of the Job |
| CST Operation Mode | INT | 1 |  | 0: Kind to Kind 1: CST to CST | W | W |  | It indicates the operation mode of cassette. |
| Substrate Type | INT | 1 |  | 0: Glass 1: Panel 2: Q-Panel | W | W | W | The Substrate Type of the job |
| Product Type | INT | 1 |  | 1:Production 2:Engineer 3:Dummy 4: Develop 5: Test 6~:Reserved | W | W |  | It indicates the type of product. Equipment can’t mix different product type in on local. |
| Job Type | INT | 1 |  | 1: TFT Glass 2: CF Glass 3: Dummy 4: Normal 5: UV Mask | W | W |  | It indicates the type of glass. |

| Item Name | Format | Word Points | Bit Points | Sub Items | Writable |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | EAS | LD | EQP |  |
|  |  |  |  | 6: Pol 7 ~ : Reserved |  |  |  |  |
| Dummy Type | INT | 1 |  | Refer to 4.4.9 Dummy Type | W | W |  | It indicates the type of dummy glass. It is valid when Job Type is Dummy. |
| Skip Flag | BIT | 2 | 1 | 1Bit/ Unit or Local 00: Processing 01: Skip | W | W |  | It indicates Host downloads a process or non process flag to EQ that to will be skip or process |
| Process Flag | BIT | 2 | 1 | 1Bits/Unit or Local 0: Skipped 1: Processed |  |  | W | It is historical Process Data that job has been processed or skipped in specific equipments. The specific equipments should update this data after processing or skip. |
| Process Reason Code | INT | 1 | 1 | 0: no Error 1: stop by OP 2: material problem 3: abort by EQP 4: abort by OP 5: Mura Reprocess 6: Reserved [need to define by vender] |  |  | W | It is historical error code of process that reason code when job has been not processed or aborted in specific equipments |
| Last Glass Flag | INT | 1 |  | 0: Not Last Glass 1: Last Glass | W | W |  | Mark last glass of CST or LOT for FIFO operation |
| First Glass Flag | INT | 1 |  | 0: Not First Run 1: First Run | W | W |  | Mark first glass of CST or LOT for FIFO operation |
| Glass Thickness | INT | 1 |  | 1: 0.3t 2: 0.4t 3: 0.5t 4:0.7t 5 ~ : Reserved | W | W |  | It indicates the thickness of glass. |
| Glass Degree | INT | 1 |  | 0: 0° 1: 90° 2: -90° 3: 180° 8: Turn over 0° 9: Turn over 90° 10: Turn over -90° 11: Turn over 180° |  | W | W | Current degree of turn/turn over from normal position. If equipment turn/turn over glass, equipment should write this data (the value is the resultant degree from normal position, not the degree amount that equipment turned) |
| Inspection Flag | BIT | 1 | 1 | 0: No Inspection 1: Inspection | W | W | W | The Inspections unit of downstream with Inline has to process an reservation flag that is set manual by OP or automatically by EQP. Such as sampling or by pass |
| Inspection Judge Data | ASC | 3(4) |  | 8bits/Equipment Max 6Equipment *Referto Judge Value* *CF Photo Special* - 4word |  |  | W | It is historical judge data that job has been judged in specific inspections. The specific inspections should update this data after inspection. |
| Job Judge | ASC | 1 |  | [Common] *refer to Judge Code | W | W | W | In case equipment makes Judge, the equipment should write this data. |
| Job Grade | ASC | 1 |  | Job Grade(2 Char) *refer to Judge Grade | W | W | W | It indicates the grade of a job. In case equipment makes Grade, the equipment should write this data |
| Job Recovery Flag | INT | 1 |  | 0: default 1: from EQ 2: from EAS | W | W | W | It indicates job recovery state at No VCR Unit. Write this data if Job is recovered from Scrap. |
| Q-Time1 | INT | 1 |  | second |  |  | W | equipment add(not overwrite) the in/out duration into Job Data before sending the Job to downstream |
| Q-Time2 | INT | 1 |  | second |  |  | W | equipment add(not overwrite) the in/out duration into Job Data before sending the Job to downstream |
| Q-Time3 | INT | 1 |  | second |  |  | W | equipment add(not overwrite) the in/out duration into Job Data before sending the Job to downstream |

| Item Name | Format | Word Points | Bit Points | Sub Items | Writable |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | EAS | LD | EQP |  |
| Q-Time Over Flag | Bit | 1 | 3 | 1st bit : Q-time 1 2nd bit : Q-time 2 3rd bit : Q-time 3 |  |  | W | This item is set by equipment of Q-time Check when Q-time is over. |
| Mode | INT | 1 |  | Ex) 1: Normal Mode 2: Cold Run Mod 3: Sorting Mode 4: Photo Dummy Mode 5: ITO Dummy Rework Mode 6: PR Rework Mode 7: Scraper Mode Refer to Indexer Operation Mode Block | W |  |  | Line Operation Mode This is the reference value when glass processing or glass flow. |
| Master Recipe | ASC or INT | 10 or 1[‘= |  |  | W | W |  | It indicates Master Recipe. (PPID) |
| Shop Special Item |  | 18 or 27 |  | Refer to below |  |  |  |  |

      - Job Data B

This Data item is brief version of Job Data. When equipment reports Job Event to Host for Job transferring between Units (Stored Job Event, Fetched Out Job Event), Equipment should report Job Data B instead of Job Data A.

| Item Name | Format | Word Points | Bit Points | Sub Items | Writable |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | EAS | LD | EQP |  |
| Cassette Sequence No | INT | 1 |  | 1~65535 | W | W |  | It is mainly to distinguish which kind of job the job is and from which cassette the job is fetched out |
| Job Sequence No | INT | 1 |  | 1~65535 | W | W |  | It indicates the position of the job in the cassette that is Slot No |
| Glass ID or PanelID | ASC | 10 |  |  | W | W | VCR | The Glass ID or Panel ID of the Job |
| Job Judge | ASC | 1 |  | [Common] *refer to Judge Code | W | W | W | It indicates the judgment of job. If operator changes it in inspection, its value should be changed. (Total result flag of all Inspection Equipment in Inline) |
| Job Grade | ASC | 1 |  | Job Grade(2 Char) *refer to Judge Grade | W | W | W |  |

      - Job Data C

This Data item is brief version of Job Data. When equipment reports Job Event to Host for Job transferring between Equipment ,Units by Cassette that is Cassette Cleaner or Multi Aging. Equipment should report Job Data C instead of Job Data A or B.

| Item Name | Format | Word Points | Bit Points | Sub Items | Writable |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | EAS | LD | EQP |  |
| Cassette Sequence No | INT | 1 |  | 1~65535 | W | W |  | It is mainly to distinguish which kind of job the job is and from which cassette the job is fetched out |
| Cassette ID | ASC | 10 |  |  | W | W | VCR |  |
| Cassette Judge | ASC | 1 |  | [Common] *refer to Judge Code | W | W | W | It indicates the judgment of CST. If operator changes it in inspection, its value should be changed. (Total result flag of all Inspection Equipment in Inline) |
| Cassette Process Flag | BIT | 1 | 1 | 1Bits/Unit of Local 0: Skipped 1: Processed |  |  | W | It is historical Process Data that job has been processed or skipped in specific equipments. The specific equipments should update this data after processing or skip. |

| Item Name | Format | Word Points | Bit Points | Sub Items | Writable |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | EAS | LD | EQP |  |
| Master Recipe | ASC or INT | 10 |  |  | W | W |  | It indicates Master Recipe. (PPID) |

      - Job Data S

This Data item is brief version of Job Data. When equipment reports Job Event to Host for Job transferring between Equipment ,Units by Cassette that is Cut Line. Equipment should report Job Data S with Job Data A that is 256 word.

※ This feature is used only devices needed.

| Item Name | Format | Word Points | Bit Points | Sub Items | Writable |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | EAS | LD | EQP |  |
| Job Data A | - | 96 |  |  | W | W | W |  |
| Panel Judge | ASC | 128 |  |  | W | W |  | this is to request information of Panels such as Risk of each Panel in a Glass (available only Glass – before cut to Panel). |
| Risk Panel Flag | ASC | 10 |  |  | W | W |  | This data should be set in Parent Glass from MES before Cutting |
| Reserved | - | 21 |  |  |  |  |  |  |

      - Array Job Data

This is additional Job Data items required for Array Shop equipment. This items should be added to Job Data A.

| Item Name | Format | Word Points | Bit Points | Sub Items | Writable |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | EAS | LD | EQP |  |
| Tray ID | INT | 1 |  | 1~65535 |  |  | W | It indicates which Tray has the glass(Array Sputter only) |
| Tray Life | INT | 1 |  | 1~65535 |  |  | W | It indicates when the Tray should be changed (Array Sputter only) |
| Wait Over Flag | BIT | 1 |  |  |  |  | W | This flag is set by Track equipment when glass is delay. EAS has to OPCall before unloading if this flag is set. |
| OffSetID1 | ASC | 3 |  |  | W |  |  | This data receives from MES that is exposure ID. |
| OffSetID2 | ASC | 3 |  |  | W |  |  | This data receives from MES that is Chamber ID. |
| Test Gap Glass Flag | INT | 1 |  | 0: Off 1: On | W | W |  | Sputter Test Gap Mode only. When this Mode has, OP should set a dummy glass for Test Gap. |
| Mode | INT | 1 |  |  | W | W |  | This item is used for validation by Equipment Operation Mode |
| Line Specific Item |  | 9 |  |  |  |  |  | Spare |

      - Cell Job Data

This is additional Job Data items required for Cell Shop equipment. This items should be added to Job Data A

        - ODF, PI, Rubbing Job data

| Item Name | Format | Word Points | Bit Points | Sub Items | Writable |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | EAS | LD | EQP |  |
| Pair Glass ID | ASC | 10 |  |  | W | W |  | When glass is paired by MES, Provide counter type(CF<>TFT)’s Glass ID |
| Pair Cassette Sequence No | INT | 1 |  | 1~65535 | W | W |  | For ODF, provide pair Glasses Cassette Sequence No |
| Pair Job Sequence No | INT | 1 |  | 1~65535 | W | W |  | For ODF, Matching Sorter, provide pair Glasses Slot No(=Job Sequence No) |
| Dummy Use Count | INT | 1 |  |  |  |  | W | For Dummy Operation |

| Item Name | Format | Word Points | Bit Points | Sub Items | Writable |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | EAS | LD | EQP |  |
| Dummy Rework Count | INT | 1 |  |  |  |  | W | For Dummy Operation |
| Re-process Flag | BIT | 1 |  | - : Default - : Set Re-process - : Re-process done 3~ : Reserved |  |  | W | For Rubbing, Seal Inspection. If process equipment received glass with Re-process flag(1), then ignore Process Flag(even it already done process for that glass), run process again |
| Re-Steam Flag | INT | 1 |  | - : Default - : Glass Turn Request - : Glass Turn Complete |  |  | W | Re-Steam Operation is assigned on this job. In the condition of Operator want re-check glass with another degree. |
| Re-Steam Degree | INT | 1 |  | 0 : 0° 1 : 90° 2 : -90° 3 : 180° |  |  | W | OP Re-Steam. In the condition of Operator want re-check glass with another degree. |
| Trouble Code 1 | BIT | 1 |  | PI only |  |  | W | BIT0 Cleaner APPM APP trouble, go Macro BIT1 Cleaner AKM AK trouble, go Macro BIT2 Cleaner EUVM EUV trouble, go Macro BIT3 Cleaner IRM IR trouble, go Macro BIT4 Cleaner VCRMI VCR Warning, go Macro & Micro BIT5 Cleaner IDMI Glass Information relate trouble, go Macro & Micro BIT6 Transfer Q-Time#1 Over Q-Time Over, go Macro BIT7 Transfer Q-Time#2 Over Q-Time Over, go Macro BIT8 Inkjet IJDM Inkjet Down trouble, go Macro BIT9 Pre-Cure PRECDM Pre-cure Down trouble, go Macro BIT10~15 : Not define |
| Trouble Code 2 | BIT | 1 |  | PI only |  |  | W | BIT0~15 : Not define |
| Special Skip Flag | BIT | 1 | 1 | 1Bit/Equipment 00: Processing 01: Skip | W | W |  | It is for ODF only that unit is used from 33 |
| Special Process Flag | BIT | 1 | 1 | 1Bit/Unit or Local 0: Skipped 1: Processed |  |  | W | It is for ODF only that unit is used from 33 |
| Temp Destination Local | INT | 1 |  | Target Local | W | W |  | It is used for flow the glass to destination which is position of target. TR will dispatch by this flag. |
| Temp Destination Slot | INT | 1 |  | Slot Position | W | W |  | It is used for flow the glass to destination which is position of target. TR will dispatch by this flag. |
| SA Steam Inspection Flag | INT | 1 |  | 0: don’t need inspection 1: need inspection |  | W |  | It is used for inspection with equipment of stand- alone after not inspection in Rubbing at Cell |
| Mode | INT | 1 |  |  | W | W |  | This item is used for validation by Equipment Operation Mode |
| Inspection Judge | ASC | 2 |  | C:CG A:AO O:OI G:GM CA:CG + AO CO:CG + OI CG:CG + GM AO:AO + OI AG:AO + GM OG:OI + GM CAO:CG + AO + OI CAG:CG + AO + GM COG:CG + O + GM AOG:AO + OI + GM CAOG:CG + AO + OI + GM - CG:Cell Gap - GM:Gravity Mura - AO:Aging Oven - OI:AOI |  |  | W | It is for ODF only Visual Inspection write Inspection Judge |
| Turn Angle | BIT | 1 | 2 | 00(0) : 0° 01(1) : 90° | W |  |  | Only use Rubbing. Turn align for Stream Inspection |

| Item Name | Format | Word Points | Bit Points | Sub Items | Writable |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | EAS | LD | EQP |  |
|  |  |  |  | 10(2) : -90° 11(3) : 180° |  |  |  |  |

        - Cutting job data

Cutting Line Scriber set Panel Recipe ID – Applied main/sub recipe id to panel, Panel Type, Panel Size, P- Code and USE TYPE on job data of Panel.

| Item Name | Format | Word Points | Bit Points | Sub Items | Writable |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | EAS | LD | EQP |  |
| Panel Position | INT | 1 |  | 0 ~ 65535 0: Glass 1~65535: Panel |  |  | W | For Cutting, The Substrate position of Glass after cutting. |
| Panel Recipe ID | ASC | 5 |  |  |  |  | W | Applied Main/Sub Cutting Recipe ID |
| Panel Type | INT | 1 |  | 1: Normal 2: MMG 3: Q-Normal 4: Q-MMG | W | W |  | This is Panel Type of Recipe |
| Panel Row Size | F | 2 |  | *Float type |  |  | W | When panel edge placed on origin, Y Size of Panel |
| Panel Col Size | F | 2 |  | *Float type |  |  | W | When panel edge placed on origin, X Size of Panel |
| P-Code | ASC | 5 |  |  |  |  | W | Code for distinguishing panel type of MMG Glass |
| USE TYPE | ASC | 5 |  |  |  |  | W | Code for managing panel cassette |
| Mode | INT | 1 |  |  | W | W |  | This item is used for validation by Equipment Operation Mode |
| Risk Panel Flag | ASC | 1 |  |  |  |  | W | This data is divided from Parent Glass, and it should be in Panel Glass after Cutting |
| Position X | INT | 1 |  |  |  |  | W | A panel’s position in parent Glass |
| Position Y | INT | 1 |  |  |  |  | W | A panel’s position in parent Glass |

        - Stand Alone

| Item Name | Format | Word Points | Bit Points | Sub Items | Writable |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | EAS | LD | EQP |  |
| Pair Job Sequence No | INT | 1 |  | 1~65535 | W | W |  | It is mainly to distinguish which kind of job is TFT + CF and from which the Pair job is fetched out |
| Panel NG Type | INT | 1 |  | 0: AG 1:1N 2:2N 3:Multi |  |  | W | For CF pre sorter |
| Next Process Code | ASC | 1 |  | 00: Not define R:Re-test W:Welding Repair B:Burning Repair S:Sorter A:Aging Oven M:Trimmer P:Dense Packer G:Shipping N:Scrap |  |  | w | For Cell Test : Sorter, Trimmer, Tester |
| Macro Flag | BIT | 1 | 1 | 0: Inspections not reserved 1: Inspections reserved |  |  | W | The Macro unit of downstream with Inline has to process an reservation flag that is set manual by OP or automatically by EQP. Such as sampling or by pass |

| Item Name | Format | Word Points | Bit Points | Sub Items | Writable |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | EAS | LD | EQP |  |
| Micro Flag | BIT | 1 | 1 | 0: Inspections not reserved 1: Inspections reserved |  |  | W | The Micro unit of downstream with Inline has to process an reservation flag that is set manual by OP or automatically by EQP. Such as sampling or by pass |
| Mode | INT | 1 |  |  | W | W |  | This item is used for validation by Equipment Operation Mode |
| Retest Ap | INT | 1 |  | - : No Retest - : Retest to AP1 - : Retest to AP2 | W | W | W | This item is used for Retest in only Cell Test Stand Alone. |

        - Job data(Include Stand-alone before Cutting)

Refer to 4.5 job data and this is additional Item

| Item Name | Format | Word Points | Bit Points | Sub Items | Writable |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | EAS | LD | EQP |  |
| Pair Glass ID | ASC | 10 |  |  | W | W |  | When glass is paired by MES, Provide counter type(CF<>TFT)’s Glass ID |
| Pair Cassette Sequence No | INT | 1 |  | 1~65535 | W | W |  | For ODF, provide pair Glasses Cassette Sequence No |
| Pair Job Sequence No | INT | 1 |  | 1~65535 | W | W |  | For ODF, Matching Sorter, provide pair Glasses Slot No(=Job Sequence No) |
| Dummy Rework Count | INT | 1 |  |  |  |  | W | For Dummy Operation |
| Re-process Flag | BIT | 1 |  | - : Default - : Set Re-process - : Re-process done 3~ : Reserved |  |  | W | For Rubbing, Seal Inspection. If process equipment received glass with Re-process flag(1), then ignore Process Flag(even it already done process for that glass), run process again |

        - Stand-alone – Steam Inspection ( if possible use Job Judge Item)

Glass after stand-alone steam inspection should unload glass by its steam inspection result So it clearly present its inspection result for indexer which conduct unload glass dispatching.

| Item Name | Format | Word Points | Bit Points | Sub Items | Writable |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | EAS | LD | EQP |  |
| SA-Steam Result | INT | 1 |  | - : Default - : Good (go ODF) - : Re-Rubbing - : PI Rework - : Scrap |  |  | W | Represent Stand Alone Steam Result |

        - Trimming Flag – Cell Test Inline/ Stand-alone Trimmer

If the panel has successful result from trimmer of Cell Test Inline, Then MES keep this result and in stand- alone trimmer, MES make equipment to skip processing for this panel. For this operation job data has SA- Trimming Skip Flag.

| Item Name | Format | Word Points | Bit Points | Sub Items | Writable |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | EAS | LD | EQP |  |
| SA-TrimmingSkip Flag | INT | 1 |  | 0 : Default 1 : ON( No need Stand- alonetrimming ) 2 : Off | W | W | W | Cell Test Inline : set ‘On’ when trimming is successfully done. Stand-alone trimmer : get job data’s value from cassette information |
| Grade Priority | INT | 1 |  | 1 : VN Rate |  |  | W | It’s used for dispatching to Port by set a priority in Port Mode |

        - Repair Flag – Cell Repair, Welding Repair, Burning Repair, Pad Repair

Repair equipment should provide its conducted repair type.

| Item Name | Format | Word Points | Bit Points | Sub Items | Writable |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | EAS | LD | EQP |  |
| Repair Flag | ASC | 1 |  | 0 : No Processing W : Welding, D : Pad, B : Burning, P : Press, O : Oven， | W | W | W | Conducted Repair Process Type |
| Step ID | ASC | 10 |  | * Cell Repair only | W | W |  | It’sused for equipment by received from MES |
| Repair Information | ASC | 1 |  | W: Welding B: Burning * Cell Repair only | W | W |  | It’sused for dispatching by received from MES |
| Grade Priority | INT | 1 |  | 1 : VN Rate |  |  | W | It’s used for dispatching to Port by set a priority in Port Mode |

        - Inspection Judge – Cell Gap, Gravity Mura

Indexer Robot Must move to Glass Next Equipment by Inspection Judge Value.

| Item Name | Format | Word Points | Bit Points | Sub Items | Writable |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | EAS | LD | EQP |  |
| Inspection Judge | ASC | 2 |  | C:C/G P:G/M R:A/O CP:C/G + G/M CR:C/G + A/O PR:G/M + A/O CPR:C/G+G/M+A/O - C/G:Cell Gap - G/M:Gravity Mura - A/O:Aging Oven | W | W |  | It’sused for equipment by received from MES |

.

        - End Cell Job Data

| Item Name | Format | Word Points | Bit Points | Sub Items | Writable |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | EAS | LD | EQP |  |
| Transfer Grade | ASC | 1 |  |  |  |  | W | It is indicates grade of transfer within inspection equipment which is End Cell only. |
| Panel Row Size | F | 2 |  | *Float type |  |  | W | When panel edge placed on origin, Y Size of Panel |
| Panel Col Size | F | 2 |  | *Float type |  |  | W | When panel edge placed on origin, X Size of Panel |
| Mode | INT | 1 |  |  | W | W |  | This item is used for validation by Equipment Operation Mode |

      - CF Job Data

This is additional Job Data items required for CF Shop equipment. This items should be added to Job Data A

| Item Name | Format | Word Points | Bit Points | Sub Items | Writable |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | EAS | LD | EQP |  |

| Item Name | Format | Word Points | Bit Points | Sub Items | Writable |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | EAS | LD | EQP |  |
| Reprocess Flag | INT | 1 |  | Local No |  |  | W | If one glass need to reprocessing, Equipment should write this flag |
| Reprocess Count | INT | 1 |  |  |  | W |  | This item is count of reprocess in photo inline. Before glass of reprocess move from Unloader to Loader, Indexer should update this value plus 1. |
| AOI Inspection Flag | INT | 1 |  |  |  |  | W | When this glass need to AOI Inspection, Sputter should write this flag ‘On’. (Only ITO Line) |
| Target Port | INT | 1 |  | Port No | W |  |  | When EAS send Cassette Map Download, EAS should write this item. TR01 should move glass to appointed port by target port item. (Only Repair & FinalMacro) |
| Rework Count | INT | 1 |  |  |  | W |  | If Auto Macro change glass Judge ‘NG’, Auto Macro should write Rework Count plus 1. (Only Rework Inline) |
| Line Number | INT | 1 |  | Refer to 11.3 CF Shop Special | W |  |  | This item indicates current line number of glass. |
| EXPO Process | INT | 1 |  | 1: Need Front 2: Need Back | W |  |  | This item is used only EXPO at New Model Change Operation. This item indicates destination of one glass at New Model Change Operation. |
| Mura Risk Flag | INT | 1 |  | 1: M flag 2: Reserved |  |  | W |  |
| Special Process Flag |  | 1 |  |  |  |  | W | Use Bit |
| Batch ID | ASC | 6 |  |  | W |  |  |  |
| Reprocess Reason Code | ASC | 8 |  |  |  |  |  | Only use Photo Line.(Reprocess Machine should write Reason Code) |
| AOI Risk Flag | INT | 1 |  | 1: K flag 2: Reserved |  |  | W | When this glass need to Repair Process, AOI should write this flag ‘on’. |
| AOI First Flag | INT | 1 |  | 0: Normal 1: Oven Pass(Oven) 2: AOI Check Complete(AOI) |  |  |  |  |
| Reserved |  | 2 |  |  |  |  |  |  |

- Common Function

  - Equipment Main Mode Management

    - Equipment Alive

Equipment Alive is the status that the equipment is in proper operation and can be controlled by PLC. Local PLC should periodically change Equipment Alive bit to be 60 seconds on and 60 seconds off. If Equipment Alive signal doesn’t change for 130 seconds, Master (EAS) should take it as communication disconnection and generate Operator Call.

- Equipment Alive Check between equipment

Besides Equipment Alive signal to Master (EAS), Equipment check Alive status of both upstream equipment and downstream equipment.

Equipment’s sends ‘Downstream Alive’ bit to its upstream equipment, and sends ‘Upstream Alive’ bit to its downstream equipment.

‘Upstream Alive’ and ‘Downstream Alive’ bit is to be set 60 seconds on and 60 seconds off, and periodically repeats this on/off set.

If equipment detects either ‘Upstream Alive’ or ‘Downstream Alive’ doesn’t change for 130 seconds, equipment raises and alarm and OP call.

    - CIM Mode

CIM Mode is to specify the communication relation between equipment and EAS. There are 2 kinds of modes, CIM ON Mode and CIM OFF Mode. Equipment should provide CIM Mode change button on touch panel.

- CIM ON Mode: The communication is normal between EAS and EQ. EQ can report all events to EAS. Also the EAS can send any command to EQ.

- CIM OFF Mode: The communication is somehow abnormal between EAS and EQ. EQ can update status data (Link Register Area), but has not to report the event to EAS (Link Relay Area). Also, equipment has not to accept any command from EAS. But, Equipment should be allow for the Command of the CIM Mode On Change.

    - CIM Mode Change By EAS

EAS can change CIM Mode by CIM Mode Change Command, for data collection.

    - Inline Mode

The inline mode is related only to job transferring and is necessary only between upstream equipment and downstream equipment. Equipment should provide inline mode change button on touch panel. The Inline Mode consists of 2 modes:

- Upstream Inline Mode On: EQ can receive a glass/Panel from upstream equipment.

- Upstream Inline Mode Off: EQ cannot receive a glass/Panel from upstream equipment.

- Downstream Inline Mode On: EQ can send a glass/Panel to downstream equipment.

- Downstream Inline Mode Off: EQ cannot send a glass/Panel to downstream equipment.

    - Local Alarm State

It shows that there is an alarm in entire equipment. If all alarms are cleared, it should be turned off.

    - PLC MAP

      - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| CIM Mode Change Command Reply Block | Return Code | 1 | Data | INT | 1: Accept 2: Already In Desired Status |

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) Action |
| --- | --- | --- | --- | --- | --- | --- |
|  | Equipment Alive | 1 | Status |  |  |  |
|  | CIM Mode | 1 | Status | - On: CIM ON Mode - Off: CIM OFF Mode |  |  |
|  |  |  |  | - On: Inline On Mode with |  |  |
|  | Upstream Inline Mode | 1 | Status | Upstream EQP - Off: Inline Off Mode with |  |  |
|  |  |  |  | Upstream EQP |  |  |
| Equipment Status Event | Downstream Inline Mode | 1 | Status | - On: Inline On Mode with Downstream EQP - Off: Inline Off Mode with |  |  |
|  |  |  |  | Downstream EQP |  |  |
|  |  |  |  | - On: There is an alarm that |  |  |
|  |  |  |  | is not cleared. |  |  |
|  | Local Alarm State | 1 | Status | - Off: There is no alarm. All |  |  |
|  |  |  |  | alarms have already been |  |  |
|  |  |  |  | cleared. |  |  |
| EASComman d Reply | CIM Mode Change Command Reply | 1 | Reply | - On: Reply - Off: Reply End | Command Reply Block |  |

      - Master (EAS) – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| CIM Mode Change Command Block | CIM Mode | 1 | Data | INT | 1: CIM On 2: CIM Off |

      - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| EAS Command | CIM Mode Change Command | 1 | CMD | - On: Fire Command - Off: Command End/Timeout | CIM Mode Change Command Block |  |

    - Communication Scenarios

      - Equipment Alive Scenario

      - CIM Mode Scenario

      - Inline Mode Scenario

      - Local Alarm State Scenario

  - Equipment Mode Change

    - Equipment Mode Change Command

If EAS need to change equipment mode, EAS sends the “Equipment Mode Change Command” to equipment.

- Normal Mode : Normal processing

- Recovery(Force clean out) Mode : Executing force clean out function

- Skip Mode : Processing Skip(or By-pass)

- Cold Run Mode : Each equipment just transfer glass without processing

- ETC..

      - PLC MAP

        - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Equipment Mode Change Return Block | Return Code | 1 | Data | INT | 1: Accepted 2: Not Accepted (Already this mode ) 3: Not Accepted (Equipment have no this mode) 4: Not Accepted (Other error) |

        - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) |
| --- | --- | --- | --- | --- | --- | --- |
| EASComma nd Reply Block | Equipment ModeChangeCommand Reply | 1 | Reply | - On: Reply - Off: Reply End | Equipment Mode ChangeReturn Block | Reply for Receiving |

        - Master (EAS) – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| EquipmentModeChangeBlock | Equipment Mode | 1 | Data | INT | 0: No Equipment Mode exist 1: Normal Mode 2: Recovery(Force Clean Out) Mode 3: Skip Mode 4: Cold Run Mode 5: ETC. |

        - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| EASComma nd | Equipment ModeChange Command | 1 | CMD | - On: Fire Event - Off: Event End | Equipment Mode ChangeBlock |  |

    - Equipment Mode Change Report

Equipment Mode can be change by operator at touch panel setting or by command from EAS. After Equipment Mode change, equipment reportsEquipment Mode Change Report

      - PLC MAP

        - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| EquipmentModeBlock | Equipment Mode | 1 | Data | INT | 0: No Equipment Mode exist |

1: Normal Mode

2: Recovery(Force Clean Out) Mode 3: Skip Mode

4: Cold Run

5: ETC..

        - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) |
| --- | --- | --- | --- | --- | --- | --- |
| EquipmentE vent | Equipment ModeChange Report | 1 | Event | - On: Fire Event - Off: Event End | Equipment Mode Block |  |

        - Master (EAS) – Link Register (W Area)

None

        - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Event Reply | Equipment Mode Change Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |

  - Equipment Management

    - Equipment Status

If equipment status is changed, the corresponding events must be reported. And it has to be displayed at the touch panel of the equipment. There is no remote command that directly changes equipment status. In EAS, equipment status is defined as one of five of the following types: state

- Idle: Equipment does not execute a process that means not exist any glass, but it is ready to start a process. This is applicable whether there is a material related to the equipment or not.

- Run: Equipment is executing a process that means exist a glass If the process is completed and there is no job for processing, the equipment turns idle again.

- PM: Equipment is initializing the module and utilities. Equipment must manage PM code list

- Down(BM): Equipment process is stopped. The equipment has been stopped by alarm, or H/W problem or other causes. During this status, the equipment has to stop job transferring.

- Pause: Equipment process is paused. In this status, equipment can resume process immediately.

- Job Change: Special Customer Requirements.

- ETC: Special Customer Requirements.

- Priority (Job Change/ETC is the same priority to PM)

Serial Structure : PM(Job Change/ETC) > DOWN(BM) > RUN > IDLE Parallel Structure : RUN > IDLE > PM(Job Change/ETC) > Down(BM)

※ Equipment Status for Loader/Un-loader/Buffer/Both

| Transfer EQ | Run | Idle |
| --- | --- | --- |

| Transfer EQ | Run | Idle |
| --- | --- | --- |
| Loader | Idle to Run: When robot or conveyor fetched out the first glass from a cassette. As long as there is one or more cassette in processing state (Port state is InUse) then Loader keeps Run status. | There is no cassette in processing state (i.e. Port State is InUse) |
| Un-loader | Idle to Run: When robot or conveyor Received a glass from upstream. If there is one or more cassette in processing state (Port state is InUse) then Un-loader keeps Run status | Un-loader has no cassette in processing (i.e. Port State is InUse), but it is ready to start glass receiving. |
| Loader/ Un-loader (Both) | Idle to Run: When robot or conveyor fetched out the first glass from a cassette. When robot or conveyor fetched out all glasses from the cassette, but if there is one or more cassette in processing state (Port state is InUse) then Loader/Un-loader keeps Running status. | There is no cassette in processing state (i.e. Port State is InUse) |
| Buffer | Idle to Run: Loader in Buffer Type: When robot or conveyor fetched out the first glass from a cassette. Buffer Type or Un-Loader in Buffer Type: When robot or conveyor Received the first glass from upstream. | Buffer has no cassette in processing (i.e. Port State is InUse), but it is ready to start glass transfer. Buffer has no glass, but it is ready to start glass transfer. |

| Equipment / Unit Status | Value | Format |
| --- | --- | --- |

| PM | 1 | INT |
| --- | --- | --- |
| DOWN | 2 | INT |
| PAUSE | 3 | INT |
| IDLE | 4 | INT |
| RUN | 5 | INT |
| JobChange | 6 | INT |
| ETC | 7 | INT |

    - Unit/Sub-Unit Status

Unit Status is only used for the equipment with multi-units, and the maximum number of units is 15. If equipment has no unit, all data should be set to zero and will be ignored. Types of Unit Status and the definitions are the same as those of Equipment Status. Each Unit statuses are independent, but Equipment status may be dependent on the statuses of units within the equipment. It is the sole responsibility of equipment to report correct Equipment Status based on the Status of its Units.

The same rule is also applied to Sub-Unit.

    - Equipment Status Reason Code

It is to classify the status based on reason code of equipment status. Reason Code is used to indicate that why the status is changed.

Reason Code is represented by 5 digit numbers that are consists of 3 different properties of code to distinguish.

Equipment has to report the current status after report the previous status when operator executes the function on touch panel which is “Recovery”, because operator should decide the reason code of status by manual if equipment reports the reason code of wrong status

| Property | Type | Category | Sub Item |
| --- | --- | --- | --- |
| Reason Code | XXXXX(Combination of each value , INT: 10000~29999) |  |  |
| Data Format | 1Digit (1 ~ 2) | 2 ~ 3 Digit (00~ 99) | 4 ~ 5 Digit (00~ 99) |
|  | By who | Category | Sub Item |
| Value Description | 1 : EQP 2 : Operator | Refer to the table In below | Refer to the table In below |
|  | * For example, Reason Code of 12001 indicates, HW Trouble happens with Sensor by EQP. |  |  |

- Common Reason Code is reserved that category : 00 ~ 85.

- Equipment Special Code is reserved that category : 86 ~ 95.

Equipment vender must use to Equipment Special Code if the reason code for vender special want to add as reason code.

Priority is related with Category that means priority must be decided order by category.

For example, when priority of EN Run is 2, if category of Dummy run is 01 and EN Run is 02, it is wrong because category code is different by order with priority.

| Status |  |  | Priority | Category |  |  |  | Sub Item |  |  |  | Format |  |  | How To Report | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | name | Code |  |  | name | Code |  |  |  |  |  |  |  |
|  |  |  |  |  | (2~3 digit) |  |  |  | (4~5 digit) |  |  |  |  |  |  |  |
| Run |  |  | 1 | Production Run |  | 00 |  | Production Run |  | 01 |  |  | INT |  | EQ Auto | MES Decision |
|  |  |  | 2 | EN Run |  | 01 |  | EN Run |  | 01 |  |  | INT |  |  |  |
|  |  |  | 3 | DV Run |  | 02 |  | DV Run |  | 01 |  |  | INT |  |  |  |
|  |  |  | 4 | Dummy Run |  | 03 |  | Dummy Run |  | 01 |  |  | INT |  | EQ Manual |  |
| Idle |  |  | 1 | No WIP | 10 |  |  | No WIP |  | 01 |  |  | INT |  | EQ Auto |  |
|  |  |  |  |  |  |  |  | No plan |  | 02 |  |  | INT |  | EQ Manual | Manual operation |
|  |  |  | 2 | No Material |  | 12 |  | No Material |  | 01 |  |  | INT |  |  |  |
|  |  |  | 3 | No CST |  | 13 |  | No CST |  | 01 |  |  | INT |  | EQ Auto | MES Decision |
|  |  |  | 4 | AMHS Down |  | 14 |  | AMHS Down |  | 01 |  |  | INT |  |  |  |
| Down |  |  | 1 | HW Failure | 20 |  |  | Equipment Hardware Error |  | 01 |  |  | INT |  |  | Manual operation is possible, and operator change is possible |
|  |  |  |  |  |  |  |  | Cannot recover trouble |  | 02 |  |  | INT |  |  |  |
|  |  |  |  |  |  |  |  | Robot Trouble |  | 03 |  |  | INT |  |  |  |
|  |  |  | 2 | SW Failure | 21 |  |  | Equipment Software Error |  | 01 |  | INT |  |  |  |  |
|  |  |  |  |  |  |  |  | Equipment Communication Error | 02 |  |  | INT |  |  |  |  |
|  |  |  |  |  |  |  |  | Glass or Panel information error | 03 |  |  | INT |  |  |  |  |
|  |  |  | 3 | Facilities Failure | 22 |  |  | Utility Error | 01 |  |  | INT |  |  |  |  |
|  |  |  | 4 | Quality Failure | 23 |  |  | Danger for human |  | 01 |  |  | INT |  |  |  |
|  |  |  |  |  |  |  |  | Glass or Panel Trouble |  | 02 |  |  | INT |  |  |  |
|  |  |  |  |  |  |  |  | Parameter overflow caused process error | 03 |  |  | INT |  |  |  |  |
|  |  |  |  |  |  |  |  | Parameter overflow caused equipment can‘t work | 04 |  |  | INT |  |  |  |  |
|  |  |  |  |  |  |  |  | Material Error | 05 |  |  | INT |  |  |  |  |
|  |  |  |  |  |  |  |  | Parameter configuration error | 06 |  |  | INT |  |  |  |  |
|  |  |  |  |  |  |  |  | other categories |  | 07 |  |  | INT |  |  |  |
|  | PM |  | 1 | PM | 30 |  |  | PM |  | 01 |  |  | INT |  | EQ Manual | Manual operation |
|  | J/C |  | 1 | Job Change |  | 40 |  | Job Change |  | 01 |  |  | INT |  |  |  |
| ETC |  |  | 1 | Material Change |  | 50 |  | Material Change |  | 01 |  |  | INT |  |  |  |
|  |  |  | 2 | Others | 51 |  |  | Set up |  | 01 |  |  | INT |  |  |  |
|  |  |  |  |  |  |  |  | Others |  | 02 |  |  | INT |  |  |  |

    - No Processing Time

Equipment should provide “No Processing Time” timer setting function on touch panel for each equipment.

If unit/equipment has sent out all Jobs (status of no Job), then equipment checks “No Processing Time” timer. If “No Processing Time” is over and there still is no Job, then equipment reports Idle.

(No Processing Time =0 : Don’t check no processing time) (Default : tact time *2)

Figure 5-1 No Process Time in case one unit in equipment

Figure 5-2 No Process Time in case upper two unit

    - PLC MAP

      - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Equipment Status Block | Equipment Status | 1 | Data | INT | Status - : PM - : Down(BM) - : Pause - : Idle 5: Run 6: Job Change |

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 7 : ETC |
|  | Equipment Reason Code | 1 | Data | INT | Refer to the Equipment Status Reason Code |
|  | Reserve | 4 | Data | INT |  |
|  | Unit Status#01 | 1 | Data | INT | Status 1 : PM 2 : Down(BM) 3 : Pause 4 : Idle 5: Run 6: Job Change 7 : ETC |
|  | Unit Reason Code#01 | 1 | Data | INT | Refer to the Equipment Status Reason Code |
|  | Unit Status#02 | 1 | Data | INT | Same as Unit Status#01 |
|  | Unit Reason Code#02 | 1 | Data | INT | Same as Unit Reason Code#01 |
|  | Unit Status#03 | 1 | Data | INT | Same as Unit Status#01 |
|  | Unit Reason Code#03 | 1 | Data | INT | Same as Unit Reason Code#01 |
|  | Unit Status#04 | 1 | Data | INT | Same as Unit Status#01 |
|  | Unit Reason Code#04 | 1 | Data | INT | Same as Unit Reason Code#01 |
|  | Unit Status#05 | 1 | Data | INT | Same as Unit Status#01 |
|  | Unit Reason Code#05 | 1 | Data | INT | Same as Unit Reason Code#01 |
|  | Unit Status#06 | 1 | Data | INT | Same as Unit Status#01 |
|  | Unit Reason Code#06 | 1 | Data | INT | Same as Unit Reason Code#01 |
|  | Unit Status#07 | 1 | Data | INT | Same as Unit Status#01 |
|  | Unit Reason Code#07 | 1 | Data | INT | Same as Unit Reason Code#01 |
|  | Unit Status#08 | 1 | Data | INT | Same as Unit Status#01 |
|  | Unit Reason Code#08 | 1 | Data | INT | Same as Unit Reason Code#01 |
|  | Unit Status#09 | 1 | Data | INT | Same as Unit Status#01 |
|  | Unit Reason Code#09 | 1 | Data | INT | Same as Unit Reason Code#01 |
|  | Unit Status#10 | 1 | Data | INT | Same as Unit Status#01 |
|  | Unit Reason Code#10 | 1 | Data | INT | Same as Unit Reason Code#01 |
|  | Unit Status#11 | 1 | Data | INT | Same as Unit Status#01 |
|  | Unit Reason Code#11 | 1 | Data | INT | Same as Unit Reason Code#01 |
|  | Unit Status#12 | 1 | Data | INT | Same as Unit Status#01 |
|  | Unit Reason Code#12 | 1 | Data | INT | Same as Unit Reason Code#01 |
|  | Unit Status#13 | 1 | Data | INT | Same as Unit Status#01 |
|  | Unit Reason Code#13 | 1 | Data | INT | Same as Unit Reason Code#01 |
|  | Unit Status#14 | 1 | Data | INT | Same as Unit Status#01 |
|  | Unit Reason Code#14 | 1 | Data | INT | Same as Unit Reason Code#01 |
|  | Unit Status#15 | 1 | Data | INT | Same as Unit Status#01 |
|  | Unit Reason Code#15 | 1 | Data | INT | Same as Unit Reason Code#01 |

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) Action |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Event | Equipment Status Change Report | 1 | Event | - On: Fire Event - Off: Event End | Equipment Status Block ※ EQ should report the event for each equipment status or unit status change | Reply for Receiving |

      - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Event Reply | Equipment Status Change Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |

  - Alarm Management

Operator should confirm all alarm when alarm is occurred even if several alarm .

* Max alarm buffering : 5 count

When reporting alarm occurrence, every Alarm Report (Set or Clear) should be reported in separate Link Area (5 report block) in ‘Round Robin’ manner.

Set and Clear state of each alarm should be reported in separate report.

Equipment should provide function to clear all alarms in equipment touch panel, and even this case each Alarm clear report should be reported separately.

When 5 report block is full, if a next alarm occurs then equipment has to keep by buffering alarms. When an alarm of within 5 block has been reported which turn off the event bit , the next alarm should be able to report with ‘Round Robin’ manner on the offed event block after get from buffering alarm.

Figure 5-3Alarm State Change Report

    - Alarm State

It is the state of the alarm that is reported.

- Alarm Set: An alarm is detected.

- Alarm Clear: An alarm is cleared.

    - Alarm Unit

When equipment reports an alarm event, it shows that which unit in the equipment the alarm is related to.

    - Alarm ID

All equipment makers must submit Alarm ID list per unit. A unit must have a unique Alarm ID regardless of error or warning alarm.

0: reserved (never used)

1 ~ 65500: Equipment can use

65501 ~ 65535: EAS Reserved

    - Alarm Code

Alarm has an associated severity code (ALCD) that is alarm type.

- 0: Danger for human

- 1: Equipment Hardware Error

- 2: Equipment Software Error

- 3: Equipment Communication Error

- 4: Glass or Panel information error

- 5: Glass or Panel Trouble

- 6: Material or Utility Error

- 7: Parameter overflow caused process error

- 8: Parameter overflow caused equipment can’t work

- 9: Parameter configuration error

- 10: Cannot recover trouble

- 11: Equipment status warning

- 12: Robot Trouble

- 13: EAS Caused

- 14: other categories

    - Alarm Level

It shows the level of the alarm. It consists of Warning and Error.

- Warning: Warning represents a light error. For warnings, the equipment should keep on processing, buzz operator, and display the message on the touch panel.

- Error: Alarm represents a serious error. When alarm is occurred, the equipment must stop the processing (For some equipments, it may cause only process pause), buzz operator to check the equipment, and display a message on the touch panel.

    - Alarm Text

It contains a brief textual description of an alarm.All the text character should in the scope of ASCII (H20-H7E, except H22,( “ ) and H27(‘)).

    - Error Recovery and Remove Job

When an alarm is occurred, or discrepancy between job existence and Job Data is found, and in consequence the data needs to be changed or removed, equipment should provide functions as below.

- Alarm to notify operator to correct it.

- Provide “job removal procedure” and “job position display” to recover the above discrepancy trouble or alarm situation.

Job removal procedure should contain at least three items.

- Physical glass/Panel removal

- Report removed Job Data to EAS

- Job deletion from equipment on touch panel

Detail error recovery scenario should be provided by equipment vender.

    - PLC MAP

      - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Alarm Block#1 | Alarm State | 1 | Data | INT | 0: Alarm Clear 1: Alarm Set |
|  | Alarm Unit | 1 | Data | INT | 0: Equipment Level Alarm 1~: Alarm Occurred Unit Number |
|  | Alarm ID | 1 | Data | INT | 1~65535 |
|  | Alarm Code | 1 | Data | INT | Refer to “5.4.4 Alarm Code” |
|  | Alarm Level | 1 | Data | INT | - : Warning - : Error |
|  | Alarm Text | 40 | Data | ASC |  |
| … |  |  |  |  |  |
| Alarm Block#n [n : 5] | Same as Alarm #1 Event Block | 45 |  |  |  |

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) Action |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Event | Alarm State Change Report#1 | 1 | Event | - On: Fire Event - Off: Event End | Alarm Block#1 | Reply for Receiving |
|  | … |  |  |  |  |  |
|  | Alarm State Change Report#n [n=5] | 1 | Event | - On: Fire Event - Off: Event End | Alarm Block#n | Reply for Receiving |

      - Master (EAS) – Link Register (W Area)

None

      - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Event Reply | Alarm State Change Report#1Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Alarm State Change Report #n Reply [n=5] | 1 | Reply | - On: Reply - Off: Reply End |  |  |

  - Job Management

The job means some processing works with a material such as glass, Panel. Each job has its own unique data for processing, which is called Job Data, including Master Recipe, cassette sequence number, job sequence number and the like. The Job Data is received from upstream equipment, as main equipment is ready to receive a job, and should be sent to downstream equipment as main equipment is ready to send out a job.

Equipment should keep and manage Job Data of each job even if the job is moved between internal zones. At the same time, equipment should provide Job Data information of each job on touch panel.

    - Job Data Transfer

If equipment sends a Job to other equipment, the equipment sending a Job is called Upstream Equipment. If equipment receives a Job from other equipment, the equipment receiving a Job is called Downstream Equipment. Equipment can have more than one Upstream Equipment and Downstream equipment. In order to distinguish which equipment is Upstream Equipment or Downstream Equipment for specific equipment, Upstream Path No. is used and managed.

When sending out a Job to a Downstream Equipment, Job Data is transferred to Downstream Equipment along with the Job, via MELSECHNET/G. For the Job Data transferring, equipment should write the data in the corresponding ‘Job Data Downstream Block’, according to downstream Equipment the Job is transferring to.

When Upstream Equipment is about to send a Job to Downstream Equipment, the Downstream Equipment first validates Job Data of the Job. In case no error is detected in the Job Data, Downstream Equipment starts Job receiving. The Job Data validation items are as below.

Glass Flow

Upstream Equipment #1

Job Data for Downstream 1 Block

Main Equipment

Job Data for Downstream 1 Block

Job Data for Downstream 2 Block

Downstream Equipment #1

Downstream Equipment #2

Figure 5-4 job Transfer

      - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Job Data for Downstream Block 1 | Job Data [Sent Out Job Event Downstream 1] | 96 or 256 | Data | - | Refer to 4.5 Job Data A or S Items or M(Only CF Mask Cleaner) Items |
| Job Data for Downstream Block 2 | Job Data [Sent Out Job Event Downstream 2] | 96 or 256 | Data | - | Refer to 4.5 Job Data A or S Items or M(Only CF Mask Cleaner) |
| .. | .. |  |  |  |  |
| Job Data for Downstream Block #N (N = 6) | Job Data [Sent Out Job Event Downstream N] | 96 or 256 | Data | - | Refer to 4.5 Job Data A or S Items or M(Only CF Mask Cleaner) |

    - Job Count

The Job Count is to report the count of the job in the entire equipment. According to the job type, equipment should distinguish whether the job is product job or dummy job. Normally, the product job means mass

product processing, but in the assembly line where there are two product jobs, TFT and CF, the product job means TFT glass processing.

Equipment should write job count before report job event to EAS.

- TFT(Array) Product Job Count: It is the current count of the TFT(Array) glasses is in the equipment.

- CF Product Job Count: It is the current count of the CF glasses is in the equipment.

- Dummy Job Count: It is the count of the dummy glasses is in the equipment.

- Total Available Count(Array Initial Cleaner Only) : The total amount of Glass Available for Unload.

- Total Glass Count(Array Initial Cleaner Only) :

L2 – Quantity of glass on conveyor out of wire cassette. L3 – The total amount of glass in the initial cleaner.

    - Job Event

The Job Data has to be reported to EAS at special time, such as received job from upstream equipment, sent out job to downstream equipment, fetched out job from a cassette or unit, stored job into a cassette or unit, and removed job from equipment.

Etch event in Details can refer operation scenario of equipment by type.

Figure 5-5 Job Data Event Report Point

Figure 5-6 Job Event Timing Chart

    - Job Event Report Timing Condition

- Sent Out Job Event

  - On the rise of Link Signal Job Transfer signal

  - Job should be completely transferred to downstream equipment.

- Received Job Event

  - After Receive Complete Signal turns off.

  - Job should be completely received by downstream equipment.

Between upstream and downstream equipment, report sequence of Sent Out Job Event from upstream equipment and Received Job Event from downstream equipment should follow physical Job transfer sequence. Exact report timing will be adjusted during and after test, if required.

    - Received Job Event

When equipment receives a job from upstream equipment, this event should be reported. The below items are included when equipment reports the Received Job event.

- Received Job: The Job Data of the job that is received from the upstream equipment.

- Upstream Path No: The Upstream Path Number of the upstream equipment

    - Sent Out Job Event

When the equipment sends out a job to downstream equipment, this event should be reported. The below items should be included when equipment reports the Sent Out Job event.

- Sent Out Job: The Job Data of the job that is sent to the downstream equipment.

- Downstream Path No: The Downstream Path Number of the downstream equipment

    - Stored Job Event

When the equipment stored a job to a unit or cassette, this event should be reported with the port number and unit number that is occurred by unit. If there are some slots, slot no should be reported.

- Stored Job: The Job Data of the job that is stored

- Unit or Port: it is used to distinguish the product stored in Unit from Port.

- Unit No: The Unit No of the Unit or Sub Unit. If the (Sub)Unit has some slots, equipment should update the Slot No.

- Sub Unit No : The level of the unit. 0 is Unit, 1~15 is Sub Unit No

- Port No: The port no of the cassette that the job is stored. Equipment should update the Slot No.

- Slot No: Slot No. If unit has no slot, this item should be cleared.

Stored Job Event should be reported in separate Link Area for each Unit/Port. Basic Link Area usage rule:

Indexer – first priority is Port , second priority is Unit

Example:

| Stored Job Event Report Area | Indexer of two Ports and one Units | Process Equipment with three Units |
| --- | --- | --- |
| Stored Job Report #1 | Port 1 | Unit 5 |
| Stored Job Report #2 | Port 3 | Unit 2 |
| Stored Job Report #3 | Unit 1 | Unit 4 |
| … |  |  |
| Stored Job Report #15 |  |  |

    - Fetched Out Job Event

When the equipment fetched out a job from a unit or cassette, this event should be reported with the port number and unit number that is occurred by unit. If there are some slots, slot no should be reported.

The Job Data of fetched job is reported while a job is being fetched out from a cassette or unit. In this case, the port number and unit number must be reported

- Fetched Out Job: The Job Data of the job that is fetched out

- Unit or Port: it is used to distinguish the product stored in Unit from Port.

- Unit No: The Unit No of the Unit or Sub Unit. If the (Sub)Unit has some slots, equipment should update the Slot No.

- Sub Unit No : The level of the unit. 0 is Unit, 1~ is Sub Unit No

- Port No: The port no of the cassette that the job is fetched. Equipment should update the Slot No.

- Slot No: Slot No. If unit has no slot, this item should be cleared.

Fetched Out Job Event should be reported in separate Link Area for each Unit/Port. Basic Link Area usage rule:

Indexer – first priority is Port , second priority is Unit Example :

| Fetched Out Job Event Report Area | Indexer of two Ports and one Units | Process Equipment with three Units |
| --- | --- | --- |
| Fetched Out Job Report #1 | Port 1 | Unit 5 |
| Fetched Out Job Report #2 | Port 3 | Unit 2 |
| Fetched Out Job Report #3 | Unit 1 | Unit 4 |
| … |  |  |
| Fetched Out Job Report #15 |  |  |

    - Removed Job Event

The Job Data of removed job is reported when a job has just been removed from equipment.

- Removed Job: The Job Data of the job that is removed by operator.

- Unit or Port: it is used to distinguish the product stored in Unit from Port.

- Unit No: The Unit No of the unit. If the unit has some slots, equipment should update the Slot No.

- Sub Unit No : The level of the unit. 0 is Unit, 1~ is Sub Unit No

- Port No: The port no of the cassette that the job is removed. Equipment should update the Slot No.

- Slot No: Slot No. If unit has no slot, this item should be cleared.

- Removed Flag : Removed type. And this is updated on touch panel

- Remove Reason Code : Removed job was happened why and this is updated on touch panel

    - Removed Job Recovery Function

Equipment should provide removed job data keeping function that stores at least the latest 20 job data removed. And function to support operator can select which Job to recovery.

Takeout Job can recovery before end of Lot processing. If Operator take out the glass after end of Lot processing, it must be took manual out tracking in Host.

    - Process Start Report

Principal unit of equipment should report when processing of a Job is started. The report timing should be comply with the ‘Process Start Time’ in Process Data Report.

ESWIN engineer will decide which Unit is required to report Process Start.

    - Process End Report

Principal unit of equipment reports the Process End Event. The report timing should be comply with the ‘Process End Time’ in Process Data Report.

ESWIN engineer will decide whether or not an important unit.

    - PLC MAP

      - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Job Count Block | TFT(Array) Product Job Count | 1 | Data | INT |  |
|  | CF Product Job Count | 1 | Data | INT |  |
|  | Dummy Job Count | 1 | Data | INT |  |
|  | Total Available Count | 1 | Data | INT | Array Initial Cleaner Only |
|  | Total Glass Count | 1 | Data | INT | Array Initial Cleaner Only |
| Received Job Data Block | Job Data for Upstream Block 1~6 | 96 or 256 | Data | - | Refer to Job Data A or S |
| Received Job Data Sub Block 1~6 | Upstream Path No | 1 | Data | INT | 1~6 |
| Sent Out Job Data Block | Job Data For Downstream Block 1~6 | 96 or 256 | Data | - | Refer to Job Data A or S |
| Sent Out Job Data Sub Block 1~6 | Downstream Path No | 1 | Data | INT | 1~6 |
| Stored Job Data | Stored Job Data | 14 | Data | - | Refer to Job Data B |

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Block #1 |  |  |  |  |  |
| Stored Job Data Sub Block #1 | Unit or Port | 1 | Data | INT | 1: Unit 2: Port |
|  | Unit No or Port No | 1 | Data | INT | 0: No Unit Event 1~: Unit No or Port No |
|  | Sub Unit No | 1 | Data | INT | 0 : Unit |
|  | Slot No | 1 | Data | INT | 0: No Slot 1~ : Slot No of Cassette/Unit |
| Stored Job Data Block #N (N: 2~15) | Stored Job Data | 14 | Data | - | Refer to Job Data B |
| Stored Job Data Sub Block #N(N: 2~15) | Unit or Port | 1 | Data | INT | 1: Unit 2: Port |
|  | Unit No or Port No | 1 | Data | INT | 0: No Unit Event |
|  | Sub Unit No | 1 | Data | INT | 0 : Unit |
|  | Slot No | 1 | Data | INT | 0: No Slot 1~ : Slot No of Cassette/Unit |
| Fetched Out Job Data Block #1 | Job Data B | 14 | Data | - | Refer to Job Data B |
| Fetched Out Job Data Sub Block#1 | Unit or Port | 1 | Data | INT | 1: Unit 2: Port |
|  | Unit No or Port No | 1 | Data | INT | 0: No Unit Event 1~: Unit No or Port No |
|  | Sub Unit No | 1 | Data | INT | 0 : Unit |
|  | Slot No | 1 | Data | INT | 0: No Slot 1~ : Slot No of Cassette/Unit |
| Fetched Out Job Data Block #N(N: 2~15) | Job Data B | 14 | Data | - | Refer to Job Data B |
| Fetched Out Job Data Sub Block #N (N: 2~15) | Unit or Port | 1 | Data | INT | 1: Unit 2: Port |
|  | Unit No or Port No | 1 | Data | INT | 0: No Unit Event 1~ : Unit No or Port No |
|  | Sub Unit No | 1 | Data | INT | 0 : Unit |
|  | Slot No | 1 | Data | INT | 0: No Slot 1~ : Slot No of Cassette/Unit |
| Removed Job Data Block | Removed Job Data | 96 or 256 | Data | - | Refer to Job Data A or C (glass code, glass ID) |
| Removed Job Data Sub Block | Unit or Port | 1 | Data | INT | 0: Others 1: Unit 2: Port |
|  | Unit No or Port No | 1 | Data | INT | 0: No Unit Event 1~: Unit No or Port No |

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
|  | Sub Unit No | 1 | Data | INT | 0 : Unit |
|  | Slot No | 1 | Data | INT | 0: No Slot 1~ : Slot No of Cassette/Unit |
|  | Removed Flag | 1 | Data | INT | 1: Scrap (broken) 2: Take out 3~: Reserved. |
| Job Process Start Report Block | Cassette Sequence No | 1 | Data | INT | CST Sequence Number (1~65535) |
|  | Job Sequence No | 1 | Data | INT | Job Sequence Number (1~65535) |
|  | Unit No | 1 | Data | INT | 0: No Unit Event 1~: Unit No |
|  | Sub Unit No | 1 | Data | INT | 0 : Unit |
|  | Slot No | 1 | Data | INT | 0: No Slot 1~ : Slot No of Cassette/Unit |
|  | Process Start Time | 3 | Data | BCD | 00 ~ 07 Bit :A.D Year(00~99) 08 ~ 15 Bit :Month(01~12) 00 ~ 07 Bit :Day(01~31) 08 ~ 15 Bit :Hour(00~23) 00 ~ 07 Bit :Minute(00~59) 08 ~ 15 Bit :Second(00~59) |
| Job Process End Report Block | Cassette Sequence No | 1 | Data | INT | CST Sequence Number (1~65535) |
|  | Job Sequence No | 1 | Data | INT | Job Sequence Number (1~65535) |
|  | Unit No | 1 | Data | INT | 0: No Unit Event 1~: Unit No |
|  | Sub Unit No | 1 | Data | INT | 0 : Unit |
|  | Slot No | 1 | Data | INT | 0: No Slot 1~ : Slot No of Cassette/Unit |
|  | Process End Time | 3 | Data | BCD | 00 ~ 07 Bit :A.D Year(00~99) 08 ~ 15 Bit :Month(01~12) 00 ~ 07 Bit :Day(01~31) 08 ~ 15 Bit :Hour(00~23) 00 ~ 07 Bit :Minute(00~59) 08 ~ 15 Bit :Second(00~59) |

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) Action |
| --- | --- | --- | --- | --- | --- | --- |
| Job Event | Received Job Report Upstream #1 | 1 | Event | - On: Fire Event - Off: Event End | Received Job Data Block Received Job Data Sub Block Job Count Block | Reply for Receiving |
|  | Received Job Report Upstream #N | 1 | Event | - On: Fire Event - Off: Event End | Received Job Data Block Received Job Data Sub Block Job Count Block | Reply for Receiving |
|  | Sent Out Job Report Downstream #1 | 1 | Event | - On: Fire Event - Off: Event End | Sent Out Job Data Block Sent Out Job Data Sub Block Job Count Block | Reply for Receiving |
|  | Sent Out Job Report Downstream #N | 1 | Event | - On: Fire Event - Off: Event End | Sent Out Job Data Block Sent Out Job Data Sub Block Job Count Block | Reply for Receiving |

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) Action |
| --- | --- | --- | --- | --- | --- | --- |
|  | Stored Job Report #1 | 1 | Event | - On: Fire Event - Off: Event End | Stored Job Block#1 Stored Job Data Sub Block#1 Job Count Block | Reply for Receiving |
|  | Stored Job Report #n (2~15) | 1 | Event | - On: Fire Event - Off: Event End | Stored Job Block#N Stored Job Data Sub Block#N Job Count Block | Reply for Receiving |
|  | Fetched Out Job Report #1 | 1 | Event | - On: Fire Event - Off: Event End | Fetched Out Job Data Block#1 Fetched Out Job Data Sub Block#1 Job Count Block | Reply for Receiving |
|  | Fetched Out Job Report #N(2~15) | 1 | Event | - On: Fire Event - Off: Event End | Fetched Out Job Data Block#N Fetched Out Job Data Sub Block#N Job Count Block | Reply for Receiving |
|  | Removed Job Report | 1 | Event | - On: Fire Event - Off: Event End | Removed Job Data Block Removed Job Data Sub Block Job Count Block | Reply for Receiving |
|  | Job Process Start Report | 1 | Event | - On: Fire Event - Off: Event End | Job Process Start Block | Reply for Receiving |
|  | Job Process End Report | 1 | Event | - On: Fire Event - Off: Event End | Job Process End Block | Reply for Receiving |

      - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| Job Event Reply | Received Job Report Reply Upstream #1 | 1 | Event | - On: Fire Event - Off: Event End |  |  |
|  | Received Job Report Reply Upstream #N ( 1~ 6) | 1 | Event | - On: Fire Event - Off: Event End |  |  |
|  | Sent Out Job Report Reply Downstream #1 | 1 | Event | - On: Fire Event - Off: Event End |  |  |
|  | Sent Out Job Report Reply Downstream #N (1~6) | 1 | Event | - On: Fire Event - Off: Event End |  |  |
|  | Stored Job Report Reply #1 | 1 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Stored Job Report Reply #N (1~6) | 1 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Fetched Out Job Report Reply #1 | 1 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Fetched Out Job Report Reply #N (1~6) | 1 | Reply | - On: Reply - Off: Reply End |  |  |

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
|  | Removed Job Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Job Process Start Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Job Process End Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |

  - Process Data Management

This function is only for process equipment, which has process data such as processed parameter values or inspection result. The data should be related to individual jobs, and reported with glass/Panel ID.

The process data item should be determined by ESWIN engineer.

- Job Data: The Job Data should be same as Job Data of Sent out Job

- Processing Time: It is actual processing time for a job in the equipment.

- Kind or Process Data: There are 2 kinds of process data. By Glass, By Time.

    - Event Report Timing

- By Glass: The Process Data(DV) of each Glass/Panel should be reported after job sent to downstream equipment, if equipment has process data. One local equipment should report process data one time per glass to EAS.

- By Time (Trace Data::SV): Equipment write current Process Data into the word area by specified time cycle. Equipment provide the function for the cycle time setting. (minimum value = 500ms)

    - DV and SV Naming Rule(recommend)

- Each equipment vendor should summit its SV and DV name with the equipment data file(EAS-equipment Data Collection.xlsx) following the standard naming rule

| Order of digit | Item |
| --- | --- |
| 1 | Layer – 1 : Equipment, 2: unit, 3: sub unit |
| 2 | Unit no. (00: if no Unit) |

Example) 1_00_MGNT-Speed : Equipment-level data

2_03_MGNT-Speed : Unit-level data, collected from Unit No. 3

    - PLC MAP: By Glass(DV)

This data should report after Sent Out Report.

      - Local PLC – Link Register (W or R Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
|  | GlassID or PanelID or MaterialID | 10 | Data | ASC |  |
|  | Process Start Time | 3 | Data | BCD | 00 ~ 07 Bit :A.D Year(00~99) |
| Process Data Block |  |  |  |  | 08 ~ 15 Bit :Month(01~12) |
|  | Process End Time | 3 | Data | BCD | 00 ~ 07 Bit :Day(01~31) |
|  |  |  |  |  | 08 ~ 15 Bit :Hour(00~23) |

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 00 ~ 07 Bit :Minute(00~59) |
|  |  |  |  |  | 08 ~ 15 Bit :Second(00~59) |
|  |  |  |  |  | Process Data Items |
|  |  |  |  |  | Default = 512 word |
|  | Process Data Items | 512 | Data | - |  |
|  |  |  |  |  | Each equipment vendor need to discuss with |
|  |  |  |  |  | customer for the size. |

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) Action |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Event | Process Data Report | 1 | Event | - On: Fire Event - Off: Event End/Timeout | Process Data Block | Reply for Receiving |

      - Master (EAS) – Link Register (W Area)

None

      - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Event Reply Block | Process Data Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |

    - PLC MAP: By Glass(DV) – CF Only

This data should report after Sent Out Report.

The data of DV Item List should request or reply with group by other because there is limit to memory size for gathering a lot of DV Item List.

Equipment should report the data with Total DV Item List Count ,Total Group Count , Current Group Count that the data is continually transferred until all data is transferred.

The condition of complete the transfer of all data is when Total Group Count and Current Group Count is same.

| Total DV Item List Count | It is total count of DV Item Lists |
| --- | --- |
| Total Group Count | It is total count of group |
| Current Group Count | It is current group count by event |

For example, if Total Group Count is 3 when Process Data Report of first is happened, then Current Group Count is 1. Equipment writes next data after EAS reads its memory map .

and when Process Data Report of next is happened, Current Group Count is 2. It will report the DV Item List until Current Group Count is 3. Equipment terminate the transition of data if Total Group Count is same with Current Group Count and then it doesn’t report next event.

      - Local PLC – Link Register (W or R Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
|  | GlassID or PanelID or MaterialID | 10 | Data | ASC |  |
|  | Process Start Time | 3 | Data | BCD | 00 ~ 07 Bit :A.D Year(00~99) |
|  |  |  |  |  | 08 ~ 15 Bit :Month(01~12) |
|  |  |  |  |  | 00 ~ 07 Bit :Day(01~31) |
|  | Process End Time | 3 | Data | BCD | 08 ~ 15 Bit :Hour(00~23) |
| Process Data Block |  |  |  |  | 00 ~ 07 Bit :Minute(00~59) 08 ~ 15 Bit :Second(00~59) |
|  |  |  |  |  | Process Data Items |
|  |  |  |  |  | Default = 512 word |
|  | Process Data Items | 512 | Data | - |  |
|  |  |  |  |  | Each equipment vendor need to discuss with customer for the size. |

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Process Data Sub Block | Total DV Item List Count | 1 | Data | INT |  |
|  | Total Group Count | 1 | Data | INT |  |
|  | Current Group Count | 1 | Data | INT |  |

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) Action |
| --- | --- | --- | --- | --- | --- | --- |
| Process Data Report Event | Process Data Report#1~#26 | 1 | Event | - On: Fire Event - Off: Event End/Timeout | - Process Data Block - Process Data Sub Block | Reply for Receiving |

      - Master (EAS) – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Process Data Report Reply Block | Return Code | 1 | Data | INT | - : OK - : NG - : continue - : DV Item List Count NG - : Group Count NG 6: Duplication Group Count NG 7 : Order by Group Count NG |

      - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Event Reply Block | Process Data Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |

      - Process Data report Sequence.

    - PLC MAP: By Time (SV)

      - Local PLC – Link Register (W or R Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 00 ~ 07 Bit :A.D Year(00~99) |
|  |  |  |  |  | 08 ~ 15 Bit :Month(01~12) |
|  | SV Time Record | 3 | Data | BCD | 00 ~ 07 Bit :Day(01~31) |
|  |  |  |  |  | 08 ~ 15 Bit :Hour(00~23) |
| Trace Data Block |  |  |  |  | 00 ~ 07 Bit :Minute(00~59) 08 ~ 15 Bit :Second(00~59) |
|  |  |  |  |  | Trace Data Items |
|  | SV Data | 640 | Data | - | Default = 640 word |
|  |  |  |  |  | Each equipment vendor need to discuss with customer for the size. |

    - Tact-Time DV Data

Equipment has to write all signal of action for tact time when glass in or out between Unit, Equipment. And When glass in or out between Robot and PLC, Transfer Robot has to write all signal of action.

When the event of Process Data Report arises from Equipment, the Tact-Time DV Data should report to Host.

It will be define by ESWIN after receives DV List by type of Equipment. The Data Format is Float type that complies with IEEE 754

For example,

| Tact-time Data | description |
| --- | --- |
| 1 | One second |
| 0.1 | 100 millisecond |
| 0.01 | 10 millisecond |

    - FAC Data Report

Equipment should report the facility data with FAC Data Report that is methodof event driven.

And also, the event has to raise by cycle of event which is set by operator on touch panel of equipment. The cycle of event is five second.

Equipment must have to write on one block of memory that is FAC Data Block#0, if the data has to be wrote by every five second, but if the data has to be wrote by every one second, equipment must have to write on etch block of memory that is from FAC Data Block#1 to FAC Data Block#5, that mean’s the etch block should have an area which is happened by every one second.

So, the five block from #1 to #5 are data to gathering during five second.

The FAC Data have to discuss by engineer of ESWIN with Vender of equipment how to use the blocks.

    - PLC MAP: By Time (SV)

      - Local PLC – Link Register (W or R Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 00 ~ 07 Bit :A.D Year(00~99) |
|  |  |  |  |  | 08 ~ 15 Bit :Month(01~12) |
|  | SV Time Record | 3 | Data | BCD | 00 ~ 07 Bit :Day(01~31) 08 ~ 15 Bit :Hour(00~23) |
|  |  |  |  |  | 00 ~ 07 Bit :Minute(00~59) |
|  |  |  |  |  | 08 ~ 15 Bit :Second(00~59) |
|  |  |  |  |  | Trace Data Items |
| FAC Data Block#0 |  |  |  |  | Default = 100 word Each equipment vendor need to discuss with customer for the size. |
|  | SV Data | 125 | Data | - | Here must be write by every five second. |
|  |  |  |  |  | *For example(ItemName=electricity) |
|  |  |  |  |  | 1.Total electricity usage(1s+2s+3s+4s+5s) |
|  |  |  |  |  | And |
|  |  |  |  |  | 2.electricity usage(5s) |
|  |  |  |  |  | 00 ~ 07 Bit :A.D Year(00~99) |
|  |  |  |  |  | 08 ~ 15 Bit :Month(01~12) |
|  | SV Time Record | 3 | Data | BCD | 00 ~ 07 Bit :Day(01~31) |
|  |  |  |  |  | 08 ~ 15 Bit :Hour(00~23) |
|  |  |  |  |  | 00 ~ 07 Bit :Minute(00~59) |
|  |  |  |  |  | 08 ~ 15 Bit :Second(00~59) |
| FAC Data Block#1 |  |  |  |  | Trace Data Items Default = 100 word |
|  |  |  |  |  | Each equipment vendor need to discuss with customer for the size. |
|  | SV Data | 100 | Data | - |  |
|  |  |  |  |  | Here must be write by every 1 second. |
|  |  |  |  |  | 1 second electricity usage or |
|  |  |  |  |  | 1 second water usage or |
|  |  |  |  |  | 1 second gas usage. |

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 00 ~ 07 Bit :A.D Year(00~99) |
|  |  |  |  |  | 08 ~ 15 Bit :Month(01~12) |
|  | SV Time Record | 3 | Data | BCD | 00 ~ 07 Bit :Day(01~31) |
|  |  |  |  |  | 08 ~ 15 Bit :Hour(00~23) |
|  |  |  |  |  | 00 ~ 07 Bit :Minute(00~59) |
|  |  |  |  |  | 08 ~ 15 Bit :Second(00~59) |
| FAC Data Block#2 |  |  |  |  | Trace Data Items Default = 100 word |
|  |  |  |  |  | Each equipment vendor need to discuss with customer for the size. |
|  | SV Data | 100 | Data | - |  |
|  |  |  |  |  | Here must be write by every 1 second. |
|  |  |  |  |  | 1 second electricity usage or |
|  |  |  |  |  | 1 second water usage or |
|  |  |  |  |  | 1 second gas usage.. |
|  |  |  |  |  | 00 ~ 07 Bit :A.D Year(00~99) |
|  |  |  |  |  | 08 ~ 15 Bit :Month(01~12) |
|  | SV Time Record | 3 | Data | BCD | 00 ~ 07 Bit :Day(01~31) |
|  |  |  |  |  | 08 ~ 15 Bit :Hour(00~23) |
|  |  |  |  |  | 00 ~ 07 Bit :Minute(00~59) |
|  |  |  |  |  | 08 ~ 15 Bit :Second(00~59) |
| FAC Data Block#3 |  |  |  |  | Trace Data Items Default = 100 word |
|  |  |  |  |  | Each equipment vendor need to discuss with customer for the size. |
|  | SV Data | 100 | Data | - |  |
|  |  |  |  |  | Here must be write by every 1 second. |
|  |  |  |  |  | 1 second electricity usage or |
|  |  |  |  |  | 1 second water usage or |
|  |  |  |  |  | 1 second gas usage.. |
|  |  |  |  |  | 00 ~ 07 Bit :A.D Year(00~99) |
|  |  |  |  |  | 08 ~ 15 Bit :Month(01~12) |
|  | SV Time Record | 3 | Data | BCD | 00 ~ 07 Bit :Day(01~31) |
|  |  |  |  |  | 08 ~ 15 Bit :Hour(00~23) |
|  |  |  |  |  | 00 ~ 07 Bit :Minute(00~59) |
|  |  |  |  |  | 08 ~ 15 Bit :Second(00~59) |
| FAC Data Block#4 |  |  |  |  | Trace Data Items Default = 100 word |
|  |  |  |  |  | Each equipment vendor need to discuss with customer for the size. |
|  | SV Data | 100 | Data | - |  |
|  |  |  |  |  | Here must be write by every 1 second. |
|  |  |  |  |  | 1 second electricity usage or |
|  |  |  |  |  | 1 second water usage or |
|  |  |  |  |  | 1 second gas usage. |

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 00 ~ 07 Bit :A.D Year(00~99) |
|  |  |  |  |  | 08 ~ 15 Bit :Month(01~12) |
|  | SV Time Record | 3 | Data | BCD | 00 ~ 07 Bit :Day(01~31) |
|  |  |  |  |  | 08 ~ 15 Bit :Hour(00~23) |
|  |  |  |  |  | 00 ~ 07 Bit :Minute(00~59) |
|  |  |  |  |  | 08 ~ 15 Bit :Second(00~59) |
| FAC Data Block#5 |  |  |  |  | Trace Data Items Default = 100 word |
|  |  |  |  |  | Each equipment vendor need to discuss with customer for the size. |
|  | SV Data | 100 | Data | - |  |
|  |  |  |  |  | Here must be write by every 1 second. |
|  |  |  |  |  | 1 second electricity usage or |
|  |  |  |  |  | 1 second water usage or |
|  |  |  |  |  | 1 second gas usage.. |

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) Action |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Event | FAC Data Report | 1 | Event | - On: Fire Event - Off: Event End/Timeout | FAC Data Block#0 … FAC Data Block#5 | Reply for Receiving |

      - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Event Reply Block | FAC Data Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |

  - Job Information Management

The EAS provide Job information download function.

    - Job Data Request

If equipment has lost Job Data or needs to update from Host, equipment can request Job Data to EAS. This Job Data could be available for Glass, Panel (after cut), or a type of material that is specifically managed, such as Mask.

Upon the Job Data request from equipment, EAS will download the latest updated data for the Job.

Where required, equipment should provide function for operator to enter a Job ID or select a specific Job when requesting Job Data to EAS.

Regardless of this function, all equipment should also provide function for manual entering and revision of data by operator for a specific glass being held in the equipment.

In Case of E.Cell except for Cutting Line, EAS send Job Data without Master Recipe for Issue case between Panel Size and Recipe.

    - Job Data Change

When glass data is changed by workers from the device, it needs to be reported to EAS Equipment vender provide the data input screen in touch panel

    - PLC Map

      - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 1: Glass ID |
|  |  |  |  |  | 2: Material ID(Mask) |
|  | Job(Glass or Material) Data Request Option | 1 | Data | INT | 3: Glass CST Seq /job Seq No |
|  |  |  |  |  | 4: Reserved |
| Job Data |  |  |  |  | 5: Glass ID for VCR Mismatch |
| Request Block | Request Job (Glass or Material) ID | 10 | Data | ASC | Job ID (Glass or Material) |
|  | Request Job CST Sequence Number | 1 | Data | INT | CST Sequence Number (1~65535) |
|  | Request Job Job Sequence Number | 1 | Data | INT | Job Sequence Number (1~65535) |
| Job Data Request Sub Block | Reserved | 20 | Data | - | It is used for line special if the special items must be not changed in job data. It will be decided by ESWIN Engineer. |
| Job Data Change Block | Changed Job Data | 96 or 23 or 256 | Data | - | Refer to Job Data A or C or S |

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) Action |
| --- | --- | --- | --- | --- | --- | --- |
| Job Event | Job (Glass or Material) DataRequest | 1 | Event | - On: Fire Event - Off: Event End | - Job Data Request Block - Job Data Request Sub Block |  |
|  | Job Data ChangeReport | 1 | Event | - On: Fire Event - Off: Event End | Job Data Change Block |  |

      - Master (EAS) – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Job Data Request Reply Block | Job Data Send | 96 or 23 or 256 | Data | - | Refer to Job Data A or C or S **E.Cell(not Cutting) : Not including Master Recipe |
|  | Job Data Request Ack | 1 | Data | INT | 1: Job Data Exist 2: Job Data no Exist |

      - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| Job Event Reply | Job Data Request reply | 1 | Reply | - On: Reply - Off: Reply End | Job Data Request Reply Block |  |
|  | Job Data Change Report reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |

  - Recipe Management

EAS download job data in Cassette Map to Loader/Un-loader/Buffer at Indexer. This job data include Master Recipe (PPID). When job is sent to downstream equipment, Maser Recipe is also sent to downstream equipment as in Job Data. For other equipment, it is received from upstream equipment and should be sent to downstream equipment without any modification.

0)	Master Recipe (PPID) Sample Format: Refer to Equipment Operation Scenario. Master Recipe (PPID) = ABDC123.. or 999

* Master Recipe (PPID) represent a set of Local Recipe IDs of each Equipment, which are to be used for processing according to a specific Host Recipe instruction..

    - Auto Recipe Change Function

All equipment should provide auto recipe change function. Equipment should provide auto recipe change enable / disable change function on equipment touch panel.

Enable: Equipment should change Recipe ID by automatic. Disable: Equipment should change Recipe ID by manual.

    - Current Recipe ID

Current recipe is the equipment setting Recipe ID that currently being used.

When current recipe is changed, equipment should report to EAS with corresponding Recipe ID. Current Recipe change at equipment should be allowed only when there is no Glass in equipment. Equipment should not allow to change Current Recipe that is not found in Recipe Register Table in the equipment.

    - Equipment Recipe Validation

When cassette is loaded on port , on CIM On status , EAS has to request Master Recipe (PPID) to MES and validate with the Master Recipe (PPID) in EAS , if current Master Recipe (PPID) not exist then EAS send the OP Call to EQ

if the registration of master recipe is OK then EAS download to Indexer the Cassette Map for processing but if master recipe is not exist in equipment then EAS doesn’t have to send the Cassette Map to indexer.

Master Recipe (PPID) is transferred from Loader to downstream equipment, and the equipment should validate Master Recipe (PPID) and corresponding Recipe ID before receiving Glass from upstream equipment

Figure 5-7 Equipment Master Recipe ID

- Master Recipe (PPID) transferring

  - EAS downloads “Master Recipe (PPID)” for each Job to Indexer when downloading Cassette information

  - Indexer sends Job data including Master Recipe (PPID) to next Equipments

Figure 5-8 Master Recipe Structure

Recipe table has to synchronize between EAS & each Equipments

- Local Recipe setting

  - EAS should provide function for setting Master-local recipe table, and should maintain the recipe table.

  - Equipment should provide function for setting Master-local recipe table, and should maintain the recipe table.

  - When receiving Job from upstream, Equipment gets Master Recipe ID from Job Data, and look into Master-Local Recipe table to identify the required Local Recipe to use to process the Job.

- Local Recipe setting

  - If Master Recipe (PPID) received from upstream equipment is match with Current Recipe ID then equipment can receive glass from upstream.

  - If Master Recipe (PPID) received from upstream equipment is not match with Current Recipe ID but found in Recipe Register Table then equipment should change recipe by automatic and receive glass from upstream equipment.

  - If Master Recipe (PPID) received from upstream equipment is not match with Current Recipe ID and not found in Recipe Register Table then equipment should raise an alarm and don’t receive glass.

After Recipe Register by operator in equipment then equipment should check Master Recipe (PPID) one more time and change recipe by automatic. If Master Recipe (PPID) received from upstream equipment is match with Current Recipe ID then receive glass from upstream equipment.

- Auto Recipe Change Disable Case

  - If PPID received from upstream Equipment matches with Current Recipe ID then Equipment can receive glass from upstream.

  - If PPID received from upstream Equipment does not match with Current Recipe ID then Equipment should generate an alarm and don’t receive glass.

After Current Recipe ID change by operator, Equipment should check PPID one more time. If PPID received from upstream Equipment matches with Current Recipe ID then receive glass from upstream Equipment.

    - Master Recipe List (auto change only)

Equipment should send to EAS the master recipe list for validate of register or Viewing of EAS

Equipment reports the list of all Equipment Recipe (Master Recipe) registered at the equipment according to request from EAS.

Equipment reports the list of all registered Equipment Recipe also when a change has been made to the list (addition/deletion revision of Equipment Recipe, etc.).

Equipment always should be set registered Equipment Recipe Number with index number.(See below) When Recipe ID is String type, it has to use the data structure in below which is ASC 10 word.

※ Master Recipe List Table

| Index number (INT) | Master Recipe (ASC 10 Word or INT 1 Word) |
| --- | --- |
| 1 | Master Recipe ID |
| 2 | … |
| 3 | … |
| .. | … |
| N | Master Recipe ID |

For example)

If the PPID is empty then the index number and PPID doesn’t report to EAS.

| Index number (INT) | Master Recipe (ASC 10 Word or INT 1 Word) |
| --- | --- |
| 1 | 3 |
| 2 | 2 |
| 3 | 5 |
| 4 |  |
| 5 | 1 |

| 6 | 8 |
| --- | --- |
| 7 |  |
| 8 | 4 |

Here is the map that is Master Recipe List.

|  | Index number | Master recipe | Index number | Master recipe | Index number | Master recipe | Index number | Master recipe | Index number | Master recipe | Index number | Master recipe | Index number | Master recipe | Index number | Master recipe |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| W | +0 | +1 | +2 | +3 | +4 | +5 | +6 | +7 | +8 | +9 | +A | +B | +C | +D | +E | +F |
| Master Recipe List | 1 | 3 | 2 | 2 | 3 | 5 | 5 | 1 | 6 | 8 | 8 | 4 |  |  |  |  |

    - Master Recipe List Report

Equipmentshould report to EAS when Master Recipe ID is changed, such like Create, Delete, Modify or when equipment receives a request of master recipe list from EAS that is Master Recipe List Request Command.

EAS should send the result of validation to Equipment with Return Code in below.

| Master Recipe List Return Code Block | Description |
| --- | --- |
| 1 | OK : Recipe List Received Result is all ok |
| 2 | Continue : Equipment still has to send the master recipe list to EAS because EAS is not yet received all master recipe list. |
| 3 | Master Recipe list Count NG : Total Group Count is same as Current Group Count but it’s different with Total Master Recipe List Count. |
| 4 | Group Count NG : Total Group Count is different with Current Group Count but it’s same with Total Master Recipe Count. |
| 5 | Duplication Group Count NG : Current Group Count is duplicate with previous group count. Equipment has to send the next group count to EAS. |
| 6 | Order by Group Count NG : Group Count is not sent order by next group count to EAS. Group Count has to send by order the next group count |

Figure 5-9 Mater Recipe List Change by Equipment

Figure 5-10 Master Recipe List Request Command

    - Current Recipe Change Report

Equipment should report with Current Recipe ID to EAS when the recipe is changed by auto or manual. In this time , Recipe parameters are able to validate by RMS

    - Recipe Key Parameter Validation

Equipment has to report the key parameters to EAS when it’s reached in condition by validation type. The key parameters should be determined by ESWIN engineer.

      - Recipe Version Check

When auto recipe is changed and before Cassette is loaded , if equipment missed a report of its change or if operator changes to wrong recipe then can be problem. To prevent changing a wrong recipe, Equipment send a version to EAS when Cassette is loaded. RMS should check the version before Process Start.

Any recipe parameter changed, the recipe version number should be updated. The recipe version number format☹e.g.: 20181127132826)

      - Key-Parameter Synchronization

The data of parameters should request or reply with group by other because there is limit to memory size for gathering a lot of parameter.

Equipment should report the data with Total Parameter Count ,Total Group Count , Current Group Count that the data is continually transferred until all data is transferred.

The condition of complete the transfer of all data is when Total Group Count and Current Group Count is same.

| Total Parameter Count | It is total count of parameters |
| --- | --- |
| Total Group Count | It is total count of group |
| Current Group Count | It is current group count by event |

For example, if Total Group Count is 3 when Recipe Parameter Report of first is happened, then Current Group Count is 1. Equipment writes next data after EAS reads its memory map .

and when Recipe Parameter Report of next is happened, Current Group Count is 2. It will report the key- parameters until Current Group Count is 3. Equipment terminate the transition of data if Total Group Count is same with Current Group Count and then it doesn’t report next event.

| Report Type | Description |
| --- | --- |
| 1 | Create |
| 2 | Modify |
| 3 | Delete |
| 4 | Request from EAS |

Equipment should send Report Type 1 ~ 3 to EAS when key-parameters are changed.

Equipment should send Report Type 4 to EAS when receives a request from EAS that’s Recipe Key- Parameters.

      - Recipe Parameter Report

Equipment should report to the EAS when the recipe parameter is changed, such like Create, Delete , Modify or when equipment receives a request of recipe parameter from Host.

Engineer should manage the parameters in equipment and RMS when change recipe parameter. The recipe parameter has to upload to RMS when recipe parameter of equipment is changed.

EAS should send the result of validation to Equipment with Return Code in below.

| Recipe Parameter Report Return Code | Description |
| --- | --- |
| 1 | OK : Recipe validation result by RMS is all ok |
| 2 | NG : Recipe validation result by RMS is NG, Equipment has to recovery to previous parameter. |

| 3 | Continue : Equipment still has to send parameter to EAS because EAS is not yet received all parameter. |
| --- | --- |
| 4 | Parameter Count NG : Total Group Count is same as Current Group Count but it’s different with Total Parameter Count. |
| 5 | Group Count NG : Total Group Count is different with Current Group Count but it’s same with Total Parameter Count. |
| 6 | Duplication Group Count NG : Current Group Count is duplicate with previous group count. Equipment has to send the next group count to EAS. |
| 7 | Order by Group Count NG : Group Count is not sent order by next group count to EAS. Group Count has to send by order the next group count |

Figure 5-11 Recipe Parameter Change Sequence

Figure 5-12 Recipe Parameter Request sequence by EAS

    - Recipe Key Parameter or Master Recipe List at abnormal

Equipment should re-send the Recipe Parameter Report or the Mater Recipe List Report to EAS that is cause by operator after OP Call when T1 or T2 Timeout occurs that the event from the first start.

And also Equipment should restart all Event from the first group count that is cause by operator after OP Call, if the Return Code is NG.

Case of Master Recipe List is same as the abnormal sequence in below.

Time out of the Event should can be changed on touch panel by operator because the message of reply can be received after time over that is able to depend on network traffic and message handshake.

Figure 5-13 abnormal scenario at recipe parameter report

    - Local Recipe Request (Array Only)

This is used for the Equipment requiring Local Recipe. This is the signal requesting Local Recipe to EAS. It is used with Local Recipe request ‘Master Recipe ID’, ‘Unit ID’in the W area.

The timing of requesting Local Recipe is basically conducted when confirming ‘Glass data is being displayed’ from the upstream side before receiving glass from the upstream.

If there is no response against Local Recipe request for a certain time, it should be available for manual input on the operation screen.

The Local Recipe request is basically done when receiving new PPID glass from the upstream, when receiving glass of new LOT, and when the auto-starting is started again(when CIM Mode Off -> On).

    - PLC MAP (Cell Only)

      - Local PLC – Link Register (W or R Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Current Recipe Block | Unit No or Local No | 1 | Data | INT | 0 : Local Others : Unit No |
|  | Current Master Recipe ID | 1 | Data | INT |  |
|  | Local Recipe ID | 1 | Data | INT |  |
| Recipe Parameter Return Code Block | Return Code | 1 | Data | INT | 1 : OK 2 : Recipe not Registered 3 : Parameter not Registered |
| Recipe Key Parameter Block | Master Recipe ID | 1 | Data | INT |  |
|  | Local Recipe ID | 1 | Data | INT |  |
|  | Unit No or Local No | 1 | Data | INT | 0 : Local Others : Unit No |
|  | Report Type | 1 | Data | INT | 1: Create 2: Modify 3: Delete 4: Request from EAS |
|  | Total Parameter Count | 1 | Data | INT |  |
|  | Total Group Count | 1 | Data | INT |  |
|  | Current Group Count | 1 | Data | INT |  |
|  | Request Recipe Parameter Items | 250 | Data | ASC |  |
| Master Recipe Block | Unit No or Local No | 1 | Data | INT | 0 : Local Others : Unit No |
|  | Master Recipe List | 250 | Data | INT |  |
|  | Report Type | 1 | Data | INT | 1: Create 2: Modify 3: Delete 4: Request from EAS |
|  | Total Master Recipe Count | 1 | Data | INT |  |
|  | Total Group Count | 1 | Data | INT | It is total count of group of Master Recipe |
|  | Current Group Count | 1 | Data | INT | It is current group count of Master Recipe by event |
| Recipe Parameter Monitoring Block | Master Recipe ID | 1 | Data | INT |  |
|  | Local Recipe ID | 1 | Data | INT |  |
|  | Unit No or Local No | 1 | Data | INT | 0 : Local Others : Unit No |
|  | Current Recipe Parameter Items | 250 | Data | ASC | Cell only |

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Status Event | Auto Recipe Change Mode | 1 | Status | - On: Enable - Off: Disable |  |  |
| Equipment Event | Current Recipe Change Report | 1 | Event | - On: Fire Event - Off: Event End | Current Recipe Block | Reply for Receiving |

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) |
| --- | --- | --- | --- | --- | --- | --- |
|  | Master Recipe List Report | 1 | Event | - On: Fire Event - Off: Event End | Master Recipe Block | Reply for Receiving |
|  | Recipe Parameter Report | 1 | Event | - On: Fire Event - Off: Event End | -Recipe Key Parameter Block *it has to be wrote by Recipe Parameter Request Command | Reply for Receiving |
| EAS Command Reply | Master Recipe List Request Command Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Recipe Parameter Request Command Reply | 1 | Reply | - On: Reply - Off: Reply End | Recipe Parameter Return Code Block |  |

      - Master (EAS) – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Master |  |  |  |  |  |
| Recipe List Request | Unit No or Local No | 1 | Data | INT | 0 : Local Others : Unit No |
| Block |  |  |  |  |  |
| Recipe | Master Recipe ID | 1 | Data | INT |  |
|  | Local Recipe ID | 1 | Data | INT |  |
| Parameter |  |  |  |  |  |
| Request |  |  |  |  | 0 : Local |
| Block | Unit No or Local No | 1 | Data | INT | Others : Unit No |
|  |  |  |  |  | 1 : OK |
| Recipe Parameter |  |  |  |  | 2 : NG 3 : continue |
| Report | Return Code | 1 | Data | INT | 4 : Parameter Count NG |
| Return Code Block |  |  |  |  | 5: Group Count NG |
|  |  |  |  |  | 6 : Duplication Group Count NG |
|  |  |  |  |  | 7 : Order by Group Count NG |
|  |  |  |  |  | 1 : OK |
| Master Recipe List Return Code Block | Return Code | 1 | Data | INT | - : continue - : Master Recipe Count NG - : Group Count NG - : Duplication Group Count NG |
|  |  |  |  |  | 6 : Order by Group Count NG |

      - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Event Reply | Current Recipe Change Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Master Recipe List Report Reply | 1 | Reply | - On: Reply - Off: Reply End | Master Recipe List Return Code Block |  |

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
|  | Recipe Parameter Report Reply | 1 | Reply | - On: Reply - Off: Reply End | Recipe Parameter Report Return Code Block |  |
| EAS Command | Master Recipe List Request Command | 1 | CMD | - On: Fire Command - Off: Command End | Master Recipe List Request Block | Reply for Receiving |
|  | Recipe Parameter Request Command | 1 | CMD | - On: Fire Command - Off: Command End | Recipe Parameter Request Block | Reply for Receiving |

    - PLC MAP (Array Only)

      - Local PLC – Link Register (W or R Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Current Recipe Block | Unit No or Local No | 1 | Data | INT | 0 : Local Others : Unit No |
|  | Current Master Recipe ID | 10 | Data | ASC |  |
|  | Local Recipe ID | 10 | Data | ASC |  |
| Recipe Parameter Return Code Block | Return Code | 1 | Data | INT | 1 : OK 2 : Recipe not Registered 3 : Parameter not Registered |
| Recipe Key Parameter Block | Local Recipe ID | 10 | Data | ASC |  |
|  | Local Recipe ID | 10 | Data | INT |  |
|  | Unit No or Local No | 1 | Data | INT | 0 : Local Others : Unit No |
|  | Report Type | 1 | Data | INT | 1: Create 2: Modify 3: Delete 4: Request from EAS |
|  | Total Parameter Count | 1 | Data | INT |  |
|  | Total Group Count | 1 | Data | INT |  |
|  | Current Group Count | 1 | Data | INT |  |
|  | Request Recipe Parameter Items | 250 | Data | ASC |  |
| Local Recipe Request Block | Master Recipe ID | 10 | Data | ASC |  |
|  | Unit No | 1 | Data | INT | 0: Local |
| Local Recipe Block | Unit No or Local No | 1 | Data | INT | 0 : Local Others : Unit No |
|  | Local Recipe List | 250 | Data | ASC |  |
|  | Report Type | 1 | Data | INT | 1: Create 2: Modify 3: Delete 4: Request from EAS |
|  | Total LocalRecipe Count | 1 | Data | INT |  |
|  | Total Group Count | 1 | Data | INT | It is total count of group of Master Recipe |
|  | Current Group Count | 1 | Data | INT | It is current group count of Master Recipe by event |
| Recipe | Master Recipe ID | 10 | Data | ASC |  |

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Version Block | Local Recipe ID | 10 | Data | ASC |  |
|  | Recipe Version | 3 | Data | BCD | 00 ~ 07 Bit :A.D Year(00~99) 08 ~ 15 Bit :Month(01~12) |
|  |  |  | Data | BCD | 00 ~ 07 Bit :Day(01~31) 08 ~ 15 Bit :Hour(00~23) |
|  |  |  | Data | BCD | 00 ~ 07 Bit :Minute(00~59) 08 ~ 15 Bit :Second(00~59) |

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Status Event | Auto Recipe Change Mode | 1 | Status | - On: Enable - Off: Disable |  |  |
| Equipment Event | Current Recipe Change Report | 1 | Event | - On: Fire Event - Off: Event End | Current Recipe Block | Reply for Receiving |
|  | Local Recipe List Report | 1 | Event | - On: Fire Event - Off: Event End | Local Recipe Block | Reply for Receiving |
|  | Recipe Parameter Report | 1 | Event | - On: Fire Event - Off: Event End | Recipe Parameter Block | Reply for Receiving |
| EAS Command Reply | Local Recipe List Request Command Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Recipe Parameter Request Command Reply | 1 | Reply | - On: Reply - Off: Reply End | Recipe Parameter Return Code Block |  |
|  | Recipe Version Request Command Reply | 1 | Reply | - On: Reply - Off: Reply End | Recipe Version Block |  |
| Equipment Special Event | LocalRecipeRequest | 1 | Event | - On: Fire Event - Off: Event End | Recipe Request Block | Reply for Receiving |

      - Master (EAS) – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Local Recipe List Request Block | Unit No or Local No | 1 | Data | INT | 0 : Local Others : Unit No |
| Recipe Parameter Request Block | Master Recipe ID | 10 | Data | ASC |  |
|  | LocalRecipe ID | 10 | Data | ASC |  |
|  | Unit No or Local No | 1 | Data | INT | 0 : Local Others : Unit No |
| Recipe Version Request Block | Master Recipe ID | 10 | Data | ASC |  |
|  | Local Recipe ID | 10 | Data | ASC |  |

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Recipe Parameter Report Return Code Block | Return Code | 1 | Data | INT | 1 : OK 2 : NG 3 : continue 4 : Parameter Count NG 5: Group Count NG 6: Duplication Group Count NG 7 : Order by Group Count NG |
| Local Recipe List Return Code Block | Return Code | 1 | Data | INT | 1 : OK 2 : continue 3 : LocalRecipe Count NG 4 : Group Count NG 5: Duplication Group Count NG 6 : Order by Group Count NG |

      - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment EventReply | Current Recipe Change Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Local Recipe List Report Reply | 1 | Reply | - On: Reply - Off: Reply End | Local Recipe List Return Code Block |  |
|  | Recipe Parameter Report Reply | 1 | Reply | - On: Reply - Off: Reply End | Recipe Parameter Report Return Code Block |  |
| EAS Command | Local Recipe List Request Command | 1 | CMD | - On: Fire Command - Off: Command End | Local Recipe List Request Block | Reply for Receiving |
|  | Recipe Parameter Request Command | 1 | CMD | - On: Fire Command - Off: Command End | Recipe Parameter Request Block | Reply for Receiving |
|  | Recipe Version Request Command | 1 | CMD | - On: Fire Command - Off: Command End | Recipe Version Request Block | Reply for Receiving |
| Equipment Special Event Reply | LocalRecipeRequest Reply | 1 | Reply | - On: Reply - Off: Reply End | Recipe Request Confirm Block |  |

| Index number (INT) | Local Recipe (ASCII 10W) |
| --- | --- |
| 1 | Local Recipe ID #1 |
| 2 | Local Recipe ID #2 |
| 3 | Local Recipe ID #3 |
| … |  |
| N | Local Recipe ID #N |

  - Recipe Management (CF Shop Only)

    - Recipe Table

EAS holds ‘Recipe Mapping Table’ and ‘Current Recipe Table’ for the each processing.

Each Equipment could has one or more unit recipes. [EAS Recipe Mapping Table]

Figure 5-X EAS Recipe Mapping Table

    - Glass Flow by Recipe

EAS download Master Recipe to indexer. CST info ( Job data including Master Recipe).

All Equipments send the Job data(=Glass Data ) to DownStream.

DownStream Equipment gets the local Unit recipe from EAS. When new glass’s master recipe is not same to ppid of last receiving glass from UpStream machine.

Downstream Equipment should check the new glass’s master recipe(ppid) when Upstream machine set “Send Able”.

Figure 5-X EAS Recipe Management

- All Equipments should provide automatic recipe change function. Different kind of product can’t be mixed in one local Machine.

- Different kind of product can be loaded at the same time on the indexer.

- Dummy glass also needs recipe management.

- All Equipments should provide auto recipe change function.

Equipment should provide auto recipe change enable / disable change function on machine touch Panel.

  - Enable : Machine should change Recipe automatically

  - Disable: Machine should change Recipe manually.

    - Local Recipe List

Report the Status of all Local Recipe registered at the device.

It is used to report the case to EAS where the Local Recipe List is changed in the device (addition/deletion revision of Local Recipe, etc.).

Equipment always should be set Bit of registered Local Recipe Number.( See below).

For example, If Local Recipe No.19 is registered, Bit 3 of the ‘Local Recipe Number 2nd Word’ is ON.

| W | F | E | D | C | B | A | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1st | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Recipe 0 |
| … | . . . |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 63th |  |  |  |  |  |  |  |  | Recipe 999 | 998 | 997 | 996 | 995 | 994 | 993 | 992 |

    - Local Recipe Request

This is used for the Equipment requiring Local Recipe. This is the signal requesting Local Recipe to EAS. It is used with ‘Local Recipe request Master Recipe ID’, ‘Unit ID’in the W area.

The timing of requesting Local Recipe is basically conducted when confirming ‘Glass data is being displayed’ from the upstream side before receiving glass from the upstream.

If there is no response against Local Recipe request for a certain time, it should be available for manual input on the operation screen.

The Local Recipe request is basically done when receiving new PPID glass from the upstream, when receiving glass of new LOT, and when the auto-starting is started again, when CIM Mode Off -> On, PPID Pre Down Load status.

    - Recipe Parameter Data Request (Recipe Parameter Data Report)

EAS requests Recipe Parameter Data to each Equipment.

In case of using Numeric(1~999) Local Recipe, Local Recipe to be reported will be entered into ‘Recipe Parameter Request Local Recipe’ in a word.

    - Recipe Parameter Data Change Report

When the Recipe Parameter Data corresponding to recipe of the device is changed, it will be reported to EAS. This is reported from the device to EAS.

    - Processing Master Recipe ID / Local Recipe Change Report

This is reported from the device to EAS.

This reports Master Recipe ID currently being processed and Local Recipe being processed in the device.

If new glass of Master Recipe ID /Local Recipe is received before one Master Recipe ID /Local Recipe is processed, it is updated with new data.

    - Recipe State Change Event Report

Equipment report to the EAS for the recipe state change event such like Add, Delete and Parameter Update. There are 2 items. (Recipe ID, Event Code)

    - PLC Map

      - Local PLC – Link Register (W or R Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Current Recipe Block | Current Master Recipe ID | 1 | Data | INT |  |
|  | Processing Local Recipe ID | 1 | Data | INT |  |
|  | Unit No | 1 | Data | INT | 0 : Local No |
| Recipe Request Block | Master Recipe ID | 1 | Data | INT |  |
|  | Unit No | 1 | Data | INT | 0: Local |
| Recipe List Report Block | Local Recipe List | 63 | Data | BIT |  |
| Recipe State Block | Local Recipe ID | 1 | Data | INT |  |
|  | Unit No | 1 | Data | INT | 0 : Local No |
|  | Event Code | 1 | Data | INT | 1 : Recipe Add 2 : Recipe Delete 3 : Recipe Parameter Update |
| Recipe key Parameter Block | Local Recipe ID | 1 | Data | INT |  |
|  | Unit No | 1 | Data | INT | 0 : Local No |
|  | Current Recipe Parameter Items | 512 | Data | ASCII | Current Recipe Parameter Items |
| Recipe Parameter Request Confirm Block | Return Code | 1 | Data | INT | - : OK - : Recipe not Registered - : Parameter not Registered * This item is used only for Recipe Parameter Data Request Reply. |

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Status Event | Auto Recipe Change Mode | 1 | Status | - On: Enable - Off: Disable |  |  |
| Equipment Event | Current Master Recipe Change Report | 1 | Event | - On: Fire Event - Off: Event End | Processing Recipe Block | Reply for Receiving |
|  | Local Recipe Request | 1 | Event | - On: Fire Event - Off: Event End | Recipe Request Block | Reply for Receiving |
|  | Recipe List Change Report | 1 | Event | - On: Fire Event - Off: Event End | Recipe List Report Block | Reply for Receiving |

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) |
| --- | --- | --- | --- | --- | --- | --- |
|  | Recipe State Change Report | 1 | Event | - On: Fire Event - Off: Event End | Recipe State Block | Reply for Receiving |
|  | Recipe Parameter Data Change Report | 1 | Event | - On: Fire Event - Off: Event End | Recipe key Parameter Block | Reply for Receiving |
| EAS Command Reply | Recipe Parameter Data Request Reply | 1 | Reply | - On: Reply - Off: Reply End | Recipe Parameter Request Confirm Block |  |

      - Master (EAS) – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Recipe Request Confirm Block | Return Code | 1 | Data | INT | 1: OK 2: NG – There are no recipe in recipe list (Operator Call) 3: NG – There is not data format (Operator Call) |
|  | Send Local Recipe | 1 | Data | INT |  |
| Recipe Parameter Request Block | Local Recipe ID | 1 | Data | INT |  |
|  | Unit No | 1 | Data | INT | 0 : Local No |
| Recipe Parameter Report Reply Block | Return Code | 1 | Data | INT | 1: OK 2: NG 3: Offline OK |

      - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment EventReply | Current Master Recipe Change Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Local Recipe Request Reply | 1 | Reply | - On: Reply - Off: Reply End | Recipe Request Confirm Block |  |
|  | Recipe List Change Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Recipe State Change Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Recipe Parameter Data Change Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |
| EAS Command | Recipe Parameter Data Request | 1 | CMD | - On: Fire Command - Off: Command End | Recipe Parameter Request Block | Reply for Receiving |

Recipe Add EventScenario

Recipe Parameter Request Command Scenario

  - CIM Message Management

CIM Message is used to notify specific information to operator of specific equipment. When receiving CIM Message, equipment has to display it to operator and save the history data locally.

Equipment should support that EAS can send one or more CIM Messages to a specific touch panel of a specific equipment using touch panel number. When equipment receives a CIM Message from EAS, the equipment should display the CIM Message in a pop-up window on touch panel. The CIM Message pop-up window should provide “Confirmation” button so that, if operator confirms, the CIM Message is cleared and at that time equipment should report the confirm event (refer to CIM Message Confirm Report Area).

Equipment clears only the CIM Message that has been confirmed by operator or indicated by CIM Message Clear command from EAS. When CIM Messages are received then display like below.

CIM Message Display (T/P=1)

| No | ID | Received Time | CIM Message | Confirm |
| --- | --- | --- | --- | --- |
| 1 | 1000 | 2010/12/01 05:00:00 | Display the newest (latest) CIM Message | Confirm |
| 2 | 2000 | 2010/12/01 04:00:00 |  | Confirm |
| 3 | 3000 | 2010/12/01 03:00:00 |  | Confirm |
| 4 | 4000 | 2010/12/01 02:00:00 |  | Confirm |
| 5 | 5000 | 2010/12/01 01:00:00 | Display the earliest CIM Message | Confirm |

If Equipment has received more than one CIM Messages before confirming by Engineer or Operator, then Equipment should do buffering of CIM Message on the screen up to five CIM Messages. And if the count of CIM Message buffered is over five, then Equipment should change the buffering of CIM Message by newest (latest) 5 CIM Messages.

    - CIM Message

Equipment should reserve enough space to display some CIM messages. Equipment displays the CIM message on equipment touch panel with CIM Message ID and Touch panel number.

    - Touch Panel Number

Equipment should provide screens to display a CIM message on touch panel. Each screen has its screen number. If equipment receives the CIM Message Set, it must read this data first and display on the corresponding screen. If touch panel number does not match then equipment doesn’t display this CIM message.

    - CIM Message ID

CIM Message ID is a unique identification of message, while message contents indicate more detail information to operator. If CIM Message ID has no value, equipment should display space. According to the CIM Message ID, equipment should display the message and do some actions such as buzzing to operator and turning on a signal tower lamp.

|  | CIM Message ID |
| --- | --- |
| Signal Light Tower | 1xxxx |
| Buzzer | 2xxxx |
| Signal Light Tower + Buzzer | 3xxxx |
| Normal | 4xxxx |
| Reserved | 5xxxx |

    - CIM Message Set Command

This is Command from EAS to equipment, instructing CIM Message display on equipment Touch Panel. When received, equipment displays the message on Popup window. Every CIM Message has its unique ID, and equipment manages CIM Messages based on the CIM Message ID.

    - CIM Message Clear Command

This is command from Master to equipment, instructing clear CIM Message being displayed on touch panel. Equipment clears from CIM Message Popup window the CIM Message indicated by Master based on CIM Message ID, regardless whether the CIM Message has been confirmed by operator or not. If there is no not- confirmed CIM message in Popup window, equipment closes the Popup window. The history of CIM Message has to be preserved.

Equipment should support that EAS can clear one or more CIM Messages from a specific touch panel of a specific equipment using touch panel number and CIM Message ID. If equipment receives this command, equipment should clear only the CIM Message indicated by CIM Message ID from the touch panel indicated by Touch Panel Number.

If CIM Message are all cleared or confirmed then equipment should close CIM Message popup display, buzzer and signal tower should change to original status.

After CIM Message clear command or confirm, equipment should re-arrange popup display like below.

- CIM Message Sample

CIM Message Display (T/P=1)

| No | ID | Received Time | CIM Message | Confirm |
| --- | --- | --- | --- | --- |
| 1 | 1000 | 2010/12/01 05:00:00 | Port#1 Canceled By Host | Confirm |
| 2 | 2000 | 2010/12/01 04:00:00 | Port#2 Canceled By Host | Confirm |
| 3 | 3000 | 2010/12/01 03:00:00 | Port#3 Canceled By Host | Confirm |
| 4 |  |  |  | Confirm |
| 5 |  |  |  | Confirm |

CIM Message Display (T/P=1)

After receive CIM Message Clear Command from BC CIM Message ID = 2000

| No | ID | Received Time | CIM Message | Confirm |
| --- | --- | --- | --- | --- |
| 1 | 1000 | 2010/12/01 05:00:00 | Port#1 Canceled By Host | Confirm |
| 2 | 3000 | 2010/12/01 03:00:00 | Port#3 Canceled By Host | Confirm |
| 3 |  |  |  | Confirm |
| 4 |  |  |  | Confirm |
| 5 |  |  |  | Confirm |

    - CIM Message Confirm Event Report

This is to indicate that operator confirmed CIM Message on the equipment touch panel. Upon confirmation from operator, equipment reports the ID of the CIM Message to Master.

If CIM Message are all cleared or confirmed then equipment should close CIM Message popup display, buzzer and signal tower should change to original status.

After CIM Message clear command or confirm, equipment should re-arrange popup display like below.

- CIM Message Sample

CIM Message Display (T/P=1)

| No | ID | Received Time | CIM Message | Confirm |
| --- | --- | --- | --- | --- |
| 1 | 1000 | 2010/12/01 05:00:00 | Port#1 Canceled By Host | Confirm |
| 2 | 2000 | 2010/12/01 04:00:00 | Port#2 Canceled By Host | Confirm |
| 3 | 3000 | 2010/12/01 03:00:00 | Port#3 Canceled By Host | Confirm |
| 4 |  |  |  | Confirm |
| 5 |  |  |  | Confirm |

CIM Message Display (T/P=1)

After confirm CIM Message on Touch Panel CIM Message ID = 2000

| No | ID | Received Time | CIM Message | Confirm |
| --- | --- | --- | --- | --- |
| 1 | 1000 | 2010/12/01 05:00:00 | Port#1 Canceled By Host | Confirm |
| 2 | 3000 | 2010/12/01 03:00:00 | Port#3 Canceled By Host | Confirm |
| 3 |  |  |  | Confirm |
| 4 |  |  |  | Confirm |
| 5 |  |  |  | Confirm |

    - CIM Message History Management

Equipment should display on the equipment Touch Panel the history of at least 20 CIM Messages most recently received from EAS.

| No | Received Time | CIM Message ID | CIM Message |
| --- | --- | --- | --- |
| 1 | 2010/10/07 13:20:57 | 0113 |  Display the newest (latest) CIM Message |
| 2 | … | … | … |
| : | … | … | … |
| 20 | 2010/10/07 10:20:01 | 3333 |  Display the earliest CIM Message |

Figure5-14CIM Message History Table

    - PLC MAP

      - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| CIM Message Confirm Block | CIM Message ID | 1 | Data | INT |  |
|  | Touch Panel No | 1 | Data | INT | 1~65535 |

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Event | CIM Message Confirm Report | 1 | Event | - On: Fire Event - Off: Event End | CIM Message Confirm Block |  |
| EAS Command | CIM Message Set Command Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) |
| --- | --- | --- | --- | --- | --- | --- |
| Reply | CIM Message Clear Command Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |

      - Master (EAS) – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| CIM Message Set CMD Block | CIM Message | 16 | Data | ASC |  |
|  | CIM Message ID | 1 | Data | INT | 1~65535 |
|  | Touch Panel No | 1 | Data | INT | 0 : All 1~65535 |
| CIM Message Clear CMD Block | CIM Message ID | 1 | Data | INT | 1~65535 |
|  | Touch Panel No | 1 | Data | INT | 0 : All 1~65535 |

      - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Event Reply | CIM Message Confirm Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |
| EAS Command | CIM Message Set Command | 1 | CMD | - On: Fire Command - Off: Command End/Timeout | CIM Message Set CMD Block |  |
|  | CIM Message Clear Command | 1 | CMD | - On: Fire Command - Off: Command End/Timeout | CIM Message Clear CMD Block |  |

    - Signal Tower Management

| Type | O |  | X |
| --- | --- | --- | --- |
| Lamp | Light On | Light Flash | Light Off |
| Buzzer | Long Beep | Short Interval Beep | No Sound |

| No | Type of CIM Message | Touch Panel | Tower Lamp |  |  |  | Buzzer |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Rd | Yw | Gn | Bl |  |
| 1 | CIM Message & Tower Lamp & Buzzer (ID : 1~65535) | Display CIM Message. | Keep current status |  | Keep current status |  |  |

  - Date Time Set Command

When equipment receives this command, equipment adjusts its local system time according to the time sent by this command.

EAS will set date time to equipment when receive date time from MES or equipment CIM mode changed to on.

    - PLC MAP

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) |
| --- | --- | --- | --- | --- | --- | --- |
| EAS Command Reply | Date Time Set Command Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |

      - Master (EAS) – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Date Time Set CMDBlock | Date Time | 3 | Data | BCD | 00 ~ 07 Bit :A.D Year(00~99) 08 ~ 15 Bit :Month(01~12) |
|  |  |  | Data | BCD | 00 ~ 07 Bit :Day(01~31) 08 ~ 15 Bit :Hour(00~23) |
|  |  |  | Data | BCD | 00 ~ 07 Bit :Minute(00~59) 08 ~ 15 Bit :Second(00~59) |

      - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| EAS Command | Date Time Set Command | 1 | CMD | - On: Fire Command - Off: Command End/Timeout | Date Time Set CMD Block | Local PLC replies and should change the time of itself. |

- Port Management

  - Port Operation Mode Management

    - CST Operation Mode

It indicates the current operation mode of Loader based on Glass type. CST Operation Mode is applicable for specific equipment’s only.

- Kind to Kind Mode (Default): Glass/Panel is stored into a cassette according to product (kind) type. It does not consider FIFO sequence of glasses. Un-loader reports Unload Request event when unloading cassette is full.

- Cassette To Cassette Mode: Glass/Panel will be stored into the same slot of unloading cassette as loading cassette. If un-loader has wire cassette then glass/Panel can be stored into the another slot of unloading cassette.

    - Indexer Operation Mode(Line Operation Mode)

In a Line, there could be multiple job transfer routes between Indexer and Equipment’s according to specific manufacturing purpose. Each of a specific job transfer route is defined as an Indexer Operation Mode; one or more Indexer Operation Mode can be assigned for one Line if there exist one or more than one specific job transfer routes

Indexer Operation Mode defines job transfer routes mainly between Indexer and Process Equipment’s, and therefore is to be used by Indexer Job dispatching controller.

The below is examples of Indexer Operation Mode.

- Mode that is required for each Equipment Types will be defined in separate Equipment Operation Scenario document.

  - Sort Mode : This mode is used for sorting function (Job exchange between Cassettes). In this Mode, Job in one Cassette (source Cassette) is transferred to another Cassette (target Cassette) directly without delivering to Process Equipment’s. Source and target cassette can be determined automatically or by user-defined Sorting Plan.

  - Cold Run Mode

    - This mode is used for the purpose of the transfer test.

    - The indexer should provide Cold Run Mode function on touch screen.

    - The operator set the glass count to touch screen of the indexer and the indexer reports to the EAS.

    - The EAS control the robot for the glass transfer test and the indexer not unload the cassette, automatically change the Port Mode until glass count set.

Indexer is required to transfer Job according to Indexer Operation Mode either when Indexer is in Local Dispatching Mode or Indexer is independent type (i.e. no EAS dispatching control).

Therefore Indexer has to provide function of

- Indexer Operation Mode management (glass transfer route definition, mode change, etc.)

- Control Job transfer according to current Indexer Operation Mode..

    - PLC Map

      - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Indexer Operation Mode Block | Indexer Operation Mode | 1 | Data | INT | *common (1~9999) - : Normal - : Clean_Out 4 : Cold Run ~9999 : Etc *Array (10000~19999) 10000 : AOI_Only 10001 : Mix_Sorting 10002 : Return 10003 : Wet_Stripper_Only 10004 : Anneal_Only 10005 : Anneal_Skip 10006 : Re_Input 10007 : USC_Only 10008 : Mix_Run 10009 : HDC_Only 10010 : PECVD_Only 10011 : Reverse 10012 : Sputter_Only 10013 : Dummy_Glass 10014 : Tray_Gap_Test 10015 : Dry_Etch_Only 10016 : Normal_Independent 10017 : Sorting 10018 : Cassette_Change 10019 : TC_Bypass 10020 : Align 10021 : To_Normal_Sort 10022 : To_Wire_Sort 10023 : Turn 10024 : Cutting 10025 : Scrap 10026 : ByPass AOI Only(Photo) 10027 : HDC_SUPTTER_HDC 10028 : Normal Matching Sorter 10029 : Wire Matching Sorter ~19999 : Etc *CF (20000~29999) 20001 : BM 20002 : Blue 20003 : Green 20004 : Red 20005 : PS 20006 : OC |

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 20007 : White 20008 : ITO_Dummy_Rework 20009 : COA_Only_PR_Rework 20010 : Scraper_Cutting 20011 : Stand_Alone_Sorting *F.Cell(30000~39999) 30001 : MERGE 30002 : PAIR 30003 : AF_CV_INPUT 30004 : CLN_CIRCULATION_START 30005 : CLN_CIRCULATION_END 30006 : PARTICLE_TEST1 30007 : PARTICLE_TEST2 30008 : PARTICLE_TEST3 30009 : UV_MASK_DUMMY 30010 : LC_SD_DUMMY 30021: WIRE_BUFF_STORED_GLASS_BACK 30031 : CLN_TURN 30032 : REVERSE_REPROCESS1 30033 : REVERSE_REPROCESS2 30034 : TC_GLASS1 30035 : TC_GLASS2 30036 : TC_GLASS3 30041 : USC_PATH1 30042 : USC_PATH2 30043 : USC_PATH3 30044 : USC_PATH4 30045 : RUBBING_SAME_RECIPE 30046 : RUBBING_DIFFERENT_RECIPE 30047 : STEAM_ONLY 30048 : MMG_MODE 30049 : TC_MMG_MODE 30050 : TTCC_USC_PATH1 30100 : SORTING 30101 : INSPECTION_SKIP 30102 : AGING_OVEN 30103 : CELL_GAP 30104 : SAMPLING 30105 : FULL_INSPECTION_GM 30106 : FULL_INSPECTION_CG 30107 : FULL_INSPECTION_GM_CG 30108 : AOI 30109 : MACRO 30110 : AOI_MACRO 30111 : ODF_EV_GLASS_TURN 30112 : MOVING_WIRE_TO_NORMAL 30113 : MOVING_NORMAL_TO_WIRE |

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 30114 : MOVING_NORMAL_TO_NORMAL 30115 : SAMPLING_ONLY_ONE 30116 : SAMPLING_MORE_THEN_TWO 30117 : FULL_INSPECTION 30118 : INSPECTION_1 30119 : INSPECTION_2 30125 : PARTICLE_COUNT_ONLY 30126 : THICKNESS_MEASURE_ONLY 30127 : BOTH 30128 : PARTICLE_ONLY_TURN 30129 : BOTH_TURN 30133 : REWORK_COUNT_CHECK 30138 : AOI_ONLY 30139 : AOI_STEAM_INSPECTION 30140 : MAIN_UNIT_ONLY 30141 : BOTH_UNIT 30145 : SLOC_PRODUCT_CF_TO_CELL 30146 : SLOC_PRODUCT_CELL_TO_CF *E.Cell(40000~59999) 40000 : TFT_UP 40001 : CF_UP 40002 : POL_LOAD 40003 : POL_UNLOAD 40004 : POL_EXCHANGE 40005 : PALLET_OUT_MODE 40006 : BOX_OUT_MODE 40007 : SCRAP_MODE 40008 : UNPACKER_MODE 40009 : SINGLE_PANEL_OUT 40010 : SINGLE_PANEL_IN 40011 : NO_VCR 40012 : POL_INPUT 40013 : BYPASS_CELLTEST |
|  | Cold Run Count | 1 | Data | INT | Cold Run only |
| Indexer Operation Mode Return Block | Return Code | 1 | Data | INT | - : OK - : NG |
| CST Operation Mode Block | CST Operation Mode | 1 | Data | INT | - : Kind To Kind mode - : Cassette To Cassette mode |

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) Action |
| --- | --- | --- | --- | --- | --- | --- |
| Indexer Event | Indexer Operation Mode Change Report | 1 | Event | - On: Fire Event - Off: Event End | Indexer Operation Mode Block | Reply for Receiving |

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) Action |
| --- | --- | --- | --- | --- | --- | --- |
|  | CST Operation Mode Change Report | 1 | Event | - On: Fire Event - Off: Event End | CST Operation Mode Block | Reply for Receiving |
| Indexer Command Reply | Indexer Operation Mode Command Reply | 1 | Reply | - On: Reply - Off: Reply End | Indexer Operation Mode Return Block |  |

      - Master (EAS) – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Indexer Operation Mode Command Block | Indexer Operation Mode | 1 | CMD | INT |  |

      - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| Indexer Event Reply | Indexer Operation Mode Change Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |
|  | CST Operation Mode Change Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |
| Indexer Command | Indexer Operation Mode Change Command | 1 | CMD | - On: Fire Command - Off: Command End | Indexer Operation Mode Command Block. |  |

  - Port Type & Mode Management

    - Port Type

It is to classify a Port based on the expected operation of the Cassette currently loaded on the Port: Cassette loaded on the port is whether to input Job into Inline, or to output Job from Inline after Job has been finished its process in a line, or to buffer a Job in a line temporary.

If there is a cassette on a port, the port type of the port cannot be changed. Equipment makers should provide this function. If port type is changed, equipment should report the ‘Load Ready’ event for the port.

There are 5 kinds of port types, loader, un-loader, both and buffer, un-loader partial. Port mode is related to port type, according to the type of the port.

- Loading Port: To be used to fetch out Job from cassette loaded on the port and input into Inline.

- Unloading Port: To be used to store into Cassette the Job that finished its process, and take out the Cassette out of Inline.

- Both Port: To be used to fetch out Job from cassette loaded on the port and input into Inline and also used to store into Cassette the Job that finished its process, and take out the Cassette out of Inline

- Buffer Port: To be used to buffer Job in Inline.

  - Buffer Type : Indicates this Port is for buffering purpose only. Available for mixing of multiple types of Glasses.

  - Loader in Buffer Type : Indicates that this Buffer Port can be used for Glass Inline input, like Loading Port.

  - Un-Loader in Buffer Type : Indicates that this Buffer Port can be used for Glass Inline output, like Unloading Port.

- Unloading Partial Port : To be used to store into Cassette the Job that finished its process, and take out the Cassette out of Inline. But a Cassette with processed the job will be loaded at Unloading Port, Then the Cassette will be unloaded when it is full.

    - Port Mode

It is to classify a Port based on operational mode of Glasses to be stored in the Cassette on Port. Port Mode is used to indicate that a Port with specific Port Mode can store only a Job of specific properties.

Port Mode is represented by 5 digit numbers that are consists of 3 different properties of Job to distinguish.

| Property | Glass Type | Production Type | Use Type(without End Cell) |
| --- | --- | --- | --- |
| Port Mode Value | XXXXX(Combination of each value , INT: 10000~59999) |  |  |
| Data Format | 1 Digit (1~5) | 2~3 Digit (00~99) | 4~5 Digit (00~99) |
|  | Glass Type | Production Type | Use Type |
| Value Description | 5: Not Specific 1: TFT 2: CF 3: Both .. | 00: Not Specific 01: Production 02: Engineer 03: General Dummy 04: QC Dummy 05: Through Dummy 06: Mask Dummy 07: Screen Dummy 08: ODF Seal Dummy 09: VAS Dummy 10: Laser Seal Dummy 11: ITO Dummy 12 :Photo Dummy 13: TPCD Dummy 14: Monitor Dummy 15: Thickness Dummy 16: Rubbing Monitor Dummy 17: Rubbing Aging Dummy 18 :MM Dummy 19: Bare Dummy 20: ODF UVMask Dummy 21: ODF Bare Dummy 22: ODF LC Dummy 23: PI Inkjet Dummy | 00: Not Specific 01: G (Good) 02: N (NG) 03: T (Retest) 04: R (Repair) 05: W (Rework) 06: S (Scrap) 07: K (Process Skip) 08: P (Pending, Need PS Laser) 09: M (Mac/Mic) 10: A (Need Mac, Need Tape Repair) 11: I (Need Mic) 12: C (Need CSI) 13: V (Need CVD) 14: K (Ink Repair :CF only) 15: D (Visible NG) .. 70: AX 71: A1 72: A2 73: A3 74: A4 75: A5 76: A6 |

| Property | Glass Type | Production Type | Use Type(without End Cell) |
| --- | --- | --- | --- |
|  |  | … | 77: A7 |
|  |  |  | 78: A8 |
|  |  |  | 79: A9 |
|  |  |  | .. |
|  |  |  | 81: AOI |
|  |  |  | 82: SI(Steam Inspection) |
|  |  |  | .. |
|  |  |  | 90: OK |
|  |  |  | 91: SN |
|  |  |  | 92: MN |
|  |  |  | 93: VN(VN Rate) |
|  |  |  | 94: NP(Non Pair) |
|  |  |  | 95: SC |
|  |  |  | 96: PA(Pair) |
|  |  |  | 97: 2N |
|  |  |  | 98: RT |
|  |  |  | 99: RD |
|  |  |  | … |
| * For example, Port Mode of 10101 indicates, the Port is to be stored TFT glass/panel of normal Production whose Judge value is Good only. |  |  |  |

Port Mode is mainly used by Indexer Dispatching Controller in order to find matching Port of each Job to transfer.

In case Equipment (includes Indexer) needs to have Port Mode property management, the Equipment has to provide functions of

- Provide Port Mode change function (at Touch Panel or other type of instrument) for each Port, and report to Host when Port Mode gets changed.

- Receive Port Mode change command from Host, and set the Port Mode accordingly.

If there is a cassette on a port and clamped, the port mode of the port cannot be changed. Equipment makers should provide this function.

This is End Cell Only in below

End Cell Grade Use Type should be define by modeling that means can be flexibly set by operator. When Port Mode is changed, Indexer should report Use Type and Grades for End Cell.

- Use type indicates that to modeling dispatching rule with a lot of grade

- Priority can be use when the Use Type exists with same grade, if Operator want to dispatch the panels in other to work with priority before processing start with Cassette Loading

- Grades indicates that panel want to dispatch by grade at port which should can set on touch panel by modeling.

It will be provide by ESWIN Engineer. For example,

|  | Use Type |
| --- | --- |
| W | +0 |

| Use Type | GQ |
| --- | --- |

|  | Priority |
| --- | --- |
| W | +0 |
| Priority | 2 |

|  | Grade | Grade | Grade | Grade | Grade | Grade |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| W | +0 | +1 | +2 | +3 | +4 | +5 | … |
| Grades | QL | QV | QW | QU | QP | QR |  |

    - Port Transfer Mode

It indicates the mode for transferring of cassette between port and AMHS. There are 3 modes: AGV Mode, MGV Mode and STK Inline Mode. The AGV mode includes RGV mode. AGV mode should be switch able to MGV mode via a button press by operator.

If there is a cassette on a port, the port transfer mode of the port cannot be changed. Equipment makers should provide this function. If port transfer mode is changed, equipment should report the ‘Load Request’ event for the port.

- MGV Mode : Port interfaces with MGV.

- AGV Mode : Port interfaces with AGV.

- Stocker Inline Mode: Port interfaces with Stocker directly.

    - Port Enable Mode

Port Enable Mode means the port will be used. Port Enabled/Disabled change should be available only when there is no cassette on the Port (i.e. Load Ready status). Once Port status is set to be disabled, no Port mode or Cassette status change related with the Port should be allowed or made.

If there is a cassette on a port, the port enable mode of the port cannot be changed. Equipment makers should provide this function. If port enable mode is changed from disabled mode to enable mode, equipment should report the ‘Load Ready’ event for the port.

- Enabled Mode: This port can be used.

- Disabled Mode: This port cannot be used. Operator or other prevents to use this port

    - Port Type Auto Change Mode

In this mode, The Indexer must be automatically changing the port type.

If actual cassette is changed empty cassette then the port type should be automatically changed from loading port type to unloading port type.

If empty cassette is changed actual cassette then do unload the cassette. And, after cassette is completely unloaded, then the indexer must be automatically changed from the unloading port type to the loading port type.

- Enable Mode : This mode can automatically change the port type.

- Disable Mode : This mode cannot automatically change the port type.

    - Port Cassette Type

A specific Cassette Type can be assigned to a Port in order to indicate that only the Cassette with matching specific properties can be loaded on the Port.

In case Equipment (Indexer) needs to have Cassette Type property management, the Equipment has to provide functions of

- Providing Cassette Type change function (at Touch Panel or other type of instrument) for each Port, and report to Host when Cassette Type gets changed.

- Receiving Cassette Type change command from Host, and set the Cassette Type accordingly

- Validating a loaded Cassette ; when received Cassette data from Host, validates the Cassette property from Host data against Cassette Type set to the Port.

| Property | Not Use | Partial | Glass Type | CST Capacity | Thickness | CST Model | Reserved |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Word |  |  |  |  |  |  |  |
| Port Cassette Type |  |  |  |  |  |  |  |
| Data Format | 1Digit (1~3) | 2 Digit (0~9) | 3 Digit (0~9) | 4 Digit (0~9) | 5 Digit (0~9) | 6 Digit (0~9) | 7~10 Digit |
| Value Description | Not Use | 0: Not Specific 1: Full 2: Empty 3: Partial | 0: Not specific 1: TFT 2 : CF 3 : Panel 4 : POL | 0: Not Specific 1: 20 2: 30 3: 50 4: 80 5: 40 | Example) 0: Not Specific 1: 0.1t 2: 0.2t 3: 0.9t | 0: Not specific 1: W 2: N 3: V 4: A 5: B | 0: Not Specific |

Port Cassette Type is represented by 5 digit numbers that are consists of 5 different properties of Job to distinguish. Thickness or size is optional and 2nd word is reserved

    - PLC MAP

      - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Port#1 Block | Port Type | 1 | Data | INT | - : Loading Port - : Unloading Port - : Both Port - : Buffer Port – Buffer Type - : Buffer Port – Loader in Buffer Type - : Buffer Port – Un-loader in Buffer Type - : Unloading Partial Port |
|  | Port Mode | 1 | Data | INT | 10100 : TFT, normal Product * Details are to be defined later |

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
|  | Port Cassette Type | 2 | Data | INT | * it might be updated if special item need to discuss. |
|  | Port Transfer Mode | 1 | Data | INT | - : MGV Mode - : AGV Mode - : Stocker Inline Mode |
|  | Port Enable Mode | 1 | Data | INT | - : Port Enabled - : Port Disabled |
|  | Port Type Auto Change Mode | 1 | Data | INT | - : Auto Change Enable - : Auto Change Disable |
|  | Port Type Change Return Code | 1 | Data | INT | - : OK - : NG |
|  | Port Mode Change Return Code | 1 | Data | INT | - : OK - : NG |
|  | Port Transfer Mode Change Return Code | 1 | Data | INT | - : OK - : NG |
|  | Port Enable Mode Change Return Code | 1 | Data | INT | - : OK - : NG |
|  | Port Type Auto Change Mode Return Code | 1 | Data | INT | - : OK - : NG |
|  | Port Cassette Type Change Return Code | 1 | Data | INT | - : OK - : NG |
| Port#2 Block | Port#2 Event Report Area | 13 |  | - | Sub Items are same as Port#1 Block |
| Port#3 Block | Port#3 Event Report Area | 13 |  | - | Sub Items are same as Port#1 Block |
| Port#4 Block | Port#4 Event Report Area | 13 |  | - | Sub Items are same as Port#1 Block |
| Port#5 Block | Port#5 Event Report Area | 13 |  | - | Sub Items are same as Port#1 Block |
| Port#6 Block | Port#6 Event Report Area | 13 |  | - | Sub Items are same as Port#1 Block |
| Port#7 Block | Port#7 Event Report Area | 13 |  | - | Sub Items are same as Port#1 Block |
| Port#8 Block | Port#8 Event Report Area | 13 |  | - | Sub Items are same as Port#1 Block |
| Port#9 Block | Port#9 Event Report Area | 13 |  | - | Sub Items are same as Port#1 Block |
| Port#10 Block | Port#10 Event Report Area | 13 |  | - | Sub Items are same as Port#1 Block |
| Port#1 Grade Use Type Block | Use Type | 1 | Data | ASC | *Refer to Port Mode for End Cell |
|  | Priority | 1 | Data | INT | *Refer to Port Mode for End Cell |
|  | Grades | 62 | Data | ASC | *Refer to Port Mode for End Cell |
| … |  |  |  |  |  |
| Port#N Grade Use Type Block | Port#N Event Report Area | 64 |  | - | Sub Items are same as Port#1 Grade Use Type Block |

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) |
| --- | --- | --- | --- | --- | --- | --- |
| Port Event | Port#1 Port Type Change Report | 1 | Event | - On: Fire Event - Off: Event End/Timeout | Port#1 Block - Port Type | Reply for Receiving |
|  | Port#1 Port Mode Change Report | 1 | Event | - On: Fire Event - Off: Event End/Timeout | Port#1 Block Port#1 Grade Use Type Block - Port Mode | Reply for Receiving |

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) |
| --- | --- | --- | --- | --- | --- | --- |
|  | Port#1 Port Cassette Type Change Report | 1 | Event | - On: Fire Event - Off: Event End/Timeout | Port#1 Block -Port Cassette Type | Reply for Receiving |
|  | Port#1 Port Transfer Mode Change Report | 1 | Event | - On: Fire Event - Off: Event End/Timeout | Port#1 Block - Port Transfer Mode | Reply for Receiving |
|  | Port#1 Port Enable Mode Change Report | 1 | Event | - On: Fire Event - Off: Event End/Timeout | Port#1 Block - Port Enable Mode | Reply for Receiving |
|  | Port#1 Port Type Auto Change Mode Report | 1 | Reply | - On: Reply - Off: Reply End | Port#1 Block - Port Type Auto Change Mode | Reply for Receiving |
| Port Command Reply | Port#1 Port Type Change Command Reply | 1 | Reply | - On: Reply - Off: Reply End | Port#1 Block - Port Type Change Return Code |  |
|  | Port#1 Port Mode Change Command Reply | 1 | Reply | - On: Reply - Off: Reply End | Port#1 Block - Port Mode Change Return Code |  |
|  | Port#1 Port Transfer Mode Change Command Reply | 1 | Reply | - On: Reply - Off: Reply End | Port#1 Block - Port Transfer Mode Change Return Code |  |
|  | Port#1 Port Enable Mode Change Command Reply | 1 | Reply | - On: Reply - Off: Reply End | Port#1 Block - Port Enable Mode Change Return Code |  |
|  | Port#1 Port Type Auto Change Mode Command Reply | 1 | Reply | - On: Reply - Off: Reply End | Port#1 Block - Port Type Auto Change Mode Return Code |  |
|  | Port#1 Port Cassette Type Change Command Reply | 1 | Reply | - On: Reply - Off: Reply End | Port#1 Block - Port Cassette Type Change Return Code |  |
| Port Event & Port Command Reply | Port#2 Port Event & Command Reply Same as Port#1 Event & Command Reply | 12 | Event | - On: Fire Event - Off: Event End/Timeout |  |  |
|  | Port#3 Port Event & Command Reply Same as Port#1 Event & Command Reply | 12 | Event | - On: Fire Event - Off: Event End/Timeout |  |  |
|  | Port#4 Port Event & Command Reply Same as Port#1 Event & Command Reply | 12 | Event | - On: Fire Event - Off: Event End/Timeout |  |  |
|  | Port#5 Port Event & Command Reply Same as Port#1 Event & Command Reply | 12 | Event | - On: Fire Event - Off: Event End/Timeout |  |  |
|  | Port#6 Port Event & Command Reply Same as Port#1 Event & Command Reply | 12 | Event | - On: Fire Event - Off: Event End/Timeout |  |  |
|  | Port#7 Port Event & Command Reply | 12 | Event | - On: Fire Event - Off: Event |  |  |

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) |
| --- | --- | --- | --- | --- | --- | --- |
|  | Same as Port#1 Event & Command Reply |  |  | End/Timeout |  |  |
|  | Port#8 Port Event & Command Reply Same as Port#1 Event & Command Reply | 12 | Event | - On: Fire Event - Off: Event End/Timeout |  |  |
|  | Port#9 Port Event & Command Reply Same as Port#1 Event & Command Reply | 12 | Event | - On: Fire Event - Off: Event End/Timeout |  |  |
|  | Port#10 Port Event & Command Reply Same as Port#1 Event & Command Reply | 12 | Event | - On: Fire Event - Off: Event End/Timeout |  |  |

      - Master (EAS) – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Port#1 Command Block | Port Type | 1 | Data | INT | Same as Port Type Change Report |
|  | Port Mode | 1 | Data | INT | Same as Port Mode Change Report |
|  | Port Cassette Type | 2 | Data | INT | * it might be updated if special item need to discuss. |
|  | Port Transfer Mode | 1 | Data | INT | - : MGV Mode - : AGV Mode - : Stocker Inline Mode |
|  | Port Enable Mode | 1 | Data | INT | - : Port Enabled - : Port Disabled |
|  | Port Type Auto Change Mode | 1 | Data | INT | - : Auto Change Enable - : Auto Change Disable |
| Port#2 Command Block | Same as Port #1 | 7 | Data | INT |  |
| Port#3 Command Block | Same as Port #1 | 7 | Data | INT |  |
| Port#4 Command Block | Same as Port #1 | 7 | Data | INT |  |
| Port#5 Command Block | Same as Port #1 | 7 | Data | INT |  |
| Port#6 Command Block | Same as Port #1 | 7 | Data | INT |  |
| Port#7 Command Block | Same as Port #1 | 7 | Data | INT |  |
| Port#8 Command Block | Same as Port #1 | 7 | Data | INT |  |
| Port#9 Command Block | Same as Port #1 | 7 | Data | INT |  |
| Port#10 | Same as Port #1 | 7 | Data | INT |  |

| Command Block |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

      - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| Port Event Reply | Port#1 Port Type Change Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Port#1 Port Mode Change Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Port#1 Port Cassette Type Change Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Port#1 Port Transfer Mode Change Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Port#1 Port Enable Mode Change Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Port#1 Port Type Auto Change Mode Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |
| Port Command | Port#1 Port Type Change Command | 1 | CMD | - On: Fire Command - Off: Command End | Port#1 Command Block - Port Type |  |
|  | Port#1 Port Mode Change Command | 1 | CMD | - On: Fire Command - Off: Command End | Port#1 Command Block - Port Mode |  |
|  | Port#1 Port Transfer Mode Change Command | 1 | CMD | - On: Fire Command - Off: Command End | Port#1 Command Block - Port Transfer Mode |  |
|  | Port#1 Port Enable Mode Change Command | 1 | CMD | - On: Fire Command - Off: Command End | Port#1 Command Block - Port Enable Mode |  |
|  | Port#1 Port Type Auto Change Mode Command | 1 | CMD | - On: Fire Command - Off: Command End | Port#1 Command Block - Port Type Auto Change Mode |  |
|  | Port#1 Port Cassette Type Change Command | 1 | CMD | - On: Fire Command - Off: Command End | Port#1 Command Block - Port Cassette Type |  |
| Port Command & Port Event Reply | Port#2 Port Command & Event Reply Same as Port#1 Event & Command Reply | 12 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Port#3 Port Command & Event Reply Same as Port#1 Event & Command Reply | 12 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Port#4 Port Command & Event Reply Same as Port#1 Event & Command Reply | 12 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Port#5 Port Command & Event Reply Same as Port#1 Event & Command Reply | 12 | Reply | - On: Reply - Off: Reply End |  |  |

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
|  | Port#6 Port Command & Event Reply Same as Port#1 Event & Command Reply | 12 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Port#7 Port Command & Event Reply Same as Port#1 Event & Command Reply | 12 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Port#8 Port Command & Event Reply Same as Port#1 Event & Command Reply | 12 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Port#9 Port Command & Event Reply Same as Port#1 Event & Command Reply | 12 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Port#10 Port Command & Event Reply Same as Port#1 Event & Command Reply | 12 | Reply | - On: Reply - Off: Reply End |  |  |

  - Port Q-Time Check (except End Cell)

Indexer has to manage Q-Time in Port after Cassette is loaded until get a first glass with robot.

Q-time check is started when Cassette is Loaded and Q-time check is ended when robot gets a first glass. Indexer can set the Q-time value on touch screen.

If Q-Time is over for a cassette, then Equipment occurs OP Call. Equipment should report Q-Time Timeout Report to EAS, Unload Cassette.

Figure 6-1 Port Q-Time

    - PLC MAP

      - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Port#1 Block | Port Q-Time | 1 | Data | INT | 1~65535 minutes |
|  | Port Q-Time Setting Time | 1 | Data | INT | 1~65535 minutes |
| … | … |  |  |  |  |
| Port#N Block | Port Q-Time | 1 | Data | INT | 1~65535 minutes |
|  | Port Q-Time Setting Time | 1 | Data | INT | 1~65535 minutes |

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) |
| --- | --- | --- | --- | --- | --- | --- |
| Port Event | Port #1 Port Q-Time Change Report | 1 | Event | - On: Fire Event - Off: Event End/Timeout | Port#1 Block *That Item : Port Q-Time Setting Time | Reply for Receiving |
|  | … |  |  |  |  |  |
|  | Port #N Port Q-Time Change Report | 1 | Event | - On: Fire Event - Off: Event End/Timeout | Port#N Block | Reply for Receiving |
|  | Port#1 Port Q- TimeTimeout Report | 1 | Event | - On: Fire Event - Off: Event End/Timeout | Port#1 Block *That Item : Port Q-Time | Reply for Receiving |
|  | … |  |  |  |  |  |
|  | Port#N Port Q- TimeTimeout Report | 1 | Event | - On: Fire Event - Off: Event End/Timeout | Port#N Block | Reply for Receiving |
| Port Command Reply | Port#1 Port Q-Time Change Command Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |
|  | … |  |  |  |  |  |
|  | Port#N Port Q-Time Change Command Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |

      - Master (EAS) – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Port#1 Command Block | Port Q-Time | 1 | Data | INT | 1~65535 minutes |
| … | … |  |  |  |  |
| Port#N Command Block | Port Q-Time | 1 | Data | INT | 1~65535 minutes |

      - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| Port Event Reply | Port #1 Port Q-Time Change Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |
|  | … |  |  |  |  |  |
|  | Port #N Port Q-Time | 1 | Reply | - On: Reply |  |  |

|  | Change Report Reply |  |  | - Off: Reply End |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Port#1 Port Q- TimeTimeout Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |
|  | … |  |  |  |  |  |
|  | Port#N Port Q- TimeTimeoutReport Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |
| Port Command | Port#1 Port Q-Time Change Command | 1 | CMD | - On: Fire Command - Off: Command End | Port#1 Command Block |  |
|  | … |  |  |  |  |  |
|  | Port#N Port Q-Time Change Command | 1 | CMD | - On: Fire Command - Off: Command End | Port#N Command Block |  |

  - Port and Cassette Control Management

    - Port State

There are 6 types of port state; Empty, Load Ready, Loaded, In Use, Unload Ready, Blocked

- Load Ready: Port can receive a cassette and is waiting for cassette loading.

- Loaded : Cassette is loaded but net yet clamped.

- In Use: a cassette has been loaded on the port, and ready to use for production. On firing the event, the Cassette Sequence Number of the port should be increased automatically

- Unload Ready: a cassette is ready to unload from the port.

- Empty: cassette is removed from the port. This is the state when cassette removal from Port is detected.

- Blocked: the port is temporarily not allowed to use (pausing); for example, accessing to a cassette in the port is not allowed. Port can be changed to this state automatically if the port is out of normal functional status. This state is controlled by Indexer by its own judgment, and in this state, Robot Command for the Port will not be downloaded.

In this time, Equipment Status depends on state of the transfer unit, if all transfer unit is down then equipment status is Down, otherwise, when the transfer unit is dispatching, its status is Run.

Port States Transition Condition

Figure 6-2Port State Change Diagram

| Current State | New State | Condition / Trigger |
| --- | --- | --- |
| Empty | Load Ready | Port is ready to receive a cassette. When this state, the port has completed I/O hand-shaking process with Cassette Transfer Equipment for Cassette transfer out, and all I/O should be ready to start a Cassette receiving hand-shaking (I/O hand shaking process definition with Cassette Transfer Equipment is out of this specification scope. Any hand shaking process agreed between Indexer Vendor and Storage/Transfer Vendor could be used) |
| Load Ready | Loaded | Cassette is loaded but not yet clamped. |
| Loaded | In Use | The all following requirement has to be completed. - Cassette is loaded and clamping is completed - Cassette ID read is completed. - Cassette glass count(slot existence) mapping is completed, if Port is equipped with slot mapping sensor. - Cassette is ready to be accessed for fetching out/storing of Job; all internal processes have to be completed, if any. |
| Loaded | Unload Ready | Unclamping is completed. When Port is occurred error or need to control to unload by receiving ‘Unclamp’ command (EAS , equipment touch panel). |
| In Use | Unload Ready | Unclamping is completed. Unclamping action can be performed by receiving ‘Unclamp’ command from Host, or user operation at equipment touch panel. |
| In Use | Loaded | This state change is applied only when there was a command for re-clamping(ReClamp). But when Cassette Status is changed to Waiting for Processing or In Processing, the command never can conduct. Operator has to cancel or abort or end of lot process for re-clamp. For ‘ReClamp’ command, Unclamping action and Clamping action should be conducted automatically and consecutively. |
| Unload Ready | Empty | Cassette is removed from Port (such as sensor detects no Cassette on Port) |
| Loaded In Use Unload Ready | Blocked | If Equipment opts to decide pausing from normal processing according to pre-defined program condition (like Port is temporary out of normal functioning), or operator manually trigger process pause on Equipment (like touch panel) |

| Blocked | Loaded In Use Unload Ready | If Equipment opts to decide resume processing according to pre-defined program condition or operator manually trigger on Equipment (like touch panel) in order to be out of Blocked state. The new state should be the same as previous state before ‘Blocked’ state (i.e. In Use > Blocked > In Use, Unload Ready > Blocked > Unload Ready) |
| --- | --- | --- |

    - Cassette Status (Feeder Type & Independent Type Indexer Only)

The Cassette Status is related to processing status of Cassette. Indexer of Feeder Type cassette operation and Indexer of Independent Type (i.e. Robot dispatching from/to cassette is controlled by Indexer) should manage this data items.

- No Cassette: There is no cassette on the port.

- Waiting for Cassette Data: A cassette is currently loaded in a port and waiting for Cassette Map Data from Master (EAS) or Operator. Equipment should report this event, after clamping the cassette and (if port has BCR) reading the cassette ID and (if port has mapping unit) sensing the existence of job for each slot.

* i.e. Report this event after Port State turns in to ‘In Use’.

- Waiting for Start Command: Cassette Map has been received successfully, and waiting for Start Command from Master (EAS) or Operator.

- Waiting for Processing

  - Loading Port: Process Start Command has been received successfully, and waiting for Job fetching out start.

  - Unloading Port: Process Start Command has been received successfully, and waiting for Job inserting into Cassette.

- In Processing

  - Loading Port: The first Job has been fetched out from Cassette.

  - Unloading Port: The first Job has been stored into Cassette.

  - Buffer Port: Received Start Command.

- Process Completed: Process of Cassette has been finish, or cancel, or abort.

  - Loading Port: equipment reports when all Job has been fetched out from Cassette is empty.

  - Unloading Port: equipment reports when all Job has been stored into Cassette.

  - Equipment reports when equipment decides cancel or abort.

- Process Paused: Process of Cassette has been pause by EAS or operator.

Figure 6-3Diagram of Timing Chart for Cassette State Change

    - Cassette Control Command

The cassette control command is used to control equipment how to process the cassette on port.

- Cassette Map Download: When equipment requests the cassette map data for a port, EAS should download the data to the equipment. EAS writes the cassette map data in the local PLC of the equipment which has requested the data. And that time, EAS sends this command to the equipment. Equipment can process the cassette using the cassette map data. When Port state is ‘In Use’, Host can write the cassette map data for the cassette on the Port. Equipment has to validate the Cassette ID and Job existence of slot, and has to reply to Host.

- Clamp: this is to request to Port to conduct ‘clamping’ actions for the cassette on the Port. After ‘clamping’, the Port should follow the normal cassette loading sequence.

- Unclamp: this is to request to Port to conduct ‘unclamping’ actions for the cassette on the Port. After ‘unclamping’, the Port should follow the normal cassette loading sequence.

- ReClamp: this is to request to Port to conduct the sequential actions of ‘unclamping’ and ‘clamping’ again. After ‘clamping’, the Port should follow the normal cassette loading sequence.

The below is additional Cassette Control Command that is required for Feeder Type, Independent Type Indexer (and, all Indexer when Auto Dispatching mode is ‘Local Mode’) and operation by Cassette.

- Cassette Process Start: If the cassette map data that received from the EAS is normal, Equipment should request a cassette process start command to EAS. When receives this command from EAS, Equipment can start the cassette.

- Cassette Process Start By Count : Some Equipment need specific glass quantity to be started. This command is used for these types and includes the glass quantity to by started. Operation is same as Cassette Process Start Command. This command only is used that operator start processing manually in Equipment touch panel.

- Cassette Process Pause: EAS can give pause to a cassette. If receives this command, Equipment must pause the process. If robot (or conveyor) is currently fetching or storing a job to the cassette, the fetching or storing operation should be completed before Equipment switches the cassette status to pause.

- Cassette Process Resume: If process of a cassette is paused, Equipment can restart the process by this command.

- Cassette Process Cancel: If a cassette is not being processed, EAS can cancel the process. EAS uses this command. Equipment unclamps the cassette on the port.

- Cassette Process Abort: If a cassette is being processed, EAS can abort the process using this command. If robot (or conveyor) is currently fetching or storing a job to the cassette, the fetching or storing operation should be completed before Equipment switches the cassette status to abort.

- Cassette Process End : If a cassette is being processed, EAS can end the process using this command. Equipment unclamps the cassette on the port.

Figure 6-4Clamp/Reclamp Sequence

    - Return Code

It is a Return Code for the Cassette Control Command from Host. Equipment has to reply with Return Code against the following command.

- Cassette Map Download.

- Clamp, Unclamp, Reclamp.

    - Cassette Map Data

It is the Job Data of each slot in a cassette. EAS can write the data to local PLC of the equipment after cassette loading process has been completed.

    - Cassette Mapping State

It is state that has to skip of mapping or mapping in Port. This can set on touch panel of equipment before loading the cassette.

If mapping state is skip then equipment is not mapping.

    - PLC MAP

      - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Port#1 Port Status Block | Port Status | 1 | Data | INT | - : Load Ready(Load Request) - : Loaded - : In Use (Load Complete) - : Unload Ready (Unload Request) - : Empty (Unload Complete) - : Blocked |
|  | Cassette Sequence No | 1 | Data | INT | 1~65535 * Include this item only when cassette exists |
|  | Cassette ID (Box ID) | 10 | Data | ASC | H20 ~ H7E (except : H22( “ ), H27(‘) * Include this item only when cassette exists |
|  | Job(glass) Existence Slot | 5 | Data | Bit | On: Job Exist Off: No Job Exist [1st Word ] 00 ~ 15 Bit : Job Existence Slot 1 ~ 16 00 bit: Job Existence Slot 1 01 bit: Job Existence Slot 2 …. 15 bit: Job Existence Slot 16 [2nd Word] 00 ~ 15 Bit : Job Existence Slot 17 ~ 32 [3rd Word] 00 ~ 15 Bit : Job Existence Slot 33 ~ 48 [4th Word] 00 ~ 15 Bit : Job Existence Slot 49 ~ 64 [5h Word] 00 ~ 15it : Job Existence Slot 65 ~ 80 * Include this item only when Port Status is ‘In Use’ |
|  | Loading Cassette Type | 1 | Data | INT | 1: Actual Cassette 2: Empty Cassette * Include this item only when cassette exists |
|  | Q-Time Flag | 1 | Data | INT | 1: Normal Un-loading 2: Q-Time Over & Un-loading * Include this item only when Port Status is ‘Unload Ready’ |
|  | Cassette Mapping State | 1 | Data | INT | 1: mapping use 2: mapping not use |
|  | Cassette Status (Feeder, Independent Type only, or Local dispatching Mode) | 1 | Data | INT | - : No Cassette Exist - : Waiting for Cassette Data - : Waiting for Start Command - : Waiting for Processing - : In Processing |

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | - : Process Paused - : Process Completed |
|  | Cassette Control Command Return Code | 1 | Data | INT | [Cassette Control Command Return Code] - : Command OK - : CSTID is Invalid (Include Dense Box ID) 3: Cassette not exist - Already received - Other Error [Cassette Map Download Return Code] 1 : Command OK 2 : CSTID is Invalid (Include Dense Box ID) 4 : Already Received 5. Other Error - : Slot Information Mismatch (If no mapping sensor then don’t check) - : Job Type Mismatch - : Dummy Type Mismatch |
|  | Reserved | 9 |  |  |  |
| Port#2 Port Status Block | Same as Port#1 | 31 |  | - |  |
| Port#3 Port Status Block | Same as Port#1 | 31 |  | - |  |
| Port#4 Port Status Block | Same as Port#1 | 31 |  | - |  |
| Port#5 Port Status Block | Same as Port#1 | 31 |  | - |  |
| Port#6 Port Status Block | Same as Port#1 | 31 |  | - |  |
| Port#7 Port Status Block | Same as Port#1 | 31 |  | - |  |
| Port#8 Port Status Block | Same as Port#1 | 31 |  |  |  |
| Port#9 Port Status Block | Same as Port#1 | 31 |  |  |  |
| Port#10 Port Status Block | Same as Port#1 | 31 |  |  |  |

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) |
| --- | --- | --- | --- | --- | --- | --- |
| Port Event | Port#1 Empty Change Report | 1 | Event | - On: Fire Event - Off: Event End/Timeout | Port#1 Status Block |  |
|  | Port#1 Load Ready Change Report | 1 | Event | - On: Fire Event - Off: Event End/Timeout | Port#1 Status Block |  |
|  | Port#1 Loaded Change Report | 1 | Event | - On: Fire Event - Off: Event End/Timeout | Port#1 Status Block |  |

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) |
| --- | --- | --- | --- | --- | --- | --- |
|  | Port#1 InUse Change Report | 1 | Event | - On: Fire Event - Off: Event End/Timeout | Port#1 Status Block |  |
|  | Port#1 Unload Ready Change Report | 1 | Event | - On: Fire Event - Off: Event End/Timeout | Port#1 Status Block |  |
|  | Port#1 Blocked Change Report | 1 | Event | - On: Fire Event - Off: Event End/Timeout | Port#1 Status Block |  |
|  | Port#1 Cassette Status Change Report (Feeder, Independent Type, or local dispatching Mode) | 1 | Event | - On: Fire Event - Off: Event End/Timeout | Port#1 Port Status Block - Cassette Status |  |
| Port Command Reply | Port#1 Cassette Control Command Reply | 1 | Reply | - On: Reply - Off: Reply End | Port#1 Port Status Block - Cassette Control Command Return Code |  |
| Port Event & Command Reply | Port#2 Port Event & Command ReplySame as Port#1 | 8 | Event |  |  |  |
|  | Port#3 Port Event & Command Reply Same as Port#1 | 8 | Event |  |  |  |
|  | Port#4 Port Event & Command Reply Same as Port#1 | 8 | Event |  |  |  |
|  | Port#5 Port Event & Command Reply Same as Port#1 | 8 | Event |  |  |  |
|  | Port#6 Port Event & Command Reply Same as Port#1 | 8 | Event |  |  |  |
|  | Port#7 Port Event & Command Reply Same as Port#1 | 8 | Event |  |  |  |
|  | Port#8 Port Event & Command Reply Same as Port#1 | 8 | Event |  |  |  |
|  | Port#9 Port Event & Command Reply Same as Port#1 | 8 | Event |  |  |  |
|  | Port#10 Port Event & Command Reply Same as Port#1 | 8 | Event |  |  |  |

      - Master (EAS) – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Port#1 Cassette Control Block | Cassette Control Command | 1 | Data | INT | 1: Cassette Map Download - : Clamp - : Unclamp - : Reclamp - : Cassette Process Start - : Cassette Process Start By Count |

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | - : Cassette Process Pause - : Cassette Process Resume - : Cassette Process Abort - : Cassette Process Cancel - : Cassette Process End |
|  | Job Existence | 5 | Data | BIT | ※ This item write when cassette map download. On: Job Exist Off: No Job Exist [1st Word ] 00 ~ 15 Bit : Job Existence Slot 1 ~ 16 00 Bit: Job Existence Slot 1 01 Bit: Job Existence Slot 2 …. 15 Bit: Job Existence Slot 16 [2nd Word] 00 ~ 15 Bit : Job Existence Slot 17 ~ 32 [3rd Word] 00 ~ 15 Bit : Job Existence Slot 33 ~ 48 [4th Word] 00 ~ 07 Bit : Job Existence Slot 49 ~ 56 [5h Word] 00 ~ 15it : Job Existence Slot 65 ~ 80 ※ Mismatch case EQ function EQ has mapping unit: Return Command Error EQ has no mapping unit: When fetch out a glass, If mismatch then fire alarm and operator call. |
|  | Slot To Process | 5 | Data | BIT | Slot Map to process 1: to process 0: not to process Example ) 000011100110000 |
|  | Job Count | 1 | Data | INT | ※ This item write when Cassette Process Start By Count. [This function use special line only] 1~128 0: Process All Glass 1: Empty Cassette |
| Port#2 Cassette Control Block | Same as Port#1 | 12 | Data | INT |  |
| Port#3 Cassette Control Block | Same as Port#1 | 12 | Data | INT |  |
| Port#4 Cassette Control | Same as Port#1 | 12 | Data | INT |  |

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Block |  |  |  |  |  |
| Port#5 Cassette Control Block | Same as Port#1 | 12 | Data | INT |  |
| Port#6 Cassette Control Block | Same as Port#1 | 12 | Data | INT |  |
| Port#7 Cassette Control Block | Same as Port#1 | 12 | Data | INT |  |
| Port#8 Cassette Control Block | Same as Port#1 | 12 | Data | INT |  |
| Port#9 Cassette Control Block | Same as Port#1 | 12 | Data | INT |  |
| Port#10 Cassette Control Block | Same as Port#1 | 12 | Data | INT |  |

      - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| Port Event Reply | Port#1 Empty Change Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Port#1 Load Ready Change Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Port#1 Loaded Change Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Port#1 InUse Change Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Port#1 Unload Ready Change Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Port#1 Blocked Change Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Cassette Status Change Report Reply (Feeder, Independent Type only) | 1 | Reply | - On: Reply - Off: Reply End |  |  |
| Port Command | Cassette Control Command | 1 | CMD | - On: Fire Command - Off: Command End/Timeout | Port#1 Control Block - Cassette Control Command - Job Existence - Job Count | Local PLC should analyze the cassette control command of the Master (EAS) and reply with return code. |
| Port Event Reply & Command | Port#2 Event Reply & CommandSame as Port#1 | 8 | Reply | - On: Reply - Off: Reply End |  |  |

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
|  | Port#3 Event Reply & Command Same as Port#1 | 8 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Port#4 Event Reply & Command Same as Port#1 | 8 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Port#5 Event Reply & Command Same as Port#1 | 8 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Port#6 Event Reply & Command Same as Port#1 | 8 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Port#7 Event Reply & Command Same as Port#1 | 8 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Port#8 Event Reply & Command Same as Port#1 | 8 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Port#9 Event Reply & Command Same as Port#1 | 8 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Port#10 Event Reply & Command Same as Port#1 | 8 | Reply | - On: Reply - Off: Reply End |  |  |

      - Local PLC – File Register (R Area)

This standard cassette map data for 20 slot cassette, other type of cassette refer to PLC Address Map.

        - Standard Cassette Map Data (25 slot)

| Standard Cassette Map Data |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Port 1 |  |  | Port 2 |  |  | Port 3 |  |  | Port 4 |  |  |
| Add |  | Slot No | Add |  | Slot No | Add |  | Slot No | Add |  | Slot No |
| R | 0000 | Slot#01 Job Data | R | 3600 | Slot#01 Job Data | R | 7200 | Slot#01 Job Data | R | 10800 | Slot#01 Job Data |
| R | 0064 | Slot#02 Job Data | R | 3664 | Slot#02 Job Data | R | 7264 | Slot#02 Job Data | R | 10864 | Slot#02 Job Data |
| R | 0128 | Slot#03 Job Data | R | 3728 | Slot#03 Job Data | R | 7328 | Slot#03 Job Data | R | 10928 | Slot#03 Job Data |
| … |  | … | … |  | … | … |  | … | … |  |  |
| R | 1472 | Slot#24 Job Data | R | 5072 | Slot#24 Job Data | R | 8672 | Slot#24 Job Data | R | 12272 | Slot#24 Job Data |
| R | 1536 | Slot#25 Job Data | R | 5136 | Slot#25 Job Data | R | 8736 | Slot#25 Job Data | R | 12336 | Slot#25 Job Data |

| Standard Cassette Map Data |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Port 5 |  |  | Port 6 |  |  | Port 7 |  |  | Port 8 |  |  |
| Add |  | Slot No | Add |  | Slot No | Add |  | Slot No | Add |  | Slot No |
| R | 14400 | Slot#01 Job Data | R | 18000 | Slot#01 Job Data | R | 21600 | Slot#01 Job Data | R | 25200 | Slot#01 Job Data |
| R | 14464 | Slot#02 Job Data | R | 18064 | Slot#02 Job Data | R | 21664 | Slot#02 Job Data | R | 25264 | Slot#02 Job Data |
| R | 14528 | Slot#03 Job Data | R | 18128 | Slot#03 Job Data | R | 21728 | Slot#03 Job Data | R | 25328 | Slot#03 Job Data |
| … |  |  | … |  | … |  |  | … |  |  |  |
| R | 15872 | Slot#24 Job Data | R | 19472 | Slot#24 Job Data | R | 23072 | Slot#24 Job Data | R | 26672 | Slot#24 Job Data |
| R | 15936 | Slot#25 Job Data | R | 19536 | Slot#25 Job Data | R | 23136 | Slot#25 Job Data | R | 26736 | Slot#25 Job Data |

| Standard Cassette Map Data |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Port 9 |  |  | Port 10 |  |  |
| Add |  | Slot No | Add |  | Slot No |
| R | 28800 | Slot#01 Job Data | R | 32400 | Slot#01 Job Data |
| R | 28864 | Slot#02 Job Data | R | 32464 | Slot#02 Job Data |
| R | 28928 | Slot#03 Job Data | R | 32528 | Slot#03 Job Data |
| … |  |  | … |  | … |
| R | 30272 | Slot#24 Job Data | R | 33872 | Slot#24 Job Data |
| R | 30336 | Slot#25 Job Data | R | 33936 | Slot#25 Job Data |

    - Communication Scenario

All of Port Status Change Event must happen by order but if the fire of event by order is not guaranty then the equipment must have responsible when the related problem of port causes.

      - Cassette Normal Scenario

When cassette map downloaded in equipment , EAS automatically has to cancel if slot map mismatched.

Equipment also has to verify for slot map mismatch after being received Cassette Map Data from EAS, and report the verification result in the Reply event. In this time, if EAS receives return code ‘NG’ then sends to equipment unclamp cassette control command.

Figure 6-5 Cassette Normal Scenario

Figure 6-6 Feeder Cassette Normal Scenario

      - Unclamp

Unclamp command has to send to equipment when port status is ‘loaded’ or ‘In Use’

And Equipment can send to EAS Port Status ‘Abort/Cancel’ when need to abort/cancel the cassette on the Port.

Equipment should provide ‘Abort’, ‘Cancel’ button for operator.

When Cassette ID validate on port, EAS automatically has to unclamp if cassette ID mismatched Details are to be indexer operation scenario

- When Port status is in use

Figure 6-7 Unclamp Scenario 1

- When Port Status is loaded

Figure 6-8 Unclamp Scenario 2

- When unclamp request on equipment touch panel

Figure 6-9 Unclamp Scenario 3

      - Clamp/Unclamp

When port status is ‘Unload Ready’

Figure 6-10 Clamp/Unclamp Command

      - Clamp/Reclamp

1)  When port status is ‘InUse’

Figure 6-11 ReClamp Scenario

      - Unclamp (Feeder Type or Independent Type Indexer or Local Dispatching Mode)

Figure 6-12 Cassette Process End or Cassette Process Abort

  - Port WIP Management

Equipment is required to manage real time WIP (Job existence in each slot) in cassette on each Port.

- When received Cassette Map Download from Host, EQP writes Port WIP data.

- Update Port WIP data when a Job is fetched out and/or stored into the cassette.

  - remove Job existence from Port WIP data when the Job is fetched out

  - add Job existence to Port WIP data when the Job is stored in cassette on the Port

- Equipment has to manage Port WIP data in real-time according to actual Job existence status, regardless of CIM operation mode (online/offline, auto/manual, etc.)

- This requirement applies to Port in Indexer (Loader, Unloader) and Buffer that is accessed by transfer module of Indexer

    - PLC MAP

      - Local PLC – File Register(R Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Port #1 WIP Data Block | Slot#1 Cassette Sequence No | 1 | Data | INT | 1~65535 |
|  | Slot#1 Job Sequence No | 1 | Data | INT | 1~65535 |
|  | Slot#1 Processing State | 1 | Data | INT | - Port Type : PB , PP - : Waiting process(glass exist) - : Process completed ( The glass is got from Process Equipment) - : Processing( The glass is not exist in this CST and when Robot get the glass; or In Process Equipment) -Port Type : PL - : Waiting process(glass exist) - : Process completed (The glass is not exist in this CST and when Robot get the glass; or In Process Equipment) -Port Type : PU, PP - : Waiting process(glass not exist) - : Process completed ( The glass is got from Process Equipment) |
|  | Slot#N Cassette Sequence No | 1 | Data | INT | 1~65535 |
|  | Slot#N Job Sequence No | 1 | Data | INT | 1~65535 |
|  | Slot#N Processing State | 1 | Data | INT | Same as Slot#1 Processing State |
| Port #N WIP Data Block | Same to Port#1 WIP Data Block |  |  |  |  |

  - Port Cassette Size (only End Cell)

Indexer can set the Port Cassette Size(X, Y) value on touch screen. If Cassette Status unclamp, Port Cassette Size set

    - PLC MAP

      - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Port#1 Block | Port Cassette Size X | 1 | Data | INT | 1~65535 |
|  | Port Cassette Size Y | 1 | Data | INT | 1~65535 |
| … | … |  |  |  |  |
| Port#N Block | Port Cassette Size X | 1 | Data | INT | 1~65535 |
|  | Port Cassette Size Y | 1 | Data | INT | 1~65535 |

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) |
| --- | --- | --- | --- | --- | --- | --- |
| Port Event | Port #1 Port Cassette Size Change Report | 1 | Event | - On: Fire Event - Off: Event End/Timeout | Port#1 Block *That Item : Port Cassette Size X, Y | Reply for Receiving |
|  | … |  |  |  |  |  |
|  | Port #N Port Cassette Size Change Report | 1 | Event | - On: Fire Event - Off: Event End/Timeout | Port#N Block | Reply for Receiving |

      - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| Port Event Reply | Port #1 Port Cassette Size Change Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |
|  | … |  |  |  |  |  |
|  | Port #N Port Cassette Size Change Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |

- Robot Management

  - Robot Control Command

    - Command format

One command is composed of up to four (4) command items of unit movement. If more than one (1) command items are included, equipment has to conduct each item consecutively without unnecessary time gap between the movement.

Figure 7-1 Command Format

- SequenceNo : Command sequence number generated for each command by EAS.

- RCMD : Robot Command

- Arm No : Robot Arm number. Arm number increases from bottom to up or from left to right at the same indexer.

- Get Position : If robot is received the Get command, Robot should move here.

- Put Position : If robot is received the Put command, Robot should move here.

- Get Slot No : If robot is received the Get command, it is Glass position in cassette

If robot arm move more than one at the same time, Slot No is the Arm No#1 to work with here.

- Put Slot No : If robot is received the Put command, it is Glass position in cassette

If robot arm move more than one at the same time, Slot No is the Arm No#1 to work with here.

- Sub CMD : Preparatory action

| Sub CMD |  |  |
| --- | --- | --- |
| None | 0 | No pre-action |
| Get Ready | 1 | Indexer robot moves and waits until it can execute get action |
| Put Ready | 2 | Indexer robot moves and waits until it can execute put action |

    - Data Description

RCMD	Arm No	Index (Position /Slot)

| None | 0 | None | 0 | Position | 0 ~ 65535 |
| --- | --- | --- | --- | --- | --- |
| Robot Home (with Robot Arm Home) | 1 |  |  |  |  |
| Transfer (from-to) | 2 |  |  |  |  |
|  |  |  |  | Slot No | 1 ~ 65535 |
| Move (to) | 3 | Arm #1 | 1 |  |  |
| Get | 4 | Arm #2 | 2 |  |  |
| Put | 5 | All arm | 99 |  |  |
| Exchange | 6 |  |  |  |  |
| Two Action Exchange (required for a specific equipment only) | 7 |  |  |  |  |
| Command Clear | 8 |  |  |  |  |
| Batch Get | 9 |  |  |  |  |
| Batch Put | 10 |  |  |  |  |

- Put : Indexer arm un-loads its glass onto the unit of counter stream Equipment.

- Get : Indexer arm loads a glass onto its one of arms from the unit of counter stream Equipment.

- Transfer(from-to) : Indexer executes get command at the get-position, moves and put command at the put- position

- Move(to) : Indexer robot moves to the specific position

- Exchange : Default exchange command.

- Two action exchange : Indexer executes exchange command with z-axis moving.

This function is used only for selected equipment where this specific instruction is required by equipment vendor

- Batch Get/Put : Two or more robot arms conducts get or put action simultaneously(one-action), sequentially(two-action)

Command Example :

| RCMD |  | Arm No | Get Position | Put Position | Get Slot No | Put Slot No | Sub CMD |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Robot Home | 1 | - | - | - | - | - | -- |  |
| Transfer (from- to) | 2 | O (to get & put) | O (From) | O (To) | O (From) | O (To) | - |  |
| Move(to) | 3 | O (to get) | O |  | O |  | Get Ready, | Wait Front to Get, |
|  | 3 | O (to put) |  | O |  | O | Put Ready | Wait Front to Put |
| Get | 4 | O | O | - | O | - | - |  |
| Put | 5 | O | - | O | - | O | - |  |
| One Action Exchange | 6 | O (Get Arm No) | O | O | O | O | - | Get, Put position is same |

| Command Clear | 8 | - | - | - | - | - | -- |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Batch Get | 9 | O (All ) | O (Get) |  | O (lowest slot no) |  | - |  |
| Batch Put | 10 | O (All) |  | O (Put) |  | O (lowest slot no) | - |  |

    - Robot CMD Ack

| CODE | Comment | operation | Description |
| --- | --- | --- | --- |
| 1 | Ack is OK |  |  |
| 2 | Busy, Try again |  | When Command is executing and Command Buffer exists Command |
| 3 | CIM Mode is OFF |  |  |
| 4 | Command is already executing | Ignore | When the same command is present in Command or Command Buffer |
| 5 | At least one parameter invalid | Check the parameter | When Command parameter is wrong |
| 6 | Cannot perform now (Hardware problem) |  | When the problem occurred in Robot |
| 7 | Glass exist in slot |  | In case, Unloading |
| 8~20 | Reserved |  |  |
| 21~99 | Vendor Define |  |  |

    - PLC MAP

      - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Robot CMD Reply Block | Robot CMD Sequence No | 1 | Data | INT | This is 1st Sequence No |
|  | Robot CMD Ack | 1 | Data | INT | - : Ack is OK - : Busy, Try again - : CIM Mode is OFF - : Command is already executing - : At least on parameter invalid - : Cannot perform now 7~20: Reserved 21~99: Vendor Define |

      - L

o c a l

P L C

– L

i n k

R

e l a y

( B

A

r e a

)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) |
| --- | --- | --- | --- | --- | --- | --- |
| Robot Event | Robot CMD Reply | 1 | Event | - On: Fire Event - Off: Event End | Robot CMD Reply Block | Reply for Receiving |

      - Master (EAS) – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Robot CMD Block | 1st Sequence No | 1 | Data | INT |  |
|  | 1st RCMD | 1 | Data | INT | - : Robot Home - : Transfer - : Move - : Get - : Put - : One Action Exchange 7: Two Action Exchange: 8: Command Clear 9: Batch Get 10 Batch Put |
|  | 1st Arm No | 1 | Data | INT | 1 : Arm #1 (lower) 2 : Arm #2 (upper) |

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
|  | 1st Get Position | 1 | Data | INT |  |
|  | 1st Put Position | 1 | Data | INT |  |
|  | 1st Get Slot No | 1 | Data | INT |  |
|  | 1st Put Slot No | 1 | Data | INT |  |
|  | 1st Sub CMD | 1 | Data | INT | - : Get Ready - : Put Ready |
|  | 2st Sequence No … … | 1 | Data | INT | Same as 1st Sequence No |
|  | 3st Sequence No … … | 1 | Data | INT | Same as 1st Sequence No |
|  | 4st Sequence No | 1 | Data | INT | Same as 1st Sequence No |
|  | 4th RCMD | 1 | Data | INT |  |
|  | 4thArm No | 1 | Data | INT |  |
|  | 4th Get Position | 1 | Data | INT |  |
|  | 4th Put Position | 1 | Data | INT |  |
|  | 4th Get Slot No | 1 | Data | INT |  |
|  | 4th Put Slot No | 1 | Data | INT |  |
|  | 4th Sub CMD | 1 | Data | INT |  |

      - Master EAS – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action) |
| --- | --- | --- | --- | --- | --- | --- |
| Robot CMD | Robot CMD | 1 | CMD | - On: Fire Event - Off: Event End | Robot CMD Block | If robot is received the Robot command, Robot should execute |

    - Robot Command Buffer

- EAS sends Robot/Conveyor command to INDEXER/CONVEYOR CONTROLLER.

- CONTROLLER prepares internal command buffer for buffering EAS command.

- When current sequence of robot action is completed, controller fetches the command from the internal command buffer.

- EAScould download command anytime.

  - Command Fetch Report

When Indexer fetches command from command buffer, should report Command Fetch Repot

    - PLC MAP

      - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Robot CMD Fetch Block | Robot CMD Sequence No | 1 | Data | INT |  |
|  | RCMD | 1 | Data | INT |  |
|  | Arm No | 1 | Data | INT | - : Arm #1 (lower) - : Arm #2 (upper) |
|  | Get Position | 1 | Data | INT |  |
|  | Put Position | 1 | Data | INT |  |
|  | Get Slot No | 1 | Data | INT |  |
|  | Put Slot No | 1 | Data | INT |  |
|  | Sub CMD | 1 | Data | INT | - : Get Ready - : Put Ready |

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) |
| --- | --- | --- | --- | --- | --- | --- |
| Robot Event | Robot CMD Fetch Report | 1 | Event | - On: Fire Event - Off: Event End | Robot CMD Fetch Block | Reply for Receiving |

      - Master (EAS) – Link Register (W Area)

none.

      - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| Robot Event Reply | Robot CMD Fetch Reply | 1 | Reply | - On: Reply - Off: Reply End |  | Reply for Receiving |

When Indexer command buffer fetched command, Command Fetch Repot send to EAS.

  - Command Result Report

The indexer must be report the execution result of Robot Control Command that it is occurred by group of commands even if a lot of command of robot is executed. If the result is NG then the indexer has to OPCall.

For example, if EAS send to equipment by one command with get , put command , Equipment should report to EAS by one event with get ,put command result.

Robot CMD Sequence No is 1st Sequence No.

    - PLC MAP

      - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
|  | Robot CMD Sequence No | 1 | Data | INT | 1 ~ 65535 |
|  | 1st CMD Result Data | 1 | Data | INT | 0: None |
|  | 2nd CMD Result Data | 1 | Data | INT | 1 : OK |
|  | 3rd CMD Result Data | 1 | Data | INT | 2 : EQ Link Signal NG (Downstream Receive Able |
|  |  |  |  |  | OFF) |
|  | 4th CMD Result Data |  |  |  |  |
| Robot CMD |  |  |  |  | 3 : EQ Link Signal NG (Send Start not ON) |
| Result Data |  |  |  |  | 4 : Link Signal Timeout (T1) |
| Block |  |  |  |  | 5 : put error when the Exchange executes for |
|  |  | 1 | Data | INT | special that CVD only |
|  |  |  |  |  | 6 : others |
|  |  |  |  |  | 7: Link Signal Initial (Only use Ecell) |
|  |  |  |  |  | 99 : *PASS (Only use Ecell) |
|  | Robot Position | 1 | Data | INT | Refer to Robot Position |

* Loader need Auto-Scan function because it doesn’t slot sensor and Arm of robot must get first slot.

Robot get not first slot, than dangerous because between slot and slot very narrow in Ecell.

When Arm of robot get, if slot is empty than Arm of robot is go up to next slot by Auto-Scan function And loader have to report Robot CMD Result Data Block Sub Items is “99”

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) |
| --- | --- | --- | --- | --- | --- | --- |
| Robot Event | Robot CMD Result Report | 1 | Event | - On: Fire Event - Off: Event End | Robot CMD Result Data Block | Reply for Receiving |

      - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| Robot Event Reply | Robot Result Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |

  - Robot Arm Event

- Robot Arm Load Report:

Report Timing: When the glass is loaded on robot arm, the indexer should report Arm Load Report.

- Robot Arm Unload Report

Report Timing: When the glass is unloaded from robot arm, the indexer should report Arm Unload Report.

    - PLC MAP

      - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Robot Arm Job Data Block | 1st Arm CST Sequence No | 1 | Data | INT |  |
|  | 1st Arm Job Sequence No | 1 | Data | INT |  |
|  | 2nd Arm CST Sequence No | 1 | Data | INT |  |
|  | 2nd Arm Job Sequence No | 1 | Data | INT |  |
|  | 3rd Arm CST Sequence No | 1 | Data | INT |  |
|  | 3rd Arm Job Sequence No | 1 | Data | INT |  |
|  | 4th Arm CST Sequence No | 1 | Data | INT |  |
|  | 4th Arm Job Sequence No | 1 | Data | INT |  |
|  | Arm No | 1 | Data | INT | - : Arm #1 (lower) - : Arm #2 (upper) |
|  | Robot Position | 1 | Data | INT |  |

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) |
| --- | --- | --- | --- | --- | --- | --- |
| Robot Event | Robot Arm Substrate Load Report | 1 | Event | - On: Fire Event - Off: Event End | Robot Arm Job Data Block | Reply for Receiving |
|  | Robot Arm Substrate Unload Report | 1 | Event | - On: Fire Event - Off: Event End | Robot Arm Job Data Block | Reply for Receiving |

      - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| Robot Event Reply | Robot Arm Substrate Load Reply | 1 | Event | - On: Fire Event - Off: Event End |  | Reply for Receiving |
|  | Robot Arm Substrate Unload Reply | 1 | Event | - On: Fire Event - Off: Event End |  | Reply for Receiving |

  - Robot MonitoringBlock

Robot Arm Monitoring is area to real-time monitoring the glass information on robot arms. The indexer should update the CST Sequence No/ Job Sequence No to real-time Robot Arm Monitoring Block when glass does exist or glass does not exist. If glass does not exist then CST Sequence No/ Job Sequence No is zero.

    - PLC MAP

      - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Robot Arm Monitoring Block | Robot Arm#01 Cassette Sequence No | 1 | Data | INT |  |
|  | Robot Arm#01 Job Sequence No | 1 | Data | INT |  |
|  | Robot Arm#02Cassette Sequence No | 1 | Data | INT |  |
|  | Robot Arm#02Job Sequence No | 1 | Data | INT |  |
|  | Robot Arm#03Cassette Sequence No | 1 | Data | INT |  |
|  | Robot Arm#03Job Sequence No | 1 | Data | INT |  |
|  | Robot Arm#04Cassette Sequence No | 1 | Data | INT |  |
|  | Robot Arm#04Job Sequence No | 1 | Data | INT |  |

  - Dispatching Mode

    - Auto Dispatching

- EAS Dispatching Mode : This mode is that EAS controls Indexer robot. EAS should send robot control command to Indexer.

- Local Dispatching Mode : This mode is that EAS don’t control Indexer robot. EAS should not send robot control command to Indexer. Indexer robot should transfer the glass by own Indexer equipment.

When if equipment receives mode change command from EAS or mode change is set at equipment touch panel, and at that time while Robot is conducting a dispatching movement, the Mode change should be activated after Robot finishes current dispatching command. And also the mode has to be changed when all port of loader is unclamped but the unloader is decided by Operator.

At least One Mode need to On, but It can’t both mode On same time(EAS On -> Local OFF, EAS Off -> Local On)

    - Robot Control Mode (Indexer Only).

- Manual Mode : This is to control Job dispatching by manual at equipment. At this Mode, EAS will not download Robot Command for auto dispatching

- Auto Mode : This is to control Job dispatching by automatic Robot Command from EAS.

At least One Mode need to On, but It can’t both mode On same time(Manual On -> Auto OFF, Manual Off -> Auto On)

    - PLC MAP

      - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Indexer Dispatching Mode Block | Dispatching Mode | 1 | Data | INT | - : EAS - : Local |
|  | Robot Control Mode | 1 | Data | INT | - : Auto - : Manual |
| Indexer Dispatching Mode Return Block | Return Code | 1 | Data | INT | - : OK - : NG Equipment error - : NG other OP is working |

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) |
| --- | --- | --- | --- | --- | --- | --- |
| Indexer Event | Dispatching Mode Change Report | 1 | Event | - On: Fire Event - Off: Event End | Indexer Dispatching Mode Block -Dispatching Mode | Reply for Receiving |
|  | Robot Control Mode Change Report | 1 | Event | - On: Fire Event - Off: Event End | Indexer Dispatching Mode Block -Robot Control Mode | Reply for Receiving |
| Indexer Command Reply | Dispatching Mode Command Reply | 1 | Event | - On: Fire Event - Off: Event End | Indexer Dispatching Mode Return Block | Reply for Command |

      - Master (EAS) – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Indexer Dispatching Mode Command Block | Dispatching Mode | 1 | Data | INT | - : EAS - : Local |

      - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| Indexer Command | Dispatching Mode Change Command | 1 | CMD | - On: Fire Event - Off: Event End | Indexer Dispatching Mode Command Block |  |
| Indexer | Dispatching Mode | 1 | Event | - On: Fire Event |  |  |

| Event Reply | Change Report Reply |  |  | - Off: Event End |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Robot Control Mode Change Report Reply | 1 | Event | - On: Fire Event - Off: Event End |  |  |

  - Robot Position

Number Assignment Rule: The position number of the robot is given to In/Out Station. Ref –Equipment Operation Scenario

Unit or buffer Position in Indexer is from 90 to 99.

- Monitoring Block

  - C/V Monitoring

C/V Monitoring is area to real-time monitoring by Indexer This area is used to glass dispatch by EAS.

The indexer should write Cassette Sequence Number and Job Sequence Number at the plc address before send able signal turns on.

* Send able & Receive able Signal, refer to Link Signal Specification section 1.1

If glass does not exist on C/V then cassette sequence no & job sequence number is zero.

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| C/V Monitoring #1 Block | CV Slot#1 Cassette Sequence No | 1 | Data | INT |  |
|  | CV Slot#1 Job Sequence No | 1 | Data | INT |  |
|  | CV Slot#2 Cassette Sequence No | 1 | Data | INT |  |
|  | CV Slot#2 Job Sequence No | 1 | Data | INT |  |
| C/V Monitoring #2 Block | Same as C/V Monitoring #1 Block | 4 | Data | - |  |
| C/V Monitoring #3 Block | Same as C/V Monitoring #1 Block | 4 | Data | - |  |
| C/V Monitoring #4 Block | Same as C/V Monitoring #1 Block | 4 | Data | - |  |
| C/V Monitoring #5 Block | Same as C/V Monitoring #1 Block | 4 | Data | - |  |
| C/V Monitoring #6 Block | Same as C/V Monitoring #1 Block | 4 | Data | - |  |
| C/V Monitoring #7 Block | Same as C/V Monitoring #1 Block | 4 | Data | - |  |
| C/V Monitoring #8 Block | Same as C/V Monitoring #1 Block | 4 | Data | - |  |

  - Buffer Monitoring

* ODF equipment will use separate data are for Buffer monitoring (refer to ODF special section)

Equipment should update the Cassette Sequence No & Job Sequence No to real-time at Buffer Monitoring when glass is stored or glass is fetched out.

If glass does not exist at buffer slot then cassette sequence no & job sequence no is zero.

      - Local PLC – Link Register (R Area)

It is used the Buffer Monitoring Block in order to viewing the Buffer in Indexer when operator want to see on user interface; Such as EAS UI or Equipment touch screen.

| Block | Sub Section | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Buffer Slot#1 Cassette Sequence No | 1 | Data | INT |  |
|  |  | Buffer Slot#1 Job Sequence No | 1 | Data | INT |  |
|  |  | Buffer Slot#1 Glass ID or Panel ID | 10 | Data | ASC | GlassID |
|  |  | Buffer Slot#1 Dummy Type | 1 | Data | INT | This item is available when Job Type is Dummy. |
|  |  |  |  |  |  | 0: TFT |
|  |  |  |  |  |  | 1: CF |
|  |  | Buffer Slot#1 Job Type | 1 | Data | INT | 2: Dummy |
|  |  |  |  |  |  | 3: All |
|  |  |  |  |  |  | 4: POL |
|  |  |  |  |  |  | 5: Reserved |
|  |  | Buffer Slot#1 Used Count(only dummy) | 1 | Data | INT | Only for Dummy |
| Buffer #1 Monitoring Block | Slot#1 |  |  |  |  |  |
|  |  | Buffer Slot#1 Start Time : Stored time | 3 | Data | BCD | 00 ~ 07 Bit :A.D Year(00~99) 08 ~ 15 Bit :Month(01~12) |
|  |  |  |  | Data | BCD | 00 ~ 07 Bit :Day(01~31) 08 ~ 15 Bit :Hour(00~23) |
|  |  |  |  | Data | BCD | 00 ~ 07 Bit :Minute(00~59) |
|  |  |  |  |  |  | 08 ~ 15 Bit :Second(00~59) |
|  |  | Buffer Slot#1 Expired Time(only dummy) |  | Data | BCD | 00 ~ 07 Bit :A.D Year(00~99) 08 ~ 15 Bit :Month(01~12) |
|  |  | * This item is used by set expired count by equipment (Operator) which is the time after move to buffer when the dummy reach to expired count. | 3 | Data | BCD | 00 ~ 07 Bit :Day(01~31) 08 ~ 15 Bit :Hour(00~23) |
|  |  |  |  | Data | BCD | 00 ~ 07 Bit :Minute(00~59) 08 ~ 15 Bit :Second(00~59) |
|  |  |  |  |  |  | 0: Normal |
|  |  | Buffer Slot#1 State | 1 | Data | INT | 1: NG |
|  |  |  |  |  |  | 9: Disable |
|  | ~ Slot #n | Same as above |  |  |  |  |
| Buffer #N Monitoring Block |  | Same as Buffer #1 Monitoring Block | 23 | Data | - |  |

This is for Pol only

| Block | Sub Section | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- | --- |
| Buffer #1 Monitoring Block | Slot#1 | Buffer Slot#1 Cassette Sequence No | 1 | Data | INT |  |
|  |  | Buffer Slot#1 Job Sequence No | 1 | Data | INT |  |
|  |  | Buffer Slot#1 POL ID | 10 | Data | ASC |  |
|  |  | Buffer Slot#1 Dummy Type | 1 | Data | INT | This item is available when Job Type is Dummy. |
|  |  | Buffer Slot#1 Job Type | 1 | Data | INT | 0: TFT 1: CF |

| Block | Sub Section | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 2: Dummy 3: All 4: POL 5: Reserved |
|  |  | ProductID | 10 | Data | ASC |  |
|  |  | POL Size Width | 2 | Data | F | POL Size Width |
|  |  | POL Size Height | 2 | Data | F | POL Size Height |
|  | ~ Slot #n | Same as above |  |  |  |  |
| Buffer #N Monitoring Block |  | Same as Buffer #1 Monitoring Block | 28 | Data | - |  |

      - Local PLC – Link Register (W Area)

It is used the Buffer Monitoring Block in order to dispatch the glass on indexer with buffer.

| Block | Sub Section | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- | --- |
| Buffer #1 WIP Data Block | Slot#1 | Buffer Slot#1 Cassette Sequence No | 1 | Data | INT |  |
|  |  | Buffer Slot#1 Job Sequence No | 1 | Data | INT |  |
|  | ~ Slot #n | Same as above |  |  |  |  |
| Buffer #N WIP Data Block |  | Same as Buffer #1 WIP Data Block | 2 | Data | - |  |

This is for POL only

| Block | Sub Section | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- | --- |
| Buffer #1 WIP Data Block | Slot#1 | Buffer Slot#1 Cassette Sequence No | 1 | Data | INT |  |
|  |  | Buffer Slot#1 Job Sequence No | 1 | Data | INT |  |
|  |  | Product ID | 10 | Data | ASC |  |
|  |  | POL Size Width | 2 | Data | F | POL Size Width |
|  |  | POL Size Height | 2 | Data | F | POL Size Height |
|  | ~ Slot #n | Same as above |  |  |  |  |
| Buffer #N WIP Data Block |  | Same as Buffer #1 WIP Data Block | 2 | Data | - |  |

  - Indexer Monitoring

8.3.1.1	Local PLC – Link Register (B Area)

| Section | Sub Section | Item | Size(Bit) | Comment |
| --- | --- | --- | --- | --- |
| Indexer Monitoring Status | Robot Status (8 Bit) | Setup | 1 |  |
|  |  | Idle [When Robot has no command to conduct] | 1 |  |
|  |  | Run [When Robot is received Robot Command, and robot action something.] | 1 |  |
|  |  | Pause | 1 |  |
|  |  | Stop | 1 |  |
|  |  | Moving | 1 | 1:Robot Move Start 0:Robot Move End |
|  |  | Reserved | 1 |  |
|  |  | Reserved | 1 |  |
|  | Robot Position (8 Bit) | 0 Bit | 1 | 1 |
|  |  | 1 Bit | 1 | 2 |
|  |  | 2 Bit | 1 | 4 |
|  |  | 3 Bit | 1 | 8 |
|  |  | 4 Bit | 1 | 16 |
|  |  | 5 Bit | 1 | 32 |
|  |  | 6 Bit | 1 | 64 |
|  |  | Reserved | 1 |  |
|  | CV #1 State | Send Ready | 1 |  |
|  |  | Receive Ready | 1 |  |
|  |  | Exchange Enable | 1 |  |
|  | CV #2 State | Send Ready | 1 |  |
|  |  | Receive Ready | 1 |  |
|  |  | Exchange Enable | 1 |  |
|  | CV #3 State | Send Ready | 1 |  |
|  |  | Receive Ready | 1 |  |
|  |  | Exchange Enable | 1 |  |
|  | CV #4 State | Send Ready | 1 |  |
|  |  | Receive Ready | 1 |  |
|  |  | Exchange Enable | 1 |  |
|  | CV #5 State | Send Ready | 1 |  |
|  |  | Receive Ready | 1 |  |
|  |  | Exchange Enable | 1 |  |
|  | CV #6 State | Send Ready | 1 |  |
|  |  | Receive Ready | 1 |  |
|  |  | Exchange Enable | 1 |  |
|  | CV #7 State | Send Ready | 1 |  |
|  |  | Receive Ready | 1 |  |

|  |  | Exchange Enable | 1 |  |
| --- | --- | --- | --- | --- |
|  | CV #8 State | Send Ready | 1 |  |
|  |  | Receive Ready | 1 |  |
|  |  | Exchange Enable | 1 |  |
|  |  | Reserved | 8 |  |
|  | Arm State | Arm #1 | 1 | - : Off (Disable) - : On (Enable) |
|  |  | Arm #2 | 1 | - : Off (Disable) - : On (Enable) |
| Total |  |  | 64 |  |

  - Position State Monitoring

Position is a specific location in equipment where Job is placed, which could be a Unit, Sub Unit or a specific location in Unit/Sub Unit. The exact location of each Position is to be determined by engineer.

Equipment should update in real-time for Job existence state at each Position. If Job does not exist in a specific Position, then cassette sequence no & job sequence no for the corresponding data should be zero.

For example) Position Definition

-Indexer

- Position 01~02 : Arm#1 ~ Arm#2

-Process Equipment

- Position 01~02 : L/L#1

- Position 03~04 : L/L#2

- Position 05~08 : Transfer Chamber robot arm#1~#4

- Position 09~10 : Process Chamber stage#1~#2

    - PLC MAP

      - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| PositionStatu s Block | Position Glass Exist | 8 | Data | BIT | Bit : Glass Exist (0: Empty, 1 : Exist) |
| PositionGlass Block | Position 01 Cassette Sequence No | 1 | Data | INT | Cassette Sequence Number (1~65535) |
|  | Position 01 Job Sequence No | 1 | Data | INT | Job Sequence Number (1~65535) |
|  | Position 02 Cassette Sequence No | 1 | Data | INT | Cassette Sequence Number (1~65535) |

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
|  | Position 02 Job Sequence No | 1 | Data | INT | Job Sequence Number (1~65535) |
|  | Position 03 Cassette Sequence No | 1 | Data | INT | Cassette Sequence Number (1~65535) |
|  | Position 03 Job Sequence No | 1 | Data | INT | Job Sequence Number (1~65535) |
|  | Position 04 Cassette Sequence No | 1 | Data | INT | Cassette Sequence Number (1~65535) |
|  | Position 04 Job Sequence No | 1 | Data | INT | Job Sequence Number (1~65535) |
|  | Position 05 Cassette Sequence No | 1 | Data | INT | Cassette Sequence Number (1~65535) |
|  | Position 05 Job Sequence No | 1 | Data | INT | Job Sequence Number (1~65535) |
|  | Position 06 Cassette Sequence No | 1 | Data | INT | Cassette Sequence Number (1~65535) |
|  | Position 06 Job Sequence No | 1 | Data | INT | Job Sequence Number (1~65535) |
|  | Position 07 Cassette Sequence No | 1 | Data | INT | Cassette Sequence Number (1~65535) |
|  | Position 07 Job Sequence No | 1 | Data | INT | Job Sequence Number (1~65535) |
|  | Position 08 Cassette Sequence No | 1 | Data | INT | Cassette Sequence Number (1~65535) |
|  | Position 08 Job Sequence No | 1 | Data | INT | Job Sequence Number (1~65535) |
|  | … | 1 | Data | INT | … |
|  | Position 128 Cassette Sequence No | 1 | Data | INT | Cassette Sequence Number (1~65535) |
|  | Position 128 Job Sequence No | 1 | Data | INT | Job Sequence Number (1~65535) |

- Signal Tower Lamp & Buzzer(provisional)

Detailed information can be changed by the user’s request.

  - Signal Light Tower Specification for Process Equipment

| Color State | Red | Yellow | Green | Blue |
| --- | --- | --- | --- | --- |
| Idle | Off | Off | Blinking | On(Online) Off(Offline) (*1) |
| Run | Off | Off | On |  |
| Down | Off | Off | Off |  |
| Maintenance(Manual) | On | Off | Off |  |
| Maintenance(Stop) | Off | On | Off |  |
| Error | Blinking | Blinking | Off |  |
| Warning | Off | Blinking | Idle : Blinking Run : On |  |

(*1) Online = Communication Status with EAS is Normal Offline = Communication Status with EAS is Abnormal

- When operator pressed “Maintenance” switch, only red lamp should be light.

- If equipment mode is online, when operator pressed “Maintenance” switch, equipment send event (Change to Maintenance)to host.

- When error cleared, status return previous status.

- When power on, Equipment status is Maintenance (manual), After Original Position, Equipment status is Maintenance(Stop).

  - Signal Light Tower Specification for Indexer or Feeder

| Color State | Red | Yellow | Green | Blue |
| --- | --- | --- | --- | --- |
| Manual Operation | Off | ON | Off | On(Online) Off(Offline) (*1) |
| Auto Operation | Off | Off | ON |  |
| Cassette Exist | Off | Blinking | Off |  |
| Feeder : CV Running Index : Robot Running | Off | Off | Blinking |  |
| Error | Blinking | Blinking | Off |  |
| Warning | Blinking | Off | Off |  |

(*1) Online = Communication Status with EAS is Normal Offline = Communication Status with EAS is Abnormal

|  | ESWINWH EAS Equipment Communication Specification |  |
| --- | --- | --- |

9.3	Buzzer Specification

| Buzzer State | Buzzer 1 (Fast Intermittent) | Buzzer 2 (Slow Intermittent) |
| --- | --- | --- |
| Error | On | Off |
| Operator Call (*1) | Off | On |

(*1) Conditions will be decided after consultation with the users.

  - The Configuration of the Signal Tower

Follows the configuration of the signal tower

Detailed product specifications will be determined by the ESWIN.

- SPECIAL FUNCTION

  - VCR Management

The equipment maker must provide the screen which sets the allowable retry count for the VCR reading.

- VCR Enable/Disable Mode

  - Enable Mode: VCR is always used (default). (Retry Count of VCR Reading is 3)

  - Disable Mode: VCR is not used and . In this mode, equipment does not read ID and passes job through the VCR unit.

- VCR Read Fail Operation : EQ provide “Key In”

  - Key In: Equipment should wait for operator key in. Blank is not permitted.

If Glass ID received from Job Data from upstream is different from Glass ID read by VCR, Equipment updates Glass ID in Job Data with VCR input result.

It can refer to the common operation scenario for details.

    - PLC MAP

      - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| VCR#1 Event Report Block | Glass ID or Panel ID | 10 | Data | ASC | VCR Read Data / Key-In Data |
|  | Cassette Sequence No | 1 | Data | INT | Job Data |
|  | Job Sequence No | 1 | Data | INT | Job Data |
|  | Unit No | 1 | Data | INT | If no unit then set 0 |
|  | VCR No | 1 | Data | INT | 1 ~ N |
|  | VCR Result | 1 | Data | INT | 1: VCR Reading OK & Match With Job Data Glass ID 2: VCR Reading OK & Miss Match With Job Data Glass ID 3: VCR Reading Fail & Key In & Match With Job Data Glass ID 4: VCR Reading Fail & Key In & Miss Match With Job Data Glass ID |
| VCR#2 Event Report Block | Same as above |  |  |  |  |

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Event | VCR#1 Event Report | 1 | Event | - On: Fire Event - Off: Event End/Timeout | VCR#1 Event Report Block | Reply for Receiving |
|  | VCR#2 Event Report | 1 | Event | - On: Fire Event - Off: Event End/Timeout | VCR#2 Event Report Block | Reply for Receiving |

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Status Event | VCR#1 Enable Mode | 1 | Status | - On: VCR Enabled - Off: VCR Disabled |  |  |
|  | VCR#2 Enable Mode | 1 | Status | - On: VCR Enabled - Off: VCR Disabled |  |  |
| EAS Command Reply | VCR#1 Enable Mode Change Command Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |
|  | VCR#2 Enable Mode Change Command Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |

      - Master (EAS) – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| VCR#1 Enable Mode CMD Block | VCR No | 1 | Data | INT | 1 ~ N |
|  | VCR Enable Mode | 1 | Data | INT | - : Enable - : Disable |
| VCR#2 Enable Mode CMD Block | VCR No | 1 | Data | INT | 1 ~ N |
|  | VCR Enable Mode | 1 | Data | INT | - : Enable - : Disable |
| VCR#1 Event Report Reply Block | Return Code | 1 | Data | INT | - : OK - : NG - : To request Job (Job Data Request) - : To request Job (VCR Mismatch) |
| VCR#2 Event Report Reply Block | Return Code | 1 | Data | INT | - : OK - : NG - : To request Job (Job Data Request) - : To request Job (VCR Mismatch) |

      - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Event Reply | VCR#1 Event Report Reply | 1 | Reply | - On: Reply - Off: Reply End | VCR#1 Event Report Reply Block |  |
|  | VCR#2 Event Report Reply | 1 | Reply | - On: Reply - Off: Reply End | VCR#2 Event Report Reply Block |  |
| EAS Command | VCR#1 Enable Mode Change Command | 1 | CMD | - On: Fire Command - Off: Command End | VCR#1 Enable Mode CMD Block | VCR#1 Enable Mode status have to changed |
|  | VCR#2 Enable Mode Change Command | 1 | CMD | - On: Fire Command - Off: Command End | VCR#2 Enable Mode CMD Block | VCR#2 Enable Mode status have to changed |

    - Communication Scenario

      - VCR Enable Mode Change Command

  - Panel Data Request

This is to request information of Panels such as Judge/Grade of each Panel in a Glass (available only Glass – before cut to Panel).

This is only used for the Equipment requiring Panel Data request. This is the signal requesting panel data to EAS.

If panel data is not exist in EAS then it will be send with null.

※ This feature is used only devices needed.

    - PLC Map

      - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Panel Data Request Block | Cassette Sequence No | 1 | Data | INT |  |
|  | Job Sequence No | 1 | Data | INT |  |

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Special Event | Panel Data Request | 1 | Event | - On: Fire Event - Off: Event End | Panel Data Request Block | Reply for Receiving |

      - Master (EAS) – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Panel Data Request Confirm Block | Cassette Sequence No | 1 | Data | INT |  |
|  | Job Sequence No | 1 | Data | INT |  |
|  | Panel Judge Data | 128 | Data | ASC |  |

|  | Panel Grade Data | 256 | Data | ASC |  |
| --- | --- | --- | --- | --- | --- |

      - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Special Event Reply | Panel Data Request Reply | 1 | Reply | - On: Reply - Off: Reply End | Panel Data Request Confirm Block |  |

  - Panel Data Report

※This feature is used only devices needed..

If is required to report Panel information update if equipment changed Grade and/or Judge value of at least one Panel in a Glass. This report is required for Glass or Q-panel.

Panel Judge Data Format Example ) panel1 Judge : 0 , Panel 2 Judge : G, Panel 3 Judge : R…

| W | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | A | B | C | D | E | F | G | H |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Panel 1 Judge : 0 |  |  |  |  |  |  |  | Panel 2 Judge : G |  |  |  |  |  |  |  |
| 1 | Panel 3 Judge : R |  |  |  |  |  |  |  | Panel 4 Judge |  |  |  |  |  |  |  |
| … | … |  |  |  |  |  |  |  | … |  |  |  |  |  |  |  |
| N | Panel N-1 Judge |  |  |  |  |  |  |  | Panel N Judge |  |  |  |  |  |  |  |

Panel Grade Data Format Example ) panel 1 grade : 00 , Panel 2 grade : AA, Panel 3 grade : AB

| W | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | A | B | C | D | E | F | G | H |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Panel 1 Grade: 0 |  |  |  |  |  |  |  | Panel 1 Grade : 0 |  |  |  |  |  |  |  |
| 1 | Panel 2 Grade: A |  |  |  |  |  |  |  | Panel 2 Grade: A |  |  |  |  |  |  |  |
| 2 | Panel 3Grade: A |  |  |  |  |  |  |  | Panel 3Grade: B |  |  |  |  |  |  |  |
| … | … |  |  |  |  |  |  |  | … |  |  |  |  |  |  |  |
| N | Panel N Grade : |  |  |  |  |  |  |  | Panel N Grade |  |  |  |  |  |  |  |

Refer to Job Grade, Job Judge for Judge/Grade Code definition.

    - PLC Map

      - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Panel Data Report Block | Cassette Sequence No | 1 | Data | INT |  |
|  | Job Sequence No | 1 | Data | INT |  |
|  | Panel Judge Data | 128 | Data | ASC |  |
|  | Panel Grade Data | 256 | Data | ASC |  |

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment | Panel Data Report | 1 | Event | - On: Fire Event | Panel Data Report Block | Reply for Receiving |

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) |
| --- | --- | --- | --- | --- | --- | --- |
| Special Event |  |  |  | - Off: Event End |  |  |

      - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Special Event Reply | Panel Data Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |

  - Material Management

It is that equipment manages durable materials such as reticle and consumable materials such as Polaroid and chemical. This function is only for equipment with special material which needs to be managed. If equipment has no material to be managed, please ignore this section.

    - Material ID

The material ID of the material is being mounted or dismounted.

    - Material Status

- Mount: Materials is installed into equipment to be used for processing.

- Un-mount: Material is removed from equipment.

- Prepared : Material ready to be used for processing.

- In-Use : Material processing in the unit of equipment.

- Done : Material processing complete and ready to dismount.

Figure 10-1 State Transition of material

    - PLC Map: Material

      - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Material Status Change Report Block | Unit No | 1 | Data | INT |  |
|  | Stage No | 1 | Data | INT | Slot No |
|  | Material ID | 10 | Data | ASC |  |
|  | Material Status | 1 | Data | INT | 1: Un-mount 2: Mount 3: Prepared 4: In-Use 5: Done 6 ~ : reserved |
|  | Material type | 1 | Data | INT | 1: PR 2: APRPlate 3: ODFAuBall 4: ODFLC 5: ODFSealant 6: ODFSiBall 7: PILiquid 8: Syringe 9: Cloth 10: Roller 11: Reserve 12: Mask 13 ~ 129 : (Ref 11.1.10 Parts Management – Dry Etch Material Type) 130 ~229 : (Ref 11.1.11 Parts Management Material Type) 230~: Reserve |
| Material Data Block | Tank No | 1 | Data | INT |  |
|  | PR ID | 10 | Data | ASCII |  |
|  | Operator ID | 4 | Data | ASCII |  |
|  | PR Weight | 1 | Data | INT | Gram (g) |
| Material Data Sub Block | PPID | 1 | Data | INT |  |
|  | Operation ID | 10 | Data | ASCII |  |
| Material Validation Request Block | Material ID | 10 | Data | ASCII |  |
|  | Material Position | 1 | Data | INT | Rubbing Inline - : Roller 1 - : Roller 2 - : Double Roller - : Reserved PI Inline - : PI Cleaner Tank 1 - : PI Cleaner Tank 2 - : PI Ink Tank 1 - : PI Ink Tank 2 - : PI Ink Tank 3 |

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | - : PI Ink Tank 4 ODF Inline - : LC Head 1 - : LC Head 2 … - : LC Head 12 - : SD Head 1 - : SD Head 2 … - : SD Head 16 - : UV Mask 1 |
|  | Unit No | 1 | Data | INT |  |

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Special Event | Material Status Change Report | 1 | Event | - On: Fire Event - Off: Event End | -Material Status Change Report Block | Reply for Receiving |
|  | Material Validation Request | 1 | Event | - On: Fire Event - Off: Event End | - Material Validation Request Block | Reply for Receiving |

      - Master PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Material Status Change Report Reply Block | Unit No | 1 | Data | INT | If no unit then set 0 |
|  | Return Code | 1 | Data | INT | 1: OK 2: NG |
| Material Validation Request Reply Block | Return Code | 1 | Data | INT | 1: OK 2: NG |
|  | Used Count | 1 | Data | INT | 1~2000 |
|  | Thickness | 1 | Data | INT | 1.500~1.855(Data*1000) |
|  | Limited Count | 1 | Data | INT | 200~2000 |
|  | Life Time | 3 | Data | BCD | yymmddhhmmss(ODF, PI Only) |
|  | Due Time | 3 | Data | BCD | Yymmddhhmmss(ODF, PI Only) |

      - Master PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Special Event Reply | Material Status Change Report Reply | 1 | Reply | - On: Reply - Off: Reply End | Material Status Change Report Reply Block |  |
|  | Material Validation Request Reply | 1 | Reply | - On: Reply - Off: Reply End | Material Validation Request Reply Block |  |

  - Loading Stop (EQ to EQ)

This signal is used to request a Job transfer stop to a specific Transfer equipment (Transfer device block or Indexer) in upstream.

While equipment turn on this signal, Job in the equipment itself must flows normally.

Refer to Equipment Operation Scenario for the upstream-downstream relation where this signal is required.

    - PLC Map

      - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Loading Stop block | Loading Stop Source | 1 | Data | INT | Local No |
|  | Loading Stop Target | 1 | Data | INT | Local No |

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items |  |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment to Equipment Event | Loading StopRequest | 1 | Event | - On: Loading Stop - Off: Release | Loading Stop block |  |

  - Transfer Stop (EQ to EQ)

This signal is used to request a Job transfer stop to upstream equipment. If equipment is situation of not being able to receive Job from upstream, such as temporary overflow, the equipment can set this signal On to upstream.

While equipment turn on this signal, Job in the equipment itself must flows normally.

    - PLC Map

      - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Transfer Stop Block | Transfer Stop source | 1 | Data | INT | downstream No * refer to equipment operation scenario |

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items |  |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment to Equipment Event | Transfer Stop Request | 1 | Event | - On: Transfer Stop - Off: Release | Transfer Stop Block |  |

  - Cancel Abort Request (EQ to EAS)

When equipment need to unclamp a cassette on manual , operator can press cancel or abort request button for unclamp a cassette from port.

EAS send reply with ‘OK’ if no problem to send ‘unclamp’ port command to equipment. And Send ‘unclamp’ port command to equipment soon after.

The other case, EAS send rely with ‘NG’.

| CIM Mode | Action | Note |
| --- | --- | --- |
| CIM On | EQ requests CST Unclamp to EAS. EAS send a Unclamp command to EQ when end condition of process. | Push for 3 sec |
| CIM Off | It is has to unclamp the CST by EQ when end condition of process. | Push for 3 sec |

    - PLC MAP

      - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Cancel Abort Request Block | Port No | 1 | Data | INT |  |
|  | Cancel or Abort |  |  |  | 1: CST ID read fail 2: mapping sensor error 3: HW error 4: others error 10: CST ID validation fail ( oven etc…) |

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Indexer Event | Cancel Abort Request Event | 1 | Event | - On: Fire Event - Off: Event End | Cancel Abort request Block |

      - Master (EAS) – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Cancel Abort Request Reply Block | Return Code | 1 | Data | INT | 1: OK 2: NG |

      - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| Indexer Event Reply | Cancel Abort Request Event Reply | 1 | Reply | - On: Reply - Off: Reply End | Cancel Abort Request Reply Block |  |

  - Lot End Glass Report (EQ to EQ)

Specific equipment requires to know the last Glass of a Lot for its processing purpose such as recipe change preparation. For this purpose the last Glass of a Lot information should be transferred from Indexer to downstream equipment according to the physical location of the last Glass.

- Indexer should send Lot End Glass report to downstream after sending the last Glass of a Lot.

- If equipment receives Lot End Glass report from upstream, equipment should identify the Lot ID and keep the Lot information, and send the Lot End Glass report to downstream after sent the last Glass of the Lot.

- For example, if equipment has three Glass of Lot A when it is received Lot End Glass report (for Lot A) from upstream, equipment should send Lot End Glass report (for Lot A) to downstream when sending the last Glass of the three Lot A Glasses.

- Regardless of scrap, this report should be sent after the last Glass of a Lot.

- Within equipment, Glass FIFO between same Lot is not necessary but Lot FIFO should be maintained between Lots.

- Upstream should send Lot End Glass Report to downstream completion of transferring the Glass to downstream equipment

    - PLC Map

      - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Lot End Glass Data Report Block | Lot ID | 10 | Data | ASC |  |
|  | Cassette Sequence No | 1 | Data | INT |  |

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment to Equipment Event | Lot End Glass Report | 1 | Event | - On: Fire Event - Off: Event End | Lot End Glass Data Report Block | Reply for Receiving |

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment to Equipment Event Reply | Lot End Glass Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |

  - Cassette Move In/Out Report

Equipment should report cassette movement within equipment when cassette moves in/out of equipment, cassette move in/out between Units.

    - PLC MAP

      - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| CST Move In Report Block | Job Data C | 23 | Data |  | Refer to Job Data C |
|  | Unit or Port | 1 | Data | INT | 0: Equipment, 1: Unit 2:Port |
|  | Unit No or Port No | 1 | Data | INT | 0 if Equipment |
| CST Move Out Report Block | Same as CST Move In Report Block. | 25 | Data |  |  |
| CST Received Report Block | Same as CST Move In Report Block. | 25 | Data |  |  |
| CST Sent Out Report Block | Same as CST Move In Report Block. | 25 | Data |  |  |

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment SpecialEven t | CST Move In Report | 1 | Event | - On: Fire Event - Off: Event End | CST Move In Report Block |  |
|  | CST Move Out Report | 1 | Event | - On: Fire Event - Off: Event End | CST Move Out Report Block |  |
|  | CST Received Report | 1 | Event | - On: Fire Event - Off: Event End | CST Received Report Block |  |
|  | CST Sent Out Report | 1 | Event | - On: Fire Event - Off: Event End | CST Sent Out Report Block |  |

      - Master (EAS) – Link Register (W Area)

Equipment that processes by cassette must refer to this data for CST information instead of Common CST Map Data area.

Cassette Control Command : CST Map Download

* Equipment Applied : Cassette Cleaner (Array), Cell Test (Cell)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Port #1 CST Data Block | Job Data C | 23 | Data |  | Refer to Job Data C |
| Port #2 CST Data Block | Same as Port #1 CST Data Block | 23 | Data |  |  |
| Port #3 CST Data Block | Same as Port #1 CST Data Block | 21 | Data |  |  |
| Port #4 CST Data Block | Same as Port #1 CST Data Block | 21 | Data |  |  |
| Port #5 CST Data Block | Same as Port #1 CST Data Block | 21 | Data |  |  |
| Port #6 CST Data Block | Same as Port #1 CST Data Block | 21 | Data |  |  |
| Port #7 CST Data Block | Same as Port #1 CST Data Block | 21 | Data |  |  |
| Port #8 CST Data Block | Same as Port #1 CST Data Block | 21 | Data |  |  |
| Port #9 CST Data Block | Same as Port #1 CST Data Block | 21 | Data |  |  |
| Port #10 CST Data Block | Same as Port #1 CST Data Block | 21 | Data |  |  |

      - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Special Event Reply | CST Move In Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |
|  | CST Move Out Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |
|  | CST Received Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |
|  | CST Sent Out Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |

  - Oven Initial Heating

When Oven starts heating, the others equipment should have warning Alarm. Because when Oven power start at the same time, Power in Fab maybe down.

Equipment should send Oven Heating initial to EAS when Oven start heating

EAS send to other Oven the Heating alarm that is CIM message from EAS to equipment

    - PLC MAP

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Equipment Special Event | Oven#1 Initial Heating Start | 1 | Event | - On: Fire Event - Off: Event End |  |
|  | Oven#1 Initial Heating end | 1 | Event | - On: Fire Event - Off: Event End |  |
|  | Oven#2 Initial Heating Start | 1 | Event | - On: Fire Event - Off: Event End |  |
|  | Oven#2 Initial Heating end | 1 | Event | - On: Fire Event - Off: Event End |  |

      - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Equipment Special EventReply | Oven#1 Initial Heating Start Reply | 1 | Event | - On: Fire Event - Off: Event End | EAS Reply |
|  | Oven#1 Initial Heating end Reply | 1 | Event | - On: Fire Event - Off: Event End | EAS Reply |
|  | Oven#2 Initial Heating Start Reply | 1 | Event | - On: Fire Event - Off: Event End | EAS Reply |
|  | Oven#2 Initial Heating end Reply | 1 | Event | - On: Fire Event - Off: Event End | EAS Reply |

  - Dummy Glass Request

* ODF equipment will use separate data for Dummy Request (refer to ODF special section)

This signal is to be used when equipment requires Dummy supply from Loader.

    - PLC MAP

      - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Dummy Glass Request Block | Dummy Glass Count | 1 | Data | INT | Count (0~28) |
|  | Dummy Type | 1 | Data | INT | Dummy Type |

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (CIMPC) |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Special Event | Dummy Glass Request | 1 | Event | - On: Fire Event - Off: Event End | Dummy Glass Request Block |  |

      - Master (EAS) –– Link Relay (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Dummy Glass Request Reply Block | Dummy Glass Request Return Code | 1 | Data | INT | 1: OK 2: NG |

      - Master (EAS) –– Link Relay (B Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Equipment Special EventReply | Dummy Glass Request Reply | 1 | Reply | - On: Fire Event - Off: Event End | Dummy Glass Request Reply Block |

  - Pre-action Signal (EQ to EQ)

Pre-action signal is mainly used to decide in advance the robot waiting position, so that it is to optimize Job transferring tact time (especially for robot dispatching in Indexer or TR in equipment)

Equipment can send ‘pre-action signal ’to robot in order to notice that glass will come out soon, or glass receiving will be possible soon.

As Every equipment is different because of current tool characteristics/status and Job flow conditions, operation conditions (Grade, Judge,..), and etc., how to use Pre-action signal may different between equipment. The detail requirements on the signal ON timing and corresponding actions would be requested by equipment engineer, especially in the process of tact time optimizations.

    - Pre-action Signal Type

Two types of Pre-action Signal can be defined.

- Pre-action Signal to Downstream : to be used to notify downstream equipment (mainly TR) that Job is coming out (sending) soon

- Pre-action Signal to Upstream : to be used to notify upstream equipment (mainly TR) that Job receiving will be ready soon

Example of corresponding actions : details would be required by equipment engineer.

- Pre-action Signal to Downstream :

  - Downstream is process equipment: upon receiving of this signal, equipment prepares any available

conditions for receiving Job, such as Shutter open, Pin up, etc..

  - Downstream is TR : upon receiving of this signal, Robot can move and wait for receiving Job.

- Pre-action Signal to Upstream :

  - Upstream is process equipment: upon receiving of this signal, equipment prepares any available conditions for sending Job, such as Shutter open, etc..

  - Upstream is TR : upon receiving of this signal, Robot can move and wait for sending out Job.

    - Pre-action Signal On/Off Condition

The below is guideline of the signal on/off condition. Detail required needs to be determined through tact time optimization process.

Pre-action Signal ON :

- Pre-action Signal to Downstream :

Signal ON timing is mainly determined by the remaining time to Job send-out position (stage) or specific position in equipment in the path to downstream.

Equipment has to prepare Signal ON timing setting on touch panel :

Example )

If Signal ON timing is set as '5' seconds, then if Job reaches a position where 5 seconds before send-out position, then equipment turn this signal ON to upstream.

- Pre-action Signal to Upstream :

Signal ON timing is mainly determined by specific action or event or signal : it could be different at every equipment.

Pre-action Signal OFF :

- Pre-action Signal to Downstream/Upstream :

Turn off signal when Job transfer process is initiated with the downstream/upstream (the same downstream/upstream to which the signal is on)

If equipment has trouble and is not available to start/continue Job transfer process.

Other Conditions :

- If equipment is currently Job transfer hand shaking with upstream(, downstream), equipment should turn on any Pre-action signal to downstream(, upstream) after completion of current hand shaking process.

    - PLC Map

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items |  |
| --- | --- | --- | --- | --- | --- | --- |
| Event Area | PAStoUpstream#1 | 1 | Event | - On: Fire Event - Off: Event End |  |  |
|  | PAStoDownstream#1 | 1 | Event | - On: Fire Event - Off: Event End |  |  |
|  | ... |  |  |  |  |  |
|  | PAStoUpstream#6 | 1 | Event | - On: Fire Event - Off: Event End |  |  |

| Block | Item | Points | Type | Sub Items | Related Word Items |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PAStoDownstream#6 | 1 | Event | - On: Fire Event - Off: Event End |  |  |

  - Oven Over Bake

    - Oven Over Bake Report

When glass in oven stay more than setting time, Oven should report Overbake Report.

    - PLC MAP

      - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Oven Over Bake Block | Cassette Sequence No | 1 | Data | INT |  |
|  | Job Sequence No | 1 | Data | INT |  |
|  | Process Setting Time | 1 | Data | INT | Time is Second |

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | EAS Action |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Special Event | Oven Over Bake Report | 1 | Event | - On: Report - Off: End | Oven Over Bake Block |  |

      - Master (EAS) – Link Register (W Area)

None.

      - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Special Event Reply | Oven Over Bake Report Reply | 1 | Reply | - On: Reply - Off: End |  |  |

  - DFS

    - Cassette ID Request

Because inspection equipment of all shopneed loading cassette ID.

    - PLC MAP

      - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |

| Cassette ID Request Block | Cassette Sequence No | 1 | Data | INT |  |
| --- | --- | --- | --- | --- | --- |
|  | Job Sequence No | 1 | Data | INT |  |

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | EAS Action |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Special Event | Cassette ID Request | 1 | Event | - On: Fire Event - Off: Event End | Cassette ID Request Block |  |

      - Master (EAS) – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Cassette ID Request Confirm Block | Cassette Sequence No | 1 | Data | INT |  |
|  | Job Sequence No | 1 | Data | INT |  |
|  | Loading Cassette ID | 5 | Data | ASC |  |
|  | Return Code | 1 | Data | INT | - : OK - : NG |

      - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Special Event Reply | Cassette ID Request Reply | 1 | Reply | - On: Reply - Off: End | Cassette ID Request Confirm Block |  |

    - CIM Online Signal On/Offline status changed

Whenever On/Offline Status changed, EAS inform the equipment using CIM Online Signal One Bit. EAS only On/Off CIM Online Signal Bit and Equipment checked CIM online signal.

- Online Status (Local/Remote) : ON

: If current online status is Local/Remote, CIM online signal bit on.

- Online Status (Offline) : OFF

: If current online status is Offline, CIM online signal bit off.

10.14.2.1	Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| Special Function | CIM Online Signal | 1 | Event | - On: Local/Remote - Off: Offline |  |  |

  - Panel Repair Result Report

※This feature is used only devices needed..

Repair Equipment report this message to inform the repaired panel and repair type

| W | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | A | B | C | D | E | F | G | H |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Panel 1 Repair Result Code : D |  |  |  |  |  |  |  | Panel 2 Repair Result Code : D |  |  |  |  |  |  |  |
| 1 | Panel 3 Repair Result Code : N |  |  |  |  |  |  |  | Repaire Repair Result Code : N |  |  |  |  |  |  |  |
| … | … |  |  |  |  |  |  |  | … |  |  |  |  |  |  |  |
| N | Panel N-1 Repair Result Code : Z |  |  |  |  |  |  |  | Panel N Repaire Code : Z |  |  |  |  |  |  |  |

Panel Repair Result Code (This code could be updated by B17 User)

| A | Normal Cutting |
| --- | --- |
| B | Cutting Femto Managing |
| C | Cutting Femto No Managing |
| D | CVD（W）Managing |
| E | CVD（W） No Managing |
| F | CVD（Cobalt） Managing |
| G | CVD（Cobalt） No Managing |
| H | CVD（Metal Ink） Managing |
| I | CVD（Metal Ink） No Managing |
| N | No Repair |
| Z | Fill Blank No Managing |
| S | Risk Product Managing |

    - PLC Map

      - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Panel Repair Result Block | Cassette Sequence No | 1 | Data | INT |  |
|  | Job Sequence No | 1 | Data | INT |  |
|  | Panel Repair Result | 128 | Data | ASC |  |

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Special Event | Panel Repair Result Report | 1 | Event | - On: Fire Event - Off: Event End | Panel Data Report Block | Reply for Receiving |

      - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Special Event Reply | Panel Repair Result Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |

- Shop Special Function

  - Array Special Function

    - Remain Glass inform (EAS to EQ) – CVD Only

Indexer has Remain glass signal that EAS info to indexer That means indexer has no more input a glass to CVD.

- Remain Glass Signal : On

: If there is at least one port in InUse

- Remain Glass Signal : OFF

: If there is no Port in InUse

11.1.1.1	Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| Special Function | Remain Glass Signal | 1 | Event | - On: Remain Glass Signal On - Off: Remain Glass Signal Off |  |  |

    - Tray Data Management (EAS to EQ) - Sputter Only

Equipment has to write Tray information whenever there is a change.

      - PLC Map

        - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Tray Data Block | Tray #1 ID | 1 | Data | INT |  |
|  | Tray #1 life | 1 | Data | INT |  |
|  | Tray #1 status | 1 | Data | INT | 1: USE 2: SKIP |
|  | … |  |  |  |  |
|  | Tray #13 ID | 1 | Data | INT |  |
|  | Tray #13 life | 1 | Data | INT |  |
|  | Tray #13 status | 1 | Data | INT | 1: USE 2: SKIP |
| Tray Data Command Reply Block | Return Code | 1 | Data | INT | 1: OK 2: NG |

        - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) |
| --- | --- | --- | --- | --- | --- | --- |
| EAS Command Reply | Tray Data Command Reply | 1 | Reply | - On: Reply - Off: Reply End | Tray Data Command Reply Block |  |

        - Master (EAS) – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Tray Data Command Block | Tray ID | 1 | Data | INT |  |
|  | Tray Status | 1 | Data | INT | - : USE - : SKIP |

        - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| EAS Command | Tray Data Command | 1 | CMD | - On: Fire Command - Off: Command End | Tray Data Command Block | Reply for Receiving |

    - Glass Start Signal (EQ to EQ) – Sputter Only

If there is no glass in the sputter, the sputter facility switches to idle mode. It takes about 2 to 3 minutes to switch from idle mode back to normal mode.

When the cassette is loaded, the indexer should inform the sputter facility to switch to Normal mode in advance.

Use one bit.

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items |  |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment to Equipment Event | Glass Start Signal | 1 | Event | - On : Glass to process - Off : No glass |  |  |

    - Crack Signal (EQ to EQ) - EGIS Only

HDC has a Crack Checker(Egis) at the entrance/exit.

- When the indexer unloading, check crack for condition of the glass.

- If a crack error occurs, generates to crack result & complete signal at the indexer and EAS.

- The Indexer & HDC should be stopped(Alarm & BM/ at Link signal completed) the crack NG signal is received.

Figure 11-1 Crack Signal Time Chart

11.1.4.1	Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items |  |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment to Equipment Event | Egis Crack NG | 1 | Event | - On : detected crack by EGIS - Off : Not detected crack by EGIS |  |  |

    - Oven Auto Recipe Change

      - Oven Job Start Request

Indexer should send “Oven Job Start Request” that can possible the auto change of recipe to Oven when new recipe is loaded, because oven heating takes long time.

The Oven should reply the OK to Indexer if the recipe is ready but if it is not ready then should reply the NG. Operator can select a Cancel or Change of recipe which is function on EAS UI when the reply is NG.

If Operator selects the Change of recipe, then indexer has to send ‘Recipe Change Request’ with Cassette Sequence No and Master Recipe to Oven.

Indexer will dispatch a glass when the Oven send “Recipe Change Complete Report” to Indexer

But even if theother Cassette is loaded, dispatching of glass can’t be start until it completed the recipe change in Oven.

      - PLC MAP

        - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Oven Job Start Return Block | Return Code | 1 | Data | INT | - : OK - : NG - : Wait |
|  | Cassette Sequence No | 1 | Data | INT | That is item at Oven Job Start Request Block. |
| Oven Recipe Change Complete Block | Cassette Sequence No | 1 | Data | INT |  |
|  | Local Recipe ID | 10 | Data | ASC |  |

        - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | EAS Action |
| --- | --- | --- | --- | --- | --- | --- |
| EAS Special Command Reply | Oven Job Start Request Reply | 1 | Reply | - On: Reply - Off: End | Oven Job Start Return Block |  |

| Block | Item | Points | Type | Sub Items | Related Word Items | EAS Action |
| --- | --- | --- | --- | --- | --- | --- |
|  | Oven Job Recipe Change Request Reply | 1 | Reply | - On: Reply - Off: End |  |  |
| Equipment Special Event | Recipe change Complete Report | 1 | Event | - On: Report - Off: End | Oven Recipe Change Complete Block |  |

        - Master (EAS) – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Oven Job Start Request Block | Cassette Sequence No | 1 | Data | INT |  |
|  | Local Recipe ID | 10 | Data | ASC |  |

        - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| EAS Special Command | Oven Job Start Request | 1 | CMD | - On: Fire Event - Off: Event End | Oven Job Start Request Block |  |
|  | Oven Job Recipe Change Request | 1 | CMD | - On: Fire Event - Off: Event End | Oven Job Start Request Block |  |
| Equipment Special Event Reply | Recipe change Complete Report Reply | 1 | Reply | - On: Reply - Off: End |  |  |

    - AOI Enable Signal ( EQ to EQ) : Photo Inline Only

Photo AOI Equipment On “AOI Enable Signal” Glass Processing AOI Photo AOI Equipment Off “AOI Enable Signal” Glass Processing By Pass

      - Local PLC – Link Register (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| Equipment to Equipment Event | AOI Enable Signal | 1 | Event | - On : Glass process AOI - Off : Glass Process By Pass |  |

    - EE/TT Removed Job Signal (EQ to EQ ) : Photo Inline Only

EE(Upstream) informs to Track(Downstream) when a glass is removed from EE.

      - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| EE Removed Job Block | GlassID | 10 | Data | ASC |  |
|  | Cassette Sequence No | 1 | Data | INT |  |
|  | Job Sequence No | 1 | Data | INT |  |

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items |  |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment to Equipment Event | Removed Job Signal | 1 | Event | - On: Fire Event - Off: Event End | EE Removed Job Block |  |
| Equipment to Equipment Event Reply | Removed Job Signal Reply | 1 | Reply | - On: Reply - Off: End |  |  |

    - Mix Run Recipe Data Request/Reply

Mix Run Recipe Data Request be used for Array DryEtch Equipment, This Message Request Dry Etch Equipment Unit Recipe.

      - PLC Map

        - Master PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| EAS Special Command | Mix Run Recipe Data Request | 1 | Event | - On: Fire Event - Off: Event End | Mix Run Recipe Data Request Block | Reply for Dry Etch Unit Recipe |

        - Master PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Mix Run Recipe Data Request Block | Local Recipe | 20 | Data | ASC |  |

        - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) |
| --- | --- | --- | --- | --- | --- | --- |
| EAS Special Command Reply | Mix Run Recipe Data Request Reply | 1 | Event | - On: Fire Event - Off: Event End | Mix Run Recipe Data Request Reply Block | Reply for Receiving |

        - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Mix Run Recipe Data Request Reply Block | Local Recipe ID | 20 | Data | ASC |  |
|  | Unit1 Recipe Data | 20 | Data | ASC |  |
|  | Unit2 Recipe Data | 20 | Data | ASC |  |
|  | Unit3 Recipe Data | 20 | Data | ASC |  |

    - Mix Run Unit Recipe Change Command/Reply

Mix Run UnitRecipe Change Command be used for Array DryEtch Equipment, This Message Set Dry Etch Equipment Unit Recipe.

      - PLC Map

        - Master PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| EAS Special Command | Mix Run Unit Recipe Change Command | 1 | Event | - On: Fire Event - Off: Event End | Mix Run Recipe Data Request Block | Change Unit Recipe and Reply Result |

        - Master PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Mix Run Unit Recipe Change Command | Local Recipe ID | 20 | Data | ASC |  |
|  | Unit1 Recipe Data | 20 | Data | ASC |  |
|  | Unit2 Recipe Data | 20 | Data | ASC |  |
|  | Unit3 Recipe Data | 20 | Data | ASC |  |

        - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) |
| --- | --- | --- | --- | --- | --- | --- |
| EAS Special CommandRe ply | Mix Run Unit Recipe Change Command Reply | 1 | Event | - On: Fire Event - Off: Event End | Mix Run Unit Recipe Change Return Block | Reply Unit Recipe Change Result |

        - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Mix Run Unit Recipe Change Return Block | Return Code | 1 | Data | INT | 1:OK 2:NG |

    - Parts Management – Dry Etch

Parts Management – Dry Etch be used for Array Dry Etch Equipment, This Message Report Dry Etch Equipment Parts related Item.

      - PLC Map

        - Local PLC – Link Register (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Special Event | Parts Report | 1 | Event | - On: Fire Event - Off: Event End | Parts Block | Reply for Receiving |

        - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Parts Block | Unit No | 1 | Data | INT |  |

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
|  | Parts List | 1050 | Data | ASC | Item #1 ~ #50 |

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Parts Block | Material Type#1 | 1 | Data | INT | 13: Electrode-Up (Dry Etch) 14: ESC-BT (Dry Etch) 15: Ceramic-UP (Dry Etch) 16: Ceramic-BT (Dry Etch) 17: SIDE-WALL (Dry Etch) 18: Inner-Bottom-Plate (Dry Etch) 19: View-Port-Glass (Dry Etch) 20: View-Port-Oring (Dry Etch) 21: SIDE-Ceramic (Dry Etch) 22: LID-Oring-IN (Dry Etch) 23: LID-Oring-Out (Dry Etch) 24: Gate-Oring (Dry Etch) 25: FRC-Oring-IN (Dry Etch) 26: FRC-Oring-Out (Dry Etch) 27: APC-Oring-Up (Dry Etch) 28: APC-Oring-Low (Dry Etch) 29: APC-Oring-Cover (Dry Etch) 30: APC-Motor-Oring (Dry Etch) 31: Lifter-Pin (Dry Etch) 32: Lifter-Pin-Oring (Dry Etch) 33: Lifter-Pin-Bellows (Dry Etch) 34: Lifter-Pin-Grease (Dry Etch) 35: Cut-off-Oring (Dry Etch) 36: FRC-Block (Dry Etch) 37: Ceramic-Gate (Dry Etch) 38: Gas-Hole-Oring (Dry Etch) 39: Gas-Hole-Filter (Dry Etch) 40: TMP-net (Dry Etch) 41: Chiller (Dry Etch) 42: Dry-Pump (Dry Etch) 43: TMP-1 (Dry Etch) 44: TMP-2 (Dry Etch) 45: TMP-3 (Dry Etch) 46: TMP-4 (Dry Etch) |

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 47: TMP-5 (Dry Etch) 48: TMP-6 (Dry Etch) 49: TMP-7 (Dry Etch) 50: TMP-8 (Dry Etch) 51: TMP-9 (Dry Etch) 52: TMP-10 (Dry Etch) 53: LL-Clean (Dry Etch) 54: PIN-Ball (Dry Etch) 55: PIN-Lifter (Dry Etch) 56: Gate-Oring-VAC (Dry Etch) 57: Gate-Oring-ATM (Dry Etch) 58: Slow-Vlalve (Dry Etch) 59: Fast-Vlalve (Dry Etch) 60: VAC-Gate-Valve-Grease (Dry Etch) 61: ATM-Gate-Valve-Grease (Dry Etch) 62: Electric-Cylinder-Grease (Dry Etch) 63: Air-Cylinder-Grease (Dry Etch) 64: USC-Clean (Dry Etch) 65: USC-Grease-Path (Dry Etch) 66: USC-Grease-guide (Dry Etch) 67: USC-PIN-Align (Dry Etch) 68: USC-PIN-Lifeter (Dry Etch) 69: USC-PIN-Ball (Dry Etch) 70: TM-Clean (Dry Etch) 71: TM-PAD (Dry Etch) 72: TM-PAD-Oring (Dry Etch) 73: VAC-Arm Grease-Extend (Dry Etch) 74: VAC-Arm-Grease-Rotation (Dry Etch) 75: VAC-Arm-Grease-Lifter (Dry Etch) 76: Arm-battery (Dry Etch) 77: ATM-Arm-Grease-Extend (Dry Etch) 78: ATM-Arm-Grease-Rotation (Dry Etch) 79: ATM-Arm-Grease-Lifter (Dry Etch) 80: ATM-Arm-Grease-pathway (Dry Etch) 81~129: Reserve |
|  | Change Date#1 | 3 |  | BCD | Change Parts Date |
|  | Glass Count#1 | 2 |  | INT | Processing Glass cumulative Count |
|  | RF SO Time Current#1 | 2 |  | INT | Chamber using power total time |
|  | RF Bi Time Current#1 | 2 |  | INT | Chamber using power total time |
|  | SN No#1 | 8 |  | ASC | Parts ID |
|  | Parts Vendor#1 | 1 |  | INT | Parts Vendor - Need ESWIN provide |
|  | Clean Chamber After Glass Count #1 | 1 |  | INT | Clean Chamber After Glass Count |
|  | Day#1 | 1 |  | INT | Use Day Count (After equipment calculation report) |
|  | GLASSID | 10 | Data | ASC | GLASSID |

        - Master PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | EAS Action |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Special Event Reply | Parts Report Reply | 1 | Reply | - On: Reply - Off: End |  | Reply for Receiving |

    - Parts Management – Wet Etch/Strip, HDC, Pre Cleaner, Initial Cleaner

Parts Management - Wet Etch/Strip, HDC, Pre Cleaner, Initial Cleaner be used for Array Wet Etch/Strip, HDC, Pre Cleaner, Initial Cleaner Equipment, This Message Report Parts Related Item.In the equipment Change

One Parts report this Message

      - PLC Map

        - Local PLC – Link Register (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Special Event | Parts Report | 1 | Event | - On: Fire Event - Off: Event End | Parts Block | Reply for Receiving |

11.1.11.1.1  Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Parts Block | Unit No | 1 | Data | INT |  |
|  | Slot No | 1 | Data | INT | Part No |
|  | Material Type | 1 | Data | INT | 130: Filter 131: Bearing 132: Guide Roller 133: Gear 134: O-ring 135: Idle Roller 136: Double Roller 137: Sensor 138: Nozzle 139: Motor 140: Heater 141: Pressure Guage 142: Roller 143: Valve 144: Flow meter 145: Pump 146: PM 147: Out C/V Tilting Grease 148: Elevator Grease 149: Robot Grease 150: AK Clean 151: AC Clean 152: Dry Unit Clean 153: Cu Crystal Clean |

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 154: APP Clean 155~229: Reserve |
|  | Change Date | 3 |  | BCD | Change Parts Date |
|  | Parts Model | 30 | Data | ASC | Parts Model |
|  | Parts Vendor | 1 | Data | INT | Parts Vendor - Need ESWIN provide |
|  | Dualistic Test | 1 | Data | ASC | N:No Dualistic Test Y:Dualistic Test |
|  | Change Reason | 30 | Data | ASC |  |

11.1.11.1.1  Master PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | EAS Action |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Special Event Reply | Parts Report Reply | 1 | Reply | - On: Reply - Off: End |  | Reply for Receiving |

    - Defect Code Report

Defect Code Report be used for Array Inspection Equipment. This report need to report before glass sent out from equipment as like as Process Data Report

After glass inspection, if glasses inspection result is not OK [ Job Judge is not G ] then equipment create defect code and report to EAS. If glass inspection result is OK [ Job Judge is G ] then equipment report this with Defect Code Count as zero(0).

      - PLC Map

        - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Defect Code Report Data Block | Cassette Sequence No | 1 | Data | INT | From Job Data about related glass |
|  | Job Sequence No | 1 | Data | INT | From Job Data about related glass |
|  | Glass ID | 10 | Data | ASC | From Job Data about related glass |
|  | Defect Code Count | 1 | Data | INT |  |
|  | Defect Code #1 | 3 | Data | ASC | Fill with space if it has no data |
|  | Defect Code #2 | 3 | Data | ASC | Fill with space if it has no data |
|  | Defect Code #3 | 3 | Data | ASC | Fill with space if it has no data |
|  | Defect Code #4 | 3 | Data | ASC | Fill with space if it has no data |
|  | Defect Code #5 | 3 | Data | ASC | Fill with space if it has no data |

        - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) |
| --- | --- | --- | --- | --- | --- | --- |

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Special Event | Defect Code Report | 1 | Event | - On: Fire Event - Off: Event End | Received Job Data Block Received Job Data Sub Block Job Count Block | Reply for Receiving |

        - Master (EAS) – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| ------ No related definition ------ |  |  |  |  |  |

        - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Special Event Reply | Defect Code Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |

  - Cell Special Function

    - Common

      - Muting Down Prevention

Each equipment should provide 2 Bits for other equipment could monitoring the down status of itself. One bit refers to Equipment Alive Signal, the other one refers to Equipment Down Signal . And when Equipment Down Signal is On, the relative equipment status is Down. If one of the two conditions is satisfied, the relative equipment will be judged Down.

The equipment of Local No = N monitoring the equipment of Local No = N + 1 whether Down or not. The master equipment should monitoring 2 signal and if judging status Down, it will trigger Warning alarm.

        - PLC MAP – Local PLC – Link Relay (B Area), Local to Local

| Block | Item | Points | Type | Sub Items | Remark |
| --- | --- | --- | --- | --- | --- |
| Muting Down Block | Equipment Down | 1 | Event | - On: Equipment Down - Off: Equipment Not Down |  |

      - Conveyor Direction Mode

Equipment included in Conveyor, provide Direction of Conveyor for the Coordinate Equipment. Related Equipment : PI phase2 TRF01, Rubbing USC01 & USC02, ODF Phase1TRF01, ODF TRF06

        - PLC MAP – Local PLC – Link Relay (W Area), Local to Local

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Conveyor Direction Block | Conveyor Direction Mode | 1 | Data | INT | - : Both (default) - : Forward - : Reverse |

        - PLC MAP – Local PLC – Link Relay (B Area), Local to Local

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| Conveyor Direction Event | Conveyor Direction Mode Change Report | 1 | Event | - On: Fire Event - Off: Event End | Conveyor Direction Mode |

|  | Conveyor Direction Mode Change Report Reply | 1 | Event | - On: Fire Event - Off: Event End |  |
| --- | --- | --- | --- | --- | --- |

      - Loading Stop Request

Refer to Loading Stop in Special Function for details

When sender on ‘Loading Stop Request’, then Actor who monitoring the signal of ‘Loading Stop Request’ will be limited the flow.

| Line | Sender (Local) | Actor(Local) | Remark |
| --- | --- | --- | --- |
| PI | TRF01 (4) | Loader(2) | Refer TRF01 Buffer Operation |
| PI | TRF07 (13) | TRF01 (4) | Refer TRF07 Buffer Operation |
| ODF | TRF01 (4) | Loader (2) | When ARC1 Block has glass more than its allowable count |
| ODF | TRF10 (20) | TRF07 (17) | When UV Cure All Down |
| ODF | TRF12 (28) | TRF07 (17) | When Delay Buffer use more than limit |
| Rubbing | Rubbing (5) | Loader (2) | When Rubbing Unit All Down |

      - Buffer, Port Dummy Glass Info

Indexer & Transfer Equipment should provide dummy information (Port No. and Slot contents) to other process equipment in Inline. This Dummy information should be updated real-time and available while Dummy Port/Buffer is in ‘In Use’ state.

Process equipment that requires Dummy operation should provide screen to view Dummy information of each Dummy Port/Buffer (refer to Equipment Operation Scenario for which equipment needs to provide this function)

        - PLC MAP – Local PLC – Link Relay (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Port #n Dummy Cassette Info Block | Dummy Glass Exist | 4 | Data | INT | "On: Dummy Exist Off: No Dummy Exist [1st Word ] 00 ~ 15 Bit : Glass Existence Slot 1 ~ 16 00 Bit: Glass Existence Slot 1 01 Bit: Glass Existence Slot 2 …. 15 Bit: Glass Existence Slot 16 [2nd Word] 00 ~ 15 Bit : Glass Existence Slot 17 ~ 32 [3rd Word] 00 ~ 15 Bit : Glass Existence Slot 33 ~ 48 [4th Word] 00 ~ 15 Bit : Glass Existence Slot 49 ~ 64" |
|  | Dummy Type | 1 | Data | INT |  |
| Dummy Buffer Info Block | Dummy Glass Exist | 2 | Data | INT | "On: Dummy Exist Off: No Dummy Exist |

|  |  |  |  |  | [1st Word ] 00 ~ 15 Bit : Glass Existence Slot 1 ~ 16 00 Bit: Glass Existence Slot 1 01 Bit: Glass Existence Slot 2 …. 15 Bit: Glass Existence Slot 16 [2nd Word] 00 ~ 15 Bit : Glass Existence Slot 17 ~ 32 |
| --- | --- | --- | --- | --- | --- |

      - Block Glass Count Management

Glass count is needed to be managed by Equipment/Block. Glass count will be controlled by ‘Allowable Glass Count’ of Equipment/Block. Transfer Equipment controls transfer flow defending on each equipment’s glass count info.

        - PLC MAP – Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| Equipment Special Event | Allowable Glass Count Changed | 1 | Event | - On: Fire Event - Off: Event End | Glass Count Block |
| Equipment Special Event Reply | Allowable Glass Count Changed Event Reply | 1 | Event | - On: Fire Event - Off: Event End |  |

        - PLC MAP – Local PLC – Link Relay (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Glass Count Block | MAX Capacity | 1 | Data | INT | Max capacity of glass by structure |
|  | Allowable Glass Count | 1 | Data | INT | Max capacity of glass by managing purpose |
|  | Current Glass Count | 1 | Data | INT | Current all glass count in equipment |
|  | CF Glass Count | 1 | Data | INT | Current CF glass count in equipment |
|  | TFT Glass Count | 1 | Data | INT | Current TFT glass count in equipment |
|  | Dummy Glass Count | 1 | Data | INT | Current dummy glass count in equipment (include UV Mask) |

      - Transfer Sync

Multi Slot Structure(like Oven, Buffer) which don't have Glass Detect Sensor, It’ information and interface base on glass existence become less reliable.

Transfer manages information of Glass existence itself, and do not get it from multi slot equipment. When two transfer has same multi slot structure between them and they do In/Out action separately, for updating

information of glass existence in each slot, the transfer should send message of Glass In/Out action to relative transfer mutually.

        - PLC MAP – Local PLC – Link Relay (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |

| Transfer Sync Data Block | Slot Data #A | 1 | Data | INT | Slot No. where Glass has been stored or fetched out. |
| --- | --- | --- | --- | --- | --- |
|  | Slot Data #B | 1 | Data | INT | Slot No. where Glass has been stored or fetched out. * Used if robot transfer 2 glass at once |

        - PLC MAP – Local PLC – Link Relay (B Area), Local to Local

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| Transfer Sync Event | Glass In Slot Event | 1 | Event | - On: Fire Event - Off: Event End | Transfer Sync Data Block |
|  | Glass Out Slot Event | 1 | Event | - On: Fire Event - Off: Event End | Transfer Sync Data Block |
|  | Glass In Slot Event Reply | 1 | Event | - On: Fire Event - Off: Event End |  |
|  | Glass Out Slot Event Reply | 1 | Event | - On: Fire Event - Off: Event End |  |

      - Panel Detect Signal

Indexer should reportthe Panel Detect Signal when the condition of Cassette is empty or full at Portwhile a Cassette loaded.

The Signal is turn off when Cassette is full but it is turn on when Cassette is empty

        - PLC MAP – Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| Port Panel Detect Signal | Port#1 Panel Detect Signal A | 1 | Event | - On: Empty - Off: glass exist |  |
|  | Port#1 Panel Detect Signal B | 1 | Event | - On: Empty - Off: glass exist |  |
|  | … |  |  |  |  |
|  | Port#N Panel Detect Signal A | 1 | Event | - On: Empty - Off: glass exist |  |
|  | Port#N Panel Detect Signal B | 1 | Event | - On: Empty - Off: glass exist |  |

      - Material Use Report(Rubbing, ODF, PI)

EQP sends Material Use Report after kitting and validating complete.

        - PLC MAP – Local PLC – Link Relay (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Material Use Block | Material ID | 10 | Data | ASCII |  |
|  | Material Position | 1 | Data | INT | 1: Roller1 2: Roller2 |
|  | Unit No | 1 | Data | INT |  |
|  | Used Count | 1 | Data | INT |  |
|  | Glass ID | 10 | Data | ASCII |  |

        - PLC Map – Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (CIMPC) |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Special Event | Material Use Report | 1 | Event | - On: Fire Event - Off: Event End | Material Use Block | Reply for Receiving |

        - PLC Map –– Master(EAS) –– Link Relay (B Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Equipment Special Event Reply | Material Use Report Reply | 1 | Reply | - On: Fire Event - Off: Event End |  |

    - ODF

      - Block Partial Down Report

Line has parallel equipment/unit structure, When these structure has happen partially down, there is operation that it need to adjust ‘Allowable Glass’ in ‘Block’ or the equipment. change Partial Down Report is using for this operation. TR04 report tree SD equipment’s partial down state. TR05 report two LC equipment’s partial down state. TRF10 report two UV equipment’s partial down state. ARC Oven report two oven’s partial down state. Seal Oven report 4 oven’s partial down state.

        - PLC MAP – Local PLC – Link Relay (W Area), Local to Local

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Partial Down Report Block | Partial Down State | 1 | Data | INT | - : Default(None State) - : 1/2 Down - : 1/3 Down - : 2/3 Down - : 1/4 Down - : 3/4 Down 9 : All Down |

        - PLC MAP – Local PLC – Link Relay (B Area), Local to Local

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| Partial Down Report | Partial Down Report | 1 | Event | - On: Fire Event - Off: Event End | Partial Down State |
| Partial Down Report Reply | Partial Down Report Reply | 1 | Event | - On: Fire Event - Off: Event End |  |

      - Seal Inspection Data transfer

LC equipment in order to decide the condition of LC Dispensing, LC equipment take the seal inspection result of CF glass which is pair glass of TFT glass from Seal Inspection equipment.

        - PLC MAP – Local PLC – Link Relay (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| LC01 Seal Inspection Result | CF Glass Cassette | 1 | Data | INT |  |

| Request Data | Sequence No |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | CF Glass Job Sequence No | 1 | Data | INT |  |
|  | CF Glass Cassette Sequence No | 1 | Data | INT |  |
| LC01 Seal Inspection Result | CF Glass Job Sequence No | 1 | Data | INT |  |
| Reply Data |  |  |  |  |  |
|  |  |  |  |  | 0: No inspection result |
|  | Seal Inspection Result | 1 | Data | INT | 1: OK |
|  |  |  |  |  | 2: NG |
| LC02 Seal Inspection Result Request Data | Same as above |  |  |  |  |
| LC02 Seal Inspection Result Request Reply Data |  |  |  |  |  |
| Job Data Check Block | Job Data Check To VI | 96 | Data | - | Refer to Job Data A |

        - PLC MAP – Local PLC – Link Relay (B Area), Local to Local

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| CELL Special Event | LC01 Seal Inspection Data Request | 1 | Event | - On: Fire Event - Off: Event End | LC01 Seal Inspection Result Request Data |
|  | LC02 Seal Inspection Data Request | 1 | Event | - On: Fire Event - Off: Event End | LC01 Seal Inspection Result Reply Data |
|  | Job Data Check to VI Request | 1 | Event | - On: Fire Event - Off: Event End | Job Data Check Block |
| CELL Special Reply | LC01 Seal Inspection Data Request Reply | 1 | Event | - On: Fire Event - Off: Event End | LC02 Seal Inspection Result Request Data |
|  | LC02 Seal Inspection Data Request Reply | 1 | Event | - On: Fire Event - Off: Event End | LC02 Seal Inspection Result Reply Data |
|  | Job Data Check to VI Request Reply | 1 | Event | - On: Fire Event - Off: Event End |  |

      - Dummy Glass Request ( EQ to EQ) – ODF only

        - 10.12.1.1 Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Dummy Glass Request Block | Dummy Glass Count | 1 | Data | INT | Count (0~28) |
|  | Dummy Slot | 1 | Data | INT | Buffer Slot No |
|  | Dummy Type | 1 | Data | INT | Dummy Type |
|  | Dummy Load Type | 1 | Data | INT | 1:Load, 2:Unload, 3:Exchange |

        - 10.12.1.2 Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (CIMPC) |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Special Event | Dummy Glass Request | 1 | Event | - On: Fire Event - Off: Event End | Dummy Glass Request Block |  |

        - 10.12.1.3 Master (EAS) –– Link Relay (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |

| Equipment Special Event Reply Block | Dummy Glass Request Return Code | 1 | Data | INT | 1: OK 2: NG |
| --- | --- | --- | --- | --- | --- |

        - 10.12.1.4 Master (EAS) –– Link Relay (B Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Equipment Special Event Reply | Dummy Glass Request Reply | 1 | Reply | - On: Fire Event - Off: Event End | Dummy Glass Request Return Code |

    - PI

        - PI Line Cassette Loading Sequence

PI Loader run PI Line Cassette Loading Sequence by following flag of Indexer Operation Mode.

It defines delivered cassette amount and sequence of CF,TFT from stocker to PI loader. Detail about Indexer Operation Mode information refers to 6.1.2 Indexer Operation Mode content.

| 1 Cycle Count | Processing Cassette Loading Action | Indexer Operation Mode |
| --- | --- | --- |
| 2 | TTCCTTCCTTCC… | 3101:TCGlassMode#1 |
| 4 | TTTTCCCCTTTT… | 3102:TCGlassMode#2 |
| CF only | CCCCCCCCCCC… | 3103:TCGlassMode#3 |
| TFT only | TTTTTTTTTTTT… | 3104:TCGlassMode#3 |

        - Cleaner Turn Mode

Indexer’s downstream-cleaner has Turn Align Unit on upper end. Operator choose function to use ‘Turn unit’, Cleaner turn on ‘Cleaner Turn Mode’.

          - PLC MAP – Local PLC – Link Relay (B Area), Local to Local

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| Indexer | Cleaner Turn Mode | 1 | Event | - On: Fire Event Keep on while use Turn Unit - Off: Event End |  |

        - Common Defect Error Report

PI Inspection requests Inkjet to stop processing when it detects repeatable defect on glasses. It use ‘Common Defect Report’ for requesting.

- Event Report : PI Inspection

- Event Receive : Inkjet

          - PLC MAP – Local PLC – Link Relay (B Area), Local to Local

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |

| CELL Special Event | IJ01 Common Defect Report | 1 | Event | - On: Fire Event - Off: Event End |  |
| --- | --- | --- | --- | --- | --- |
|  | IJ02 Common Defect Report | 1 | Event | - On: Fire Event - Off: Event End |  |
| CELL Special Reply | IJ01 Common Defect Report Reply | 1 | Event | - On: Fire Event - Off: Event End |  |
|  | IJ02 Common Defect Report Reply | 1 | Event | - On: Fire Event - Off: Event End |  |

        - Unloader CST Auto End (Local to Local)

When a Group Lot starts, Loader will send Group ID & Group Lot Count(2CST/100EA standard) to Unloader. Unloader manages the Group Lot by itself.

Lot’s Group Lot Count of Group Lot of First is 100EA, when a Second Lot starts , send the sum which First Lotis added with Group Lot Count of Second Lot Group. For example, [First Lot (50EA) + Second Lot(40EA) = Group Lot Count(90EA)].

when Port Cancel & Port Abort happened among Group Lot,remain Glass Count and Group ID will been sent to Unloader.

If CST Map Info mismatch, operator checks the CST Map Info and send modified Glass Count to Unloader which should edit on touch panel of Loader

Unloader has to manage Group Lot Glass all loaded in CST, even CST is not Full CST, if the relative CST is end condition, then execute Unclamp.

- Event Report : PI Loader

- Event Receive : PI Unloader

Figure 11-2 PI CST Operation

- when the Glass out from CST every time among Group Lot, Loader has to send Glass’s Group ID, Glass ID, CST Sequence No, Job Sequence No with End Condition Glass Out Report to Unloader.

- Unloader has to manage the Group Lot itself

- When each EQ report scrap of glass which is Removed Job Report , Unloader will use the Scrap information and execute Unclamp to CST automatically. So Unloader always should scan Local #n Removed Job Report and Removed Job Data but need to reply of Removed Job Report

          - PLC MAP – Local PLC (PI Loader) – Link Relay (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| End Condition Report Block | End Condition ID | 10 | Data | ASC | Group ID or LOT(Dummy) ID |
|  | End Condition Count | 1 | Data | INT | 0 ~ 200 (Glass Count) |

          - PLC MAP – Local PLC (PI Loader) – Link Relay (B Area), Local to Local

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| CELL Special Event | End Condition Count Change Report | 1 | Event | - On: Fire Event - Off: Event End | End Condition Report Block |
| CELL Special Reply | End Condition Count Change Report Reply | 1 | Event | - On: Fire Event - Off: Event End |  |

          - PLC MAP – Local PLC – Link Relay (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| End Condition Report Out Block | End Condition ID | 10 | Data | ASC | Group ID or LOT(Dummy) ID |
|  | Glass ID | 10 | Data | ASC |  |
|  | CST Sequence No | 1 | Data | INT |  |
|  | Job Sequence No | 1 | Data | INT |  |

          - PLC MAP – Local PLC (PI Loader) – Link Relay (B Area), Local to Local

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| CELL Special Event | End Condition Glass Out Report | 1 | Event | - On: Fire Event - Off: Event End | End Condition Report Block |
| CELL Special Reply | End Condition Glass Out Report Reply | 1 | Event | - On: Fire Event - Off: Event End |  |

        - Dummy Glass Request(PI Inkjet, TR01, Loader Only, EQ to EQ)

This signal is to be used when equipment requires Dummy supply from Inkjet.

          - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Dummy Glass Request Block | Dummy Glass Count | 1 | Data | INT | Count (1~50) |
|  | Dummy Type | 1 | Data | INT | Dummy Type |
|  | Dummy Glass Request Local No | 1 | Data | INT |  |

          - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (CIMPC) |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Special Event | Dummy Glass Request | 1 | Event | - On: Fire Event - Off: Event End | Dummy Glass Request Block |  |

          - Master (EAS) –– Link Relay (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Dummy Glass Request Reply Block | Dummy Glass Request Return Code | 1 | Data | INT | 1: OK 2: NG |

          - Master (EAS) –– Link Relay (B Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Equipment to Equipment Event Reply | Dummy Glass Request Reply | 1 | Reply | - On: Fire Event - Off: Event End | Dummy Glass Request Reply Block |

        - Normal Glass Request(PI Inkjet, Loader Only, EQ to EQ)

This signal is to be used when equipment requires Normal Glass supply from Inkjet.

          - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Normal Glass Request Block | Normal Glass Count | 1 | Data | INT | Count (1~50) |
|  | Normal Type | 1 | Data | INT | Normal Type - : TFT - : CF |
|  | Normal Glass Request Local No | 1 | Data | INT |  |

          - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (CIMPC) |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment to Equipment Event | Normal Glass Request | 1 | Event | - On: Fire Event - Off: Event End | Normal Glass Request Block |  |
|  | Normal Mode Request | 1 | Event | - On: Fire Event - Off: Event End | TFT, CF Glass has been work completed. Use if the rest of the glass transfer is required. |  |

          - Master (EAS) –– Link Relay (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Normal Glass Request Reply Block | Normal Glass Request Return Code | 1 | Data | INT | 1: OK 2: NG |

          - Master (EAS) –– Link Relay (B Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Equipment to Equipment Event Reply | Normal Glass Request Reply | 1 | Reply | - On: Fire Event - Off: Event End | Normal Glass Request Reply Block |

    - Rubbing

      - Rubbing Operation Mode & Path Direction

The operator can change Path Mode by using Rubbing Touch Panel. USC01 and USC02 should support and adjust Link Signal according to numerical value of Path Mode Item.

There are 3 types of rubbing operation mode. Entire line mode control to be set with ‘Indexer Operation Mode’. But rubbing equipment's touch screen also need function for changing rubbing operation mode for operator. Rubbing sent request for changing ‘Indexer Operation Mode’ when operator conduct function. . Detail about ‘Indexer Operation Mode’ information refers to 6.1.2 ‘Indexer Operation Mode’ content.

| Rubbing Operation Mode | Remark | Indexer Operation Mode |
| --- | --- | --- |
| 1 | Two rubbing unit run with 2 different recipe | 3201:TCGlassMode#1 |
| 2 | Two rubbing unit run with same recipe | 3202:TCGlassMode#2 |

        - PLC MAP – Local PLC – Link Relay (W Area), Local to Local

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Path Mode Block | Path Mode | 1 | Data | INT | - : USC01 Loading , USC02 Unloading - : USC02 Loading , USC01 Unloading - : USC01 Loading/Unloading - : USC02 Loading/Unloading |
| Rubbing Operation Mode Block | Rubbing Operation Mode | 1 | Data | INT | - : Mode 1 - : Mode 2 - : Mode 3 - : MMG Mode - ~: Reserve |
| Rubbing Operation Mode Change Reply Block | Rubbing Operation Mode Change Reply Code | 1 | Data | INT | - : OK - : NG (Operation Mode can’t change able situation) |

        - PLC MAP – Local PLC – Link Relay (B Area) , Local to Local

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| CELL Special Event | Path Operation Change Event #1 (Rubbing) | 1 | Event | - On: Fire Event - Off: Event End | Path Mode Block |
|  | Path Operation Change Event #2 (Rubbing) | 1 | Event | - On: Fire Event - Off: Event End | Path Mode Block |
|  | Rubbing Mode Change Request (Rubbing) | 1 | Event | - On: Fire Event - Off: Event End | Rubbing Operation Mode Block |
| CELL Special Reply | Rubbing Mode Change Request Reply(Indexer) | 1 | Event | - On: Fire Event - Off: Event End | Rubbing Operation Mode Change Reply Block |
|  | Path Operation Change Event #1 Reply(USC1) | 1 | Event | - On: Fire Event - Off: Event End |  |
|  | Path Operation Change Event #2 Reply(USC2) | 1 | Event | - On: Fire Event - Off: Event End |  |

      - USC State

USC supply USC operation pattern and decides function of USC use or not.

        - PLC MAP – Local PLC – Link Relay (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| USC State Block | USC Skip Mode | 1 | Data | INT | - : ON (=Skip) - : OFF, USC Skip cause of USC trouble |
|  | Pin Align | 1 | Data | INT | - : ON, - : OFF ( = Not Use) |
|  | VCR Skip Mode | 1 | Data | INT | - : ON (=Skip) - : OFF, Not Use VCR, also not check Glass Direction |
|  | Thickness Check | 1 | Data | INT | - : ON, - : OFF ( = Not Use) |
|  | Broken Check | 1 | Data | INT | - : ON, - : OFF ( = Not Use) |
|  | Glass Exist | 1 | Data | INT | - : ON, - : OFF ( = Not Exist) |

        - PLC MAP – Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| Equipment Special Event | USC State Report | 1 | Event | - On: Fire Event - Off: Event End | USC State Block |

        - Master (EAS) – Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| Equipment Special Event Reply | USC State Report #1 Reply | 1 | Reply | - On: Fire Event - Off: Event End |  |

      - Steam Skip Mode

Indexer represents state of activation ‘Steam Skip Mode’.

When Steam Skip Mode is on, Indexer sets SA-Steam Inspection Flag Bit as on which is a part of Inspection Flag of Job Data, then this Job Data reports the Stored Job Report of returning glass to PB Port.

        - PLC MAP – Local PLC – Link Relay (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Steam Mode Block | Steam Skip Mode | 1 | Data | INT | - : ON (=Skip) - : OFF - : ALL (All Glass Steam Inspection) |

        - PLC Map – Local PLC – Link Relay(B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| Equipment Special Event | Steam Skip Mode Report | 1 | Event | - On: Fire Event - Off: Event End | Steam Mode Block |

        - Master (EAS) – Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| Equipment Special Event Reply | Steam Skip Mode Report Reply | 1 | Reply | - On: Fire Event - Off: Event End |  |

      - TC Mode

        - PLC MAP – Local PLC – Link Relay (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Rubbing TC Mode Block | Rubbing TC Mode | 1 | Data | INT | - : TFT Only Mode - : CF Only Mode - : TFT/CF Both Mode - : Dummy Mode |

        - PLC Map – Local PLC – Link Relay(B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| Equipment Special Event | Rubbing TC Mode Report | 1 | Event | - On: Fire Event - Off: Event End | Rubbing TC Mode Block |

        - Master (EAS) – Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| Equipment Special Event Reply | RubbingTC Mode Report Reply | 1 | Reply | - On: Fire Event - Off: Event End | Only Reply |

      - Material Status Sub Block

Rubbing sends Material Status to CIM with Material Status Sub Block including additional Material Info.

        - PLC MAP – Local PLC – Link Relay (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Material Status Sub Block | Material Position | 1 | Data | INT | 1: Roller1 2: Roller2 |
|  | Used Count | 1 | Data | INT | 1~2000 |
|  | Thickness | 1 | Data | INT | 1.500~1.855(Data*1000) |
|  | Torque | 1 | Data | INT | 3.00~20.00(Data*100) |
|  | Recipe No | 1 | Data | INT | 0~999 |
|  | Glass Angle | 4 | Data | ASCII | -10~10 |
|  | Work Angle | 4 | Data | ASCII | -180~270 |
|  | Head Angle | 4 | Data | ASCII | -10~10 |

|  | Operator ID | 10 | Data | ASCII |  |
| --- | --- | --- | --- | --- | --- |

      - AOI Result Report

The AOI inspection shall report the following event when reporting the inspection results.

        - PLC MAP – Local PLC – Link Relay (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| AOI ResultRepor t Block | Cassette Sequence No | 1 | Data | INT | 1~65535 |
|  | Job Sequence No | 1 | Data | INT | 1~65535 |
|  | GlassID | 10 | Data | ASCII |  |
|  | AOI Result | 1 | Data | INT | - : Communicate GLS to Steam - : Communicate GLS to Buffer04 - : Communicate GLS to Buffer01(Scrap Buffer) |

        - PLC Map – Local PLC – Link Relay(B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| Equipment Special Event | AOI Result Report | 1 | Event | - On: Fire Event - Off: Event End | AOI Result Report Block |

        - Master (EAS) – Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| Equipment Special Event Reply | AOI Result Report Reply | 1 | Reply | - On: Fire Event - Off: Event End |  |

    - Cutting

      - Cutting Line Current Recipe Management

Cutting line recipe structure have some special features. Glass’s ‘Panel Pattern’ has 2 kind of types, Normal and MMG. For Normal type, Line run with one recipe by Inch(also main). For MMG type, Line run with 2 recipe by Inch, one for main panel size, and other for sub panel size.

Cutting line’s each equipment should have same key parameters and structure of cutting line recipe. And EAS should update recipe’s key parameters from Scriber’s recipe update event.

- Panel Position Rule

| Panel Rule ID Item Name | Position Number |
| --- | --- |
| Panel #1 Rule ID | 1 |
| Panel #2 Rule ID | 2 |
| … | … |

| Panel#256 Rule ID | 256 |
| --- | --- |

        - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Scriber Current Recipe Data Block | Master Recipe ID | 1 | Data | INT | 0~999 |
|  | Panel Type | 1 | Data | INT | - : Normal - : Normal + MMG 3: Q-Normal 4: Q-Normal + Q-MMG |
|  | Sub#1 Recipe ID | 5 | Data | ASC | 0~9, A~Z, a~z (except:: I, O, i, o) |
|  | Sub#1 Panel Row Size | 2 | Data | F |  |
|  | Sub#1 Panel Col Size | 2 | Data | F |  |
|  | Sub#1 P-Code | 5 | Data | ASC | 0~9, A~Z, a~z (except:: I, O, i, o) |
|  | Sub#1 Use Type | 5 | Data | ASC | 0~9, A~Z, a~z (except:: I, O, i, o) |
|  | Sub#1 Panel Count | 1 | Data | INT | 1~256 (Main panel count will create from source glass) |
|  | Sub#1 Type Use Flag | 1 | Data | INT | - : Default - : Use - : Not use |
|  | Sub#2 Recipe ID | 5 | Data | ASC | 0~9, A~Z, a~z (except:: I, O, i, o) |
|  | Sub#2 Panel Row Size | 2 | Data | F |  |
|  | Sub#2 Panel Col Size | 2 | Data | F |  |
|  | Sub#2 P-Code | 5 | Data | ASC | 0~9, A~Z, a~z (except:: I, O, i, o) |
|  | Sub#2 Use Type | 5 | Data | ASC | 0~9, A~Z, a~z (except:: I, O, i, o) |
|  | Sub#2 Panel Count | 1 | Data | INT | 1~256 (Sub panel count will create from source glass [If Source is not MMG, value is 0]) |
|  | Sub#2 Type Use Flag | 1 | Data | INT | - : Default - : Use - : Not use |
|  | Panel #1 Rule ID | 1 | Data | ASC | Panel Naming Rule appendix, 2 Character |
|  | … |  |  |  |  |
|  | Panel #256 Rule ID | 1 | Data | ASC |  |
| Cutting Event Report Block | Cassette Sequence No | 1 | Data | INT |  |
|  | Job Sequence No | 1 | Data | INT |  |

        - Master (EAS) – Link Register (W Area)

        - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| Equipment Special Event | Scriber Current Recipe Changed Event | 1 | Event | - On: Fire Event - Off: Event End | Scriber Current Recipe Data Block Cutting Event Report Block |

        - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| Equipment Special EventReply | Scriber Current Recipe Changed Event Reply | 1 | Reply | - On: Reply - Off: Reply End |  |

      - Unload CST Check

Panel cassette is managed by panel size and cassette type(A & B). For panel cassette management, B9 provide ‘Use-Type’.

‘Use Type Check Request’ is a command for Loader/Transfer(next to Scriber) check existence of unloading port which setting with demanded ‘Use-Type’. For this command has allocated two data – Main Use-Type,

Sub Use-Type

– But it is possible tohas empty data of Sub Use-Type, if cassette information was changed to normal recipe ( not for MMG ).

‘Cassette Check Request’ is a command for TR to check existence of unloading cassette which is on the demanded ‘Use-Type’ port.

        - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Cassette Check RequestReply Block | Return Code | 1 | Data | INT | 1: Cassette Exist(Prepared) 2: Not Exist |
| Use Type CheckRequestReply Block | Return Code | 1 | Data | INT | 1: All Ready 2: Main Not Ready 3: Sub Not Ready 4: Not found Both |
| Use Type Check Data Block | Sub#1 Use-Type | 5 | Data | ASC | 0~9, A~Z, a~z (except:: I, O, i, o) |
|  | Sub Use-Type | 5 | Data | ASC | 0~9, A~Z, a~z (except:: I, O, i, o) |
| Cassette Check Data Block | Use-Type | 5 | Data | ASC | 0~9, A~Z, a~z (except:: I, O, i, o) |

        - Local PLC – Link Relay (B Area), Local to Local

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| Equipment Special Event Reply | Cassette Check Request Reply(Unloader) | 1 | Reply | - On: Reply - Off: Reply End | Cassette Check Request Reply Block |
|  | Use Type Check Request Reply(Unloader) | 1 | Reply | - On: Reply - Off: Reply End | Use Type Check Request Reply Block |
| Equipment Special Event | Use Type Check Request (Loader) | 1 | Event | -- On: Fire Event - Off: Event End | Use Type Check Data Block |
|  | Cassette Check Request (TR) | 1 | Event | - On: Fire Event - Off: Event End | Cassette Check Data Block |

      - Glass Count Management for Cutting Line

Cutting line equipment should provide word area for present Job amount inside equipment. Loader/Unloader update glass count with Job stored count in cassette.

        - PLC MAP – Local PLC – Link Relay (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Glass Count Block | Glass Count | 1 | Data | INT | Glass amount inside the equipment currently |
|  | Main Panel Count | 1 | Data | INT | Main panel amount inside the equipment currently |
|  | Sub Panel Count | 1 | Data | INT | Sub panel amount inside the equipment currently |

      - Cutting Event Report

When Scriber finish cutting, Scriber send cutting event report to EAS for send to MES

        - PLC MAP – Local PLC – Link Relay (W Area)

None.

        - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| Equipment Special Event | Cutting Event Report | 1 | Event | - On: Fire Event - Off: Event End | Scriber Current Recipe Data Block |

        - Master (EAS) – Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| Equipment Special Event Reply | Cutting Event Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |

      - Big Panel Unloading

Scriber’s ‘Big Panel Unload’ operation with pick up unit need virtual port state report for handling Line in/out job data of multiple panel.

And additionally It need to report ‘Big Panel Unloading Start Event’ and ‘Big Panel Unloading Start Event’.

        - PLC MAP – Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| Equipment Special Event | Big Panel Unloading Start Event | 1 | Event | - On: Fire Event - Off: Event End | Big Panel Cassette Info Block |
|  | Big Panel Unloading End Event | 1 | Event | - On: Fire Event - Off: Event End | Big Panel Cassette Info Block |

        - PLC MAP – Local PLC – Link Relay (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Big Panel Cassette Info Block | Cassette ID | 10 | Data | ASC | Unloading panel’s packaging Identification |
|  | Panel Count | 10 | Data | ASC | Planed(or Unloaded) panel’s Count |

        - Master (EAS) – Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |

| Equipment Event Reply | Big Panel Unloading Start Event Reply | 1 | Reply | - On: Reply - Off: Reply End |  |
| --- | --- | --- | --- | --- | --- |
|  | Big Panel Unloading End Event Reply | 1 | Reply | - On: Reply - Off: Reply End |  |

      - Auto Grind Amount Revision [TBD]

When NG continues In Edge Inspection, Equipment of Edge Inspection provides the value of Auto Grind for correct value.

This is not decided in specific contents and Size of Word in PLC will be provide after discussion.

        - PLC MAP – Local PLC – Link Relay (W Area), Local to Local

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Revision Inspection Data | Inspection Type | 1 | Data | INT | - : Not Defined - : Edge Inspection |
|  | Revision Data for Grinder | 200 | Data | ASC | [ TBD ] |

        - PLC MAP – Local PLC – Link Relay (B Area) , Local to Local

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| CELL Special Event | Auto Grind Amount Revision Data Report | 1 | Event | - On: Reply - Off: Reply End |  |
| CELL Special Reply | Auto Grind Amount Revision Data Report Reply | 1 | Event | - On: Fire Event - Off: Event End | Revision Inspection Data |

      - Panel ( Include Cell Test )

      - Switch Panel Size Report (Local to Local)

Edge Grinder, Grinder Cleaner, Edge Inspection need to report ‘Switch Panel Size Report’. This report need for MMG Main/Sub Panel Type-Model Change Operation. Each equipment report this when it complete preparation model change.

        - PLC MAP – Local PLC – Link Relay (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Switch Panel Info Data Block | Last Panel Cassette Sequence No | 1 | Data | INT | Cassette Sequence No of Last Panel of Current Model |
|  | Last Panel Job Sequence No | 1 | Data | INT | Job Sequence No of Last Panel of Current Model |
|  | Last Panel Position | 1 | Data | INT | Panel Position of Last Panel of Current Model |
|  | Next Panel P-Code | 5 | Data | ASC | Next Model Information |
|  | Next Panel Recipe ID | 5 | Data | ASC | Next Model Information |
|  | Next Panel Row | 2 | Data | F | Next Model Information |
|  | Next Panel Col | 2 | Data | F | Next Model Information |

        - PLC MAP – Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |

| CELL Special Event | Switch Panel Size Report | 1 | Event | On: Fire Event - Off: Event End | Switch Panel Info Data Block |
| --- | --- | --- | --- | --- | --- |
| CELL Special Reply | Switch Panel Size Report Reply | 1 | Event | - On: Reply - Off: Reply End |  |

      - Panel Info Monitoring

Scriber should write additional data in Panel Info Monitoring Block when equipment sends Cutting Event Report

And Scriber should write Position X, Position Y for a panel’s parent Glass in Job Data A when equipment sends Sent Out Job Report.

Especially, other equipment should not change these Items during the panel’s flow

Edge Inspection should read these data from Panel Info Monitoring Block and Job Data A when equipment sends Received Job Report

        - PLC Map – Local PLC – Link Relay (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Panel Info Monitoring Block | Main Panel Count | 1 | Data | INT | Scriber writes, Edge Inspection reads |
|  | Total Panel Count | 1 | Data | INT | Scriber writes, Edge Inspection reads |
|  | X Count | 1 | Data | INT | Scriber writes, Edge Inspection reads |
|  | Y Count | 1 | Data | INT | Scriber writes, Edge Inspection reads |

    - Panel Sorter Load

Equipment should send ‘Panel Sorter Load Complete’ to EAS in order to dispatch it when Operator load a panel or POL by manual.

      - PLC MAP – Local PLC – Link Relay (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Panel Sorter Load Block | Panel Sorter Load Type | 1 | Data | INT | 1 : Panel 2: POL |
|  | Updated Job Data | 96 | Data | - | Refer to Job Data A |

      - PLC MAP – Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| Equipment Special Event | Panel Sorter Load Complete | 1 | Event | On: Fire Event - Off: Event End | Panel Sorter Load Block |

      - Master (EAS) – Local PLC – Link Relay (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Panel Sorter Load Return Code Block | Return Code | 1 | Data | INT | 1:OK |

|  |  |  |  |  | 2 :NG |
| --- | --- | --- | --- | --- | --- |

      - Master (EAS) – Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| Equipment Special Event Reply | Panel Sorter Load Complete Reply | 1 | Event | On: Fire Event - Off: Event End | Panel Sorter Load Return Code Block |

    - Cell Test Inline

      - POL Load Request (Cell Test Inline, Cell Test Stand Alone)

VAP&AP need to request ‘POL Load Request’ to EAS in order to dispatch a POL from Buffer to Stage.

        - PLC MAP – Local PLC – Link Relay (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| POL Load Information Block | POL Product ID | 10 | Data | ASC | Product ID |
|  | POL Size Width | 2 | Data | F | POL Size Width |
|  | POL Size Height | 2 | Data | F | POL Size Height |
| Indexer POL Load Information#1 Block | POL Product ID | 10 | Data | ASC | Product ID |
|  | POL Size Width | 2 | Data | F | POL Size Width |
|  | POL Size Height | 2 | Data | F | POL Size Height |
| Indexer POL Load Information#2 Block | POL Product ID | 10 | Data | ASC | Product ID |
|  | POL Size Width | 2 | Data | F | POL Size Width |
|  | POL Size Height | 2 | Data | F | POL Size Height |

        - PLC MAP – Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| Equipment Special Event | POL Load Request | 1 | Event | On: Fire Event - Off: Event End | POL Load Information Block |
| Equipment to Equipment Event | Indexer POL Load Request#1 | 1 | Event | On: Fire Event - Off: Event End | Indexer POL Load Information#1Block |
|  | Indexer POL Load Request#2 | 1 | Event | On: Fire Event - Off: Event End | Indexer POL Load Information#2 Block |

        - Master (EAS) – Local PLC – Link Relay (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| POL Load Result Information Block | Return Code | 1 | Data | INT | 1:OK 2:NG |
| Indexer POL Load Result Iformation#1 Block | Return Code | 1 | Data | INT | 1:OK 2:NG |

| Indexer POL Load Result Information#2 Block | Return Code | 1 | Data | INT | 1:OK 2:NG |
| --- | --- | --- | --- | --- | --- |

        - Master (EAS) – Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| Equipment Special Event Reply | POL Load Request Reply | 1 | Event | On: Fire Event - Off: Event End | POL Load Result Information Block |
| Equipment to Equipment Event Reply | Indexer POL Load Request Reply#1 | 1 | Event | On: Fire Event - Off: Event End | Indexer POL Load Result Information#1 Block |
|  | Indexer POL Load Request Reply#2 | 1 | Event | On: Fire Event - Off: Event End | Indexer POL Load Result Information#2 Block |

      - POL Unload Request (Cell Test Inline, Cell Test Stand Alone)

Indexer need to request ‘POL Unload Request’ to EAS in order to dispatch a POL from Buffer to Port.

        - PLC MAP – Local PLC – Link Relay (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| POL Unload Information Block | POL Product ID | 10 | Data | ASC | Product ID |
|  | POL Size Width | 2 | Data | F | POL Size Width |
|  | POL Size Height | 2 | Data | F | POL Size Height |
| Indexer POL UnLoad Information#1 Block | POL Product ID | 10 | Data | ASC | Product ID |
|  | POL Size Width | 2 | Data | F | POL Size Width |
|  | POL Size Height | 2 | Data | F | POL Size Height |
| Indexer POL UnLoad Information#2 Block | POL Product ID | 10 | Data | ASC | Product ID |
|  | POL Size Width | 2 | Data | F | POL Size Width |
|  | POL Size Height | 2 | Data | F | POL Size Height |

        - PLC MAP – Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| Equipment Special Event | POL Unload Request | 1 | Event | On: Fire Event - Off: Event End | POL Unload Information Block |
| Equipment to Equipment Event | Indexer POL Unload Request#1 | 1 | Event | On: Fire Event - Off: Event End | Indexer POL Unload Information#1 Block |
|  | Indexer POL Unload Request#2 | 1 | Event | On: Fire Event - Off: Event End | Indexer POL Unload Information#2 Block |

        - Master (EAS) – Local PLC – Link Relay (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| POL Unload Result Information Block | Return Code | 1 | Data | INT | 1:OK 2:NG |
| Indexer POL Unload Result Information#1 Block | Return Code | 1 | Data | INT | 1:OK 2:NG |

| Indexer POL Unload Result Information#2 Block | Return Code | 1 | Data | INT | 1:OK 2:NG |
| --- | --- | --- | --- | --- | --- |

        - Master (EAS) – Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| Equipment Special Event Reply | POL Unload Request Reply | 1 | Event | On: Fire Event - Off: Event End | POL Unload Result Information Block |
| Equipment to Equipment Event Reply | Indexer POL Unload Request Reply#1 | 1 | Event | On: Fire Event - Off: Event End | Indexer POL Unload Result Information#1 Block |
|  | Indexer POL Unload Request Reply#2 | 1 | Event | On: Fire Event - Off: Event End | Indexer POL Unload Result Information#2 Block |

      - After Grade Decide Request (Local to Local)

Unloader need to request ‘After Grade Decide Request’ the other Panel Grade of except Panel Grade ‘A’ to VAP&AP. VAP&AP has to send Panel Grade after processing to Unloader.

        - PLC MAP – Local PLC – Link Relay (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| After Grade Decide Request Block | Panel ID | 10 | Data | ASC | Panel ID |
|  | Cassette Sequence No | 1 | Data | INT | Panel’ Cassette Sequence No |
|  | Job Sequence No | 1 | Data | INT | Panel’ Job Sequence No |

        - PLC MAP – Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| Equipment to Equipment Event | After Grade Decide Request | 1 | Event | On: Fire Event - Off: Event End | After Grade Decide Request Block |

        - PLC MAP – Local PLC – Link Relay (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| After Grade Decide Result Block | Result Panel Grade | 1 | Data | ASC | After Grade Decide Panel Grade |

        - PLC MAP – Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| Equipment to Equipment Event Reply | After Grade Decide Request Reply | 1 | Event | On: Fire Event - Off: Event End | After Grade Decide Result Block |

    - CST Model Change

      - CST Pitch Spec Request

Machine need to request ‘CST Pitch Spec Request’ to EAS in order to adjust the cassette pitch.

        - PLC MAP – Local PLC – Link Relay (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| CST Pitch Spec Request Block | CST ID | 10 | Data | ASC | Cassette ID |

|  | Port No | 1 | Data | INT | Port No |
| --- | --- | --- | --- | --- | --- |

        - PLC MAP – Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| Equipment Special Event | CST Pitch Spec Request | 1 | Event | On: Fire Event - Off: Event End | CST Pitch Spec Request Block |

        - Master (EAS) – Local PLC – Link Relay (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| CST Pitch Spec Request Confirm Block | CST ID | 10 | Data | ASC | Cassette ID |
|  | From Pitch | 1 | Data | INT | From Pitch Value |
|  | Target Pitch | 1 | Data | INT | Target Pitch Value |

        - Master (EAS) – Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| Equipment Special Event Reply | CST Pitch Spec Request Confirm | 1 | Event | On: Fire Event - Off: Event End | CST Pitch Spec Request Confirm |

      - CST Pitch Change Start Report

Machine need to request ‘CST Pitch Spec Request’ to EAS in order to adjust the cassette pitch.

        - PLC MAP – Local PLC – Link Relay (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| CST Pitch Change Start Report Block | CST ID | 10 | Data | ASC | Cassette ID |
|  | Port No | 1 | Data | INT | Port No |
|  | From Pitch | 1 | Data | INT | From Pitch Value |
|  | Target Pitch | 1 | Data | INT | Target Pitch Value |

        - PLC MAP – Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| Job Event | CST Pitch Change Start Report | 1 | Event | On: Fire Event - Off: Event End | CST Pitch Change Start Report Block |

        - Master (EAS) – Local PLC – Link Relay (W Area)

        - Master (EAS) – Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| Job Event Reply | CST Pitch Change Start Report Reply | 1 | Event | On: Fire Event - Off: Event End | CST Pitch Change Start Report Reply |

      - CST Pitch Change End Report

Machine need to request ‘CST Pitch Change End Report’ to EAS in order to Before the cassette pitch Change.

        - PLC MAP – Local PLC – Link Relay (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| CST Pitch Change End Report Block | CST ID | 10 | Data | ASC | Cassette ID |
|  | Port No | 1 | Data | INT | Port No |

|  | From Pitch | 1 | Data | INT | From Pitch Value |
| --- | --- | --- | --- | --- | --- |
|  | Target Pitch | 1 | Data | INT | Target Pitch Value |

        - PLC MAP – Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| Job Event | CST Pitch Change End Report | 1 | Event | On: Fire Event - Off: Event End | CST Pitch Change End Report Block |

        - Master (EAS) – Local PLC – Link Relay (W Area)

        - Master (EAS) – Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| Job Event Reply | CST Pitch Change End Report Reply | 1 | Event | On: Fire Event - Off: Event End | CST Pitch Change End Report Reply |

    - PS Height Value Management

This event reports the PS Height Average Value from the PS Inspection equipment.

The ODF LD equipment passes the PS Height value reported by the PS Inspection equipment when requested by the Glass ID.

      - PS Height Value Management

        - PLC MAP – Local PLC – Link Relay (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| PS Inspection Report Block | Glass ID | 10 | Data | ASC |  |
|  | Panel01 Height | 2 | Data | INT | Panel Average Height Value |
|  | Panel02 Height | 2 | Data | INT | Panel Average Height Value |
|  | Panel03 Height | 2 | Data | INT | Panel Average Height Value |
|  | Panel04 Height | 2 | Data | INT | Panel Average Height Value |
|  | Panel05 Height | 2 | Data | INT | Panel Average Height Value |
|  | … |  |  |  |  |
|  | Panel32 Height | 2 | Data | INT | Panel Average Height Value |
| PS Inspection Request Block | Glass ID | 10 | Data | ASC |  |

        - PLC MAP – Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| Equipment Special Event | PS Inspection Report | 1 | Event | On: Fire Event - Off: Event End | PS Inspection Report Block (PI Inspection) |
|  | PS Inspection Request | 1 | Event | On: Fire Event - Off: Event End | PS Inspection Request Block (ODF LD) |

        - Master (EAS) – Local PLC – Link Relay (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| PS Inspection Request Reply Block | Return Code | 1 | Data | INT | - : OK - : NG (There is no information about this glass) |
|  | Glass ID | 10 | Data | ASC |  |
|  | Panel01 Height | 2 | Data | INT | Panel Average Height Value |
|  | Panel02 Height | 2 | Data | INT | Panel Average Height Value |
|  | Panel03 Height | 2 | Data | INT | Panel Average Height Value |
|  | Panel04 Height | 2 | Data | INT | Panel Average Height Value |
|  | Panel05 Height | 2 | Data | INT | Panel Average Height Value |
|  | … |  |  |  |  |
|  | Panel32 Height | 2 | Data | INT | Panel Average Height Value |

        - Master (EAS) – Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items |
| --- | --- | --- | --- | --- | --- |
| Equipment Special Event Reply | PS Inspection Report Reply | 1 | Event | On: Fire Event - Off: Event End |  |
|  | PS Inspection Request Reply | 1 | Event | On: Fire Event - Off: Event End | PS Inspection Request Reply Block |

  - CF Special Function

    - Job Route Decision

      - Reprocess Flag & Reprocess Count& Reprocess Reason Code.

If one glass need to reprocessing, Equipment should write this flag (value = local No.).

When glass move from Unloader to Loader(ready to reprocessing), Unloader should reset Reprocess Flag (value=blank) and update Reprocess Count (+1)

Reprocess Flag Signal bit On Equipment : PPCL, COAT, HPCP, MCPD, MURA, EXPO Reprocess Reason Code

- Line Name List :

| Line | FITO01 | FITO02 | FITO03 | FPHT01 | FPHT02 | FPHT03 | FPHT04 | FPHT05 | FPHT06 | FPHT07 | FPHT08 | FPHT09 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Code | T1 | T2 | T3 | P1 | P2 | P3 | P4 | P5 | P6 | P7 | P8 | P9 |
| Line | FPHT10 | FPHT11 | FPHT12 | FPHT13 | FPHT14 | FPHT15 | FMES01 | FMES02 | FMES03 | FMRP01 | FCRP01 | FCRP02 |
| Code | PA | PB | PC | PD | PE | PF | S1 | S2 | S3 | M1 | C1 | C2 |
| Line | FCRP03 | FCRP04 | FFRP01 | FFRP02 | FFRP03 | FFRP04 | FXIN01 |  |  |  |  |  |
| Code | C3 | C4 | F1 | F2 | F3 | F4 | X1 |  |  |  |  |  |

- EQP Name List :

| EQP | Load | TR01 | TR02 | TR03 | TR04 | TR05 | UNLD | APP | Cleaner | Coater | VCD | HP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Code | LD | T1 | T2 | T3 | T4 | T5 | UD | AP | CL | CT | VC | HP |
| EQP | CP | Mura | EXPO | TITL | DEVP | AOI | Macro | MCPD | TPCD | PSHT | CDME | THCV |
| Code | CP | MU | EX | TT | DE | AO | MA | MC | TP | PS | CD | TH |
| EQP | Sputter | Oven | Repair |  |  |  |  |  |  |  |  |  |
| Code | SP | OV | RP |  |  |  |  |  |  |  |  |  |

- Reason Name List :

| Reason | Coater particle | Uncoating | Coater vertical Line | Coater horizontal Line | Cracks | Chip | APP Mura | CLN Delay |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Code | C1 | UC | C2 | C3 | SC | CH | AP | CS |
| Reason | AK Delay | EXPO particle | Electric discharge Error | TP Error | Color map Error | DEV Delay | Over Bake | Micro Out Spec |
| Code | KS | E1 | SP | TP | XY | DS | OB | RP |
| Reason | Overflow | Stage Mura | DEV horizontal Line | Oil contamination |  |  |  |  |
| Code | OF | SM | D1 | OL |  |  |  |  |

* Refer to CF Job Data : Reprocess Flag, Reprocess Count, Reprocess Reason Code

      - AOI Inspection Flag

This Flag only use for Sputter.

When this glass need to AOI Inspection, Sputter should write this flag ‘on’. This flag indicates the Job has to be inspected at MES Line AOI. AOI Inspection has to turn off this flag after having received Job.

* Refer to CF Job Data : AOI Inspection Flag

      - Target Port (Repair , FinalMacro)

This item use only for Sorting Mode in Repair, Final Macro inline.

When EAS send Cassette Map Download, EAS should write this item. TR01 should move glass to appointed port by target port item.

- Refer to CF Job Data : Target Port

Index Operation Mode(Line Operation Mode)

This is the reference value when glass processing or glass flow. Loader, when transferring Job to downstream, writes this information in Job Data based on current Line Operation Mode.

- Refer to CF Job Data : Indexer Operation Mode , 6.1.2 Indexer Operation Mode(Line Operation Mode)

      - AOI Risk Flag

This Flag only use for AOI.

When this glass need to Repair Process, AOI should write this flag ‘on’. This flag indicates the Job has to be inspected at MES Line Repair. Repair Process has to turn off this flag after having received Job.

* Refer to CF Job Data : AOI Risk Flag

    - Product Stop Signal

      - Product Stop Signal Flag

If glass judge and setting judge in equipment are equal, Equipment should write this flag ‘on’. Method of using this stop signal refer to Operation Scenario.

      - PLC MAP

        - Local PLC – Link Register (W Area)

None

        - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | EAS Action |
| --- | --- | --- | --- | --- | --- | --- |
| Product Stop Signal Block | HPCP Buffer Stop Signal | 1 | Event | - On: Stop - Off: Release |  |  |
|  | MCPD NG Stop Signal | 1 | Event |  |  |  |
|  | Mura Stop Signal | 1 | Event |  |  |  |
|  | Coater Purge Signal | 1 | Event |  |  |  |
|  | AOI Mark NG Stop Signal #A | 1 | Event |  |  |  |
|  | AOI Mark NG Stop Signal #B | 1 | Event |  |  |  |
|  | Shot Common Defect Stop Signal | 1 | Event |  |  |  |
|  | Titler Reading NG Stop Signal | 1 | Event |  |  |  |
|  | TP NG Stop Signal | 1 | Event |  |  |  |
|  | CD NG Stop Signal | 1 | Event |  |  |  |
|  | Macro NG Stop Signal | 1 | Event |  |  |  |
|  | Sputter Down Stop Signal | 1 | Event |  |  |  |
|  | Out Handling Stop Signal | 1 | Event |  |  |  |
|  | OVEN Down Stop Signal | 1 | Event |  |  |  |
|  | In Handling Stop Signal | 1 | Event |  |  |  |
|  | Strip Down or Trouble | 1 | Event |  |  |  |

| Block | Item | Points | Type | Sub Items | Related Word Items | EAS Action |
| --- | --- | --- | --- | --- | --- | --- |
|  | Auto Macro faulty detection | 1 | Event |  |  |  |
|  | PS Height NG Stop Signal | 1 | Event |  |  |  |
|  | AIS Standard Position NG | 1 | Event |  |  |  |

        - Master (EAS) – Link Register (W Area)

None.

        - Master (EAS) – Link Relay (B Area)

None.

    - PPID PreDownload

      - PPID PreDownload Flag

When CST include new batch “CST Data Download” and there are no old batch glass in Indexer(old batch glass count =0), Indexer should send PPID Pre-Download to downstream Equipment.(Indexer Only)

The Equipment which received PPID Pre-Download should send PPID Pre-Download to downstream Equipment when there are no old batch glass(old batch glass count = 0).(Equipment Only)

      - PLC MAP

        - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| PPID PreDownload Data Block | PPID | 1 | Data | INT |  |
|  | Batch ID | 10 | Data | ASC |  |

        - Local PLC – Link Relay (B Area) - Local to Local

| Block | Item | Points | Type | Sub Items | Related Word Items | EAS Action |
| --- | --- | --- | --- | --- | --- | --- |
| PPIDPreDownloa d Block | PPID PreDownload Flag | 1 | Event | - On: Stop - Off: Release | PPID PreDownload Data Block | ON must be kept more than 600ms |
|  | PPID PreDownload Flag Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |

        - Master (EAS) – Link Register (W Area)

None.

        - Master (EAS) – Link Relay (B Area)

None.

    - Block Stop

      - Block Stop Flag

When equipment receive PPID Pre-Download, equipment should write Block Stop Signal ‘on’.

Block Stop Flag bit On Equipment : refer to PLC Map or CF Shop Operation Scenario.

      - PLC MAP

        - Local PLC – Link Register (W Area)

None.

        - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | EAS Action |
| --- | --- | --- | --- | --- | --- | --- |
| Block Stop Block | Block Stop Flag | 1 | Event | - On: Stop - Off: Release |  |  |

        - Master (EAS) – Link Register (W Area)

None.

        - Master (EAS) – Link Relay (B Area)

None.

    - COAT PR

      - Coat PR Emptied Report

COAT should report this item when COAT PR empty.

      - PR ID Report

When PR change, COAT should report this item with PR ID.

      - Tank NO

Tank No which put PR.

      - Coat PR ID Report Return Code

| Return Code | Value | Format |
| --- | --- | --- |
| OK | 1 | INT |
| NG | 2 | INT |

      - PLC MAP

        - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Material Data Block | Tank No | 1 | Data | INT |  |
|  | PR ID | 10 | Data | ASC |  |
|  | Operator ID | 4 | Data | ASC |  |
|  | PR Weight | 1 | Data | INT | Gram (g) |

        - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | EAS Action |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment | Coat PR Emptied Report | 1 | Event | - On: Report | Material Data Block |  |

| Block | Item | Points | Type | Sub Items | Related Word Items | EAS Action |
| --- | --- | --- | --- | --- | --- | --- |
| Special Event |  |  |  | - Off: End |  |  |

        - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Special Event Reply | Coat PR Emptied Report Reply | 1 | Reply | - On: Reply - Off: End |  |  |

    - Block Stop Signal Off Command

      - Block Stop Signal Off Command

When Equipment which Block Stop Signal ‘On’ receive Block Off Command from EAS, Equipment should this bit ‘Off’.

      - PLC Map

        - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| EAS Command | Block Stop Signal Off Command | 1 | Reply | - On: Fire Command - Off: Command End/Timeout |  |  |

    - Equipment Reason Code Event

      - Sent Out Job Report

If the Equipment has made a decision as NG(Manual or Auto), it should provide a reason code for judging.

Manual NG, all Equipment should provide the ability to select NG Reason

Auto NG, some Equipment(COAT,MURA,EXPO,AOI,MACR,TPCD,PSHT,MCPD,THCV,REPAIR,CDME)

Possible auto select NG Reason. automatic decision-making is available.

- Line Name List :

| Line | FITO01 | FITO02 | FITO03 | FPHT01 | FPHT02 | FPHT03 | FPHT04 | FPHT05 | FPHT06 | FPHT07 | FPHT08 | FPHT09 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Code | T1 | T2 | T3 | P1 | P2 | P3 | P4 | P5 | P6 | P7 | P8 | P9 |
| Line | FPHT10 | FPHT11 | FPHT12 | FPHT13 | FPHT14 | FPHT15 | FMES01 | FMES02 | FMES03 | FMRP01 | FCRP01 | FCRP02 |
| Code | PA | PB | PC | PD | PE | PF | S1 | S2 | S3 | M1 | C1 | C2 |

| Line | FCRP03 | FCRP04 | FFRP01 | FFRP02 | FFRP03 | FFRP04 | FXIN01 | FREW01 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Code | C3 | C4 | F1 | F2 | F3 | F4 | X1 | R1 |  |  |  |  |

- EQP Name List :

| EQP | Load | TR01 | TR02 | TR03 | TR04 | TR05 | UNLD | APP | Cleaner | Coater | VCD | HP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Code | LD | T1 | T2 | T3 | T4 | T5 | UD | AP | CL | CT | VC | HP |
| EQP | CP | Mura | EXPO | TITL | DEVP | AOI | Macro | MCPD | TPCD | PSHT | CDME | THCV |
| Code | CP | MU | EX | TT | DE | AO | MA | MC | TP | PS | CD | TH |
| EQP | Sputter | Oven | Repair | CUTT |  |  |  |  |  |  |  |  |
| Code | SP | OV | RP | CU |  |  |  |  |  |  |  |  |

- Reason Name List :

| Reason | Coater particle | Uncoating | Coater vertical Line | Coater horizontal Line | Cracks | Chip | APP Mura | CLN Delay |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Code | C1 | UC | C2 | C3 | SC | CH | AP | CS |
| Reason | AK Delay | EXPO particle | Electric discharge Error | TP Error | Color map Error | DEV Delay | Over Bake | Micro Out Spec |
| Code | KS | E1 | SP | TP | XY | DS | OB | RP |
| Reason | Overflow | Stage Mura | DEV horizontal Line | Oil contamination |  |  |  |  |
| Code | OF | SM | D1 | OL |  |  |  |  |

      - PLC Map

        - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | EAS Action |
| --- | --- | --- | --- | --- | --- | --- |
| Job Event | Sent Out Job Report Downstream #N | 1 | Event | - On: Report - Off: End |  |  |

        - Local PLC – Link Relay (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Job Count Block | TFT(Array) Product Job Count | 1 | Data | INT |  |
|  | CF Product Job Count | 1 | Data | INT |  |
|  | Dummy Job Count | 1 | Data | INT |  |
| Sent Out Job Data Block | Job Data For Downstream Block 1~6 | 96 or 256 | Data | - | Refer to Job Data A or S |
| Sent Out Job Data Sub Block 1~6 | Downstream Path No | 1 | Data | INT | 1~6 |
| Equipment Reason Code Block | NG Judge Reason Code | 4 | Data | ASC |  |

        - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | EAS Action |
| --- | --- | --- | --- | --- | --- | --- |
| Job Event | Sent Out Job Report Reply Downstream #N | 1 | Event | - On: Reply - Off: End |  |  |

      - Removed Job Report

If the Equipment has made a decision Scrap, it should provide a reason code for scrap.

- Line Name List :

| Line | FITO01 | FITO02 | FITO03 | FPHT01 | FPHT02 | FPHT03 | FPHT04 | FPHT05 | FPHT06 | FPHT07 | FPHT08 | FPHT09 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Code | T1 | T2 | T3 | P1 | P2 | P3 | P4 | P5 | P6 | P7 | P8 | P9 |
| Line | FPHT10 | FPHT11 | FPHT12 | FPHT13 | FPHT14 | FPHT15 | FMES01 | FMES02 | FMES03 | FMRP01 | FCRP01 | FCRP02 |
| Code | PA | PB | PC | PD | PE | PF | S1 | S2 | S3 | M1 | C1 | C2 |
| Line | FCRP03 | FCRP04 | FFRP01 | FFRP02 | FFRP03 | FFRP04 | FXIN01 | FREW01 |  |  |  |  |
| Code | C3 | C4 | F1 | F2 | F3 | F4 | X1 | R1 |  |  |  |  |

- EQP Name List :

| EQP | Load | TR01 | TR02 | TR03 | TR04 | TR05 | UNLD | APP | Cleaner | Coater | VCD | HP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Code | LD | T1 | T2 | T3 | T4 | T5 | UD | AP | CL | CT | VC | HP |
| EQP | CP | Mura | EXPO | TITL | DEVP | AOI | Macro | MCPD | TPCD | PSHT | CDME | THCV |
| Code | CP | MU | EX | TT | DE | AO | MA | MC | TP | PS | CD | TH |
| EQP | Sputter | Oven | Repair | CUTT |  |  |  |  |  |  |  |  |
| Code | SP | OV | RP | CU |  |  |  |  |  |  |  |  |

- Reason Name List :

| Reason | Coater particle | Uncoating | Coater vertical Line | Coater horizontal Line | Cracks | Chip | APP Mura | CLN Delay |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Code | C1 | UC | C2 | C3 | SC | CH | AP | CS |
| Reason | AK Delay | EXPO particle | Electric discharge Error | TP Error | Color map Error | DEV Delay | Over Bake | Micro Out Spec |
| Code | KS | E1 | SP | TP | XY | DS | OB | RP |
| Reason | Overflow | Stage Mura | DEV horizontal Line | Oil contamination |  |  |  |  |
| Code | OF | SM | D1 | OL |  |  |  |  |

      - PLC Map

        - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | EAS Action |
| --- | --- | --- | --- | --- | --- | --- |

| Block | Item | Points | Type | Sub Items | Related Word Items | EAS Action |
| --- | --- | --- | --- | --- | --- | --- |
| Job Event | Removed Job Report | 1 | Event | - On: Report - Off: End |  |  |

        - Local PLC – Link Relay (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Removed Job Data Block | Removed Job Data | 96 | Data | ASC |  |

        - Local PLC – Link Relay (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Removed Job Data Sub Block | Unit or Port | 1 | Data | INT | 1: Unit 2: Port |
|  | Unit No or Port No | 1 | Data | INT | 0: No Unit Event 1~: Unit No or Port No |
|  | Sub Unit No | 1 | Data | INT | 0 : Unit |
|  | Slot No | 1 | Data | INT | 0: No Slot 1~ : Slot No of Cassette/Unit |
|  | Removed Flag | 1 | Data | INT | - Scrap - Take Out - Reserved~ |

        - Local PLC – Link Relay (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Equipment Reason Code Block | Scrap Reason Code | 4 | Data | ASC |  |

        - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| Job Event Reply | Removed Job Report Reply | 1 | Reply | - On: Reply On - Off: Reply Off |  |  |

    - EXPO Mask

      - EXPO Mask Check Request

EAS should command this item when Mask Check Request.

      - EXPO Mask Load Request/Complete

EXPO should report this item when Mask Load Request/Complete.

      - EXPO Mask Unload Request/Complete

EXPO should report this item when Mask Unload Request/Complete.

      - EXPO Mask Failed Cancel Command

EAS should command this item when Mask Load Failed.

      - EXPO Mask Move Command

EAS should command this item when Mask need Move.

      - PLC MAP

        - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Mask Data Block | PPID | 1 | Data | INT |  |
|  | Mask ID | 15 | Data | ASC |  |

        - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | EAS Action |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Event | Mask Load Request | 1 | Event | - On: Report - Off: End | Mask Data Block |  |
|  | Mask Load Complete | 1 | Event | - On: Report - Off: End | Mask Data Block |  |
|  | Mask Unload Request | 1 | Event | - On: Report - Off: End | Mask Data Block |  |
|  | Mask Unload Complete | 1 | Event | - On: Report - Off: End | Mask Data Block |  |

        - Master (EAS) – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Mask Load Request Reply Block | Return Code | 1 | Data | INT | - : OK - : Select Mask not Available - : Others |

        - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Event Reply | Mask Load Request Reply | 1 | Reply | - On: Reply - Off: End | Mask Load Request Reply Block |  |
|  | Mask Load Complete Reply | 1 | Reply | - On: Reply - Off: End |  |  |
|  | Mask Unload Request Reply | 1 | Reply | - On: Reply - Off: End |  |  |

|  | Mask Unload Complete Reply | 1 | Reply | - On: Reply - Off: End |  |  |
| --- | --- | --- | --- | --- | --- | --- |

        - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Mask Check Request Reply Block | Return Code | 1 | Data | INT | - : OK - : Mask Not Available - : PPID Not Available - : Already In Use (MASK) - : OFFSETID is invalid - : Other |
| Mask Command Reply Block | Return Code | 1 | Data | INT | - : OK - : NG - : Others |

        - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | EAS Action |
| --- | --- | --- | --- | --- | --- | --- |
| EAS Command Reply | Mask Check Request Reply | 1 | Event | - On: Reply - Off: End | Mask Check Request Reply Block |  |
|  | Mask Unload Command Reply | 1 |  |  | Mask Command Reply Block |  |
|  | Mask Cancel Command Reply | 1 |  |  |  |  |

        - Master (EAS) – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Mask Command Block | Mask ID | 15 | Data | ASC |  |
| Mask Check Request Block | PPID | 1 | Data | INT |  |
|  | Mask ID | 15 | Data | ASC |  |

        - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| EAS Command | Mask Check Request | 1 | Command | - On: Reply - Off: End | Mask Check Request Block |  |
|  | Mask Unload Command |  |  |  | Mask Command Block |  |
|  | Mask Cancel Command |  |  |  |  |  |

    - Mask Data Management

        - Job Data M(Only Use Mask Cleaner)

This Data item is brief version of Job Data. When equipment reports Job Event to Host for Job transferring between Equipment ,Units by Mask Cleaner that is Mask Cleaner Line. Use Mask Equipment should report Job Data M

※ This feature is used only devices needed.

| Item Name | Format | Word Points | Bit Points | Sub Items | Writable |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | EAS | LD | EQP |  |
| Cassette Sequence No | INT | 1 |  |  | W | W | W | 1~65536 It is mainly to distinguish which kind of job the job is and from which cassette the job is fetched out. (When port Status In Use, Case Sequence No +1 |
| Job Sequence No | INT | 1 |  |  | W | W | W | 1 Fix |
| Mask Name | ASC | 15 |  |  | W | W | W |  |
| Mask Location | ASC | 5 |  |  | W | W |  |  |
| Mask Status | INT | 1 |  | 0：Disable 1：Clean 2：Dirty |  | W |  |  |
| Cleaning Time | ASC | 6 |  |  |  | W | W |  |
| EXPO Count | ASC | 3 |  |  |  | W | W |  |
| Count on Equipment | INT | 1 |  |  |  | W | W |  |
| Cleaning Recipe | INT | 1 |  |  |  | W | W |  |
| Use Mask Equipment | ASC | 2 |  |  |  | W | W |  |
| Priority | INT | 1 |  |  |  | W | W |  |
| Mask Judge | ASC | 1 |  |  |  |  | W |  |
| reserved |  | 58 |  |  |  |  |  |  |

        - Mask Data Event

The Mask Data has to be reported to EAS at special time, such as received job from upstream equipment, sent out job to downstream equipment, fetched out job from a Case or unit, stored job into a Case or unit, and removed Mask from equipment.

Etch event in Details can refer operation scenario of equipment by type.

Figure 11-3 Job Data Event Report Point

Figure 11-4 Job Event Timing Chart

        - Mask Job Event Report Timing Condition

          - Sent Out Job Event

            - On the rise of Link Signal Job Transfer signal

            - Job should be completely transferred to downstream equipment.

          - Received Job Event

            - After Receive Complete Signal turns off.

            - Job should be completely received by downstream equipment.

Between upstream and downstream equipment, report sequence of Sent Out Job Event from upstream equipment and Received Job Event from downstream equipment should follow physical Job transfer sequence. Exact report timing will be adjusted during and after test, if required.

        - Mask Received Job Event

When equipment receives a job from upstream equipment, this event should be reported. The below items are included when equipment reports the Received Job event.

          - Received Job: The Job Data of the job that is received from the upstream equipment.

          - Upstream Path No: The Upstream Path Number of the upstream equipment

        - Mask Sent Out Job Event

When the equipment sends out a job to downstream equipment, this event should be reported. The below items should be included when equipment reports the Sent Out Job event.

          - Sent Out Job: The Job Data of the job that is sent to the downstream equipment.

          - Downstream Path No: The Downstream Path Number of the downstream equipment

        - Mask Stored Job Event

When the equipment stored a job to a unit or cassette, this event should be reported with the port number and unit number that is occurred by unit. If there are some slots, slot no should be reported.

          - Stored Job: The Job Data of the job that is stored

          - Unit or Port: it is used to distinguish the product stored in Unit from Port.

          - Unit No: The Unit No of the Unit or Sub Unit. If the (Sub)Unit has some slots, equipment should update the Slot No.

          - Sub Unit No : The level of the unit. 0 is Unit, 1~15 is Sub Unit No

          - Port No: The port no of the cassette that the job is stored. Equipment should update the Slot No.

          - Slot No: Slot No. If unit has no slot, this item should be cleared.

Stored Job Event should be reported in separate Link Area for each Unit/Port. Basic Link Area usage rule:

Indexer - first priority is Port , second priority is Unit Example:

| Stored Job Event Report Area | Indexer of two Ports and one Units | Process Equipment with three Units |
| --- | --- | --- |
| Stored Job Report #1 | Port 1 | Unit 5 |
| Stored Job Report #2 | Port 3 | Unit 2 |
| Stored Job Report #3 | Unit 1 | Unit 4 |
| ... |  |  |
| Stored Job Report #15 |  |  |

        - Mask Fetched Out Job Event

When the equipment fetched out a job from a unit or cassette, this event should be reported with the port number and unit number that is occurred by unit. If there are some slots, slot no should be reported.

The Job Data of fetched job is reported while a job is being fetched out from a cassette or unit. In this case, the port number and unit number must be reported

          - Fetched Out Job: The Job Data of the job that is fetched out

          - Unit or Port: it is used to distinguish the product stored in Unit from Port.

          - Unit No: The Unit No of the Unit or Sub Unit. If the (Sub)Unit has some slots, equipment should update the Slot No.

          - Sub Unit No : The level of the unit. 0 is Unit, 1~ is Sub Unit No

          - Port No: The port no of the cassette that the job is fetched. Equipment should update the Slot No.

          - Slot No: Slot No. If unit has no slot, this item should be cleared.

Fetched Out Job Event should be reported in separate Link Area for each Unit/Port. Basic Link Area usage rule:

Indexer - first priority is Port , second priority is Unit Example :

| Fetched Out Job Event Report Area | Indexer of two Ports and one Units | Process Equipment with three Units |
| --- | --- | --- |
| Fetched Out Job Report #1 | Port 1 | Unit 5 |
| Fetched Out Job Report #2 | Port 3 | Unit 2 |
| Fetched Out Job Report #3 | Unit 1 | Unit 4 |
| ... |  |  |
| Fetched Out Job Report #15 |  |  |

        - Mask Removed Job Event

The Job Data of removed job is reported when a job has just been removed from equipment.

          - Removed Job: The Mask Data of the job that is removed by operator.

          - Unit or Port: it is used to distinguish the product stored in Unit from Port.

          - Unit No: The Unit No of the unit. If the unit has some slots, equipment should update the Slot No.

          - Sub Unit No : The level of the unit. 0 is Unit, 1~ is Sub Unit No

          - Port No: The port no of the cassette that the job is removed. Equipment should update the Slot No.

          - Slot No: Slot No. If unit has no slot, this item should be cleared.

          - Removed Flag : Removed type. and this is updated on touch panel

          - Remove Reason Code : Removed job was happened why and this is updated on touch panel

        - Mask Process Start Report

Principal unit of equipment should report when processing of a Job is started. The report timing should be comply with the 'Process Start Time' in Process Data Report.

ESWIN engineer will decide which Unit is required to report Process Start.

        - Mask Process End Report

Principal unit of equipment reports the Process End Event. The report timing should be comply with the 'Process End Time' in Process Data Report.

ESWIN engineer will decide whether or not an important unit.

    - Mask Data PLC MAP

      - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Job Count Block | Total Glass Count | 1 | Data | INT |  |
| Received Job Data Block | Job Data for Upstream Block 1~2 | 37 | Data | - | Refer to Job Data M(Only CF Mask Cleaner) |
| Received Job Data Sub Block 1~2 | Upstream Path No | 1 | Data | INT | 1~2 |
| Sent Out Job Data Block | Job Data For Downstream Block 1~2 | 37 | Data | - | Refer to Job Data M(Only CF Mask Cleaner) |
| Sent Out Job Data Sub Block 1~2 | Downstream Path No | 1 | Data | INT | 1~2 |
| Stored Job Data Block #1 | Stored Job Data | 37 | Data | - | Refer to Job Data M(Only CF Mask Cleaner) |
| Stored Job Data Sub Block #1 | Unit or Port | 1 | Data | INT | 1: Unit 2: Port |
|  | Unit No or Port No | 1 | Data | INT | 0: No Unit Event 1~: Unit No or Port No |

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
|  | Sub Unit No | 1 | Data | INT | 0 : Unit |
|  | Slot No | 1 | Data | INT | 0: No Slot 1~ : Slot No of Cassette/Unit |
| Stored Job Data Block #N (N: 2~15) | Stored Job Data | 37 | Data | - | Refer to Job Data M(Only CF Mask Cleaner) |
| Stored Job Data Sub Block #N(N: 2~15) | Unit or Port | 1 | Data | INT | 1: Unit 2: Port |
|  | Unit No or Port No | 1 | Data | INT | 0: No Unit Event 1~: Unit No or Port No |
|  | Sub Unit No | 1 | Data | INT | 0 : Unit |
|  | Slot No | 1 | Data | INT | 0: No Slot 1~ : Slot No of Cassette/Unit |
| Fetched Out Job Data Block #1 | Job Data M | 37 | Data | - | Refer to Job Data M(Only CF Mask Cleaner) |
| Fetched Out Job Data Sub Block#1 | Unit or Port | 1 | Data | INT | 1: Unit 2: Port |
|  | Unit No or Port No | 1 | Data | INT | 0: No Unit Event 1~: Unit No or Port No |
|  | Sub Unit No | 1 | Data | INT | 0 : Unit |
|  | Slot No | 1 | Data | INT | 0: No Slot 1~ : Slot No of Cassette/Unit |
| Fetched Out Job Data Block #N(N: 2~15) | Job Data M | 37 | Data | - | Refer to Job Data M(Only CF Mask Cleaner) |
| Fetched Out Job Data Sub Block #N (N: 2~15) | Unit or Port | 1 | Data | INT | 1: Unit 2: Port |
|  | Unit No or Port No | 1 | Data | INT | 0: No Unit Event 1~ : Unit No or Port No |
|  | Sub Unit No | 1 | Data | INT | 0 : Unit |
|  | Slot No | 1 | Data | INT | 0: No Slot 1~ : Slot No of Cassette/Unit |
| Removed Job Data Block | Removed Job Data | 37 | Data | - | Refer to Job Data M(Only CF Mask Cleaner) |
| Removed Job Data Sub Block | Unit or Port | 1 | Data | INT | 0: Others 1: Unit 2: Port |
|  | Unit No or Port No | 1 | Data | INT | 0: No Unit Event 1~: Unit No or Port No |
|  | Sub Unit No | 1 | Data | INT | 0 : Unit |
|  | Slot No | 1 | Data | INT | 0: No Slot 1~ : Slot No of Cassette/Unit |
|  | Removed Flag | 1 | Data | INT | 1: Scrap (broken) 2: Take out |

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 3~: Reserved. |
| Job Process Start Report Block | Cassette Sequence No | 1 | Data | INT | 1~65535 |
|  | Job Sequence No | 1 | Data | INT | 1 FIX |
|  | Unit No | 1 | Data | INT | 0: No Unit Event 1~: Unit No |
|  | Sub Unit No | 1 | Data | INT | 0 : Unit |
|  | Slot No | 1 | Data | INT | 0: No Slot 1~ : Slot No of Cassette/Unit |
|  | Process Start Time | 3 | Data | BCD | 00 ~ 07 Bit :A.D Year(00~99) 08 ~ 15 Bit :Month(01~12) 00 ~ 07 Bit :Day(01~31) 08 ~ 15 Bit :Hour(00~23) 00 ~ 07 Bit :Minute(00~59) 08 ~ 15 Bit :Second(00~59) |
| Job Process End Report Block | Cassette Sequence No | 1 | Data | INT | 1~65535 |
|  | Job Sequence No | 1 | Data | INT | 1 FIX |
|  | Unit No | 1 | Data | INT | 0: No Unit Event 1~: Unit No |
|  | Sub Unit No | 1 | Data | INT | 0 : Unit |
|  | Slot No | 1 | Data | INT | 0: No Slot 1~ : Slot No of Cassette/Unit |
|  | Process End Time | 3 | Data | BCD | 00 ~ 07 Bit :A.D Year(00~99) 08 ~ 15 Bit :Month(01~12) 00 ~ 07 Bit :Day(01~31) 08 ~ 15 Bit :Hour(00~23) 00 ~ 07 Bit :Minute(00~59) 08 ~ 15 Bit :Second(00~59) |

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) Action |
| --- | --- | --- | --- | --- | --- | --- |
| Job Event | Received Job Report Upstream #1 | 1 | Event | - On: Fire Event - Off: Event End | Received Job Data Block Received Job Data Sub Block Job Count Block | Reply for Receiving |
|  | Received Job Report Upstream #N | 1 | Event | - On: Fire Event - Off: Event End | Received Job Data Block Received Job Data Sub Block Job Count Block | Reply for Receiving |
|  | Sent Out Job Report Downstream #1 | 1 | Event | - On: Fire Event - Off: Event End | Sent Out Job Data Block Sent Out Job Data Sub Block Job Count Block | Reply for Receiving |
|  | Sent Out Job Report Downstream #N | 1 | Event | - On: Fire Event - Off: Event End | Sent Out Job Data Block Sent Out Job Data Sub Block Job Count Block | Reply for Receiving |
|  | Stored Job Report #1 | 1 | Event | - On: Fire Event - Off: Event End | Stored Job Block#1 Stored Job Data Sub Block#1 Job Count Block | Reply for Receiving |

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) Action |
| --- | --- | --- | --- | --- | --- | --- |
|  | Stored Job Report #n (2~15) | 1 | Event | - On: Fire Event - Off: Event End | Stored Job Block#N Stored Job Data Sub Block#N Job Count Block | Reply for Receiving |
|  | Fetched Out Job Report #1 | 1 | Event | - On: Fire Event - Off: Event End | Fetched Out Job Data Block#1 Fetched Out Job Data Sub Block#1 Job Count Block | Reply for Receiving |
|  | Fetched Out Job Report #N(2~15) | 1 | Event | - On: Fire Event - Off: Event End | Fetched Out Job Data Block#N Fetched Out Job Data Sub Block#N Job Count Block | Reply for Receiving |
|  | Removed Job Report | 1 | Event | - On: Fire Event - Off: Event End | Removed Job Data Block Removed Job Data Sub Block Job Count Block | Reply for Receiving |
|  | Job Process Start Report | 1 | Event | - On: Fire Event - Off: Event End | Job Process Start Block | Reply for Receiving |
|  | Job Process End Report | 1 | Event | - On: Fire Event - Off: Event End | Job Process End Block | Reply for Receiving |

      - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| Job Event Reply | Received Job Report Reply Upstream #1 | 1 | Event | - On: Fire Event - Off: Event End |  |  |
|  | Received Job Report Reply Upstream #N ( 1~ 2) | 1 | Event | - On: Fire Event - Off: Event End |  |  |
|  | Sent Out Job Report Reply Downstream #1 | 1 | Event | - On: Fire Event - Off: Event End |  |  |
|  | Sent Out Job Report Reply Downstream #N (1~2) | 1 | Event | - On: Fire Event - Off: Event End |  |  |
|  | Stored Job Report Reply #1 | 1 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Stored Job Report Reply #N (1~2) | 1 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Fetched Out Job Report Reply #1 | 1 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Fetched Out Job Report Reply #N (1~2) | 1 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Removed Job Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |
|  | Job Process Start Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
|  | Job Process End Report Reply | 1 | Reply | - On: Reply - Off: Reply End |  |  |

    - Mask Clean Priority Management.

      - Mask Clean Wait List Request Command

Principal unit of equipment Request the Mask Wait List. ESWIN engineer will decide whether or not an important unit.

      - Mask Clean Wait List Request Confirm

Principal unit of equipment Reply the Wait List Request Event. The Reply timing should be comply with the 'Mask Clean Priority' in Mask Clean Wait List Request Confirm..

ESWIN engineer will decide whether or not an important unit.

      - Change Mask Clean Priority Request Command

Principal unit of equipment Request the change Mask Priority. The Request timing should be comply with the 'Mask ID' and ‘Mask Priority’ in Change Mask Case Priority Request..

ESWIN engineer will decide whether or not an important unit.

      - Change Mask Clean Priority Request Confirm

Principal unit of equipment Reply the change Mask Priority Request Reply. The Reply timing should be comply with the 'Mask ID' and ‘Mask Priority’ in Change Mask Case Priority Request Reply..

ESWIN engineer will decide whether or not an important unit.

      - Mask Clean Request Command

Principal unit of equipment Request the special Mask Clean. The Request timing should be comply with the 'Mask ID' in Mask Clean Request..

ESWIN engineer will decide whether or not an important unit.

      - Mask Clean Request Confirm

Principal unit of equipment Reply the change Mask Clean Request Reply. The Reply timing should be comply with the 'Mask ID' and ‘Mask Priority’ in Mask Clean Request Reply..

ESWIN engineer will decide whether or not an important unit.

    - Mask Clean Priority PLC Map

      - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Mask Clean Wait List Request Confirm Block | Mask ID | 15 | Data | ASCII |  |
|  | Priority | 1 | Data | INT |  |
|  | Position | 1 | Data | INT |  |
|  | Mask ID | 15 | Data | ASCII |  |
|  | Priority | 1 | Data | INT |  |
|  | Position | 1 | Data | INT |  |

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
|  | Mask ID | 15 | Data | ASCII |  |
|  | Priority | 1 | Data | INT |  |
|  | Position | 1 | Data | INT |  |
|  | Mask ID | 15 | Data | ASCII |  |
|  | Priority | 1 | Data | INT |  |
|  | Position | 1 | Data | INT |  |
|  | Mask ID | 15 | Data | ASCII |  |
|  | Priority | 1 | Data | INT |  |
|  | Position | 1 | Data | INT |  |
|  | Mask ID | 15 | Data | ASCII |  |
|  | Priority | 1 | Data | INT |  |
|  | Position | 1 | Data | INT |  |
|  | Mask ID | 15 | Data | ASCII |  |
|  | Priority | 1 | Data | INT |  |
|  | Position | 1 | Data | INT |  |
|  | Mask ID | 15 | Data | ASCII |  |
|  | Priority | 1 | Data | INT |  |
|  | Position | 1 | Data | INT |  |
|  | Mask ID | 15 | Data | ASCII |  |
|  | Priority | 1 | Data | INT |  |
|  | Position | 1 | Data | INT |  |
|  | Mask ID | 15 | Data | ASCII |  |
|  | Priority | 1 | Data | INT |  |
|  | Position | 1 | Data | INT |  |
| Change Mask Clean Priority Request Confirm Block | Mask ID | 15 | Data | ASCII |  |
|  | Priority | 1 | Data | INT | 1~10 |
| Mask Clean Request Confirm Block | Mask ID | 15 | Data | ASCII |  |
|  | Priority | 1 | Data | INT | 1~10 |

      - Local PLC – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) Action |
| --- | --- | --- | --- | --- | --- | --- |
| EAS Command Reply | Mask Clean Wait List Request Confirm | 1 | Event | - On: Fire Event - Off: Event End | Mask Clean Wait List Request Confirm Block | Reply for Receiving |
|  | Change Mask Clean Priority Request Confirm | 1 | Event | - On: Fire Event - Off: Event End | Change Mask Clean Priority Request Confirm Block | Reply for Receiving |

| Block | Item | Points | Type | Sub Items | Related Word Items | Master (EAS) Action |
| --- | --- | --- | --- | --- | --- | --- |
|  | Mask Clean Request Confirm | 1 | Event | - On: Fire Event - Off: Event End | Mask Clean Request Confirm Block | Reply for Receiving |

      - Master(EAS) – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Mask Clean Wait List Request Command Block | Equipment Name | 8 | Data | INT |  |
| Change Mask Clean Priority Request Command Block | Mask ID | 15 | Data | ASCII |  |
|  | Priority | 1 | Data | INT | 1~10 |
| Mask Clean Request Block | Mask ID | 15 | Data | ASCII |  |

      - Master (EAS) – Link Relay (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| EAS Command | Mask Clean Wait List Request Command | 1 | Event | - On: Fire Event - Off: Event End |  |  |
|  | Change Mask Clean Priority Request | 1 | Event | - On: Fire Event - Off: Event End |  |  |
|  | Mask Clean Request Command | 1 | Event | - On: Fire Event - Off: Event End |  |  |

    - CF Oven Reverse Flow

      - AOI First Flow Command

Principal unit of equipment Command the special AOI First Flow. The Command timing should be comply with the 'Lot ID', 'Glass Count' in AOI First Flow Command..

ESWIN engineer will decide whether or not an important unit.

Use Only CF Photo Line

      - Local PLC – Link Register (W Area)

-

      - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| EAS Command Reply | AOI First Flow Command Reply | 1 | Event | - On: Fire Event - Off: Event End |  |  |

      - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| AOI First Flow Command Block | LOT ID | 10 | Data | ASCII |  |
|  | Count | 1 | Data | INT |  |

      - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| EAS Command | AOI First Flow Command | 1 | Event | - On: Fire Event - Off: Event End |  |  |

    - CF AOI Mask Info Request

      - Mask Info Reqeust

Principal unit of equipment Request the special Mask Info. The Request timing should be comply with the 'Cassette Sequence No', 'Job Sequence No', ‘Glass ID’ in Mask Info Request.

      - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Mask Info Request Block | Cassette Sequence No | 1 | Data | INT |  |
|  | Job Sequence No | 1 | Data | INT |  |
|  | Glass ID | 10 | Data | ASCII |  |

      - Local PLC – Link Register (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Event | Mask Info Request | 1 | Event | - On: Fire Event - Off: Event End |  |  |

      - Local PLC – Link Register (W Area)

| Block | Item | Points | Type | Format | Sub Items |
| --- | --- | --- | --- | --- | --- |
| Mask Info Request Reply Block | Mask ID | 10 | Data | INT |  |

      - Local PLC – Link Register (B Area)

| Block | Item | Points | Type | Sub Items | Related Word Items | Local PLC Action |
| --- | --- | --- | --- | --- | --- | --- |
| Equipment Event Reply | Mask Info Request Reply | 1 | Event | - On: Fire Event - Off: Event End |  |  |

- End of document -