1

Equipment HSMS Communication Specification

| DOCUMENT ID: ESWIN Equipment HSMS Communication Specification |  | NUMBER OF PAGES:119 |  |
| --- | --- | --- | --- |
| REVISION NO: v0.2 |  | STATUS: Release |  |
|  |  | PREPARED BY: SIGNATURE | DATE: 2018/11/28 |
| REVIEWED BY: SIGNATURE | DATE | REVIEWED BY: SIGNATURE | DATE |
| REVIEWED BY: SIGNATURE | DATE | REVIEWED BY: SIGNATURE | DATE |
| APPROVED BY: SIGNATURE | DATE | APPROVED BY: SIGNATURE | DATE |

1 1

- Caution -

- This specification is distributed in pursuant to the business procedure in the ESWIN.

- If the publishing company takes losses due to illegal distribution of this specification outside the company by a party, it shall take all the civil and criminal responsibilities in pursuant to acts on preventing unfair competition, protecting business secrets, as well as punishment in pursuant to regulations of the company and compensation for losses.

- ESWIN have publishing rights and copyrights of this specification and it shall not be copied, amended or distributed without prior consent.

Table of Contents

Table of Contents	3

Revision History	8

Scope & Basic Description	10

  - Scope	10

    - About Test and Support	10

    - About Documentation	10

    - Response Report	10

  - Equipment Operation Screen Specification	11

  - Handshake Signal Screen	11

  - Equipment Type	11

  - Equipment Structure	11

- Communication Overview	13

  - Line Architecture	13

  - Definitions	13

    - Control Mode Definitions	13

    - Equipment Processing State Definitions	13

    - Lot Status	14

    - Engineer Process Data	15

    - Material Status Definitions	15

    - Equipment Status Reason Code	15

- Common Function Requirements	20

  - Equipment Function	20

    - Communication Check Function	20

    - Status Request Function	20

    - Control State Change Report Function	20

    - Status Change Report Function	20

    - Recipe Modified Date/Time and Body Upload Function	20

    - Glass Process Data Report Function	20

    - Lot Process Data Report Function	20

    - Utility Report Function	21

    - Alarm Report Function	21

    - SECS Error Report Function	21

    - Glass Scrap/Removal Report	21

    - Glass History Report Function	21

    - CIM Message Management Function	21

    - Equipment Status Reason Code	22

- SECSII Stream Function	28

  - Stream Function List	28

  - Stream Function Description	31

    - SnF0 Abort Transaction	32

    - S1F1 Are You There Request	32

    - S1F2 Online Data	32

    - S1F5 Formatted Status Request	33

    - S1F6 Formatted Status Data	33

    - S1F13 Establish Communications Request	40

    - S1F14 Establish Communications Request Acknowledge	40

    - S1F15 Offline Request	41

    - S1F16 Offline Acknowledge	41

    - S1F17 Online Request	41

    - S1F18 Online Acknowledge	42

    - S2F15 New Equipment Constant Send	42

    - S2F16 New Equipment Constant Acknowledge	42

    - S2F17 Date and Time Request	43

    - S2F18 Date and Time Data	43

    - S2F23 Trace Initialize Send	43

    - S2F24 Trace Initialize Acknowledge	44

    - S2F25 Loop-back Diagnostic Request	44

    - S2F26 Loop-back Diagnostic Data	45

    - S2F29 Equipment Constant Name list Request	45

    - S2F30 Equipment Constant Namelist	45

    - S2F31 Date and Time Set Request	46

    - S2F32 Date and Time Set Acknowledge	46

    - S2F41 Host Command Send	46

    - S2F42 Host Command Acknowledge	47

    - S2F105 LotEnd Inform send	48

    - S2F106 LotEnd Inform Acknowledge	48

    - S2F107 Last Glass Inform send (Array Only)	49

    - S2F108 Last GlassInform Acknowledge (Array Only)	49

    - S2F111 Forced Glass Pass Command Send	50

    - S2F112 Forced Glass Pass Command Acknowledge	50

    - S2F113 Cassette Map Data Download	50

    - S2F114 Cassette Map Data Download Acknowledge	52

    - S2F115 Glass Data Download	53

    - S2F116 Glass Data Download Acknowledge	55

    - S2F117 Glass Erase/Recovery Information Send	55

    - S2F118 Glass Erase/Recovery Information Acknowledge	56

    - S2F119 Equipment Operation Mode Change Command Send	56

    - S2F120 Equipment Operation Mode Change Command Acknowledge	57

    - S2F121 APC Data Download Command Send	57

    - S2F122 APC Data Download Command Acknowledge	58

    - S2F123 Material Control Request	58

    - S2F124 Material Control Reply	59

    - S5F1 Alarm Report Send	60

    - S5F2 Alarm Report Acknowledge	62

    - S5F5 List Alarms Request	62

    - S5F6 List Alarm Data	62

    - S6F1 Trace Data Send	63

    - S6F1 Trace Data Send (FAC)	64

    - S6F2 Trace Data Acknowledge	65

    - S6F3 Discrete Variable Data Send	65

    - S6F4 Discrete Variable Data Acknowledge	70

    - S6F11 Event Report Send	70

    - S6F12 Event Report Acknowledge	90

    - S6F31 OXR Information Download Request	90

    - S6F32 OXR Information Download Data	91

    - S6F33 Cassette ID Request	92

    - S6F34 Cassette ID Request Reply	93

    - S7F15 Equipment Recipe Request	93

    - S7F16 Equipment Recipe Reply	94

    - S7F19 Current EPPD Request	94

    - S7F20 Current EPPD Data	94

    - S7F25 Formatted Process Program Request	95

    - S7F26 Formatted Process Program Data	95

    - S7F31 Formatted Process Program Send	97

    - S7F32 Formatted Process Program Acknowledge	98

    - S7F73 Recipe ID Check	98

    - S7F74 Recipe ID Check Acknowledge	99

    - S9F1 Unrecognized Device ID	99

    - S9F3 Unrecognized Stream Type	99

    - S9F5 Unrecognized Function Type	99

    - S9F7 Illegal Data	100

    - S9F9 Transaction Timer Timeout	100

    - S9F11 Data Too Long	100

    - S10F3 Terminal Display, Signal	100

    - S10F4 Terminal Display, Signal Acknowledge	101

    - S64F1 Measurement Result Send	101

    - S64F2 Measurement Result Send Acknowledge	102

    - S64F3 Feedback Result Send	102

    - S6F111 OP Login Report	102

    - S6F112 OP Login Report Acknowledge	103

- SECSII Stream Function Scenario	104

  - Online Scenario (Offline → Online by Equipment)	104

  - Offline Scenario (Online → Offline by Equipment)	104

  - Online Scenario (Offline → Online by Host)	105

  - Offline Scenario (Online → Offline by Host)	105

  - Equipment Information Inquire Scenario	106

  - Utility Data Inquire Scenario	106

  - Material Data Inquire Scenario	106

  - New Equipment Constant Send Scenario	106

  - EquipmentConstant Namelist Scenario	107

  - Date and Time Set By Host Scenario	107

  - List Alarms Request Scenario	107

  - Alarm Report Scenario	107

  - Equipment Event Notification Scenario	107

  - Glass Process Data Information Report Scenario	108

  - Glass History Data Report Scenario	108

  - Lot Process Data Information Report Scenario	108

  - Host Request Process Program Parameter Information Scenario	108

  - Host Request Current EPPD Request Scenario	108

  - Host Check Recipe ID Scenario	109

  - Online Normal Scenario	109

  - Online Abnormal Scenario (Last Glass is broken)	111

  - Auto Feedback Scenario	112

  - Glass Data Request Scenario	112

  - Equipment Recipe Request Scenario	112

  - Formatted Process Program Send Scenario	112

- Line Special Scenario	114

  - Cassette Map Download Scenario	114

  - Cassette Map Download Abnormal Scenario	114

    - ACKC<>0 from EQUIPMENT	114

    - Cancel from Loader	114

  - Forced Glass Pass Command Send Scenario	115

    - Forced Glass Pass Command Send By Host	115

  - Material Status Change Scenario	116

  - Material Status Change Abnormal Scenario	116

    - Material ID NG	116

    - Material Mount NG	117

    - Material In-USE NG	117

  - Equipment Operation Mode Change Command Send Scenario	118

    - Equipment Operation Mode Change By Host	118

    - Material Control Request By Host(Material Existence Check)	118

    - Material Control Request By Host(Material Pre-load)	119

    - VCR mismatch Report	119

    - Glass(Panel) Data Change Report	119

    - Glass(Panel) Process Result Report	119

  - Equipment Login Scenario	120

    - Equipment login on Touch Panel	120

Revision History

| No | Date | Version | Contents |
| --- | --- | --- | --- |
| 1 | 2018.10.10 | V0.1 | Reference version initialed |
| 2 | 2018.11.28 | V0.2 | Add 4.2.51 The process data item should be determined by ESWIN engineer. 4.2.64 Recipe Key parameter determined by ESWIN engineer Any recipe parameter changed, the recipe version number should be updated. And recipe version number format:(e.g.:20181127132826) |

※Some details are subject to change.

For the specification, ESWIN reserves the right of final decision in any disputes. During the testing term, whether it is specificati on discrepancy or software bug, ESWIN can change the specification without any charge by vendor.

Scope & Basic Description

This specification is used for ESWIN Display Technology Co., Ltd. B17 Automation Projects equipment automation common specification. It applies only to the equipment that uses the SEMI Equipment Communication Standards (SECS) as the communication protocol.

Basically, this specification only includes the common parts based on SEMI standard. For the items this specification does not include or the items that cannot be applied, caused by the special characteristics of hardware, a separated discussion is necessary and the final specification should use the discussing results as standards.

  - Scope

    - About Test and Support

All interfaces between CIMPC and equipment must be fully tested by the equipment maker and ESWIN S/W engineer. Tests must be performed first prior to the equipment being shipped to ESWIN. The equipment maker should receive written permission from ESWIN's S/W engineer before shipping.

When ESWIN implements an online program to interface with the host computer, the equipment maker must open technical data and offer technical support in compliance with ESWIN.

The equipment maker should not change the concept or method of the equipment without written permission from the ESWIN.

    - About Documentation

The equipment maker should inform the ESWIN of the information needed for the variation of equipment before implementation and send a document containing that information.

The equipment maker should supply final specification documents in English (2 copies of normal papers and 2 copies of clean papers). These documents should include an Alarm list.

The equipment maker must prepare the operating manual for CIM screen and function in English.

Special functions (Equipment operational logic and additional items) should be discussed and through deliberations between the ESWIN and the maker and should be recorded in a written document.

    - Response Report

The upstream equipment maker and the downstream equipment maker must discuss the detailed flow base on this specification; the upstream equipment maker then draws up the results in detail and submits it to the ESWIN.

If this specification is not sufficient for equipment, discussions with the ESWIN are necessary.

If some cases of errors are not specified in this specification, the equipment maker must notify the ESWIN. After receiving this specification, the equipment maker has to submit it within 2 weeks.

  - Equipment Operation Screen Specification

The equipment must support GUI in basic operational display.

The equipment maker must provide the function whereby the operator is able to input the cassette ID and Glass ID at a touch panel.

Cassette ID & Glass ID or Chip ID input fields are necessary in the screen

The equipment must support an interface status monitoring screen, and it must include CIMPC, upstream equipment and downstream equipment.

The operation screen of the equipment must be discussed between the equipment maker and the ESWIN engineer before programming, and is recommended to follow the ESWIN standard.

  - Handshake Signal Screen

Provide a screen on the touch panel for equipment operation to monitor the handshake signal for hand shaking between upstream equipment and downstream equipment. The status of the handshake signal should preferably be confirmed on the screen. The fundamental rule is real-time display.

  - Equipment Type

The equipment types are divided into 4 types according to function.

Loader (LD): In charge of normal input of glasses into equipment. It is generally composed of ports and robot.

Un-loader (ULD): In charge of normal output of glasses from equipment. It is generally composed of ports and robot. Process Equipment (PM): Denotes equipment to process glasses, etc., rather than handling the job transferring. It also includes inspection or measurement equipment. It is generally composed of process unit or inspection unit.

Buffer Equipment (BF): In charge of interleaving and buffering Job in the equipment. It is generally composed of buffer port and robots.

  - Equipment Structure

Equipment is equipment formed by the interconnection of one or more units of equipment. Equipment is composed of the following.

Sub Unit: The latest component

Unit: distinguished by its function, Equipment design or Management rule (TRF, BF, etc. ) Equipment: Normally distinguish by local No. It can include one or more unit(Indexer, Cleaner ... etc.)

Block: Block is concept for EAS. Block is logical group of dispatching for operation of glass flow, It can include one or more Equipment

Line : Entire system (Photo#1, SDEtch#1)

Equipment: Equipment is composed of more than one Indexer (Cassette loading/Unloading) and more than one process equipment. This Equipment is regarded as one equipment to Host CIM program, and its communication with Host is regarded as such.

Figure 1-1 Line Structure

- Communication Overview

  - Line Architecture

The overall architecture of Inline Equipment is as below：

Figure 1-1Line Architecture

  - Definitions

This section defines the important terminology that will be used in this document. The terms that are defined in this section should be applied to the entire document except that extra descriptions are included at the individual sections.

    - Control Mode Definitions

The control mode defines the level of cooperation between EQUIPMENT and CIMPC.

| State | Description |
| --- | --- |
| OFFLINE | Only the message for establishing communication is possible. Any message for automation purpose is severely restricted. Operator performs the necessary step at EQUIPMENT operator console. |
| ONLINE | The equipment side communicates with host and whenever equipment or lot information is changed, it should send related reports to host. OFFLINE Mode will change to ONLINE anytime. |

    - Equipment Processing State Definitions

The equipment processing state is used to record the equipment performance. The equipment should generate event reports as well

as status variables for each processing state transition.

| State | Description |
| --- | --- |
| IDLE | When Equipment have run glass ability, but there is no Job in processing. |
| RUN | When equipment starts processing or sends a “PROCESS START” event report to host, go into process state. |
| DOWN | When equipment stops abnormally or urgently. |
| PAUSE | When Pause button attached |
| PM | Status of preventive maintenance activities. EAS provides all button, also Equipment’s Status change button is allowed on touch panel. |
| JOB CHANGE | Special Customer Requirements. |
| E-TIME | Special Customer Requirements. |
| ETC | Special Customer Requirements. |

Equipment should provide “No Processing Time” timer setting function on touch panel or console for each unit and equipment. If no processing time over then equipment should report Idle. (No Processing Time =0 : Don’t check no processing time) (Default : tact time *2)

Equipment Status Change Diagram is below:

Figure 1-2 Equipment Status Diagram

    - Lot Status

| State | Description |
| --- | --- |
| Wait for Processing | This status will be continued until the actual process is started. |
| In Processing | From the start of first sheet (of the Lot) process to the end of sheet process. |
| Cancel End | Before in processing, the lot is deleted/ dropped. |
| Process End | After sending actual process data, the Lot has been processed without any error. |
| Abort End | Process has been aborted by operator or process stop because of equipment break-down. (There might have some sheet not being processed.) |

| Abnormal End | Process has been ended as the result of some error such as alarm. And the alarm might impact the lot(s) those are in processing. All the sheets would be claimed “Abnormal End” |
| --- | --- |

    - Engineer Process Data

| Item | Description |
| --- | --- |
| By Glass | Every processed glass. |

    - Material Status Definitions

The following table defines the possible states of resist/mask.

| Material Status | Description |
| --- | --- |
| Empty | Material is taken off in the Library. |
| Mount | Material is mounted by BCR Read |
| In-Use | Material is in-use. |
| Prepare | Material is prepared by Material Reservation |

    - Equipment Status Reason Code

It is to classify the status based on reason code of equipment status. Reason Code is used to indicate that why the status is changed.

Reason Code is represented by 5 digit numbers that are consists of 3 different properties of code to distinguish.

| Property | Type | Category | Sub Item |
| --- | --- | --- | --- |
| Reason Code | XXXXX(Combination of each value , INT: 10000~29999) |  |  |
| Data Format | 1Digit (1 ~ 2) | 2 ~ 3 Digit (00~ 99) | 4 ~ 5 Digit (00~ 99) |
|  | By who | Category | Sub Item |
| Value Description | 1 : EQP 2 : Operator | Refer to the table In below | Refer to the table In below |
|  | * For example, Reason Code of 12001 indicates, HW Trouble happens with Sensor by EQP. |  |  |

Common Reason Code is reserved that category : 00 ~ 79. Equipment Special Code is reserved that category : 80 ~ 95.

| Status | Priority | Category |  | Sub Item |  | Format | How To Report | Descrip tion |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | name | Code | name | Code |  |  |  |
|  |  |  | (2~3 digit) |  | (4~5 digit) |  |  |  |
| RUN | 1 | Product Run | 00 | Product Run | 01 | INT | EQ Auto | Array CF |
|  | 2 | EN Run | 01 | EN Run | 01 | INT |  |  |

|  | 3 | Dummy Run | 02 | Dummy Run | 01 | INT |  | Cell |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | Develop Run | 04 | Develop Run | 01 | INT |  |  |
|  | 6 | Test Run | 05 | Test Run | 01 | INT |  |  |
| IDLE | 1 | Waiting CST | 10 | Waiting CST | 01 | INT |  |  |
|  | 2 | No WIP | 11 | No WIP | 01 | INT |  |  |
| DOWN | 1 | HW Trouble | 20 | Sensor damage | 01 | INT | Auto/Manual |  |
|  |  |  |  | Motor damage | 02 | INT |  |  |
|  |  |  |  | Bearing damage | 03 | INT |  |  |
|  |  |  |  | Robot damage | 04 | INT |  |  |
|  |  |  |  | Other Power Module damage | 05 | INT |  |  |
|  |  |  |  | Machine Part damage | 06 | INT |  |  |
|  |  |  |  | others | 07 | INT |  |  |
|  | 2 | Information Error | 21 | Glass Broken | 01 | INT |  |  |
|  |  |  |  | Glass and data is not match | 02 | INT |  |  |
|  |  |  |  | Handshake Error | 03 | INT |  |  |
|  |  |  |  | Interlock Error | 04 | INT |  |  |
|  |  |  |  | Communition Error | 05 | INT |  |  |
|  |  |  |  | others | 06 | INT |  |  |
|  | 3 | Parameter Error | 22 | Exhaust Error | 01 | INT |  |  |
|  |  |  |  | Flow Error | 02 | INT |  |  |
|  |  |  |  | Gap Error | 03 | INT |  |  |
|  |  |  |  | Temperature Error | 04 | INT |  |  |
|  |  |  |  | Press Error | 05 | INT |  |  |
|  |  |  |  | others | 06 | INT |  |  |
|  | 4 | User Action | 23 | Stop Push Wrong | 01 | INT |  |  |
|  |  |  |  | Trouble Inspection | 02 | INT |  |  |
|  |  |  |  | Emergence | 03 | INT |  |  |
|  |  |  |  | Safety Door | 04 | INT |  |  |
|  |  |  |  | Safety Grating | 05 | INT |  |  |
|  |  |  |  | others | 06 | INT |  |  |
|  | 5 | Job Change | 24 | Same Job Change | 01 | INT |  |  |
|  |  |  |  | Different Job | 02 | INT |  |  |

|  |  |  |  | Change |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | others | 03 | INT |  |  |
|  | 6 | E-Time | 25 | Process Miss | 01 | INT |  |  |
|  |  |  |  | Equipment Miss | 02 | INT |  |  |
|  |  |  |  | Development Miss | 03 | INT |  |  |
|  |  |  |  | Production Miss | 04 | INT |  |  |
|  |  |  |  | others | 05 | INT |  |  |
|  | 7 | Quality | 26 | Mura | 01 | INT |  |  |
|  |  |  |  | Particle | 02 | INT |  |  |
|  |  |  |  | Material Faulty | 03 | INT |  |  |
|  |  |  |  | Glass Scrap | 04 | INT |  |  |
|  |  |  |  | Measurement Over Spec | 05 | INT |  |  |
|  |  |  |  | others | 06 | INT |  |  |
| PM | 1 | Plan-PM | 30 | Weekly Plan | 01 | INT | Manual | Array CF Cell |
|  |  |  |  | Monthly Plan | 02 | INT |  |  |
|  |  |  |  | Quarterly Plan | 03 | INT |  |  |
|  |  |  |  | Semi annual Plan | 04 | INT |  |  |
|  |  |  |  | Annual Pan | 05 | INT |  |  |
|  |  |  |  | Other Plan | 06 | INT |  |  |
|  |  |  |  | Daily maintenance | 07 | INT |  | Cell |
|  | 2 | Non-Plan | 31 | Non-Plan PM | 01 | INT |  | Array,C F,Cell |
|  | 3 | Routine maintenance | 32 | Electrode to be replaced | 01 | INT |  | Array |
|  |  |  |  | Change filter | 02 | INT |  | Array |
|  |  |  |  | Clean pressure | 03 | INT |  | Array |
|  |  |  |  | Cu Equipment clean | 04 | INT |  | Array |
|  |  |  |  | Mask Clean | 05 | INT |  | Array |
|  |  |  |  | Change pump | 06 | INT |  | Array |
|  |  |  |  | Change Slit value oring | 07 | INT |  | Array |
|  | 4 |  | 33 | Other PM | 01 | INT |  | Array |
| E-Time | 1 | Test | 50 | RUN "D" TYPE | 01 | INT |  | Array, |

|  |  |  |  | LOT |  |  |  | Cell |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | RUN "E" TYPE LOT | 02 | INT |  | Array, Cell |
|  |  |  |  | Developing /EN Overtime | 03 | INT |  | CF |
|  | 2 | Dummy Run | 51 | Dummy Run | 01 | INT |  | Array, Cell |
|  | 3 | Make Cell | 52 | Make Cell Dummy | 01 | INT |  | Array |
|  | 4 | Idle | 53 | DV/EN IDLE | 01 | INT |  | Cell |
|  |  |  |  | Dummy IDLE | 02 | INT |  | Cell |
|  |  |  |  | others | 03 | INT |  | Cell |
|  | 5 | Down | 54 | DV/EN Down | 01 | INT |  | Cell |
|  |  |  |  | Dummy Down | 02 | INT |  | Cell |
|  |  |  |  | others | 03 | INT |  | Cell |
|  | 6 | Others | 55 | others | 01 | INT |  | Array, Cell |
| Job Change | 1 | J/C Test | 60 | J/C Test | 01 | INT |  | Array |
|  | 2 | Job Change | 61 | Adhesive J/C normal | 01 | INT |  | CF |
|  |  |  |  | nonadhesive J/C normal | 02 | INT |  | CF |
|  |  |  |  | Same type Change | 03 | INT |  | Cell |
|  |  |  |  | Model Change | 04 | INT |  | Cell |
|  |  |  |  | Same thickness glass Change | 05 | INT |  | Cell |
|  |  |  |  | Different thickness glass Change | 06 | INT |  | Cell |
|  |  |  |  | Replaceable PU Change | 07 | INT |  | Cell |
|  |  |  |  | No replacement PU Change | 08 | INT |  | Cell |
|  |  |  |  | J/C Overtime | 09 | INT |  | CF, Cell |
|  | 3 | others | 62 | others | 01 | INT |  | Array, |

|  |  |  |  |  |  |  |  | Cell |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ETC | 1 | Materiel Change | 70 | Product Materiel Change | 01 | INT |  | Cell |
|  |  |  |  | Spare parts change | 02 | INT |  | Cell |
|  |  |  |  | others | 03 | INT |  | CF |
|  | 2 | Technology & EQ improved. | 71 | Parameter | 01 | INT |  | Array,C F,Cell |
|  | 3 | First process | 72 | Engineer to borrow equipment | 01 | INT |  | Cell |
|  | 4 | IDLE | 73 | Product Monitor IDLE | 01 | INT |  | Cell |
|  |  |  |  | Dummy Monitor IDLE | 02 | INT |  | Cell |
|  |  |  |  | Defect verification | 03 | INT |  | Cell |
|  |  |  |  | others | 04 | INT |  | Cell |
|  | 5 | Down | 74 | Materiel Change Down | 01 | INT |  | Cell |
|  |  |  |  | Spare parts Change Down | 02 | INT |  | Cell |
|  |  |  |  | Product Monitor Down | 03 | INT |  | Cell |
|  |  |  |  | Dummy Monitor Down | 04 | INT |  | Cell |
|  |  |  |  | others | 05 | INT |  | Cell |
| Equipment Special Reserved |  |  | 80 ~ 95 |  | 0 | INT |  |  |

- Common Function Requirements

  - Equipment Function

    - Communication Check Function

The objective of this function is to check the reliability of communication between Equipment and Host. When Equipment receives loop-back diagnostic request from Host, it should reply back to the host with the same string. Also, Host can perform the same action to Equipment when operating in any time (no matter Online and offline Mode).

    - Status Request Function

Host can send the status query request to Equipment at any time in Online status. The every status query includes the following：

Equipment status query

Control state query, Material list or status query.

    - Control State Change Report Function

When operator changes the control state at equipment or indicate command from CIMPC, equipment should report the state change event to CIMPC.

    - Status Change Report Function

When equipment status changes, it has to report status change event to Host under Online Mode. When material status changes, it has to report status change event to Host under Online Mode.

    - Recipe Modified Date/Time and Body Upload Function

If equipment modified the recipe (process program) parameter contents, equipment should report the modified recipe number to Host in Online Mode.

If Host queries this recipe at any time in Online Mode, equipment should be able to upload latest recipe body (key parameter) to Host including that recipe modified last date/time.

    - Glass Process Data Report Function

After each glass process finishes, equipment should send process data report to Host by glass in Online Mode.

    - Lot Process Data Report Function

After each lot process finishes, equipment should send process data report to Host in Online Mode.

    - Utility Report Function

There are some materials like gas, water, or chemical will be supplied to equipment. Equipment should collect these data for Host query or report by time setting.

    - Alarm Report Function

When Equipment have any alarm happened, equipment should send alarm report to Host (Only when Control Mode is Online Mode). The alarm report items including alarm level and alarm event, alarm code and alarm text that affected by the alarm. Also, when alarm already is cleared, equipment should send alarm clear event and report to Host

    - SECS Error Report Function

Equipment sent SECS error report to Host, when the block message cannot be handled or timeout error cause transaction failure. (Refer to S9Fx)

    - Glass Scrap/Removal Report

Equipment need report glass scrap or removal and its reason code to host when glass is scraped/removal from equipment.

    - Glass History Report Function

For multi-unit equipment, host need to trace glass-visited history. That meaning is after glass process completed, equipment should be send the glass through which one sub-equipment/chamber/unit/slot and its start process start and process end time.

    - CIM Message Management Function

CIM Message is used to notify specific information to operator of specific equipment. When receiving CIM Message, equipment has to display it to operator and save the history data locally.

Equipment should support that CIMPC can send one or more CIM Messages to a specific touch panel (or equipment’s controller screen) of a specific equipment. When equipment receives a CIM Message from CIMPC, the equipment should display the CIM Message in a pop-up window on touch panel (or equipment’s controller screen). The CIM Message pop-up window should provide “Confirmation” button so that, if operator confirms, the CIM Message is cleared.

CIM Message Display (T/P=1)

When CIM Messages are received then display like below.

| No | ID | Received Time | CIM Message | Confirm |
| --- | --- | --- | --- | --- |
| 1 | 1000 | 2010/12/01 05:00:00 | Display the newest (latest) CIM Message | Confirm |
| 2 | 2000 | 2010/12/01 04:00:00 |  | Confirm |
| 3 | 3000 | 2010/12/01 03:00:00 |  | Confirm |
| 4 | 4000 | 2010/12/01 02:00:00 |  | Confirm |
| 5 | 5000 | 2010/12/01 01:00:00 | Display the earliest CIM Message | Confirm |

If Equipment has received more than one CIM Messages before confirming by Engineer or Operator, then Equipment should do buffering of CIM Message on the screen up to five CIM Messages. And if the count of CIM Message buffered is over five, then

Equipment should change the buffering of CIM Message by newest (latest) 5 CIM Messages.

If CIM Message are all cleared or confirmed then equipment should close CIM Message popup display, buzzer and signal tower should change to original status.

After the CIM Message is confirmed, equipment should re-arrange popup display like below.

CIM Message Display (T/P=1)

CIM Message Sample

| No | ID | Received Time | CIM Message | Confirm |
| --- | --- | --- | --- | --- |
| 1 | 1000 | 2010/12/01 05:00:00 | Port#1 Canceled By Host | Confirm |
| 2 | 2000 | 2010/12/01 04:00:00 | Port#2 Canceled By Host | Confirm |
| 3 | 3000 | 2010/12/01 03:00:00 | Port#3 Canceled By Host | Confirm |
| 4 |  |  |  | Confirm |
| 5 |  |  |  | Confirm |

CIM Message Display (T/P=1)

After receive CIM Message Clear Command from BC CIM Message ID = 2000

| No | ID | Received Time | CIM Message | Confirm |
| --- | --- | --- | --- | --- |
| 1 | 1000 | 2010/12/01 05:00:00 | Port#1 Canceled By Host | Confirm |
| 2 | 3000 | 2010/12/01 03:00:00 | Port#3 Canceled By Host | Confirm |
| 3 |  |  |  | Confirm |
| 4 |  |  |  | Confirm |
| 5 |  |  |  | Confirm |

| No | Received Time | CIM Message ID | CIM Message |
| --- | --- | --- | --- |
| 1 | 2010/10/0713:20:57 | 0113 |  Display the newest (latest) CIM Message |
| 2 | ... | … | … |
| : | ... | … | … |
| 20 | 2010/10/0710:20:01 | 3333 |  Display the earliest CIM Message |

Equipment should display on the equipment Touch Panel (or equipment’s controller screen) the history of at least 20 CIM Messages most recently received from CIMPC.

    - Equipment Status Reason Code

It is to classify the status based on reason code of equipment status. Reason Code is used to indicate that why the status is changed.

Reason Code is represented by 5 digit numbers that are consists of 2 different properties of code to distinguish.

Equipment has to report the current status after report the previous status when operator executes the function on touch panel which is “Recovery”, because operator should decide the reason code of status by manual if equipment reports the reason code of wrong status

| Property | Type | Category | Sub Item |
| --- | --- | --- | --- |
| Reason Code | XXXXX(Combination of each value , INT: 10000~29999) |  |  |
| Data Format | 1Digit (1 ~ 2) | 2 ~ 3 Digit (00~ 99) | 4 ~ 5 Digit (00~ 99) |
|  | By who | Category | Sub Item |
| Value Description | 1 : EQP 2 : Operator | Refer to the table In below | Refer to the table In below |
|  | * For example, Reason Code of 12001 indicates, HW Trouble happens with Sensor by EQP. |  |  |

Common Reason Code is reserved that category : 00 ~ 50. Equipment Special Code is reserved that category : 51 ~ 99.

equipment vender must use to Equipment Special Code if the reason code for vender special want to add as reason code.

Priority is related with Category that means priority must be decided order by category.

For example, when priority of EN Run is 2,	if category of Dummy run is 01 and EN Run is 02, it is wrong because category code is different by order with priority.

| Status | Priority | Category |  | Sub Item |  | Format | How To Report | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | name | Code | name | Code |  |  |  |
|  |  |  | (2~3 digit) |  | (4~5 digit) |  |  |  |
| RUN | 1 | Product Run | 00 | Product Run | 01 | INT | EQ Auto | Array CF Cell |
|  | 2 | EN Run | 01 | EN Run | 01 | INT |  |  |
|  | 3 | Dummy Run | 02 | Dummy Run | 01 | INT |  |  |
| IDLE | 1 | Waiting CST | 10 | Waiting CST | 01 | INT |  |  |
|  | 2 | No WIP | 11 | No WIP | 01 | INT |  |  |
| DOWN | 1 | HW Trouble | 20 | Sensor damage | 01 | INT | Auto/Manual |  |
|  |  |  |  | Motor damage | 02 | INT |  |  |
|  |  |  |  | Bearing damage | 03 | INT |  |  |
|  |  |  |  | Robot damage | 04 | INT |  |  |
|  |  |  |  | Other Power Module damage | 05 | INT |  |  |
|  |  |  |  | Machine Part damage | 06 | INT |  |  |
|  |  |  |  | others | 07 | INT |  |  |
|  | 2 | Information Error | 21 | Glass Broken | 01 | INT |  |  |
|  |  |  |  | Glass and data | 02 | INT |  |  |

|  |  |  |  | is not match |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Handshake Error | 03 | INT |  |  |
|  |  |  |  | Interlock Error | 04 | INT |  |  |
|  |  |  |  | Communition Error | 05 | INT |  |  |
|  |  |  |  | others | 06 | INT |  |  |
|  | 3 | Parameter Error | 22 | Exhaust Error | 01 | INT |  |  |
|  |  |  |  | Flow Error | 02 | INT |  |  |
|  |  |  |  | Gap Error | 03 | INT |  |  |
|  |  |  |  | Temperature Error | 04 | INT |  |  |
|  |  |  |  | Press Error | 05 | INT |  |  |
|  |  |  |  | others | 06 | INT |  |  |
|  | 4 | User Action | 23 | Stop Push Wrong | 01 | INT |  |  |
|  |  |  |  | Trouble Inspection | 02 | INT |  |  |
|  |  |  |  | Emergence | 03 | INT |  |  |
|  |  |  |  | Safety Door | 04 | INT |  |  |
|  |  |  |  | Safety Grating | 05 | INT |  |  |
|  |  |  |  | others | 06 | INT |  |  |
|  | 5 | Job Change | 24 | Same Job Change | 01 | INT |  |  |
|  |  |  |  | Different Job Change | 02 | INT |  |  |
|  |  |  |  | others | 03 | INT |  |  |
|  | 6 | E-Time | 25 | Process Miss | 01 | INT |  |  |
|  |  |  |  | Equipment Miss | 02 | INT |  |  |
|  |  |  |  | Development Miss | 03 | INT |  |  |
|  |  |  |  | Production Miss | 04 | INT |  |  |
|  |  |  |  | others | 05 | INT |  |  |
|  | 7 | Quality | 26 | Mura | 01 | INT |  |  |
|  |  |  |  | Particle | 02 | INT |  |  |
|  |  |  |  | Material Faulty | 03 | INT |  |  |

|  |  |  |  | Glass Scrap | 04 | INT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Measurement Over Spec | 05 | INT |  |  |
|  |  |  |  | others | 06 | INT |  |  |
| PM | 1 | Plan-PM | 30 | Weekly Plan | 01 | INT | Manual | Array CF Cell |
|  |  |  |  | Monthly Plan | 02 | INT |  |  |
|  |  |  |  | Quarterly Plan | 03 | INT |  |  |
|  |  |  |  | Semi annual Plan | 04 | INT |  |  |
|  |  |  |  | Annual Pan | 05 | INT |  |  |
|  |  |  |  | Other Plan | 06 | INT |  |  |
|  |  |  |  | Daily maintenance | 07 | INT |  | Cell |
|  | 2 | Non-Plan | 31 | Non-Plan PM | 01 | INT |  | Array,CF,Cell |
|  | 3 | Routine maintenance | 32 | Electrode to be replaced | 01 | INT |  | Array |
|  |  |  |  | Change filter | 02 | INT |  | Array |
|  |  |  |  | Clean pressure | 03 | INT |  | Array |
|  |  |  |  | Cu Equipment clean | 04 | INT |  | Array |
|  |  |  |  | Mask Clean | 05 | INT |  | Array |
|  |  |  |  | Change pump | 06 | INT |  | Array |
|  |  |  |  | Change Slit value oring | 07 | INT |  | Array |
|  | 4 |  | 33 | Other PM | 01 | INT |  | Array |
| E-Time | 1 | Test | 50 | RUN "D" TYPE LOT | 01 | INT |  | Array, Cell |
|  |  |  |  | RUN "E" TYPE LOT | 02 | INT |  | Array, Cell |
|  |  |  |  | Developing /EN Overtime | 03 | INT |  | CF |
|  | 2 | Dummy Run | 51 | Dummy Run | 01 | INT |  | Array, Cell |
|  | 3 | Make Cell | 52 | Make Cell Dummy | 01 | INT |  | Array |
|  | 4 | Idle | 53 | DV/EN IDLE | 01 | INT |  | Cell |
|  |  |  |  | Dummy IDLE | 02 | INT |  | Cell |
|  |  |  |  | others | 03 | INT |  | Cell |

|  | 5 | Down | 54 | DV/EN Down | 01 | INT |  | Cell |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Dummy Down | 02 | INT |  | Cell |
|  |  |  |  | others | 03 | INT |  | Cell |
|  | 6 | Others | 55 | others | 01 | INT |  | Array, Cell |
| Job Change | 1 | J/C Test | 60 | J/C Test | 01 | INT |  | Array |
|  | 2 | Job Change | 61 | Adhesive J/C normal | 01 | INT |  | CF |
|  |  |  |  | nonadhesive J/C normal | 02 | INT |  | CF |
|  |  |  |  | Same type Change | 03 | INT |  | Cell |
|  |  |  |  | Model Change | 04 | INT |  | Cell |
|  |  |  |  | Same thickness glass Change | 05 | INT |  | Cell |
|  |  |  |  | Different thickness glass Change | 06 | INT |  | Cell |
|  |  |  |  | Replaceable PU Change | 07 | INT |  | Cell |
|  |  |  |  | No replacement PU Change | 08 | INT |  | Cell |
|  |  |  |  | J/C Overtime | 09 | INT |  | CF, Cell |
|  | 3 | others | 62 | others | 01 | INT |  | Array, Cell |
| ETC | 1 | Materiel Change | 70 | Product Materiel Change | 01 | INT |  | Cell |
|  |  |  |  | Spare parts change | 02 | INT |  | Cell |
|  |  |  |  | others | 03 | INT |  | CF |
|  | 2 | Technology & EQ improved. | 71 | Parameter | 01 | INT |  | Array,CF,Cell |
|  | 3 | First process | 72 | Engineer to borrow equipment | 01 | INT |  | Cell |
|  | 4 | IDLE | 73 | Product Monitor IDLE | 01 | INT |  | Cell |

|  |  |  |  | Dummy Monitor IDLE | 02 | INT |  | Cell |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Defect verification | 03 | INT |  | Cell |
|  |  |  |  | others | 04 | INT |  | Cell |
|  | 5 | Down | 74 | Materiel Change Down | 01 | INT |  | Cell |
|  |  |  |  | Spare parts Change Down | 02 | INT |  | Cell |
|  |  |  |  | Product Monitor Down | 03 | INT |  | Cell |
|  |  |  |  | Dummy Monitor Down | 04 | INT |  | Cell |
|  |  |  |  | others | 05 | INT |  | Cell |
| Equipment Special Reserved |  |  | 80 ~ 95 |  | 0 | INT |  |  |

- SECSII Stream Function

  - Stream Function List

| Stream | Function | SFCD , CEID ,DVID | Description | Direction |
| --- | --- | --- | --- | --- |
| Sn | F0 |  | Abort Transaction | H↔E |
| S1 | F1 |  | Are You There Request | H↔E |
|  | F2 |  | Online Data | H↔E |
|  | F5 |  | Formatted Status Request | H→E |
|  |  | 1 | Equipment Information Inquire |  |
|  |  | 2 | Utility Data Inquire |  |
|  |  | 3 | Material Data Inquire |  |
|  |  | 4 | Buffer Data Inquire |  |
|  |  | 5 | Glass Data Request |  |
|  | F6 |  | Formatted Status Data | H←E |
|  | F13 |  | Establish Communications Request | H↔E |
|  | F14 |  | Establish Communications Request Acknowledge | H↔E |
|  | F15 |  | Offline Request | H→E |
|  | F16 |  | Offline Acknowledge | H←E |
|  | F17 |  | Online Request | H→E |
|  | F18 |  | Online Acknowledge | H←E |
| S2 | F15 |  | New Equipment Constant Send | HE |
|  | F16 |  | New Equipment Constant Acknowledge | HE |
|  | F17 |  | Date and Time Request | H←E |
|  | F18 |  | Date and Time Data | H→E |
|  | F23 |  | Trace Initialize Send | HE |
|  | F24 |  | Trace Initialize Acknowledge | HE |
|  | F25 |  | Loop-back Diagnostic Request | H↔E |
|  | F26 |  | Loop-back Diagnostic Data | H↔E |
|  | F29 |  | Equipment Constant Namelist Request | HE |
|  | F30 |  | Equipment Constant Namelist | HE |
|  | F31 |  | Date and Time Set Request | HE |
|  | F32 |  | Date and Time Set Acknowledge | HE |
|  | F41 |  | Host Command Send | HE |

|  | F42 |  | Host Command Acknowledge | HE |
| --- | --- | --- | --- | --- |
|  | F105 |  | Lot end inform send | HE |
|  | F106 |  | Lot end inform send Acknowledge | HE |
|  | F107 |  | Last Glass inform send | HE |
|  | F108 |  | Last GlassInform Acknowledge | HE |
|  | F111 |  | Forced Clean Out Command Send | HE |
|  | F112 |  | Forced Clean Out Command Acknowledge | HE |
|  | F113 |  | Cassette Map Data Download | HE |
|  | F114 |  | Cassette Map Data Download Acknowledge | HE |
|  | F115 |  | Glass Data Download | HE |
|  | F116 |  | Glass Data Download Acknowledge | HE |
|  | F117 |  | Glass Erase/Recovery Information Send | HE |
|  | F118 |  | Glass Erase/Recovery Information Acknowledge | HE |
|  | F119 |  | Equipment Operation Mode Change Command | HE |
|  | F120 |  | Equipment Operation Mode Change Command Acknowledge | HE |
|  | F121 |  | APC Data Download Command Send | HE |
|  | F122 |  | APC Data Download Command Acknowledge | HE |
|  | F123 |  | Material Control Request | HE |
|  | F124 |  | Material Control Reply | HE |
| 5 | F1 |  | EQP Alarm Report Send | HE |
|  | F2 |  | EQP Alarm Report Acknowledge | HE |
|  | F5 |  | List Alarms Request | HE |
|  | F6 |  | List Alarms Data | HE |
| S6 | F1 |  | Trace Data Send | HE |
|  | F2 |  | Trace Data Acknowledge | HE |
|  | F3 |  | Discrete Variable Data Send | HE |
|  |  | 1 | Glass Process Data Information Report |  |
|  |  | 2 | Lot Process Data Informtion Report |  |
|  |  | 3 | Glass History Data Report |  |
|  |  | 7 | Tact Time Data Report |  |
|  | F4 |  | Discrete Variable Data Acknowledge | HE |
|  | F11 |  | Event Report Send | HE |
|  |  | 01 | Control Mode Change |  |
|  |  | 02 | EquipmentOperation Mode Change |  |
|  |  | 03 | Equipment Status Change |  |
|  |  | 04 | Material Status Change |  |
|  |  | 05 | Process Program Change |  |

|  |  | 06 | Process Program Modify/Create |  |
| --- | --- | --- | --- | --- |
|  |  | 07 | Glass Erase Event |  |
|  |  | 08 | Job in EQP Report |  |
|  |  | 09 | Job out EQP Report |  |
|  |  | 10 | Job process start |  |
|  |  | 11 | Job process end |  |
|  |  | 12 | Dummy Request |  |
|  |  | 14 | Glass data request |  |
|  |  | 15 | Glass position Report |  |
|  |  | 16 | Glass Recovered Request |  |
|  |  | 18 | Glass Data Edit Request |  |
|  |  | 19 | JobIn UNIT Report |  |
|  |  | 20 | Job Out UNIT Report |  |
|  |  | 21 | Job Hold Request |  |
|  |  | 22 | OXR InformationUpdate Report |  |
|  |  | 23 | Panel Data Request |  |
|  |  | 24 | VCR Mismatch Report |  |
|  |  | 25 | Glass(Panel) Data Change Report |  |
|  |  | 26 | Glass(Panel) Process Result Report |  |
|  |  | 27 | Loading Stop Request |  |
|  |  | 28 | Loading Stop Release |  |
|  |  | 50 | Unit Process start Report |  |
|  |  | 30 | Unit Status Change |  |
|  |  | 51 | Unit Process end Report |  |
|  |  | 110 | Material Cleaning Complete Report |  |
|  |  | 131 | Material Load Request |  |
|  |  | 133 | Material Load Complete |  |
|  |  | 134 | Material Unload Request |  |
|  |  | 135 | Material Unload Complete |  |
|  |  | 200 | Unit Process Program Change |  |
|  |  | 201 | Route Process Program Change |  |
|  | F12 |  | Event Report Acknowledge | HE |
|  | F31 |  | OXR Information Download Request | HE |
|  | F32 |  | OXR Information Download Data | HE |
| S7 | F15 |  | Current EPPD Request | HE |
|  | F16 |  | Current EPPD Data | HE |
|  | F19 |  | Formatted Process Program Request | HE |

|  | F20 |  | Formatted Process Program Data | HE |
| --- | --- | --- | --- | --- |
|  | F25 |  | Formatted Process Program Send | HE |
|  | F26 |  | Formatted Process Program Acknowledge | HE |
|  | F31 |  | Recipe ID Check Request | HE |
|  | F32 |  | Recipe ID Check Acknowledge | HE |
|  | F73 |  | Unrecognized Device ID | HE |
|  | F74 |  | Unrecognized Stream Type | HE |
| S9 | F1 |  | Unrecognized Function Type | HE |
|  | F3 |  | Illegal Data | HE |
|  | F5 |  | Transaction Timer Timeout | HE |
|  | F7 |  | Data Too Long | HE |
|  | F9 |  | Terminal Display, Single | HE |
|  | F11 |  | Terminal Display, Single Acknowledge | HE |
| S10 | F3 |  | Measurement Result Send | HE |
|  | F4 |  | Measurement Result Send Acknowledge | HE |
| S64 | F1 |  | Feedback Result Send | HE |
|  | F2 |  |  | HE |
|  | F3 |  |  | HE |

  - Stream Function Description

S=single, M=multi-block

Short mnemonic

Stream n

Valid direction H=host, E=equipment

Function m (odd)

reply option

S, M

Sn, Fm name of function (xxx)

prime

H→E, H←E,

H←→E

reply (required) [reply] (optional) “blank” (never)

The sections are described in a standard form which involves specification of the number, name, single or multiple blocks, direction of communication, nature of reply required, description, variable definition, and the detailed structure of the message in terms of lists and items. Double lines separate streams, and single lines separate transactions to aid readability. The abbreviations used in each transaction are as follows:

Description: A description of the action generated by the function. Item Definition: It’s definition of each item by the function.

if item format is ASCII, Item’s value is character. when the intended value does not fully meet with ASCII byte, the empty spaces

should be filled by space character to meet the setting Bytes.

Example) <SOFTREV>:[ASCII 6 Bytes] Software Version S1F2

L,2

“Model “ /<MDLN>: [ASCII 6 Bytes] Model Name. “V1.0 “ /<SOFTREV>:[ASCII 6 Bytes] Software Version

Structure: Detailed structure showing lists and defined items. Lists are denoted by a capital L followed by the length separated by a comma. The individual elements in the list are numbered on separate lines. Nested lists are indented to emphasize the structure. The detailed form of the items is given in the define section at the beginning of the transaction. The symbols “<” and “>” are used to enclose each item in the structure data and imply that there is an item header. A detailed description of each data item as well as a list of the allowable data formats can be found in the item description. Exception: special cases in the structure that have a different meaning.

    - SnF0 Abort Transaction

Sn, F0 Abort Transaction	S,H↔E.

Description: Used in lieu of an expected reply to abort a transaction. Stream n and Function 0 are defined in every stream and has the same meaning in every stream.

Item Definition: None. Structure: Header only. Exception: None.

    - S1F1 Are You There Request

S1, F1 Are You There Request	S,H↔E, reply.

Description: Establishes if the Equipment is on-line. A function 0 response to this message means the communication is inoperative. In the Equipment, a function 0 is equivalent to a timeout on the receive timer after issuing S1, F1 to the host.

Item Definition: None. Structure: Header only. Exception: None.

    - S1F2 Online Data

S1, F2 Online Data	S,H↔E.

Description: Data signifying that Equipment is alive. Item Definition:

<MDLN>:	[ASCII 6 Bytes] Model Name.

<SOFTREV>:	[ASCII 6 Bytes] Software Version.

Structure: L, 2

- <A[6] MDLN>

- <A[6] SOFTREV>

Exception: The host sends a zero-length list to the Equipment.

    - S1F5 Formatted Status Request

S1, F5 Formatted Status Request	S,H→E, reply.

Description: A request for the Equipment to report the status according to a pre-defined fixed format. The host may send this message to the Equipment at anytime.

Item Definition:

<SFCD>:	[ 1 Byte] Status Form Code.

1 = Equipment Information Inquire.

2 = Utility Data Inquire.

3 = Material Data Inquire

4 = Buffer Data Inquire

5 = Glass Data Request.

<EQUIPMENTID>:	[ASCII 22 Bytes] Equipment ID.

Structure: L, 2

- <B[1] SFCD>

- <A[22] EQUIPMENTID>

Exception: None.

    - S1F6 Formatted Status Data

S1, F6 Formatted Status Data	M,H←E.

Description: The Equipment reports the value of status variables according to the SFCD. Item Definition:

<SFCD>:	[Binary 1 Byte] Status Form Code.

1 = Equipment Information Inquire.

2 = Utility Data Inquire.

3 = Material Data Inquire

4 = Buffer Data Inquire

5 = Glass Data Request

<SUBCD>:	[UINT 4 Bytes] Sub Code.

1 = First Step Process Data.

2 = Second Step Process Data.

：	：

99 = Final Step Process Data.

<CTRLMODE>:	[ASCII 1 Byte] Control Mode.

1 = Offline Mode.

2 = Online Mode.

<EQUIPMENTID>:	[ASCII 22 Bytes] Equipment ID.

<GLASSID>:	[ASCII 20 Bytes] Glass ID.

<EQUIPMENTMODE>:	[ASCII 4 Bytes] Equipment Operation Mode.

NORN = Normal Mode. PASS = Pass Mode.

LT =LT Mode.(CVD Only) HT = HT Mode.(CVD Only)

HTS = HTS Mode.(CVD Only) PVA = PVA Mode.(CVD Only)

FPM = Full PM Mode.(Dry Etcher Only) PPM = Partial PM Mode. (Dry Etcher Only)

<EQUIPMENTST>:	[ASCII 1 Byte] Equipment Status.

I = Idle. R = Run.

D = DOWN.

P = Pause. M = PM

J = Job Change T = E-Time

E = ETC

<STATUS_REASONCODE>:	[Unsigned Integer 1 Byte] Status Reason Code.

<PPID>:	[ASCII 20 Bytes] Process Program ID

<PPVER>:	[ASCII 14 Bytes] Process Program Version.

PPVER means recipe last modified date time. Format：YYYYMMDDHHMMSS.

<RPTTIME>:	[ASCII 14 Bytes] Utility Report Time.

Format：YYYYMMDDHHMMSS.

<MATERIALID>:	[ASCII 30 Bytes] Material ID.

<MATERIALSL>:	[ASCII 2 Bytes] Material Slot.

<MATERIALST>:	[ASCII 1 Byte] Material Status.

“ ” = Material is taken off in the Library(Empty)/” ” is space

character1

M = Material is mounted by operator (Mount). I = Material is In-Use (In-Use).

P = Material is in prepare buffer (Prepare).

<RECOVERY>:	[ASCII 1 Byte]Recovery Mode

= 0: Normal Mode.

= 1: Recovery Mode.

<GLASS_COUNT>:	[ASCII 4Bytes]Glass Count in Equipment 0~9999

<AUTO>:	[ASCII 1 Byte]Auto Mode

= 0: Non Auto Mode.

= 1: Auto Mode.

<MANUAL>:	[ASCII 1 Byte]Manual Mode

= 0: Non Manual Mode.

= 1: Manual Mode.

<BUFFERSL>:	[ASCII 2 Bytes] Buffer Slot.

<BUFFERST>:	[ASCII 1 Byte] Buffer Status.

= 0: normal

= 1: NG

= 9: disable

=2~8:Reserved

<BUFFERSTIME>:	[ASCII 14 Bytes] Buffer Start Time.

Format：YYYYMMDDHHMMSS.

<BUFFERETIME>:	[ASCII 14 Bytes] Buffer Expired Time Format：YYYYMMDDHHMMSS.

<BUFFER_USEDCOUNT>:	[ASCII 4Bytes] Used Count in Buffer

0~9999

<BUFFER_JOBTYPE>:	[ASCII 1Bytes] Used Count in Buffer

= 1: General Dummy

= 2~9: Reserved

<BUFFER_DUMMYTYPE>:	[ASCII 1Bytes] Buffer Dummy Type

= 0: TFT

= 1: CF

= 2: Dummy

= 3: ALL

= 4: Reserved

<INLINE>:	[ASCII 1 Byte]Inline Mode

= 0: Both Upstream and Downstream Off

= 1: Both Upstream and Downstream On

= 2: Upstream Off/Downstream On

= 3: Upstream On/Downstream Off

<ALID>:	[UINT 4 Bytes] Alarm ID

<ALTX>:	[ASCII 80 bytes] Alarm TEXT

<UNITID>:	[ASCII 16 Bytes] Unit ID

<SUNITID>:	[ASCII 16 Bytes] Sub Unit ID

<CSTSEQ>:	[ASCII 5 Bytes] Cassette Sequence No.

<SLOT>:	[ASCII 2 Bytes] Slot number

<DCNAME>:	[ASCII 16 Bytes] Utility Item Name.

<DCTYPE>:	[ASCII 3 Bytes] Utility Item Type.

<DCVALUE>:	[ASCII 16 Bytes] Utility Item Value.

If there is no value of item, which should be presented by space

If there is no alarm, please use all space

Structure:

      - [SFCD = 1] Equipment Information Inquire.

L, 11

        - <B[1] SFCD>

        - <A[22] EQUIPMENTID>

        - <A[1] CTRLMODE>

        - <A[4] EQUIPMENTMODE>

        - <A[1] EQUIPMENTST>

        - <U2[1] STATUS_REASONCODE >

        - <A[1]RECOVERY>

        - <A[1]INLINE>

        - <A[20] PPID>

        - <A[4] GLASS_COUNT>

        - L,n (Unit count)	/* if not Unit, please set this List is L, 0 */

          - L, 12

            - <A[16] UNITID>

            - <A[4] EQUIPMENTMODE>

            - <A[1] UNITST>

            - <U2[1] STATUS_REASONCODE >

            - <A[20] PPID>

            - <A[4] GLASS_COUNT>

            - <A[1]RECOVERY>

......

.....

n. L, 12

            - <A[1]INLINE>

            - <A[1]AUTO>

            - <A[1]MANUAL>

            - <U4[1] ALID>	/* It’s always space, except EQUIPMENTST = DOWN. */

            - <A[80] ALTX>

- <A[16] UNITID>

- <A[4] EQUIPMENTMODE>

- <A[1] UNITST>

- <U2[1] STATUS_REASONCODE >

- <A[20] PPID>

- <A[4] GLASS_COUNT>

- <A[1]RECOVERY>

- <A[1]INLINE>

- <A[1]AUTO>

- <A[1]MANUAL>

- <U4[1] ALID>	/* It’s always space, except EQUIPMENTST = DOWN. */

- <A[80] ALTX>

Exception: A zero-length item means that no report can be made.

      - [SFCD = 2] Utility Data Inquire.

- Process data information report with first step: SUBCD=1.

- Process data information report with second step: SUBCD=2.

- Process data information report with n step: SUBCD=n.

- Process data information report with final step: SUBCD=99.

- If Equipment has only less then 600 items process data information of glass, it should be report SUBCD=99.

L, 5

- <B[1] SFCD>

- <U4[1] SUBCD>

- <A[4] EQUIPMENTMODE>

- <A[1] EQUIPMENTST>

- L, n	/* (Equipment Count or Unit count) */

  - L, 2

    - <A[16] UNITID>	/* If don’t have Unit, Fill with Equipment-id

    - L, m (1~1500)	/* m: Max data collection Item = 1500. */

      - L, 3

- <A[16] DCNAME>

- <A[3] DCTYPE>

- <A[16] DCVALUE>

：	：

m. L, 3

n. L, 2

  - <A[16] DCNAME>

  - <A[3] DCTYPE>

  - <A[16] DCVALUE>

- <A[16] UNITID>

- L, m (1~1500)	/* m: Max data collection Item = 1500. */

  - L, 3

    - <A[16] DCNAME>

    - <A[3] DCTYPE>

    - <A[16] DCVALUE>

：	：

m. L, 3

- <A[16] DCNAME>

- <A[3] DCTYPE>

- <A[16] DCVALUE>

Exception: A zero-length item means that no report can be made.

      - [SFCD = 3] Material Data Inquire.

L, 2

        - <B[1] SFCD>

        - L, m	/* m: Count of Material Slot */

          - L, 3

m. L, 3

- <A[2] MATERIALSL1>

- <A[30] MATERIALID1>

- <A[1] MATERIALST1>

..

- <A[2] MATERIALSLm>

- <A[30] MATERIALIDm>

- <A[1] MATERIALSTm>

Exception: MATERIALID and MATERIALST are space means there is no material in that slot.

      - [SFCD = 4] Buffer Data Inquire.

L, 2

        - <B[1] SFCD>

        - L, a /* a: Count of Buffer */

- L 2

- <A[16] UNITID>

- L, m	/* m: Count of Buffer Slot */

  - L, 8

…

    - L 2

      - <A[2] BUFFERSL1>

      - <A[1] BUFFERST1>

      - <A[14] BUFFERSTIME1 >

      - <A[14] BUFFERETIME1 >

      - <A[4] BUFFER_USEDCOUNT1>

      - <A[1] BUFFER_JOBTYPE1>

      - <A[14] BUFFER_DUMMYTYPE1 >

      - <A[20]GLASSID1>

- <A[16] UNITID>

- L, m	/* m: Count of Buffer Slot */

  - L, 8

    - <A[2] BUFFERSLm>

    - <A[1] BUFFERSTm>

    - <A[14] BUFFERSTIMEm>

    - <A[14] BUFFERETIMEm>

    - <A[4] BUFFER_USEDCOUNTm>

    - <A[1] BUFFER_JOBTYPEm>

    - <A[14] BUFFER_DUMMYTYPEm>

    - <A[20]GLASSIDm>

…

      - [SFCD = 5] Glass Data Request.

L, 3

        - <B[1] SFCD>

        - <A[22] EQUIPMENTID>

        - L, n /* (Unit Count) */

          - L, 3

            - <A[16] UNITID>

            - L,m /* (Glass Count of Unit) */ L3 /* Count 1 */

              - <A[5] CSTSEQ>

              - <A[2] SLOT>

              - <A[20] GLASSID>

…

L3 /* Count m */

- <A[5] CSTSEQ>

- <A[2] SLOT>

- <A[20] GLASSID>

            - L,k	/* (Sub unit Count) */

              - L, 4

                - <A[16] SUNITID>

                - <A[5] CSTSEQ>

                - <A[2] SLOT>

                - <A[20] GLASSID>

    - S1F13 Establish Communications Request

S1, F13 Establish Communications Request	S,H↔E, reply.

Description: The purpose of this message is to provide a formal means of initializing communications both on power-up and following a break in communications. An attempt to send an Establish Communications Request (S1,F13) should be repeated at programmable intervals until an Establish Communications Acknowledge(S1,F14) is received within the transaction timeout period with an acknowledgement code accepting the establishment.

Item Definition:

<MDLN>:	[ASCII 6 Bytes] Model Name.

<SOFTREV>:	[ASCII 6 Bytes] Software Version.

Structure: L, 2

- <A[6] MDLN>

- <A[6] SOFTREV>

Exception: The host sends a zero-length list to the Equipment.

    - S1F14 Establish Communications Request Acknowledge

S1, F14 Establish Communications Request Acknowledge	S,H↔E

Description: Accept or deny Establish Communications Request (S1,F13). MDLN and SOFTREV are on-line data and are valid only if COMMACK = 0.

Item Definition:

Structure: L, 2

<COMMACK>:	[Binary 1 Byte] Establish Communications Request Acknowledge Code 0= Accept

1= Deny or retry 2 ~ 63: Reserved

<MDLN>:	[ASCII 6 Bytes] Model Name.

<SOFTREV>:	[ASCII 6 Bytes] Software Version.

- <B[1] COMMACK>

- L, 2

  - <A[6] MDLN>

  - <A[6] SOFTREV>

Exception: The host sends a zero-length list for item 2 to the Equipment.

    - S1F15 Offline Request

S1, F15 Offline Request	S,H→E, reply.

Description: The Host requests that the Equipment transition to the OFF-Line state. Item Definition: None.

Structure: Header only. Exception: None.

    - S1F16 Offline Acknowledge

S1, F16 Offline Acknowledge	S,H←E.

Description: Acknowledge or error in response to S1, F15. Item Definition:

<OFLACK>:	[Binary 1 Byte] Acknowledge for Request Offline

0 = OK, Accepted.

1 = NG, Already Off-Line, Not Accepted. 2~63 = Error, Not Accepted.

Structure:

<B[1] OFLACK>

Exception: None.

    - S1F17 Online Request

S1, F17 Online Request	S,H→E, reply.

Description: The Host requests that the Equipment transition to the ON-Line state. Item Definition: None.

Structure: Header only. Exception: None.

    - S1F18 Online Acknowledge

S1, F18 Online Acknowledge	S,H←E.

Description: Acknowledge or error in response to S1, F17. Item Definition:

<ONLACK>:	[Binary 1 Byte] Acknowledge for Request Online

0 = OK, Accepted.

1 = NG, Not permit.

2 = NG, Already Online, Not Accepted. 3~63 = Error, Not Accepted.

Structure:

<B[1] ONLACK>

Exception: None.

    - S2F15 New Equipment Constant Send

S2, F15 New Equipment Constant Send	S,H→E, reply.

Description: Change one or more Equipment constants. Item Definition:

<ECID>:	[UINT 4 Bytes] Equipment Constant ID

<ECV>:	[ASCII 16 Bytes] Equipment Constant Value

Structure: L,n

1. L,2

n. L,2

- <U4[1] ECID1>

- <A[16] ECV1>

:

- <U4[1] ECIDn>

- <A[16] ECVn>

Note: The definition of ECID & ECV will be discussed with Equipment Engineer later.

    - S2F16 New Equipment Constant Acknowledge

S2F16 New Equipment Constant Acknowledge	S,H←E.

Description: Acknowledge or error in response to S2F15. If EAC contains a non-zero error code, the Equipment should not change any of the ECIDs specified in S2F15.

Item Definition:

<EAC>:	[Binary 1 Byte] New Equipment Constant Acknowledge 0 = Accepted

Structure:

<B[1] EAC>

1 = Denied, at least one constant does not exist

2 = Denied, busy

3 = Denied, at least one constant is outside the range to 63: Reserved

    - S2F17 Date and Time Request

S2, F17 Date and Time Request	S,H↔E, reply.

Description: Useful to check Equipment time base or for Equipment to synchronize with the host time base. Item Definition: None.

Structure: Header only. Exception: None.

    - S2F18 Date and Time Data

S2, F18 Date and Time Data	S,H↔E.

Description: Actual time data. Item Definition:

<DATETIME>:	[ASCII 14 Bytes] Date and Time Data.

Format：YYYYMMDDHHMMSS.

Structure:

<A[14] DATETIME>

Exception: None.

    - S2F23 Trace Initialize Send

S2,F23 Trace Initialize Send	S,H↔E, reply.

Description: Status variables exist at all times. This function provides a way to sample a subset of those status variables as a functi on of time. The trace data is returned on S6, F1 and is related to the original request by the TRID multiple trace requests may be m ade to that Equipment allowing it. If Equipment receives S2, F23 with the same TRID as a trace function that is currently in progre ss, the Equipment should terminate the old trace and then initiate the new trace. A trace function currently in progress may be term inated by S2, F23 with TRID of that trace and TOTSMP=0. But TOTSMP= -1 that is infinity

The Each Equipment shall document its trace performance limits. The host computer shall not send an S2, F23 which exceeds the Equipment’s performance limits, or the Equipment may operate incorrectly.

Item Definition:

<TRID>	[ASCII 20 Bytes] Trace Data ID

<DSPER>	[ASCII 8 Bytes] Data Sample Period format : hhmmsscc

<TOTSMP>	[ASCII 5 Bytes] Total Samples to be made

<REPGSZ>	[ASCII 3 Bytes] Reporting Group Size

Structure L, 5

<SVID>	[ASCII 5 Bytes] Status Variable ID

- <A[20] TRID>

- <A[8] DSPER>

- <A[5] TOTSMP>

- <A[3] REPGSZ>

- L, n

  - <A[5] SVID>

….

n .<A[5] SVID>

Exception: None.

    - S2F24 Trace Initialize Acknowledge

S2, F24 Trace Initialize Acknowledge	S,H↔E.

Description : Acknowledge or error Item Definition

<TIAACK>	[Binary 1 Bytes]

0 = Everything correct

1 = Too many SVIDs

2 = No more traces allowed

3 = Invalid period 4= SVID is invalid 5=Others

Structure

<B[1] TIAACK>

Exception: None.

    - S2F25 Loop-back Diagnostic Request

S2, F25 Loop-back diagnostic Request	S,H↔E, reply.

Description: A diagnostic message for checkout of protocol and communication circuits. The binary string sent is echoed back. Item Definition:

<ABS>:	[Binary 10 Bytes] Any Binary string.

Structure:

<B[10] ABS>

Exception: None.

    - S2F26 Loop-back Diagnostic Data

S2, F26 Loop-back Diagnostic Data	S,H↔E.

Description: The echoed binary string. Item Definition:

<ABS>:	[Binary 10 Bytes] Any Binary string.

Structure:

<B[10] ABS>

Exception: None.

    - S2F29 Equipment Constant Name list Request

S2,F29 Equipment Constant Name list Request	S,H→E, reply

Description: This function allows the host to retrieve basic information about what Equipment constants are available in the Equipment.

Item Definition:

<ECID>:	[UINT 4 Bytes] Equipment Constant ID

Structure: L, n

1.<U4[1] ECID1>

:

n.<U4[1] ECIDn>

Exception: A zero-length list means send information for all ECIDs.

    - S2F30 Equipment Constant Namelist

S2,F30 Equipment Constant Namelist	M,HE

Description: Data Response to S2F29. Item Definition:

<ECID>	[UINT 4 Bytes] EquipmentConstant ID

<ECNAME>	[ASCII 16 Bytes] Equipmentconstant name

<ECMIN>	[ASCII 16 Bytes] Equipmentconstant minimum value

<ECMAX>	[ASCII 16 Bytes] Equipmentconstant maximum value

<ECDEF>	[ASCII 16 Bytes] Equipmentconstant default value

<UNITS>	[ASCII 16 Bytes] Unit identifier

Structure:

L,n (n represents the number of Equipment constants)

1. L,6

- <U4[1] ECID1>

- <A[16] ECNAME1>

- <A[16] ECMIN1>

:

n. L,6

- <A[16] ECMAX1>

- <A[16] ECDEF1>

- <A[16] UNITS1>

- <U4[1] ECIDn>

- <A[16] ECNAMEn>

- <A[16] ECMINn>

- <A[16] ECMAXn>

- <A[16] ECDEFn>

- <A[16] UNITSn>

Exceptions: Space ASCII items for ECNAME, ECMIN, ECMAX, ECDEF, and UNITS indicates that the ECID does not exist.

    - S2F31 Date and Time Set Request

S2, F31 Date and Time Set Request	S,H→E, reply.

Description: Useful to synchronize the Equipment time with the host time base. Item Definition:

<DATETIME>:	[ASCII 14 Bytes] Date and Time Data.

Format：YYYYMMDDHHMMSS.

Structure:

<A[14] DATETIME>

Exception: None.

    - S2F32 Date and Time Set Acknowledge

S2, F32 Date and Time Set Acknowledge	S,H←E.

Description: Useful to synchronize the Equipment time with the host time base. Item Definition:

<TIACK>:	[Binary 1 Byte] Date and Time set Acknowledge Code

0 = OK, Accepted.

1 = NG, Not Accepted. 2~63 = Error, Not Accepted.

Structure:

<B[1] TIACK>

Exception: None.

    - S2F41 Host Command Send

S2,F41 Host Command Send	S,HE, reply

Description: The Host requests the Equipment perform the specified remote command with the

associated parameters. Item Definition:

<RCMD>:	[ASCII 10 Bytes] Remote command

START, CANCEL, ABORT, PAUSE, RESUME

<CPNAME>:	[ASCII 16 Bytes] Command parameter CSTSEQ: Cassette Sequence No

<CPVAL>:	[ASCII 10 Bytes] Command parameter value

Structure: L,2

- <A[10] RCMD>

- L,n * number of parameters

  - L,2

:

n. L,2

- <A[16] CPNAME1> * parameter 1 name

- <A[10] CPVAL1> * parameter 1 value

- <A[16] CPNAMEn> * parameter n name

- <A[10] CPVALn> * parameter n value

    - S2F42 Host Command Acknowledge

S2,F42 Host Command Acknowledge	S,HE.

Description: Acknowledge Host command or error. If command is not accepted due to one or more invalid parameters, then a list of invalid parameters will be returned containing the parameter name and reason for being invalid.

Item Definition:

<HCACK>	[Binary 1 Byte] Host command parameter acknowledge code

0 = Acknowledged. Command has been executed.

1 = Command does not exist.

2 = Command cannot be executed now.

3 = At least one parameter is invalid.

4 = Acknowledged. Command is executed and the completion is to be

advised by the event.

5 = Denied. Already in requested status.

6 = The specified object does not exit. 7-63: Reserved

<CPNAME>	[ASCII 16 Bytes] Command parameter name

<CPACK>	[Binary 1 Byte] Command parameter acknowledge code

1 = Parameter name (CPNAME) does not exist.

2 = Illegal value was specified for the CPVAL

Structure: L,2

- <B[1] HCACK>

- L,n * number of parameters

  - L,2

3 = Illegal format was specified for the CPVAL

>3 = Other errors 4-63: Reserved

:

n. L,2

- <A[16] CPNAME1> * parameter 1 name

- <B[1] CPACK1> * parameter 1 reason

- <A[16] CPNAMEn> * parameter n name

- <B[1] CPACKn> * parameter n reason

Exception: If there are no invalid parameters, then a list of zero length will be sent for item 2.

    - S2F105 LotEnd Inform send

S2F105 Lot End inform send	S,HE, reply

Description: This message is Host send the lot end inform to equipment. Item Definition:

<GLASSID>:	[ASCII 20 Bytes] Glass ID.

<PPID>:	[ASCII 20 Bytes] Process Program ID

<CSTSEQ>:	[ASCII 5 Bytes] Cassette Sequence No.

1~9999

<SLOT>:	[ASCII 2 Bytes] Slot number 1~60

Structure: L, 4

- <A[5]CSTSEQ>

- <A[2] SLOT>

- <A[20]GLASSID>

- <A[20]PPID>

    - S2F106 LotEnd Inform Acknowledge

S2F106 Lot End inform Acknowledge	S,H←E,.

Description: Acknowledge or error. Item Definition:

<ACKC5>:	[Binary 1 Byte] Acknowledge code

Structure L, 1

1.<B[1] ACKC5>

0 = Accepted

>0 = Error, not accepte

Exception: None.

    - S2F107 Last Glass Inform send (Array Only)

S2F107 Last Glass inform send	S,HE, reply

Description: This message is Host send the lot end inform to equipment. Item Definition:

<LOTID>:	[ASCII 20 Bytes] Lot ID.

<GLASSID>:	[ASCII 20 Bytes] Glass ID.

<PPID>:	[ASCII 20 Bytes] Process Program ID

< GLASS_CODE >:	[Unsigned Integer 2 Byte, Unsigned Integer 2 Byte] Cassette Sequence No,

Slot No

Structure: L, 4

- <A[20] LOTID>

- <A[20] GLASSID>

- <U2[2] GLASS_CODE>

- <A[20]PPID>

    - S2F108 Last GlassInform Acknowledge (Array Only)

S2F108 Last Glass inform Acknowledge	S,H←E,.

Description: Acknowledge or error. Item Definition:

<ACKC5>:	[Binary 1 Byte] Acknowledge code 0 = Accepted

>0 = Error, not accepted

Structure L, 1

1.<B[1] ACKC5>

Exception: None.

    - S2F111 Forced Glass Pass Command Send

S2F111 Forced Glass Pass Command Send	S,HE, reply Description: This message is Host send forced clean out command to equipment.

Item Definition:

<EQUIPMENTID>:	[ASCII 22 Bytes] Equipment ID

<CLEANOUT>:	[ASCII 1 Byte]Forced Glass PassCommand

= 0: Forced Glass Pass Mode Off.

= 1: Clean Out Mode On

= 2 : Recovery Mode On

Structure: L, 2

- <A[22] EQUIPMENTID>

- <A[1] CLEANOUT>

    - S2F112 Forced Glass Pass Command Acknowledge

S2F112 Forced Glass Pass Command Acknowledge	S,H←E. Description: Acknowledge or error.

Item Definition:

<ACKC>:	[Binary 1 Byte] Acknowledge code 0 = Accepted

>0 = Error, not accepte

Structure:

L, 1

1.<B[1] ACKC>

Exception: None.

    - S2F113 Cassette Map Data Download

S2F113Cassette Map Data Download	S,HE, reply Description: This message is used to send the mapping data of cassette to Equipment..

Item Definition:

<EQUIPMENTID>:	[ASCII 22 Bytes] Equipment ID

<CSTSEQ>:	[ASCII 5 Bytes] Cassette Sequence No.

1~9999

<CSTOPERMODE>:	[ASCII 1 Byte] Cassette Operation Mode.

= 0: Kind to Kind

= 1: CST to CST

<SUBSTRATETYPE>:	[ASCII 1 Byte] Cassette Operation Mode.

= 0: Glass

= 1: Chip

= 2: CST

<SLOT>:	[ASCII 2 Bytes] Slot number 1~60

<GLASSID>:	[ASCII 20 Bytes] Glass ID.

<ALLPPID>:	[ASCII 40 Bytes] All Process Program ID

<PRODUCT>:	[ASCII 16 Bytes] Product Type 1~65535

<JOBTYPE>:	[ASCII 1 Byte] Job Type

= 1: TFT Glass

= 2: CF Glass

= 3:Dummy Glass

= 4: UV Mask

<DUMMYTYPE>:	[ASCII 1 Byte] Dummy Type

= 1: General Dummy

= 2: Through Dummy

= 3:Thickness Dummy

= 4: ITO Dummy

= 5:Bare Dummy

<JOBJUDGE>:	[ASCII 1 Byte] Job Judge

= 0: Inspection Judge or No Judge

= 1: OK

= 2: NG - Insp. Result

= 3: RW – Required Rework

= 4: RJ – Reject (Scrap)

= 5: RP – Required Repair

= 6: RD – Re-Judge

<JOBGRADE>:	[ASCII 4 Bytes] Job Grade

Y, SK, RW, RD, RL, RC, RJ, etc

<OXR>:	[ASCII 52 Bytes] OXR information of chips in a glass.

1 byte per chip. Max 52 chips

0 = No Panel Exist.

1 = G : Good.

2 = A : No Dark Point.

3 = B : Panel with Pad Repair.

4 = R : Final Repair Judge for LCVD Repair.

5 = L : LCVD Repair OK.

6 = S : Scrap.

7 = D : Dark Point Repaired.

8 = N : NG

9 = O : OK. Array Initial Grade

A = X : Real Scrap. Panel don’t need to be tested.

Structure: L, 5

- <A[22] EQUIPMENTID>

- <A[5] CSTSEQ>

- < A[1] CSTOPERMODE>

- < A[1] SUBSTRATETYPE>

- L, n (n = Glass Count)

  - L, 9

Exception:

n. L, 9

- <A[2] SLOT>

- <A[20] GLASSID>

- <A[40] ALLPPID>

- <A[16] PRODUCT>

- <A[1] JOBTYPE>

- <A[1] DUMMYTYPE>

- <A[1] JOBJUDGE>

- <A[4] JOBGRADE>

- <A[52] OXR>

- <A[2] SLOT>

- <A[20] GLASSID>

- <A[40] ALLPPID>

- <A[16] PRODUCT>

- <A[1] JOBTYPE>

- <A[1] DUMMYTYPE>

- <A[1] JOBJUDGE>

- <A[4] JOBGRADE>

- <A[52] OXR>

    - S2F114 Cassette Map Data Download Acknowledge

S2F114Cassette Map Data Download Acknowledge	S,H←E, reply

Description: Acknowledge or error. Item Definition:

<ACKC>:	[Binary 1 Byte] Acknowledge code 0 = Accepted

>0 = Error, not accept

Structure:

L, 1

1.<B[1] ACKC>

Exception: None.

    - S2F115 Glass Data Download

S2F115Glass Data Download	S,HE, reply

Description: This message is used to send the data for the requested glass (S6F11,CEID=14) to Equipment. Item Definition:

<EQUIPMENTID>:	[ASCII 22 Bytes] Equipment ID

<CSTSEQ>:	[ASCII 5 Bytes] Cassette Sequence No.

1~9999

<CSTOPERMODE>:	[ASCII 1 Byte] Cassette Operation Mode.

= 0: Kind to Kind

= 1: CST to CST

<SUBSTRATETYPE>:	[ASCII 1 Byte] Cassette Operation Mode.

= 0: Glass

= 1: Chip

= 2: CST

<GLSRESULT>	[ASCII 1 Byte] Glass Exist a Lot.

= 0: OK

= 1: NG

<SLOT>:	[ASCII 2 Bytes] Slot number 1~60

<GLASSID>:	[ASCII 20 Bytes] Glass ID.

<ALLPPID>:	[ASCII 40 Bytes] All Process Program ID

<PRODUCT>:	[ASCII 16 Bytes] Product Type 1~65535

<JOBTYPE>:	[ASCII 1 Byte] Job Type

= 1: TFT Glass

= 2: CF Glass

= 3:Dummy Glass

= 4: UV Mask

Structure: L, 6

<DUMMYTYPE>:	[ASCII 1 Byte] Dummy Type

= 1: General Dummy

= 2: Through Dummy

= 3:Thickness Dummy

= 4: ITO Dummy

= 5:Bare Dummy

<JOBJUDGE>:	[ASCII 1 Byte] Job Judge

= 0: Inspection Judge or No Judge

= 1: OK

= 2: NG - Insp. Result

= 3: RW – Required Rework

= 4: RJ – Reject (Scrap)

= 5: RP – Required Repair

= 6: RD – Re-Judge

<JOBGRADE>:	[ASCII 4 Bytes] Job Grade

Y, SK, RW, RD, RL, RC, RJ, etc

<OXR>:	[ASCII 52 Bytes] OXR information of chips in a glass.

1 byte per chip. Max 52 chips

0 = No Panel Exist.

1 = G : Good.

2 = A : No Dark Point.

3 = B : Panel with Pad Repair.

4 = R : Final Repair Judge for LCVD Repair.

5 = L : LCVD Repair OK.

6 = S : Scrap.

7 = D : Dark Point Repaired.

8 = N : NG

9 = O : OK. Array Initial Grade

A = X : Real Scrap. Panel don’t need to be tested.

- <A[22] EQUIPMENTID>

- <A[5] CSTSEQ>

- <A[1] CSTOPERMODE>

- <A[1] SUBSTRATETYPE>

- <A[1] GLSRESULT

- L, 1

  - L, 9

    - <A[2] SLOT>

    - <A[20] GLASSID>

    - <A[40] ALLPPID>

    - <A[16] PRODUCT>

    - <A[1] JOBTYPE>

    - <A[1] DUMMYTYPE>

    - <A[1] JOBJUDGE>

    - <A[4] JOBGRADE>

    - <A[52] OXR>

Exception:

    - S2F116 Glass Data Download Acknowledge

S2F116Glass Data Download Acknowledge	S,H←E. Description: Acknowledge or error.

Item Definition:

<ACKC>:	[Binary 1 Byte] Acknowledge code 0 = Accepted

>0 = Error, not accept

Structure:

L, 1

1.<B[1] ACKC>

Exception: None.

    - S2F117 Glass Erase/Recovery Information Send

S2F117Glass Erase/Recovery Information Send	S,HE, reply

Description: This message is used to send the information for erased or recovered glass to Equipment itself and other equipments. Item Definition:

<CSTSEQ>:	[ASCII 5 Bytes] Cassette Sequence No.

1~9999

<SLOT>:	[ASCII 2 Bytes] Slot number 1~60

<GLASSID>:	[ASCII 20 Bytes] Glass ID.

<ER>:	[ASCII 1 Byte] Erase or Recovery.

1 = Erase.

2 = Recovery.

<LASTGLSFLAG>:	[ASCII 1 Byte] Last Glass Flag 0 = Not Last Glass.

Structure: L, 5

- <A[5] CSTSEQ>

- < A[2] SLOT>

- <A[20] GLASSID>

- < A[1] ER>

- < A[1] LASTGLSFLAG>

1 = Last Glass.

Exception:

    - S2F118 Glass Erase/Recovery Information Acknowledge

S2F118Glass Erase/Recovery Information Acknowledge	S,H←E. Description: Acknowledge or error.

Item Definition:

<ACKC>:	[Binary 1 Byte] Acknowledge code 0 = Accepted

>0 = Error, not accept

Structure:

L, 1

1.<B[1] ACKC>

Exception: None.

    - S2F119 Equipment Operation Mode Change Command Send

S2F119EquipmentOperation Mode Change Command Send	S,HE, reply Description: This message is used to send Equipment operation mode change command to EQUIPMENT.

Item Definition:

<EQUIPMENTID>:	[ASCII 22 Bytes] Equipment ID

<EQUIPMENTMODE>:	[ASCII 4 Bytes] EquipmentOperation Mode.

NORN = Normal Mode.

PASS = Pass Mode.(=Skip Mode) LT =LT Mode.(CVD Only)

HT = HT Mode.(CVD Only) HTS = HTS Mode.(CVD Only) PVA = PVA Mode.(CVD Only)

FPM = Full PM Mode.(Dry Etcher Only) PPM = PartialPM Mode. (Dry Etcher Only) INLN = Inline Mode (Common)

LDST = Loading Stop Send Mode (Common)

Structure: L, 2

- <A[22] EQUIPMENTID>

- <A[4] EQUIPMENTMODE>

COLD = Cold Run Mode(Common) FOUT = Force Clean Out Mode(Common) RCVR = Recovery Mode(Photo Only)

CPND = Cassette Pending Mode(Photo Only) RPRO = Reprocess Mode

    - S2F120 Equipment Operation Mode Change Command Acknowledge

S2F120EquipmentOperation Mode Change Command Acknowledge	S,H←E. Description: Acknowledge or error.

Item Definition:

<ACKC>:	[Binary 1 Byte] Acknowledge code 0 = Accepted

>0 = Error, not accept

Structure:

L, 1

1.<B[1] ACKC>

Exception: None.

    - S2F121 APC Data Download Command Send

S2F121 APC Data DownloadCommand Send	S,HE, reply Description: This message is used to downloadAPC Data to Equipment.

Item Definition:

<EQUIPMENTID>:	[ASCII 22 Bytes] Equipment ID

<APCNAME>:	[ASCII 16 Bytes] AdvancedParameter Control Item Name.

<APCVALUE>:	[ASCII 16 Bytes] Advanced Parameter Control Item Value.

Structure: L, 2

- <A[22] EQUIPMENTID> /* If Equipment has Unit, It is filled with Unit ID

- L, m(1~600)	/* m: Max data collection item = 1-600. */

  - L, 2

    - <A[16] APCNAME>

    - <A[16] APCVALUE>

：	：

m. L, 2

- <A[16] APCNAME>

- <A[16] APCVALUE>

    - S2F122 APC Data Download Command Acknowledge

S2F122 APC Data Download Command Acknowledge	S,H←E. Description: Acknowledge or error.

Item Definition:

<ACKC>:	[Binary 1 Byte] Acknowledge code 0 = Accepted

>0 = Error, not accept

Structure:

L, 1

1.<B[1] ACKC>

Exception: None.

    - S2F123 Material Control Request

S2, F123MaterialControl Request	S,H→E, reply.

Description: CIM-PC checks the process possibility of exposure equipment& send process start or abort command to indexer. according the condition & defined rule. This message is for that mask is ready in the mask-library.

Item Definition:

<CCODE> :	[Binary 1 Byte] Control Code.

1 = Material Existence Check.

2 = Material Reservation

3 = Material Pre-load （When OIC Send Manual Transfer Comma

nd send to the target Machine）

4 = Material Change

134 = Material Unload

135 = Material Cancel

<MATERIALTYPE>:	[Binary 1 Byte] Material Type

1 = Mask.

2 = PR

3 = PI

<PPID>:	[ASCII 20 Bytes] Process Program ID

<EQUIPMENTID>:	[ASCII 22 Bytes] Equipment ID

Structure: L, 7

<MATERIALID>:	[ASCII 30 Bytes] Material ID

< OFFSETID1>:	[ASCII 10 Bytes] Offset ID 1

< OFFSETID2>:	[ASCII 10 Bytes] Offset ID 2

- <B[1] CCODE>

- <B[1]MATERIALTYPE>

- <A[20] PPID>

- <A[22] EQUIPMENTID>

- <A[30] MATERIALID >

- <A[10] OFFSETID1>

- <A[10] OFFSETID2>

    - S2F124 Material Control Reply

S2, F124Material Control Reply	S,H←E.

Description: Acknowledge or error in response to S2, F123. Item Definition:

<CCODE> :	[Binary 1 Byte] Control Code.

1 = Material Existence Check.

2 = Material Reservation

3 = Material Pre-load

4 = Material Change

134 = Material Unload

135 = Material Cancel

<MATERIALTYPE>:	[Binary 1 Byte] Material Type

1 = Mask.

2 = PR

3 = PI

<PPID>:	[ASCII 20 Bytes] Process Program ID

<EQUIPMENTID>:	[ASCII 22 Bytes] Equipment ID

<MATERIALID>:	[ASCII 30 Bytes] Material ID

< OFFSETID1>:	[ASCII 10 Bytes] Offset ID 1

< OFFSETID2>:	[ASCII 10 Bytes] Offset ID 2

<ONLACK>:	[Binary 1 Byte] Acknowledge for Request Online

0 = OK, Accepted or Mask exists for the requested PPID.

Structure: L, 8

- <B[1] CCODE>

- <B[1]MATERIALTYPE>

- <A[20] PPID>

- <A[22] EQUIPMENTID>

- <A[30] MATERIALID >

- <A[10] OFFSETID1>

- <A[10] OFFSETID2>

- <B[1] ONLACK>

1 = NG, Mask is not exists for the requested PPID.

2 = NG, PPID is not exists.

3 = NG, Already In Use.(MASK) ; it has to apply without CCode 1

4 = NG, OFFSETID is invalid 5~63 = Error, Not Accepted.

Exception: None.

    - S5F1 Alarm Report Send

S5, F1 Alarm Report Send	S,H←E, reply.

Description: This message reports a change in or presence of an alarm condition. One message will be issued when the alarm is set and one message will be issued when the alarm is cleared. Irrecoverable errors and attention flags may not have a corresponding clear message.

Item Definition:

<UNITID>:	[ASCII 16 Bytes] Unit ID

<ALCD>:	[Binary 1 Byte] Alarm code byte bit 8 = 1 means alarm set

bit 8 = 0 means alarm cleared bit 7-1 is alarm category

0: Danger for human

1: Equipment	Hardware Error 2: Equipment Software Error

3: Equipment Communication Error 4: Glass or Panel information error 5: Glass or Panel Trouble

6: Material or Utility Error

7: Parameter overflow caused process error

8: Parameter overflow caused equipment can’t work

9: Parameter configuration error 10: Cannot recover trouble

11: Equipment status warning 12: Robot Trouble

13: EAS Caused

14: other categories

15-63 Reserved

<ALID>:	[UINT 4 Bytes] Alarm ID

<ALTX>:	[ASCII 80 Bytes] Alarm text

<GLASSID>:	[ASCII 20 Bytes] Glass ID.

<ALDT>:	[ASCII 20 Bytes] Alarm Data.

<ALTP>:	[UINT 1 Byte] Alarm Type

0= Warning(Light Alarm)

1= Error(Heavy Alarm)

Structure: L,7

- <A[16] UNITID>/* If don’t have Unit, Fill with Equipment-ID

- <U1[1] ALTP>

- <B[1] ALCD>

- <U4[1] ALID>

- <A[80] ALTX>

- <A[20] ALDT>/* If don’t have Alarm Data, Fill with space(20)

- L,m /* (m: Glass Count of Unit)	If don’t have Glass, m=0

  - <A[20] GLASSID>

…

m. <A[20] GLASSID>

Exception: None. Note:

This stream contains messages regarding binary and analog Equipment exceptions. Exceptions are classified into two categories: errors and alarms.

- personal safety — Condition may be dangerous to people.

- Equipment safety — Condition may harm Equipment.

- parameter control warning — Parameter variation outside of preset limits — may harm product.

- parameter control error — Parameter variation outside of reasonable control limits — may indicate an Equipment malfunction.

- irrecoverable error — Intervention required before normal use of Equipment can resume.

- Equipment status warning — An unexpected condition has occurred, but operation can continue.

- attention flags — A signal from a process program indicating that a particular step has been reached.

- data integrity — A condition which may cause loss of data; usually related to Stream 6.

    - S5F2 Alarm Report Acknowledge

S5, F2 Alarm Report Acknowledge	S,H→E.

Description: Acknowledge or error. Item Definition:

<ACKC5>	[Binary 1 Byte] Acknowledge code 0 = Accepted

>0 = Error, not accepted 1-63 Reserved

Structure:

<B[1] ACKC5>

Exception: None.

    - S5F5 List Alarms Request

S5,F5 List Alarms Request	S,HE, reply

Description: This message requests the Equipment to send alarm list which happened but not reset yet information to the host. Item Definition: None

Structure: Header only. Exception: None

    - S5F6 List Alarm Data

S5,F6 List Alarm Data	M,HE

Description: This message contains the alarm data known to the Equipment. There may be multiple alarms in the list. Item Definition:

<UNITID>:	[ASCII 16 Bytes] Unit ID

<ALCD>:	[Binary 1 Byte] Alarm code byte bit 8 = 1 means alarm set

bit 8 = 0 means alarm cleared bit 7-1 is alarm category

0: Danger for human

1: Equipment	Hardware Error 2: Equipment Software Error

3: Equipment Communication Error 4: Glass or Panel information error 5: Glass or Panel Trouble

6: Material or Utility Error

Structure: L,m

7: Parameter overflow caused process error

8: Parameter overflow caused equipment can’t work 9: Parameter configuration error

10: Cannot recover trouble 11: Equipment status warning 12: Robot Trouble

13: EAS Caused

14: other categories

15-63 Reserved

<ALID>:	[UINT 4 Bytes] Alarm ID

<ALTX>:	[ASCII 80 Bytes] Alarm text

- L,4

:

m. L,4

  - <A[16] UNITID1>/* If don’t have Unit, Fill with Equipment-ID

  - <B[1] ALCD1>

  - <U4[1] ALID1>

  - <A[80] ALTX1>

- <A[16] UNITIDm>/* If don’t have Unit, Fill with Equipment-ID

- < B[1] ALCDm>

- < U4[1] ALIDm>

- <A[80] ALTXm>

Exception: m=0 means there is no any alarm is happening.

    - S6F1 Trace Data Send

S6, F1 Trace Data Send (W bit =0)	M,H←E

Description : This function sends samples to the host according to the trace setup done by S2,F23 Item Definition

<TRID>	[ASCII 20 Bytes] Trace Data ID

<SMPLN>	[ASCII 5 Bytes] Sample Number

<STIME>	[ASCII 16 Bytes] Sample Time

Format : YYYYMMDDhhmmsscc

<SV>	[ASCII 40 Bytes] Status Variable Value

Structure:

L, 4

- <A[20] TRID>

- <A[5] SMPLN>

- <A[16] STIME>

- L, m (m = Report Group Size) 4.L, n (n = SV Counts)

  - <A[40] SV>

:

n.<A[40] SVn>

Exception : None

Note: SV List order must match S2F23 SVID list order.

    - S6F1 Trace Data Send (FAC)

Equipment should report the facility data with FAC Data Report that is method of event driven.

And also, the event has to raise by cycle of event which is set by operator on touch panel of equipment. The cycle of event is five second.

Equipment must have to write on one block of memory that is FAC Data Block#0, if the data has to be wrote by every five second,	but if the data has to be wrote by every one second, equipment must have to write on etch block of memory that is from FAC Data Block#1 to FAC Data Block#5, that mean’s the etch block should have an area which is happened by every one second.

So, the five block from #1 to #5 are data to gathering during five second.

S6, F1 Trace Data Send (W bit =0)	M,H←E

Item Definition

<TRID>	[ASCII 20 Bytes] Trace Data ID (value : FAC)

<SMPLN>	[ASCII 5 Bytes] Sample Number

<STIME>	[ASCII 16 Bytes] Sample Time

Format : YYYYMMDDhhmmsscc

<SV>	[ASCII 40 Bytes] Status Variable Value

Structure: L, 4

- <A[20] TRID ‘FAC’>

- <A[5] SMPLN>

- <A[16] STIME>

- L, 6

L, n (n = SV Counts)	(SV: value of 5 second)

  - <A[40] SV>

:

n.<A[40] SVn>

L, n (n = SV Counts)	(SV: value of 1 second)

1.<A[40] SV>

:

n.<A[40] SVn>

L, n (n = SV Counts)	(SV: value of 1 second, SV same the second List)

1.<A[40] SV>

:

n.<A[40] SVn>

L, n (n = SV Counts)	(SV: value of 1 second, SV same the second List)

1.<A[40] SV>

:

n.<A[40] SVn>

L, n (n = SV Counts)	(SV: value of 1 second, SV same the second List)

1.<A[40] SV>

:

n.<A[40] SVn>

L, n (n = SV Counts)	(SV: value of 1 second, SV same the second List)

1.<A[40] SV>

:

n.<A[40] SVn>

    - S6F2 Trace Data Acknowledge

S6, F2 Trace Data Acknowledge	S,H→E.

Description : Acknowledge or error Item Definition

<ACKC7>	[ASCII 1 Bytes] Acknowledge Code

0 = Accepted

1 = Permission not granted

2 = Parameter not exist

3 = Parameter value error

4 = PPID is in use

5 = Other error

>5 = Reserved

Structure:

<B[1] ACKC7>

Exception : None

    - S6F3 Discrete Variable Data Send

S6, F3 Discrete Variable Data Send	M,H←E, reply.

Description: Any data report which is initiated by an event, such as the completion of a measurement, rather than passage of time

is called a discrete variable. Reports requiring only one block of data may report directly to the host with this message. Item Definition:

The process data item should be determined by ESWIN engineer.

<DVID>:	[Binary 1 Byte] Discrete Variable Data ID

1 = Glass Process Data Information Report.

2 = Lot Process Data Information Report.

3 = Glass History Data Report. 7 = Tact Time Data Report.

<SUBCD>:	[UINT 4 Bytes] Sub Code.

1 = First Step Process Data.

2 = Second Step Process Data.

：	：

99 = Final Step Process Data.

<EQUIPMENTID>:	[ASCII 22 Bytes] Equipment ID.

<OPERATIONID>:	[ASCII 20 Bytes] Operation ID.

<RPTTIME>:	[ASCII 14 Bytes] Utility Report Time.

Format：YYYYMMDDHHMMSS.

<GLASSID>:	[ASCII 20 Bytes] Glass ID.

<SLOT>:	[ASCII 2 Bytes] Slot number 1~60

<PPID>:	[ASCII 20 Bytes] Process Program ID

<CSTSEQ>:	[ASCII 5 Bytes] Cassette Sequence No.

1~9999

<UNIQUEID>:	[ASCII 6 Bytes] Job Unique ID

<LOTSTARTTIME>:	[ASCII 14 Bytes] Lot Start Time.

Format：YYYYMMDDHHMMSS.

<LOTENDTIME>:	[ASCII 14 Bytes] Lot End Time.

Format：YYYYMMDDHHMMSS.

<PDCNAME>:	[ASCII 32 Bytes] Process Data Collection Item Name.

<PDCTYPE>:	[ASCII 3 Bytes] Process Data Collection Item Type.

<PDCVALUE>:	[ASCII 32 Bytes] Process Data Collection Item Value.

If there is no value of item, which should be presented by space

<UNITID>:	[ASCII 16 Bytes] Unit ID

<GSTARTIME>:	[ASCII 14 Bytes] Glass Process Start Time.

Format：YYYYMMDDHHMMSS.

<GPRENTIME>:	[ASCII 14 Bytes] Glass Process End Time.

Format：YYYYMMDDHHMMSS

Structure:

<GPROCTIME>:	[ASCII 4 Bytes] Glass Process Time.

<UGSTARTIME>:	[ASCII 14 Bytes] Unit Process Start Time.

Format：YYYYMMDDHHMMSS

<UGPRENTIME>:	[ASCII 14 Bytes] Unit Process End Time.

Format：YYYYMMDDHHMMSS

<UGPROCTIME>:	[ASCII 4 Bytes] Unit Process Time.

<PPVER>:	[ASCII 14 Bytes] Process Program Version.

<CARRIERID>:	[ASCII 10 Bytes] Carrier ID

<PROCESS_RESULT>:	[ASCII 1 Byte] Process Result

0 = Not Processed (Not In EQUIPMENT)

1 = Normal Processed (In EQUIPMENT)

2 = Abnormal Processed (In EQUIPMENT)

3 = Process Skip (In EQUIPMENT)

<EQUIPMENTMODE>:	[ASCII 4 Bytes] Equipment Operation Mode.

NORN = Normal Mode. PASS = Pass Mode.

LT =LT Mode.(CVD Only) HT = HT Mode.(CVD Only)

HTS = HTS Mode.(CVD Only) PVA = PVA Mode.(CVD Only)

FPM = Full PM Mode.(Dry Etcher Only) PPM = PartialPM Mode. (Dry Etcher Only)

<EQUIPMENTST>:	[ASCII 1 Byte] Equipment Status.

I = Idle. R = Run.

D = DOWN.

P = Pause. M = PM

J = Job Change T = E-Time

E = ETC

- Process data information report with first step: SUBCD=01.

- Process data information report with second step: SUBCD=02.

- Process data information report with n step: SUBCD=n.

- Process data information report with final step: SUBCD=99.

- If Equipment has only less then 600 items process data information of glass, it should be report SUBCD=99.

      - [DVID = 1] Glass Process Data Information Report.

L, 7

        - <B[1] DVID>

        - <U4[1] SUBCD>

        - <A[22] EQUIPMENTID>

        - <A[4] EQUIPMENTMODE>

        - <A[1] EQUIPMENTST>

        - <A[20] OPERATIONID>

        - L, 9

          - <A[20] GLASSID>

          - <A[2] SLOT>

          - <A[20] PPID>

          - <A[5] CSTSEQ>

          - <A[6] UNIQUEID>	/ *for Coater/Developer and Exposure*/

          - <A[14] PPVER>

          - <A[10] CARRIERID>	/* CSTID or TrayID */

          - < A[1] PROCESS_RESULT>

          - L, m(1~1500)	/* m: Max data collection item = 1-1500. */

            - L, 3

              - <A[32] PDCNAME>

              - <A[3] PDCTYPE>

              - <A[32] PDCVALUE>

：	：

m. L, 3

Exception: None.

- <A[32] PDCNAME>

- <A[3] PDCTYPE>

- <A[32] PDCVALUE>

      - [DVID = 2] Lot Process Data Information Report.

L, 7

        - <B[1] DVID>

        - <U4[1] SUBCD>

        - <A[22] EQUIPMENTID>

        - <A[4] EQUIPMENTMODE >

        - <A[1] EQUIPMENTST >

        - <A[20] OPERATIONID>

        - L, 6

          - <A[5] CSTSEQ>

          - <A[6] UNIQUEID>

          - <A[14] LOTSTARTTIME>

          - <A[14] LOTENDTIME>

          - <A[14] PPVER>

          - L, m(1~600)	/* m: Max data collection item = 1-600. */

- L, 3

- <A[32] PDCNAME>

- <A[3] PDCTYPE>

- <A[32] PDCVALUE>

：	：

m. L, 3

- <A[32] PDCNAME>

- <A[3] PDCTYPE>

- <A[32] PDCVALUE>

      - [DVID = 3] Glass History Report

Exception: None.

      - [DVID = 7] Tact Time Data Report.

L, 7

        - <B[1] DVID>

        - <U4[1] SUBCD>

        - <A[22] EQUIPMENTID>

        - <A[4] EQUIPMENTMODE>

        - <A[1] EQUIPMENTST>

        - <A[20] OPERATIONID>

        - L, 9

          - <A[20] GLASSID>

          - <A[2] SLOT>

          - <A[20] PPID>

          - <A[5] CSTSEQ>

          - <A[6] UNIQUEID>	/ *for Coater/Developer and Exposure*/

          - <A[14] PPVER>

          - <A[10] CARRIERID>	/* CSTID or TrayID */

          - < A[1] PROCESS_RESULT>

          - L, m(1~1500)	/* m: Max data collection item = 1-1500. */

            - L, 3

              - <A[32] PDCNAME>

              - <A[3] PDCTYPE>

              - <A[32] PDCVALUE>

：	：

m. L, 3

Exception: None.

- <A[32] PDCNAME>

- <A[3] PDCTYPE>

- <A[32] PDCVALUE>

    - S6F4 Discrete Variable Data Acknowledge

S6, F4 Data Variable Acknowledge	S,H→E.

Description: Acknowledge or error. Item Definition:

<ACK6C>:	[Binary 1 Byte] Acknowledge Code.

0 = Accepted

>0 = Error, not accepted 1-63 Reserved

Structure:

<B[1] ACKC6>

Exception: None.

    - S6F11 Event Report Send

S6, F11 Event Report Send	M,H←E, reply.

Description: The purpose of this message is for the Equipment to send a defined, linked, and enabled group of reports to the host upon the occurrence of an event (CEID).

Item Definition:

<CEID>:	[ASCII 3 Bytes] Collect Event ID.

01 = Control Mode Change.

02 = Equipment Operation Mode Change. 03 = Equipment Status Change.

04 = Material Status Change. 05 = Process Program Change.

06 = Process Program Modify Or Create Or Delete . 07 = Glass Erase Event

08=Job in EQUIPMENT Report

09=Job out EQUIPMENT Report 10=Job Process start Report 11=Job Process end Report 12=Dummy Request

13=Panel Data Report 14=Glass data Request 15=Glass position Report 16=Glass Recovered Request 18=Glass data Edit Request 19=JOB IN Unit Report 20=JOB OUT Unit Report 21=Job Hold Request

22=OXR Information Update Report 23=Panel Data Request

24=VCR Mismatch Report 25=Glass(Panel) Data Change Report 26=Glass(Panel) Process Result Report 27=Loading Stop Request

28=Loading Stop Release 30=Unit Status Change 50=Unit Process start Report 51=Unit Process end Report

110=Material Cleaning Complete Report 131=Material Load Request 133=Material Load Complete 134=Material Unload Request 135=Material Unload Complete 200=Unit Process Program Change

201=Route Process Program Modify or Create or Delete

<CTRLMODE>:	[ASCII 1 Byte] Control Mode.

1 = Offline Mode.

2 = Online Mode.

<EQUIPMENTID>:	[ASCII 22 Bytes] Equipment ID.

<UNITID>:	[ASCII 16 Bytes] Unit ID

<EQUIPMENTMODE>:	[ASCII 4 Bytes] Equipment Operation Mode.

NORN = Normal Mode.

PASS = Pass Mode.(=Skip Mode)

LT =LT Mode.(CVD Only) HT = HT Mode.(CVD Only)

HTS = HTS Mode.(CVD Only) PVA = PVA Mode.(CVD Only)

FPM = Full PM Mode.(Dry Etcher Only) PPM = Partial PM Mode. (Dry Etcher Only) INLN = Inline Mode (Common)

LDST = Loading Stop Send Mode (Common) COLD = Cold Run Mode(Common)

FOUT = Force Clean Out Mode(Common) RCVR = Recovery Mode(Photo Only)

CPND = Cassette Pending Mode(Photo Only) RPRO = Reprocess Mode

<EQUIPMENTST>:	[ASCII 1 Byte] Equipment Status.

I = Idle. R = Run.

D = DOWN.

P = Pause. M = PM

J = Job Change T = E-Time

E = ETC

<STATUS_REASONCODE>:	[Unsigned Integer 1 Byte] Status Reason Code.

<UNITST>:	[ASCII 1 Byte] Unit Status.

I = Idle. R = Run.

D = DOWN.

P = Pause. M = PM

<ALID>:	[UINT 4 Bytes] Alarm ID

<ALTX>:	[ASCII 0-80 Bytes] Alarm text

<ALDT>:	[ASCII 0-80 Bytes] Alarm data

<MATERIALID>:	[ASCII 30 Bytes] Material ID.

<MATERIALSL>:	[ASCII 2 Bytes] Material Slot.

<MATERIALST>:	[ASCII 1 Byte] Material Status.

“ ” = Material is taken off in the Library(Empty)/” ” is space

character1

M = Material is mounted (Mount). I = Material is In-Use (In-Use).

P = Material is in prepare buffer (Prepare).

<PPID>:	[ASCII 20 Bytes] Process Program ID.

<TIME>:	[ASCII 14 Bytes] YYYYMMDDHHmmss

<REASON CODE>:	[ASCII 3 Bytes] 001~999

001: Causes of Equipment Remove 002: Causes of manual Remove 003: Causes of Equipment Broken 004: Causes of Manual Broken 005~999: Reserve

<OPERATORID>:	[ASCII 10Bytes] Operator ID

<CSTSEQ>:	[ASCII 5 Bytes] Cassette Sequence No.

1~9999

<SLOT>:	[ASCII 2 Bytes] Slot number 1~60

<OLDCSTSEQ>:	[ASCII 5 Bytes] Old Cassette Sequence 1~9999

<OLDSLOT>:	[ASCII 2 Bytes] Old Slot number 1~60

<GLSPOSITION>:	[ASCII 16 Bytes] Glass position

<GLASS_COUNT>:	[ASCII 4 Bytes] Glass count in Equipment 0~9999

<RECOVERY>:	[ASCII 1 Byte]Recovery Mode

= 0: Normal Mode.

= 1: Recovery Mode.

<AUTO>:	[ASCII 1 Byte]Auto Mode

= 0: Non Auto Mode.

= 1: Auto Mode.

<MANUAL>:	[ASCII 1 Byte]Manual Mode

= 0: Non Manual Mode.

= 1: Manual Mode.

<INLINE> :	[ASCII 1 Byte]Inline Mode

= 0: Both Upstream and Downstream Off

= 1: Both Upstream and Downstream On

= 2: Upstream Off/Downstream On

= 3: Upstream On/Downstream Off

<INPUT_SLOT>:	[ASCII 2 Bytes] Slot number 1~60

<OUTPUT_SLOT>:	[ASCII 2 Bytes] Slot number 1~60

<CLASS>:	[ASCII 1 Byte] Classify Process Program action.

1 = Modify.

2 = Create.

3 = Delete.

<OXR>:	[ASCII 52 Bytes] OXR information of chips in a glass.

1 byte per chip. Max 52 chips

0 = No Panel Exist.

1 = G : Good.

2 = A : No Dark Point.

3 = B : Panel with Pad Repair.

4 = R : Final Repair Judge for LCVD Repair.

5 = L : LCVD Repair OK.

6 = S : Scrap.

7 = D : Dark Point Repaired.

8 = N : NG

9 = O : OK. Array Initial Grade

A = X : Real Scrap. Panel don’t need to be tested.

<RCODE>:	[ASCII 4 Bytes] Reason Code for equipment status change. "LDST" = Paused by "Loading Stop"

"STRL" = Released by "Stop Release"

<DIR>:	[ASCII 4 Bytes] Direction only for the Inline Mode.

No use for other mode.

"UPST" : Upstream inline mode "DNST" : Downstream inline mode

<OLDMODEVALUE>:	[ASCII 1 Bytes] Old mode value before changing.

"0" = Mode Off for the specified 'EQUIPMENTMODE ' "1" = Mode On for the specified 'EQUIPMENTMODE '

<NEWMODEVALUE>:	[ASCII 1 Bytes] New mode value after changing.

"0" = Mode Off for the specified 'EQUIPMENTMODE ' "1" = Mode On for the specified 'EQUIPMENTMODE '

<PPVER>:	[ASCII 14 Bytes] Process Program Version.

PPVER means recipe last modified date time. Format：YYYYMMDDHHMMSS.

<PPNAME>:	[ASCII 32 Bytes] Process Program Parameter Item Name.

<PPVALUE>:	[ASCII 16 Bytes] Process Program Parameter Item Value.

If there is no value of item, which should be presented by space

<SubRPID>	[ASCII 32 Bytes] Sub Process Program ID

<RGLASSID>:	[ASCII 20 Bytes] Glass ID VCR Read

<JGLASSID>:	[ASCII 20 Bytes] Glass ID in Job Data

<LOADINGSTOP_REASONCODE>: [ASCII 1 Byte] Loading Stop Reason Code

1: Status (PM or Run)

2: Engineer (Stop or Start)

3: Downstream (Dead-Lock or Recovery) 4: Continuous Defect

5: Machine Dependent 6~ : Reserved

<GLASS_JUDGE>:	[ASCII 2 Byte] Glass(Job) Judge

<Mode>:	[ASCII 2 Byte] Mode

<DEFECT_CODE>:	[ASCII 14 Byte] Defect Code

Slot No

-	PLC Job Data or Up-stream EQP, Refer PLC Specification

<PRODUCTID>:	[ASCII 20 Bytes] Product ID. From

<LOTID>:	[ASCII 20 Bytes] Lot ID.

<GLASSID>:	[ASCII 20 Bytes] Glass ID.

<OPERATIONID>:	[ASCII 20 Bytes] Operation ID.

< GLASS_CODE >:	[Unsigned Integer 2 Byte, Unsigned Integer 2 Byte] Cassette Sequence No,

<Glass_Type>:	[ASCII 1 Bytes] Glass Type.

<Proc_Flag>:	[ASCII 32 Bytes] Process Flag, historical Process Data that job has been

processed or skipped in specific equipments.

<INSPECTION_JUDGE>:	[ASCII 6 Bytes] Inspection Judge.

<Skip_Flag>:	[ASCII 32 Bytes] Skip Flag.

< INSPECTION_FLAG>:	[ASCII 1 Bytes] Inspection Flag

0 = default

1 = Inspection is required

Structure:

      - [CEID = 01] Control Mode Change.

L, 2

        - <A[3] CEID>

        - L, 2

Exception: None.

- <A[22] EQUIPMENTID>

- <A[1] CTRLMODE>

      - [CEID = 02] Equipment Operation Mode Change.

L, 2

        - <A[3] CEID>

        - L, 5

          - <A[22] EQUIPMENTID>

          - <A[4] EQUIPMENTMODE>

          - <A[4] DIRNO>

          - <A[1] OLDMODEVALUE>

          - <A[1] NEWMODEVALUE>

Exception: None.

      - [CEID = 03] Equipment Status Change.

L, 12

        - < A[3] CEID >

        - <A[22] EQUIPMENTID>

        - <A[1] CTRLMODE>

        - <A[4] EQUIPMENTMODE>

        - <A[1] EQUIPMENTST>

        - <U2[1] STATUS_REASONCODE >

        - <A[1]RECOVERY>

        - <A[1]INLINE>

        - <A[20] PPID>

        - <A[4] GLASS_COUNT>

        - <A[4] RCODE>	*added for B17, "LDST"(Loading Stop), "STRL"(Stop Release)

        - L,n (Unit count) /* if not Unit, please set this List is L, 0 */

          - L, 12

            - <A[16] UNITID>

            - <A[4] EQUIPMENTMODE>

            - <A[1] UNITST>

            - <U2[1] STATUS_REASONCODE >

            - <A[20] PPID>

            - <A[4] GLASS_COUNT>

            - <A[1]RECOVERY>

Exception: None.

......

n. L, 12

            - <A[1]INLINE>

            - <A[1]AUTO>

            - <A[1]MANUAL>

            - <U4[1] ALID>	/* It’s always space, except EQUIPMENTST = DOWN. */

            - <A[80] ALTX>

- <A[16] UNITID>

- <A[4] EQUIPMENTMODE>

- <A[1] UNITST>

- <U2[1] STATUS_REASONCODE >

- <A[20] PPID>

- <A[4] GLASS_COUNT>

- <A[1]RECOVERY>

- <A[1]INLINE>

- <A[1]AUTO>

- <A[1]MANUAL>

- <U4[1] ALID>	/* It’s always space, except EQUIPMENTST = DOWN. */

- <A[80] ALTX>

      - [CEID = 04] Material Status Change.

L, 4

        - < A[3] CEID >

        - <A[22] EQUIPMENTID>

        - <A[1] MATERIALTRANSFER> /* " E"=Enable、"D"=Disable */

        - L, m	/* m: Count of Material Slot */ /* Nikon only; 1 to 5=Slot 1 to 5, 6=Inline Port */

          - L, 3

..

m. L, 3

- <A[2] MATERIALSL1> /* "01" */

- <A[30] MATERIALID1>

- <A[1] MATERIALST1> /* Nikon only ; " "、"M"、"I"、"P" */

- <A[2] MATERIALSLm> /* Nikon only ; if m=6, then "00" */

- <A[30] MATERIALIDm>

- <A[1] MATERIALSTm>

Exception: MATERIALID and MATERIALST are space means there is no material in that slot.

      - [CEID = 05] Process Program Change

L, 2

        - <A[3] CEID >

        - L, 6

          - <A[22] EQUIPMENTID>

          - <A[4] EQUIPMENTMODE>

          - <A[1] EQUIPMENTST>

          - <A[20] PPID>

          - <A[14] PPVER>

          - L, n	/* n: Each SubProcess Programcount */ 1.L, 2

            - <A[32] SubRPID>	/*IF Equipment have not sub RPID, this value same is PPID*/

            - L,m	/* m: Each Process Program Parameter item */

1. L, 2

n. L, 2

…

m. L, 2

- <A[32] PPNAME1>

- <A[16] PPVALUE1>

- <A[32] PPNAMEm>

- <A[16] PPVALUEm>

: :

  - <A[32] SubRPID>

  - L,m	/* m: Each Process Program Parameter item */

1. L, 2

…

m. L, 2

- <A[32] PPNAME1>

- <A[16] PPVALUE1>

- <A[32] PPNAMEm>

- <A[16] PPVALUEm>

Exception: If there are no process program ID, then a list of zero length will be sent for item 6(L, n) and PPVER is space. Exception: None.

      - [CEID = 06] Process Program Modify or Create or Delete.

L, 4

        - <A[3] CEID >

        - <A[22] EQUIPMENTID>

        - <A[1] CLASS>

        - L, n(changed count) 1.L, 2

.... n.L, 2

Exception: None.

- <A[20] PPID>

- <A[14] TIME>

- <A[20] PPID>

- <A[14] TIME>

      - [CEID = 07] Glass Erase Report

L, 2

        - <A[3] CEID>

        - <A[22] EQUIPMENTID>

        - L, 6

Exception: None.

- <A[20] GLASSID>

- <A[5] CSTSEQ>

- <A[2] SLOT>

- <A[16] GLSPOSITION

- <A[3] REASON CODE>

- <A[10] OPERATORID>

      - [CEID = 08] JOB IN EQUIPMENT Report

L, 3

        - <A[3] CEID>

        - <A[22] EQUIPMENTID>

        - L, 15

          - <A[20] PRODUCTID>

          - <A[20] LOTID>

          - <A[20] GLASSID>

          - <A[20] OPERATIONID>

          - <A[20] PPID>

          - <A[2] GLASS_JUDGE>	* refer the PLC Common Spec.

          - <U2[2] GLASS_CODE>	* refer the PLC Common Spec.

          - <A[1] GLASS_TYPE>	* refer the PLC Common Spec.

          - <A[32] PROC_FLAG>	* refer the PLC Common Spec.

          - <A[6] INSPECTION_JUDGE>	* refer to Inspection Judge Data of the PLC Common Spec.

          - <A[32] SKIP_FLAG>	* refer the PLC Common Spec.

          - <A[1] INSPECTION_FLAG>	* refer the PLC Common Spec.

          - <A[2] MODE>

          - <A[14] DEFECT_CODE>

          - <A[14] TIME>

Exception: None.

      - [CEID = 09] JOB OUT EQUIPMENT Report

L, 3

        - <A[3] CEID>

        - <A[22] EQUIPMENTID>

        - L, 15

          - <A[20] PRODUCTID>

          - <A[20] LOTID>

          - <A[20] GLASSID>

          - <A[20] OPERATIONID>

          - <A[20] PPID>

          - <A[2] GLASS_JUDGE>	* refer the PLC Common Spec.

          - <U2[2] GLASS_CODE>	* refer the PLC Common Spec.

          - <A[1] GLASS_TYPE>	* refer the PLC Common Spec.

          - <A[32] PROC_FLAG>	* refer the PLC Common Spec.

          - <A[6] INSPECTION_JUDGE>	* refer to Inspection Judge Data of the PLC Common Spec.

          - <A[32] SKIP_FLAG>	* refer the PLC Common Spec.

          - <A[1] INSPECTION_FLAG>	* refer the PLC Common Spec.

          - <A[2] MODE>

          - <A[14] DEFECT_CODE>

          - <A[14] TIME>

Exception: None.

      - [CEID = 10] JOB PROCESS START Report

L, 2

        - <A[3] CEID>

        - L, 4

          - <A[20] GLASSID>

          - <A[22] EQUIPMENTID>

          - <A[20] PPID>

          - <A[14] TIME>

Exception: None.

      - [CEID = 11] JOB PROCESS END Report

L, 2

        - <A[3] CEID>

        - L, 4

Exception: None.

- <A[20] GLASSID>

- <A[22] EQUIPMENTID>

- <A[20] PPID>

- <A[14] TIME>

      - [CEID = 12] Dummy Request

L, 2

        - <A[3] CEID>

        - L, 1

          - <A[22] EQUIPMENTID>

Exception: None.

      - [CEID = 13] Panel Data Report

L, 2

        - <A[3] CEID>

        - L, 4

Exception: None.

- <A[5]CSTSEQ>

- <A[2] SLOT>

- <A[128]PANELJUDGEDATA>

- <A[256]PANELGRADEDATA>

      - [CEID = 14] Glass data Request

L, 2

        - <A[3] CEID>

        - L, 3

Exception: None.

- <A[22] EQUIPMENTID>

- <A[5]CSTSEQ>

- <A[2] SLOT>

      - [CEID = 15] Glass position report

L, 2

        - <A[3] CEID>

        - L, 3

          - <A[22] EQUIPMENTID>

          - <A[4] GLASS_COUNT>

3.L,n(n = total glass count)

- L,3

Exception: None.

.......

n. L,3

- <A[5] CSTSEQ>

- <A[2] SLOT>

- <A[16] GLSPOSITION>

- <A[5] CSTSEQ>

- <A[2] SLOT>

- <A[16] GLSPOSITION>

      - [CEID = 16] Glass Recovered Report

L, 3

        - <A[3] CEID>

        - <A[22] EQUIPMENTID>

        - L, 6

Exception: None

- <A[20] GLASSID>

- <A[5] CSTSEQ>

- <A[2] SLOT>

- <A[16] GLSPOSITION

- <A[3] REASON CODE>

- <A[10] OPERATORID>

      - [CEID = 18] Glass Data Edit

L, 2

        - <A[3] CEID>

        - L, 5

          - <A[22] EQUIPMENTID>

          - <A[5] CSTSEQ>

Exception: None

          - <A[2] SLOT>

          - <A[5] OLDCSTSEQ>

          - <A[2] OLDSLOT>

      - [CEID = 19] JOB IN Unit Report

L, 2

        - <A[3] CEID>

        - L, 5

Exception: None.

- <A[20] GLASSID>

- <A[22] EQUIPMENTID>

- <A[16] UNITID>

- <A[2] INPUT_SLOT>	/* If Unit has no slot, this item is filled with space.

- <A[14] TIME>

      - [CEID = 20] JOB OUT Unit Report

L, 2

        - <A[3] CEID>

        - L, 5

Exception: None.

- <A[20] GLASSID>

- <A[22] EQUIPMENTID>

- <A[16] UNITID>

- <A[2] OUTPUT_SLOT>	/* If Unit has no slot, this item is filled with space.

- <A[14] TIME>

      - [CEID = 21] JOB Hold Request

L, 2

        - <A[3] CEID>

        - L, 4

Exception: None.

- <A[22] EQUIPMENTID>

- <A[16] UNITID>

- <A[2] SLOT>

- <A[20] GLASSID>

      - [CEID = 22] OXR Information Update Report

L, 2

        - <A[3] CEID>

        - L, 15

          - <A[22] EQUIPMENTID>

          - <A[5] CSTSEQ>

          - <A[2] SLOT>

          - <A[20] GLASSID>

          - <A[52] OXR>

          - <A[150] GATECUTREPAIRTYPE>

          - <A[150] GATECVDREPAIRTYPE>

          - <A[150] SDCUTREPAIRTYPE>

          - <A[150] SDCVDREPAIRTYPE>

          - <A[150] FINALCUTREPAIRTYPE>

          - <A[150] FINALCVDREPAIRTYPE>

          - <A[150] ARRAYTESTPRODUCTJUDGE>

          - <A[150] ASSYPRODUCTJUDGE>

          - <A[150] SUBPRODUCTGRADES>

          - <A[150] SUBPRODUCTJUDGES>

Exception: None.

      - [CEID = 23] Panel data Request

L, 2

        - <A[3] CEID>

        - L, 4

Exception: None.

- <A[22] EQUIPMENTID>

- <A[20] GLASSID>	/* Glass ID or Panel ID */

- <A[5]CSTSEQ>

- <A[2] SLOT>

      - [CEID = 24] VCR Mismatch Report

L, 2

        - <A[3] CEID>

        - L, 5

          - <A[22] EQUIPMENTID>

          - <A[20] RGLASSID>	/*Read Glass ID or Panel ID */

          - <A[20] JGLASSID>	/*Job Data Glass ID or Panel ID*/ 4.<A[5]CSTSEQ>

5.<A[2] SLOT>

      - [CEID = 25] Glass Data Change Report

L, 3

        - <A[3] CEID>

        - <A[22] EQUIPMENTID>

        - L, 15	/* New Glass Data */

          - <A[20] PRODUCTID>

          - <A[20] LOTID>

          - <A[20] GLASSID>

          - <A[20] OPERATIONID>

          - <A[20] PPID>

          - <A[2] GLASS_JUDGE>	* refer the PLC Common Spec.

          - <U2[2] GLASS_CODE>	* refer the PLC Common Spec.

          - <A[1] GLASS_TYPE>	* refer the PLC Common Spec.

          - <A[32] PROC_FLAG>	* refer the PLC Common Spec.

          - <A[6] INSPECTION_JUDGE>	* refer to Inspection Judge Data of the PLC Common Spec.

          - <A[32] SKIP_FLAG>	* refer the PLC Common Spec.

          - <A[1] INSPECTION_FLAG>	* refer the PLC Common Spec.

          - <A[2] MODE>

          - <A[14] DEFECT_CODE>

          - <A[14] TIME>

[CEID = 26] Glass Process Result Report L, 3

- <A[3] CEID>

- <A[22] EQUIPMENTID>

- L, 15

  - <A[20] PRODUCTID>

  - <A[20] LOTID>

  - <A[20] GLASSID>

  - <A[20] OPERATIONID>

  - <A[20] PPID>

  - <A[2] GLASS_JUDGE>	* refer the PLC Common Spec.

  - <U2[2] GLASS_CODE>	* refer the PLC Common Spec.

  - <A[1] GLASS_TYPE>	* refer the PLC Common Spec.

Exception: None.

  - <A[32] PROC_FLAG>	* refer the PLC Common Spec.

  - <A[6] INSPECTION_JUDGE>	* refer to Inspection Judge Data of the PLC Common Spec.

  - <A[32] SKIP_FLAG>	* refer the PLC Common Spec.

  - <A[1] INSPECTION_FLAG>	* refer the PLC Common Spec.

  - <A[2] MODE>

  - <A[14] DEFECT_CODE>

  - <A[14] TIME>

      - [CEID = 27] Loading Stop Request

      - [CEID = 28] Loading Stop Release

L, 3

- <A[3] CEID>

- <A[22] EQUIPMENTID>

- L, 2

Exception: None

- <A[16] UNITID>

- <A[1] LOADINGSTOP_REASONCODE>

      - [CEID = 30] Unit Status Change.

L, 14

        - <A[3] CEID>

        - <A[22] EQUIPMENTID>

        - <A[4] EQUIPMENTMODE>

        - <A[16] UNITID>

        - <A[1] UNITST>

        - <U2[1] STATUS_REASONCODE >

        - <A[20] PPID>

        - <A[4] GLASS_COUNT>

        - <A[1]RECOVERY>

        - <A[1]INLINE>

        - <A[1]AUTO>

        - <A[1]MANUAL>

        - <U4[1] ALID>	/* It’s always space, except EQUIPMENTST = DOWN. */

        - <A[80] ALTX>

Exception: None.

      - [CEID = 50] Unit PROCESS START Report

L, 2

        - <A[3] CEID>

        - L, 9

Exception: None.

- <A[20] GLASSID>

- <A[20] LOTID>

- <A[5] CSTSEQ>

- <A[2] SLOT>

- <A[20] PRODUCTID>

- <A[10] OPERATORID>

- <A[22] EQUIPMENTID>

- <A[16] UNITID>

- <A[32] SubRPID>

      - [CEID = 51] Unit PROCESS END Report

L, 2

        - <A[3] CEID>

        - L, 9

Exception: None.

- <A[20] GLASSID>

- <A[20] LOTID>

- <A[5] CSTSEQ>

- <A[2] SLOT>

- <A[20] PRODUCTID>

- <A[10] OPERATORID>

- <A[22] EQUIPMENTID>

- <A[16] UNITID>

- <A[32] SubRPID>

      - [CEID = 110] Material Cleaning Complete Report.

L, 6

        - <A[3] CEID>

        - <A[22] EQUIPMENTID>

        - <A[20] PPID>

        - <A[30] MATERIALID >

        - <A[14] START_TIME>

        - <A[14] END_TIME>

Exception: None.

      - [CEID = 131] Material Load Request.

      - [CEID = 133] Material Load Complete.

      - [CEID = 134] Material Unload Request.

      - [CEID = 135] Material Unload Complete.

L, 4

        - <A[3] CEID>

        - <A[22] EQUIPMENTID>

        - <A[20] PPID>

        - L, n(n = total Material count)

          - <A[30] MATERIALID >

…

n.<A[30] MATERIALID >

Exception: None.

      - [CEID = 200] Unit Process Program Change

L, 2

        - <A[3] CEID>

        - L, 3

          - <A[22] EQUIPMENTID>

          - <A[1] CLASS>

          - L, n	/* n: Each Unit count */

            - L,2

              - <A[16] UNITID>

              - L,k	/* k: SubRPID Count */

                - L,2

1.<A[32] SubRPID>

2. L,m	/*if CLASS is 3(Delete) m=0 */

1. L, 2

…

m. L, 2

- <A[32] PPNAME1>

- <A[16] PPVALUE1>

- <A[32] PPNAMEm>

- <A[16] PPVALUEm>

k. L,2

: :

  - <A[32] SubRPID>

  - L,m

    - L, 2

…

m. L, 2

: :

- <A[32] PPNAME1>

- <A[16] PPVALUE1>

- <A[32] PPNAMEm>

- <A[16] PPVALUEm>

n. L, 2

  - <A[16] UNITID>

  - L,k

    - L,2

- <A[32] SubRPID>

- L,m

…

k. L,2

m. L, 2

: :

- L, 2

- <A[32] PPNAME1>

- <A[16] PPVALUE1>

- <A[32] PPNAMEm>

- <A[16] PPVALUEm>

  - <A[32] SubRPID>

  - L,m

    - L, 2

…

m. L, 2

- <A[32] PPNAME1>

- <A[16] PPVALUE1>

- <A[32] PPNAMEm>

- <A[16] PPVALUEm>

Exception: If there are no process program ID, then a list of zero length will be sent for item 6(L, n) and PPVER is space. Exception: None.

      - [CEID = 201] Route Process Program Modify or Create or Delete.

L.5

        - <A[3] CEID>

        - <A[22] EQUIPMENTID>

        - <A[1] CLASS>

        - <A[20] PPID>

        - L, n		/* n: Each Unit count. if if CLASS is 3(Delete) n=0 */ 1.L, 2

Exception: None.

.... n.L, 2

- <A[16] UNITID>

- <A[32] SubRPID>

- <A[16] UNITID>

- <A[32] SubRPID>

    - S6F12 Event Report Acknowledge

S6, F12 Event Report Acknowledge	S,H→E.

Description: Acknowledge or error. Item Definition:

<ACK6>:	[Binary 1 Byte] Acknowledge for Event Report 0 = OK, Accepted.

1~63 = NG, Error, Not Accepted.

Structure:

<B[1] ACK6>

Exception: None.

    - S6F31 OXR Information Download Request

S6,F31 OXR Information Download Request	S,H←E Description: This message is used to request OXR information of chips in a glass to host.

Item Definition:

<EQUIPMENTID>:	[ASCII 22 Bytes] Equipment ID.

<CSTSEQ>:	[ASCII 5 Bytes] Cassette Sequence No.

1~9999

Structure: L, 4

<SLOT>:	[ASCII 2 Bytes] Slot number 1~60

<GLASSID>:	[ASCII 20 Bytes] Glass ID.

- <A[22] EQUIPMENTID>

- <A[5] CSTSEQ>

- <A[2] SLOT>

- <A[20] GLASSID>

    - S6F32 OXR Information Download Data

S6,F32 OXR Information Download Information Data	S,HE Description: This message is used to download OXR information for the requested glass to Equipment.

Item Definition:

<RETURN>:	[ASCII 1 Byte] Return Code

1 = OK

2 = NG.

<EQUIPMENTID>:	[ASCII 22 Bytes] Equipment ID.

<CSTSEQ>:	[ASCII 5 Bytes] Cassette Sequence No.

1~9999

<SLOT>:	[ASCII 2 Bytes] Slot number 1~60

<GLASSID>:	[ASCII 20 Bytes] Glass ID.

<OXR>:	[ASCII 52 Bytes] OXR information of chips in a glass.

1 byte per chip. Max 52 chips

0 = No Panel Exist.

1 = G : Good.

2 = A : No Dark Point.

3 = B : Panel with Pad Repair.

4 = R : Final Repair Judge for LCVD Repair.

5 = L : LCVD Repair OK.

6 = S : Scrap.

7 = D : Dark Point Repaired.

8 = N : NG

9 = O : OK. Array Initial Grade

A = X : Real Scrap. Panel don’t need to be tested.

Structure:

L, 2

- <A[1] RETURN>

- L, 14

  - <A[5] CSTSEQ>

  - <A[2] SLOT>

  - <A[20] GLASSID>

  - <A[150] OXR>

  - <A[150] GATECUTREPAIRTYPE>

  - <A[150] GATECVDREPAIRTYPE>

  - <A[150] SDCUTREPAIRTYPE>

  - <A[150] SDCVDREPAIRTYPE>

  - <A[150] FINALCUTREPAIRTYPE>

  - <A[150] FINALCVDREPAIRTYPE>

  - <A[150] ARRAYTESTPRODUCTJUDGE>

  - <A[150] ASSYPRODUCTJUDGE>

  - <A[150] SUBPRODUCTGRADES>

  - <A[150] SUBPRODUCTJUDGES>

  - <A[150] PIREWORK>

  - <A[150] CELLREPAIR>

Exception: If when RETURN is 2, The host sends a zero-length list to the Equipment.

    - S6F33 Cassette ID Request

S6,F33 Cassette ID Request	S,H←E

Description: This message is used to request Cassette ID of a glass to host. Item Definition:

<EQUIPMENTID>:	[ASCII 22 Bytes] Equipment ID.

<CSTSEQ>:	[ASCII 5 Bytes] Cassette Sequence No.

1~9999

<SLOT>:	[ASCII 2 Bytes] Slot number 1~60

<GLASSID>:	[ASCII 20 Bytes] Glass ID.

Structure: L, 4

- <A[22] EQUIPMENTID>

- <A[5] CSTSEQ>

- <A[2] SLOT>

- <A[20] GLASSID>

    - S6F34 Cassette ID Request Reply

S6,F34 Cassette ID Request Reply	S,HE

Description: This message is used to provide requested Cassette ID of a glass to Equipment. Item Definition:

<CSTIDACK>:	[ASCII 1 Byte] Return Code

1 = OK

2 = NG.

<EQUIPMENTID>:	[ASCII 22 Bytes] Equipment ID.

<CSTSEQ>:	[ASCII 5 Bytes] Cassette Sequence No.

1~9999

<SLOT>:	[ASCII 2 Bytes] Slot number 1~60

<GLASSID>:	[ASCII 20 Bytes] Glass ID.

<CASSETTEID>:	[ASCII 10 Bytes] Loading Cassette ID

Structure: L, 2

L, 5

- <A[1] CSTIDACK>

- <A[22] EQUIPMENTID>

- <A[5] CSTSEQ>

- <A[2] SLOT>

- <A[20] GLASSID>

- <A[10] CASSETTEID>

Exception: If when CSTIDACK is 2, The host sends a zero-length list to the Equipment.

    - S7F15 Equipment Recipe Request

S7,F15 Equipment Recipe Request	S, HE

Description: This message is used to request recipe ID that a equipment requires to CIM-PC. Item Definition:

<EQUIPMENTID>:	[ASCII 22 Bytes] Equipment ID.

<PPID>:	[ASCII 20 Bytes] Process Program ID

Structure:

L, 2

- <A[22] EQUIPMENTID>

- <A[20] PPID>

    - S7F16 Equipment Recipe Reply

S7,F16 Equipment Recipe Reply	S,HE

Description: This message is used to reply to S7, F15. Item Definition:

<EQUIPMENTID>:	[ASCII 22 Bytes] Equipment ID.

<PPID>:	[ASCII 20 Bytes] Process Program ID

<SubRPID>	[ASCII 32 Bytes] Sub Process Program ID

Structure: L, 3

- <A[22] EQUIPMENTID>

- <A[20] PPID>

3, L, n		/* n: Each SubProcess Programcount */ L,2

  - <A[16] UNITID>

  - <A[32] SubRPID>

Exception: If the requested PPID of S7, F15 is not valid, list n value would be 0.

    - S7F19 Current EPPD Request

S7,F19 Current EPPD Request	S,HE

Description: This message is used to request the transmission of the current Equipment process

program directory (EPPD). This is a list of all the PPIDs of the process programs stored in the Equipment. Item Definition: None

Structure: Header only

    - S7F20 Current EPPD Data

S7,F20 Current EPPD Data	M,HE

Description: This message is used to transmit the current EPPD. Item Definition:

<PPID>:	[ASCII 20 Bytes] Process Program ID

Structure:

L,n * number of process programs in the directory

1.<A[20] PPID1>

: n.<A[20] PPIDn>

    - S7F25 Formatted Process Program Request

S7, F25 Formatted Process Program Request	S,HE, reply. Description: This message is used by either Equipment or host to request a particular process program from the other. Item Definition:

<PPID>:	[ASCII 20 Bytes] Process Program ID.

Structure:

L, 1

1.<A[20] PPID>

Exception: None.

    - S7F26 Formatted Process Program Data

S7, F26 Formatted Process Program Data	M,HE.

Description: This message transfers a process program in response to a request for the ERNO.

Recipe Key parameter determined by ESWIN engineer

Any recipe parameter changed, the recipe version number should be updated. And recipe version number format:(e.g.:20181127132826)

Item Definition:

<EQUIPMENTID>:	[ASCII 22 Bytes] Equipment ID.

<EQUIPMENTMODE>:	[ASCII 4 Bytes] EquipmentOperation Mode.

NORN = Normal Mode. PASS = Pass Mode.

LT =LT Mode.(CVD Only) HT = HT Mode.(CVD Only)

HTS = HTS Mode.(CVD Only) PVA = PVA Mode.(CVD Only)

FPM = Full PM Mode.(Dry Etcher Only) PPM = PartialPM Mode. (Dry Etcher Only)

<EQUIPMENTST>:	[ASCII 1 Byte] Equipment Status.

I = Idle. R = Run.

D = DOWN.

P = Pause. M = PM

J = Job Change T = E-Time

E = ETC

<PPID>:	[ASCII 20 Bytes] Process Program ID

<PPVER>:	[ASCII 14 Bytes] Process Program Version.

PPVER means recipe last modified date time.

Format：YYYYMMDDHHMMSS.

<PPNAME>:	[ASCII 32 Bytes] Process Program Parameter Item Name.

<PPVALUE>:	[ASCII 16 Bytes] Process Program Parameter Item Value.

If there is no value of item, which should be presented by space

<UNITID>:	[ASCII 16 Bytes] Unit ID

<SubRPID>	[ASCII 32 Bytes] Sub Process Program ID

Structure: L, 6

- <A[22] EQUIPMENTID>

- <A[4] EQUIPMENTMODE>

- <A[1] EQUIPMENTST>

- <A[20] PPID>

- <A[14] PPVER>

- L, n	/* n: Each SubProcess Programcount */ 1.L, 3

  - <A[16] UNITID>

  - <A[32] SubRPID>	/*IF Equipment have not sub RPID, this value same is PPID

  - L,m	/* m: Each Process Program Parameter item */

    - L, 2

n. L, 3

…

m. L, 2

- <A[32] PPNAME1>

- <A[16] PPVALUE1>

- <A[32] PPNAMEm>

- <A[16] PPVALUEm>

: :

  - <A[16] UNITID>

  - <A[32] SubRPID>

  - L,m	/* m: Each Process Program Parameter item */

    - L, 2

…

m. L, 2

- <A[32] PPNAME1>

- <A[16] PPVALUE1>

- <A[32] PPNAMEm>

- <A[16] PPVALUEm>

Exception: If there are no process program ID, then a list of zero length will be sent for item 6(L, n) and PPVER is space.

    - S7F31 Formatted Process Program Send

S7, F31 Formatted Process Program Send	S,HE, reply. Description: This message transfers a process program to a equipment.

Item Definition:

<EQUIPMENTID>:	[ASCII 22 Bytes] Equipment ID.

<PPID>:	[ASCII 20 Bytes] Process Program ID

<PPVER>:	[ASCII 14 Bytes] Process Program Version.

PPVER means recipe last modified date time. Format：YYYYMMDDHHMMSS.

<PPNAME>:	[ASCII 32 Bytes] Process Program Parameter Item Name.

<PPVALUE>:	[ASCII 16 Bytes] Process Program Parameter Item Value.

If there is no value of item, which should be presented by space

<SubRPID>	[ASCII 32 Bytes] Sub Process Program ID

<UNITID>:	[ASCII 16 Bytes] Unit ID

Structure: L, 4

- <A[22] EQUIPMENTID>

- <A[20] PPID>

- <A[14] PPVER>

- L, n	/* n: Each SubProcess Programcount */ 1.L, 3

  - <A[16] UNITID>

  - <A[32] SubRPID>	/*IF Equipment have not sub RPID, this value same is PPID*/

  - L,m	/* m: Each Process Program Parameter item */

    - L, 2

…

m. L, 2

- <A[32] PPNAME1>

- <A[16] PPVALUE1>

- <A[32] PPNAMEm>

- <A[16] PPVALUEm>

    - S7F32 Formatted Process Program Acknowledge

S7, F32Formatted Process Program Acknowledge	S,H←E. Description: Acknowledge to S7, F31.

Item Definition:

<EQUIPMENTID>:	[ASCII 22 Bytes] Equipment ID.

<PPID>:	[ASCII 20 Bytes] Process Program ID

<PPVER>:	[ASCII 14 Bytes] Process Program Version.

<ONLACK>:	[Binary 1 Byte] Acknowledge for Request Online

0 = OK, Accepted for the requested PPID.

1 = NG, Not Accepted for the requested PPID. 2~63 = Error, Not Accepted.

Structure: L, 4

- <A[22] EQUIPMENTID>

- <A[20] PPID>

- <A[14] PPVER>

- <B[1] ONLACK>

Exception: None.

    - S7F73 Recipe ID Check

S7,F73 Recipe ID Check	S,HE

Description: this function is for check the Recipe ID that is registered or not Item Definition:

<PPID>	[ASCII 20 Bytes] Process Program ID

<PPVER>:	[ASCII 14 Bytes] Process Program Version.

<EQUIPMENTID>:	[ASCII 22 Bytes] Equipment ID.

Structure: L, 2

- <A[22] EQUIPMENTID>

- L,n (n= number of need checked Recipe count)

  - L, 2

n. L, 2

- <A[20] PPID>

- <A[14] PPVER>

.....

- <A[20] PPID>

- <A[14] PPVER>

    - S7F74 Recipe ID Check Acknowledge

S7,F74 Recipe ID Check Acknowledge	S,H←E

Description: Acknowledge or error. Item Definition:

<ACK1>:	[Binary 1 Byte] Acknowledge for Event Report

0 = OK

1 = NG.

Structure:

<B[1] ACK1>

Exception: None.

    - S9F1 Unrecognized Device ID

S9, F1 Unrecognized Device ID	S,H←E.

Description: The device ID in the message block header did not correspond to any known device ID in the node detecting the error. Item Definition:

<MHEAD>:	[Binary 10 Bytes] SECS Message block header associated with message block in error.

Structure:

<B[10] MHEAD>

Exception: None.

    - S9F3 Unrecognized Stream Type

S9, F3 Unrecognized Stream Type	S,H←E.

Description: The Equipment does not recognize the stream type in the message block header. Item Definition:

<MHEAD>:	[Binary 10 Bytes] SECS Message block header associated with message block in error.

Structure:

<B[10] MHEAD>

Exception: None.

    - S9F5 Unrecognized Function Type

S9, F5 Unrecognized Stream Type	S,H←E.

Description: This message indicates that the function in the message ID is not recognized by the receiver. Item Definition:

<MHEAD>:	[Binary 10 Bytes] SECS Message block header associated with message block in error.

Structure:

<B[10] MHEAD>

Exception: None.

    - S9F7 Illegal Data

S9, F7 Illegal Data	S,H←E.

Description: This message indicates that the stream and function were recognized, but the associated data format could not be interpreted.

Item Definition:

<MHEAD>:	[Binary 10 Bytes] SECS Message block header associated with message block in error.

Structure:

<B[10] MHEAD>

Exception: None.

    - S9F9 Transaction Timer Timeout

S9, F9 Transaction Timer Timeout	S,H←E.

Description: This message indicates that a transaction (receive) timer has timed out and that the corresponding transaction has been aborted. It is up to the host to respond to this error in an appropriate manner to keep the system operational.

Item Definition:

<SHEAD>:	[Binary 10 Bytes] Stored header related to the transaction timer.

Structure:

<B[10] MHEAD>

Exception: None.

    - S9F11 Data Too Long

S9, F11 Data Too Long	S,H←E.

Description: This message to the host indicates that the Equipment has been sent more data than it can handle. Item Definition:

<MHEAD>:	[Binary 10 Bytes] SECS Message block header associated with message block in error.

Structure:

<B[10] MHEAD>

Exception: None.

    - S10F3 Terminal Display, Signal

S10, F3 Terminal Display, Single	S,H→E, reply.

Description: Data to be displayed. Item Definition:

<TEXT>:	[ASCII 80 Bytes] a single line of characters.

Structure:

<A[80] TEXT>

Exception: None.

    - S10F4 Terminal Display, Signal Acknowledge

S10, F4 Terminal Display, Single Acknowledge	S,H←E. Description: Acknowledge or error.

Item Definition:

<ACK10C>:	[Binary 1 Byte] Acknowledge Code.

0 = OK, Accepted.

1 ~ 63 = Error, Not Accepted.

Structure:

<B[1] ACK10C>

Exception: None.

    - S64F1 Measurement Result Send

S64, F1 Measurement Result Send	S,H→E, reply.

Description: The Host sends Measurement Result Data to the Equipment. Item Definition:

<PPID>:	[ASCII 20 Bytes] Process Program ID

<GLASSID>:	[ASCII 20 Bytes] Glass ID.

<INSPDT>:	[ASCII 14 Bytes] Inspected Date and Time Format : YYYYMMDDHHMMSS.

<SHOTNO>:	[UINT 1 Byte] Shot No.

<MPX>:	[ASCII 9 Bytes] Measured Position X “+####.###” (mm).

<MPY>:	[ASCII 9 Bytes] Measured Position Y “+####.###” (mm).

<ODX>:	[ASCII 9 Bytes] Overlay Difference X “+####.###” (um).

<ODY>:	[ASCII 9 Bytes] Overlay Difference Y “+####.###” (um).

Structure: L,2

- <A[20] PPID>

- L,n(1~60) /* n: Glass Count = 1-60. */

  - L, 3

    - <A[20] GLASSID>

    - <A[14] INSPDT>

    - L,m(1~1080) /* m: Measurement Count = 1-1080. */

      - L, 5

        - <U1[1] SHOTNO1>

        - <A[9] MPX1>

        - <A[9] MPY1>

        - <A[9] ODX1>

        - <A[9] ODY1>

:

m. L, 5

- <U1[1] SHOTNOm>

- <A[9] MPXm>

- <A[9] MPYm>

- <A[9] ODXm>

- <A[9] ODYm>

    - S64F2 Measurement Result Send Acknowledge

S64, F2 Measurement Result Send Acknowledge	S,H←E. Description: The Equipment sends Measurement Result Send Acknowledge to the Host.

Item Definition:

<ACKC64>:	[Binary 1 Byte] Acknowledge code

0 = OK, Accepted

1 = NG, Error, Not Accepted

Structure:

<B[1] ACKC64>

Exception: None.

    - S64F3 Feedback Result Send

S64, F3 Feedback Result Send	S,H←E.

Description: The Equipment sends Feedback Result Send to the Host. Item Definition:

<RTNCODE>:	[UINT 1 Byte] Feedback Result Code

0 = Updated

1 = Canceled

2 = Skipped

3 = Error

<PPID>	[ASCII 20 Bytes] Process Program ID /* Copy from S64F1 */

Structure: L,2

- <U1[1] RTNCODE>

- <A[20] PPID>

Exception: None.

    - S6F111 OP Login Report

S6, F111 OP Login Report	S,H→E, reply.

Description: The equipment sends the request login to Host. Item Definition:

<OPID>:	[ASCII 8 Bytes] Operator ID

<STATUS>:	[ASCII 1 Bytes] Login Status.

- : Login In

- : Login Out

<PASSWORD>:	[ASCII 8 Bytes] Password of login Not use : “ ‘ space

Structure: L,3

- <A[8] OPID>

- <A[1] STATUS>

- <A[8] PASSWORD>

    - S6F112 OP Login Report Acknowledge

S6, F112 OP Login Report Acknowledge	S,H→E. Description: The Equipment sends Measurement Result Send Acknowledge to the Host.

Item Definition:

<LOGINRESULT>:	[ASCII 1 Bytes] Login Result

1 = OK, Accepted

2 = NG, Error, Not Accepted

<PERMISSION>:	[ASCII 1 Bytes] Permission Level

- : CIMPC is OFFLINE

- ~	: TBD

Structure: L,2

- <A[1] LOGINRESULT>

- <A[1] PERMISSION>

Exception: None.

- SECSII Stream Function Scenario

  - Online Scenario (Offline → Online by Equipment)

| Comment | Host | Equipment | Comment |
| --- | --- | --- | --- |
| Operator actuates Online switch when Equipment Offline state is active. |  |  |  |
|  |  |  S1, F13 | Establish Communication Request |
| COMMACK=0 | S1, F14  |  |  |
|  |  | ← S1, F1 | Requests Online. |
| [IF] Host accepts Online [THEN] Host grants Online [ELSE] Host denies Online [END_IF] | S1, F2 → S1, F0 → |  |  |
|  |  | ← S2, F17 | [IF] Host denies Online [THEN] Keep Original Mode. [ELSE] Request Date and Time. [END_IF] |
| Host reply Date/Time | S2, F18 → |  |  |
|  |  | ← S6, F11 (1) | Report Control Mode (Offline → Online). |
| Host reply Ack | S6, F12 → |  |  |
| Formatted Status Request | S1, F5(1) → |  |  |
|  |  | ← S1, F6 | Formatted Status Data |
|  |  | ← S6, F11 (2) | Report EquipmentOperation Mode |
| Host reply Ack | S6, F12 → |  |  |
|  |  | ← S6, F11 (3) | Report Equipment Status |
| Host reply Ack | S6, F12 → |  |  |
|  |  | ← S6, F11 (4) | Report Material Status |
| Host reply Ack | S6, F12 → |  |  |

  - Offline Scenario (Online → Offline by Equipment)

| Comment | Host | Equipment | Comment |
| --- | --- | --- | --- |
| Operator actuates Offline switch when Equipment Online state is active. |  |  |  |
|  |  | ← S6, F11 (1) | Report Control Mode Change. (Online → Offline). |

| Host reply Ack | S6, F12 → |  |  |
| --- | --- | --- | --- |
| If Host reply ACK6 = 0, Equipment turns control mode to Offline Mode (CEID=1). If Host reply ACK6 <> 0 or host no response, Equipment still changes to Offline. |  |  |  |

  - Online Scenario (Offline → Online by Host)

| Comment | Host | Equipment | Comment |
| --- | --- | --- | --- |
| Host can query Equipment control status from Offline state to Online by option setting. |  |  |  |
| Establish Communication Request | S1, F13  |  |  |
|  |  | S1, F14 | COMMACK=0 |
| Online Request | S1, F17 → |  |  |
|  |  | ← S1, F18 | Online Acknowledge |
|  |  | ← S2, F17 | [IF] Equipment denies Online [THEN] Keep Original Mode. [ELSE] Request Date and Time. [END_IF] |
| Host reply Date/Time | S2, F18 → |  |  |
|  |  | ← S6, F11 (1) | Report Control Mode (Offline → Online). |
| Host reply Ack | S6, F12 → |  |  |
| Formatted Status Request | S1, F5(1) → |  |  |
|  |  | ← S1, F6 | Formatted Status Data |
|  |  | ← S6, F11 (2) | Report EquipmentOperation Mode |
| Host reply Ack | S6, F12 → |  |  |
|  |  | ← S6, F11 (3) | Report Equipment Status |
| Host reply Ack | S6, F12 → |  |  |
|  |  | ← S6, F11 (4) | Report Material Status |
| Host reply Ack | S6, F12 → |  |  |

  - Offline Scenario (Online → Offline by Host)

| Comment | Host | Equipment | Comment |
| --- | --- | --- | --- |
| Host can query Equipment control status from Online state to Offline by option setting. |  |  |  |
| Offline Request | S1, F15 → |  |  |

|  |  | S1, F0 ← S1, F16  S6, F11(1) | [IF] Equipment is OFF-LINE [THEN] Equipment denies requests. [ELSE] Equipment is ON-LINE Equipment acknowledges. Report Offline Event |
| --- | --- | --- | --- |
| Acknowledge | S6, F12  |  | [END_IF] |

  - Equipment Information Inquire Scenario

| Comment | Host | Equipment | Comment |
| --- | --- | --- | --- |
| Host can query Equipment information to Equipment by option setting. |  |  |  |
| Host requests Equipment Information | S1, F5 (1) → |  |  |
|  |  | ← S1, F6 | Reply Equipment Information. |

  - Utility Data Inquire Scenario

| Comment | Host | Equipment | Comment |
| --- | --- | --- | --- |
| Host can query Utility Data to Equipment by option setting. |  |  |  |
| Host requests Utility Data | S1, F5 (2) → |  |  |
|  |  | ← S1, F6 | Reply Utility Data. |

  - Material Data Inquire Scenario

| Comment | Host | Equipment | Comment |
| --- | --- | --- | --- |
| Host can query Utility information to Equipment by option setting. |  |  |  |
| Host requests Material Data | S1, F5 (3) → |  |  |
|  |  | ← S1, F6 | Reply Material Data. |

  - New Equipment Constant Send Scenario

| Comment | Host | Equipment | Comment |
| --- | --- | --- | --- |
| Host can indicate Equipment Constant to Equipment. |  |  |  |
| Host send new constant | S2, F15 → |  |  |
|  |  | ← S2, F16 | Reply Ack |

  - EquipmentConstant Namelist Scenario

| Comment | Host | Equipment | Comment |
| --- | --- | --- | --- |
| Host can requests Equipment Constant Namelist (for S2, F15 using) |  |  |  |
| Host requests Equipment Constant Namelist | S2, F29 → |  |  |
|  |  | ← S2, F30 | Reply Equipment Constant Namelist |
| Space ASCII items for ECNAME, ECMIN, ECMAX, ECDEF, and UNITS indicates that the ECID does not exist. |  |  |  |

  - Date and Time Set By Host Scenario

| Comment | Host | Equipment | Comment |
| --- | --- | --- | --- |
| Host can set the date and time information to Equipment by option setting. |  |  |  |
| Host requests Date and Time Set | S2, F31 → |  |  |
|  |  | ← S2, F32 | Date and Time Set Acknowledge. |
| If Equipment reply S2F32 = OK, Equipment change date/time same with host date/time. If Equipment reply S2F32 = NG, Equipment keep original date/time. |  |  |  |

  - List Alarms Request Scenario

| Comment | Host | Equipment | Comment |
| --- | --- | --- | --- |
| Host can query then alarm information which is occurred but not reset yet. |  |  |  |
| Host requests Alarm List | S5, F5 → |  |  |
|  |  | ← S5, F6 | List Alarm Data. |
| SECS Item L, m=0 means there is no any alarm is happening. |  |  |  |

  - Alarm Report Scenario

| Comment | Host | Equipment | Comment |
| --- | --- | --- | --- |
| When Equipment was occurred or reset alarm event , Equipment needs to report this event to host automatically. |  |  |  |
|  |  | ← S5, F1 | Report Alarm Information |
| Host reply result | S5, F2 → |  |  |
|  |  | ← S6, F11 (3) | Report Equipment Status Change. |
| Host reply result | S6, F12 → |  |  |

  - Equipment Event Notification Scenario

| Comment | Host | Equipment | Comment |
| --- | --- | --- | --- |
| When Equipment collection event occurs, Equipment needs to report event to host automatically. |  |  |  |

|  |  | ← S6, F11 | Report Event Change. |
| --- | --- | --- | --- |
| Host reply Ack | S6, F12 → |  |  |

  - Glass Process Data Information Report Scenario

| Comment | Host | Equipment | Comment |
| --- | --- | --- | --- |
| If glass is finished process, Equipment needs to report glass process data to host automatically. |  |  |  |
|  |  | ← S6, F3 (1) | Report Glass Process Data. |
| Host reply result | S6, F4 → |  |  |

  - Glass History Data Report Scenario

| Comment | Host | Equipment | Comment |
| --- | --- | --- | --- |
| If glass is finished process, Equipment needs to report glass history data to host automatically. |  |  |  |
|  |  | ← S6, F3 (3) | Report Glass History Data |
| Host reply result | S6, F4 → |  |  |

  - Lot Process Data Information Report Scenario

| Comment | Host | Equipment | Comment |
| --- | --- | --- | --- |
| If lot is finished process, Equipment needs to report lot process data to host automatically. |  |  |  |
|  |  | ← S6, F3 (2) | Report Lot Process Data |
| Host reply result | S6, F4 → |  |  |

  - Host Request Process Program Parameter Information Scenario

| Comment | Host | Equipment | Comment |
| --- | --- | --- | --- |
| Host can request inline Equipment process program parameter information form Equipment. |  |  |  |
| Host requests Process Program Parameter Information | S7, F25 → |  |  |
|  |  | ← S7, F26 | Reply Process Program Parameter Information. |

  - Host Request Current EPPD Request Scenario

| Comment | Host | Equipment | Comment |
| --- | --- | --- | --- |
| Host can request the Equipment process program data form Equipment. |  |  |  |
| Host requests Process Program data | S7, F19 → |  |  |
|  |  | ← S7, F20 | Reply Process Program Data List. |

  - Host Check Recipe ID Scenario

| Comment | Host | Equipment | Comment |
| --- | --- | --- | --- |
| Host can request the Equipment process program data form Equipment. |  |  |  |
| Host Check the Equipment’s RecipeI D | S7, F73 → |  |  |
|  |  | ← S7, F74 | Reply the Result to Host. |

  - Online Normal Scenario

| Comment | Host | Equipment | Comment |
| --- | --- | --- | --- |
| This Scenario is a normal scenario when EQUIPMENT Run at online mode |  |  |  |
|  |  | 1st glass move in EQUIPMENT |  |
|  |  | ← S6, F11(08) | Job In EQUIPMENT |
| Acknowledge | S6,F12 → |  |  |
|  |  | ← S6, F11(15) | Glass count/Position change |
| Acknowledge | S6,F12 → |  |  |
|  |  | ← S6, F11(03) | EQUIPMENT Status change(RUN) |
| Acknowledge | S6,F12 → |  |  |
|  |  | ← S6, F11(19) | If EQUIPMENT have unit(s), JOB IN Unit Report |
| Acknowledge | S6,F12 → |  |  |
|  |  | ← S6, F11(10) | Process Start |
| Acknowledge | S6,F12 → |  |  |
|  |  | Whenever a glass is moved into a EQUIPMENT from upstream EQUIPMENT, EQUIPMENT should be reported the order of the above. |  |
| In Processing… |  |  |  |
|  |  | Nth glass move in EQUIPMENT |  |
|  |  | ← S6, F11(08) | Job In EQUIPMENT |
| Acknowledge | S6,F12 → |  |  |
|  |  | ← S6, F11(15) | Glass count/Position change |
| Acknowledge | S6,F12 → |  |  |
|  |  | ← S6, F11(03) | If EQUIPMENT Status is not changed, Don’t report.(Other glasses are processing) |
| Acknowledge | S6,F12 → |  |  |
|  |  | ← S6, F11(19) | If EQUIPMENT have Unit, JOB IN |

|  |  |  | Unit Report |
| --- | --- | --- | --- |
| Acknowledge | S6,F12 → |  |  |
|  |  | ← S6, F11(10) | Process Start |
| Acknowledge | S6,F12 → |  |  |
| In Processing… |  |  |  |
|  |  | 1st glass move out EQUIPMENT |  |
|  |  | ← S6, F11(11) | Process End |
| Acknowledge | S6,F12 → |  |  |
|  |  | ← S6, F11(20) | If EQUIPMENT have Unit, JOB Out Unit Report |
| Acknowledge | S6,F12 → |  |  |
|  |  | ← S6, F3(1) | Glass Data Report |
| Acknowledge | S6,F4→ |  |  |
|  |  | ← S6, F3(3) | Glass History Report |
| Acknowledge | S6,F4→ |  |  |
|  |  | ← S6, F11(09) | Job Out EQUIPMENT |
| Acknowledge | S6,F12 → |  |  |
|  |  | ← S6, F11(15) | Glass count/Position change |
| Acknowledge | S6,F12 → |  |  |
|  |  | ← S6, F11(03) | If EQUIPMENT Status is not changed, Don’t report.(Other glasses are processing) |
| Acknowledge | S6,F12 → |  |  |
|  |  | Whenever a glass is moved out from EQUIPMENT, EQUIPMENT should be reported the order of the above. |  |
| In Processing… |  |  |  |
|  |  | Last glass move out EQUIPMENT |  |
|  |  | ← S6, F11(11) | Process End |
| Acknowledge | S6,F12 → |  |  |
|  |  | ← S6, F11(20) | If EQUIPMENT have Unit, JOB Out Unit Report |
| Acknowledge | S6,F12 → |  |  |
|  |  | ← S6, F3(1) | Glass Data Report |
| Acknowledge | S6,F4→ |  |  |
|  |  | ← S6, F3(2) | Lot Data Report |
| Acknowledge | S6,F4→ |  |  |
|  |  | ← S6, F3(3) | Glass History Report |

| Acknowledge | S6,F4→ |  |  |
| --- | --- | --- | --- |
|  |  | ← S6, F11(09) | Job Out EQUIPMENT |
| Acknowledge | S6,F12 → |  |  |
|  |  | ← S6, F11(15) | Glass count/Position change |
| Acknowledge | S6,F12 → |  |  |
|  |  | ← S6, F11(03) | EQUIPMENT Status change(IDLE) |
| Acknowledge | S6,F12 → |  |  |

  - Online Abnormal Scenario (Last Glass is broken)

| Comment | Host | Equipment | Comment |
| --- | --- | --- | --- |
| This Scenario is a abnormal scenario when last glass is broken. |  |  |  |
|  |  | ← S6, F11(10) | Process Start(n-1) st |
| Acknowledge | S6,F12 → |  |  |
| In Processing… Last glass is broken |  |  |  |
|  |  | ← S6, F11(07) | Glass Erase Event |
| Acknowledge | S6,F12 → |  |  |
| Host send a new last glass id to EQUIPMENT. In case of inline EQUIPMENT, Host send to all EQUIPMENT. |  |  |  |
| Glass Erase/Recovery Information Send | S2, F117 → |  |  |
|  |  | ← S2, F118 | Glass Erase/Recovery Information Acknowledge |
|  |  | ← S6, F11(11) | Process End(n-1) st |
| Acknowledge | S6,F12 → |  |  |
|  |  | ← S6, F11(20) | If EQUIPMENT have Unit, JOB IN Unit Report |
| Acknowledge | S6,F12 → |  |  |
|  |  | ← S6, F3(1) | Glass Data Report |
| Acknowledge | S6,F4→ |  |  |
|  |  | ← S6, F3(2) | Lot Data Report |
| Acknowledge | S6,F4→ |  |  |
|  |  | ← S6, F3(3) | Glass History Report |
| Acknowledge | S6,F4→ |  |  |
|  |  | ← S6, F11(09) | Job OutEQUIPMENT |
| Acknowledge | S6,F12 → |  |  |
|  |  | ← S6, F11(15) | Glass count/Position change |

| Acknowledge | S6,F12 → |  |  |
| --- | --- | --- | --- |
|  |  | ← S6, F11(03) | EQUIPMENT Status change(IDLE) |
| Acknowledge | S6,F12 → |  |  |
| If a new last glass from host is already passed, EQUIPMENT can ignore it. 5.21 Auto Feedback Scenario Comment Host Equipment Comment Host can indicates the auto feedback data to Equipment. Host send Measurement Result Send S64, F1 → ← S64, F2 [IF] ACKC64 > 0, Error happened [THEN] END [ELSE] ACKC64 = 0, Accepted ← S64, F3 Auto Feedback Result [END_IF] |  |  |  |

  - Glass Data Request Scenario

| Comment | Host | Equipment | Comment |
| --- | --- | --- | --- |
| Host can indicates the auto feedback data to Equipment. |  |  |  |
|  |  | ← S6, F11(14) | Glass Data Request |
| Acknowledge | S6,F12 → |  |  |
| Glass Data Download | S2, F115 → |  |  |
|  |  | ← S2, F115 | Acknowledge |

  - Equipment Recipe Request Scenario

| Comment | Host | Equipment | Comment |
| --- | --- | --- | --- |
| Equipment checks process possibility for the load waiting glass and requests recipe ID to CIM-PC if the glass'sPPID is different to the current PPID. |  |  |  |
|  |  | ← S7, F15 | Equipment Recipe Request |
| Equipment Recipe Reply | S7,F16→ |  |  |

  - Formatted Process Program Send Scenario

| Comment | Host | Equipment | Comment |
| --- | --- | --- | --- |
| CIM-PC could send equipment's recipe parameter when recipe parameter is changed by R2R system. |  |  |  |
|  |  | ← S7, F31 | Formatted Process Program Send |
| Formatted Process Program | S7,F32→ |  |  |

| Acknowledge |  |  |  |
| --- | --- | --- | --- |

- Line Special Scenario

  - Cassette Map Download Scenario

| Comment | Host | Equipment | Comment |
| --- | --- | --- | --- |
| When a cassette is loaded on port, Host send cassette map data to equipment |  |  |  |
| Load Complete Host Send Cassette Map data to Loader via PLC Only if the return code of Cassette Map Download is OK, Host Send S2F113 |  |  |  |
| Cassette Map Data Send | S2, F113 → |  |  |
|  |  | ← S2, F114 | ACKC= 0 |
| Host Command Send | S2, F41 → <RCMD>=’START’ <CPNAME>=’CSTSEQ’ <CPVAL>=CST Seq No. |  |  |
|  |  | ← S2, F42 | HACK=0 |
| In Processing. |  |  |  |

  - Cassette Map Download Abnormal Scenario

    - ACKC<>0 from EQUIPMENT

| Comment | Host | Equipment | Comment |
| --- | --- | --- | --- |
| When a cassette is loaded on port, Host send cassette map data to equipment |  |  |  |
| Load Complete Host Send Cassette Map data to Loader via PLC Only if the return code of Cassette Map Download is OK, Host Send S2F113 |  |  |  |
| Cassette Map Data Send | S2, F113 → |  |  |
|  |  | ← S2, F114 | ACKC<> 0 |
| Equipment should be occur alarm. Host send Cassette Cancel Command to Loader. |  |  |  |

    - Cancel from Loader

| Comment | Host | Equipment | Comment |
| --- | --- | --- | --- |
| When a cassette is loaded on port, Host send cassette map data to equipment |  |  |  |
| Load Complete Host Send Cassette Map data to Loader via PLC |  |  |  |

| Only if the return code of Cassette Map Download is OK, Host Send S2F113 |  |  |  |
| --- | --- | --- | --- |
| Cassette Map Data Send | S2, F113 → |  |  |
|  |  | ← S2, F114 | ACKC= 0 |
| Host Command Send | S2, F41 → <RCMD>=’START’ <CPNAME>=’CSTSEQ’ <CPVAL>=CST Seq No. |  |  |
|  |  | ← S2, F42 | HACK=0 |
| CST Cancel occur on Loader before process start. |  |  |  |
| Host Command Send | S2, F41 → <RCMD>=’CANCEL’ <CPNAME>=’CSTSEQ’ <CPVAL>=CST Seq No. |  |  |
|  |  | ← S2, F42 | HACK=0 |

  - Forced Glass Pass Command Send Scenario

    - Forced Glass Pass Command Send By Host

| Comment | Host | Equipment | Comment |
| --- | --- | --- | --- |
| When operator push the ‘Forced Clean Out’ button on CIMPC Screen, Send this message to Equipment. |  |  |  |
| In Processing. Operator Push the Forced Clean Out’ button. Host send Forced Glass Pass Command to Loader via PLC Host send Forced Glass Pass Command to other equipment via HSMS or PLC |  |  |  |
| Forced Glass Pass Command Send | S2, F111 → |  | <CLEANOUT>=1 |
|  |  | ← S2, F112 | Acknowledge |
|  |  | ← S6, F11(2) | <EQUIPMENTMODE>=PASS |
| Acknowledge | S6, F12 → |  |  |
|  |  | ← S6, F11(3) | <EQUIPMENTMODE>=PASS <RECOVERY>=0 |
| Acknowledge | S6, F12 → |  |  |
| : When all of Job in equipment is sent out Operator Push the Forced Clean Out Reset button. : |  |  |  |

| Forced Glass Pass Command Send | S2, F111 → |  | <CLEANOUT>=0(Reset) |
| --- | --- | --- | --- |
|  |  | ← S2, F112 | Acknowledge |
|  |  | ← S6, F11(2) | <EQUIPMENTMODE>=NORN |
| Acknowledge | S6, F12 → |  |  |
|  |  | ← S6, F11(3) | <EQUIPMENTMODE>=NORN <RECOVERY>=0 |
| Acknowledge | S6, F12 → |  |  |

  - Material Status Change Scenario

| Comment | Host | Equipment | Comment |
| --- | --- | --- | --- |
| Equipment report Material status Change. |  |  |  |
| Read the material id by BCR |  |  |  |
|  |  | ← S6, F11(4) | <MATERIALST> is blank |
| Host check the Material id. When the result is OK, Host send OK reply |  |  |  |
| Acknowledge | S6, F12 → (ACK6=0) |  |  |
| When the material is mounted |  |  |  |
|  |  | ← S6, F11(4) | <MATERIALST>=M |
| Acknowledge | S6, F12 → (ACK6=0) |  |  |
| When the material is used |  |  |  |
|  |  | ← S6, F11(4) | <MATERIALST>=I |
| Acknowledge | S6, F12 → (ACK6=0) |  |  |

  - Material Status Change Abnormal Scenario

    - Material ID NG

| Comment | Host | Equipment | Comment |
| --- | --- | --- | --- |
| Equipment report Material status Change. |  |  |  |
| Read the material id by BCR |  |  |  |
|  |  | ← S6, F11(4) | <MATERIALST> is blank |

| Host check the Material id. When the result is NG, Host send NG Reply |  |  |  |
| --- | --- | --- | --- |
| Acknowledge | S6, F12 → (ACK6<>0) |  |  |

    - Material Mount NG

| Comment | Host | Equipment | Comment |
| --- | --- | --- | --- |
| Equipment report Material status Change. |  |  |  |
| Read the material id by BCR |  |  |  |
|  |  | ← S6, F11(4) | <MATERIALST> is blank |
| Host check the Material id. When the result is OK, Host send OK reply |  |  |  |
| Acknowledge | S6, F12 → (ACK6=0) |  |  |
| When the material is mounted |  |  |  |
|  |  | ← S6, F11(4) | <MATERIALST>=M |
| Host check the Material id. When the result is NG, Host send NG reply |  |  |  |
| Acknowledge | S6, F12 → (ACK6<>0) |  |  |

    - Material In-USE NG

| Comment | Host | Equipment | Comment |
| --- | --- | --- | --- |
| Equipment report Material status Change. |  |  |  |
| Read the material id by BCR |  |  |  |
|  |  | ← S6, F11(4) | <MATERIALST> is blank |
| Host check the Material id. When the result is OK, Host send OK reply |  |  |  |
| Acknowledge | S6, F12 → (ACK6=0) |  |  |
| When the material is mounted |  |  |  |
|  |  | ← S6, F11(4) | <MATERIALST>=M |
| Host check the Material id. |  |  |  |

| When the result is OK, Host send OK reply |  |  |  |
| --- | --- | --- | --- |
| Acknowledge | S6, F12 → (ACK6=0) |  |  |
| When the material is used |  |  |  |
|  |  | ← S6, F11(4) | <MATERIALST>=I |
| Host check the Material id. When the result is NG, Host send NG reply |  |  |  |
| Acknowledge | S6, F12 → (ACK6<>0) |  |  |

  - Equipment Operation Mode Change Command Send Scenario

    - Equipment Operation Mode Change By Host

| Comment | Host | Equipment | Comment |
| --- | --- | --- | --- |
| When operator push equipment mode button(ex, Loading stop send mode) on CIMPC Screen, Send this message to Equipment. |  |  |  |
| In Processing. Operator Push the equipment mode button. Host send Equipment Operation Mode Change Command to Loader via PLC Host send Equipment Operation Mode Change Command to other equipment via HSMS or PLC |  |  |  |
| Equipment Operation Mode Change Send | S2, F119 → |  | <EQUIPMENTMODE>=LDST |
|  |  | ← S2, F120 | Acknowledge |
|  |  | ← S6, F11(2) | Equipment Operation Mode Change. |
| Acknowledge | S6, F12 → |  |  |
|  |  | ← S6, F11(3) | If it is needed, |
| Acknowledge | S6, F12 → |  |  |

    - Material Control Request By Host(Material Existence Check)

| Comment | Host | Equipment | Comment |
| --- | --- | --- | --- |
| CIM-PC checks the process possibility & send process start or abort command to indexer. according the condition & defined rule. This message is for that mask is ready in the mask-library |  |  |  |
| In Processing. When new cassette loads on indexer, CIM-PC sends Material Existence Check to a specific equipmentvia HSMS. |  |  |  |
| Material Control Request Send | S2, F123 → |  | < CCODE>=1 (Material Existence |

|  |  |  | Check) |
| --- | --- | --- | --- |
|  |  | ← S2, F124 | Acknowledge |

    - Material Control Request By Host(Material Pre-load)

| Comment | Host | Equipment | Comment |
| --- | --- | --- | --- |
| CIM-PC sends material pre-load(prepare) requests to a specific equipment according to the condition & defined rule. |  |  |  |
| In Processing. When Lot Start glass arrives at a specific position, CIM-PC sends Material Pre to a specific equipment via HSMS. |  |  |  |
| Material Control Request Send | S2, F123 → |  | <CCODE>=3 (Material Pre-load) |
|  |  | ← S2, F124 | Acknowledge |

    - VCR mismatch Report

| Comment | Host | Equipment | Comment |
| --- | --- | --- | --- |
| If veri-code reading ID is different to the glass ID of job data, the equipment sends mismatch message to CIM-PC. |  |  |  |
| In Processing. VCR reading complete and if reading ID mismatches the glass ID of job data, the equipment report via HSMS. |  |  |  |
|  |  | ← S6, F11(24) | VCR Mismatch Report Send |
| Acknowledge | S6, F12→ |  |  |

    - Glass(Panel) Data Change Report

| Comment | Host | Equipment | Comment |
| --- | --- | --- | --- |
| If a equipment change job transfer data update, it should report the updated job transfer data with CEID 25. |  |  |  |
| In Processing. After update job transfer data, send S6, F11(CEID:25) to CIM-PC via HSMS. |  |  |  |
|  |  | ← S6, F11(25) | Glass(Panel) Data Change Report Send |
| Acknowledge | S6, F12→ |  |  |

    - Glass(Panel) Process Result Report

| Comment | Host | Equipment | Comment |
| --- | --- | --- | --- |
| Inspection or measure equipment send judge result to CIM-PC with CEID 26. |  |  |  |
| In Processing. After making judge for a glass, send S6, F11(CEID:26) to CIM-PC via HSMS. |  |  |  |
|  |  | ←S6, F11(26) | Glass(Panel) Process Result Report Send |
| Acknowledge | S6, F12→ |  |  |

  - Equipment Login Scenario

    - Equipment login on Touch Panel

| Comment | Host | Equipment | Comment |
| --- | --- | --- | --- |
| When operator login on touch panel of equipment, Send this message to Host. |  |  |  |
| On Touch Panel Operator push the equipment login button. Log in window is pop up After Operator inputs the ID and Password, press OK button Host send the Result and permission Level to equipment |  |  |  |
|  |  | ← S6, F111 | OP Login Report |
| Acknowledge | S6, F112 → |  | If the result is OK then the login success by permission level Else if the result is NG then the login is failed. |